TSG SA Meeting #SP-94E	SP-211321r01

14 - 20 December 2021, Electronic meeting

TSG SA Rel-18 Prioritization Workshop	SP-211174r01

9-10 December 2021, Electronic meeting                                  (Revision of SP-211159)

SA WG2 Meeting #S2-148E	S2-2109327

15 - 19 November, 2021, E-Meeting	(Revision of S2-2108437r02)

Source:	China Telecom, CATT, China Mobile, China Unicom, Ericsson, Huawei, Intel, Oracle, Tencent, vivo, ZTE, CAICT, CEPRI, Inspur, Vodafone, Facebook, Charter, Telstra

Title:	New SID on enhancement of 5G AM Policy

Document for:	Approval

Agenda Item:	9.1.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	New SID on enhancement of 5G AM Policy

Acronym:	FS_eAMP

Unique identifier:	940064

Potential target Release:	Rel-18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X |  | X |  | X |
| Don't know |  | X |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

3	Justification

The access and mobility policy control introduced in 5G encompasses the management of service area restrictions, the management of the RFSP functionalities, the management of the UE-AMBR, the management of the UE Slice-MBR and the management of the SMF selection by PCF.

Some potential enhancements for study in AM policy are identified based on the Rel-17 SA2 issues and requirements:

- AM Policy is only provided by PCF to AMF in 4G/5G interworking scenario and not supported in EPC. In some scenarios, RFSP index may be updated to direct a UE from 5G to 4G, while the subscription data or locally configured policy of the UE is of 5G prioritization. Because the MME is not able to receive RFSP Index update from PCF, the ping-pong issue is likely to happen if the network delivered different RFSP Index in 4G and 5G. Detailed description may refer to Discussion Paper S2-2103936 “Discussion on PCF providing RFSP Index to MME/RAN”. If it keeps the “back to 4G” RFSP index from 5G to 4G, the core network has no way to ask the UE to come back to 5G. As such, mechanism needs to be investigated to support AM policy update for the EPC side.

4	Objective

WT #1: Study the current mechanisms to provide AM policy control when UE moves from/to 5GC to/from EPC and define enhancements in current mechanism if needed.

WT #2: void.

WT #3: void.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is depended on completion of other WTs |
| --- | --- | --- | --- | --- |
| WT #1 | 1.5 | 0.5 | No | Self-contained |
| WT #2 | void | void | void | void |
| WT #3 | void | void | void | void |

Total TU estimates for the study phase: 1.5

Total TU estimates for the normative phase: 0.5

Total TU estimates: 1.5+0.5= 2

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.XXX | Study on enhancement of 5G AM Policy | TSG#96 (June, 2022) | TSG#97 (Sep, 2022) | Zhuoyi Chen, China Telecom (chenzy34@chinatelecom.cn) |

NOTE: the need for a TR will be checked during study phase.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text} |

6	Work item Rapporteur(s)

Chen, Zhuoyi, China Telecom, chenzy34@chinatelecom.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

None.

9	Supporting Individual Members

Supporting IM name

China Telecom

CATT

China Mobile

China Unicom

Ericsson

Huawei

Intel

Oracle

Tencent

vivo

ZTE

CBN

CAICT

CEPRI

Inspur

Vodafone

Facebook

Charter

Telstra