3GPP TSG SA Meeting #94E	SP-211552

# E-meeting, December 14 - 20, 2021	(revision of S2-2108095)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3256 | rev | 5 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE onboarding architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm, Lenovo, Motorola Mobility, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Intel, Nokia, Nokia Shanghai Bell, Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-12-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The architecture for UE onboarding, and especially the DCS, has not yet been concluded. The architecture includes AUSF and then a DCS which potentially can either include a AAA server or a AUSF. SA3 agreed the following: For initial access for UE onboarding, the following authentication methods are concluded to be specified in normative work: - Primary authentication between UE and AUSF in the O-SNPN. No interaction with DCS during primary authentication. As currently specified in TS 33.501 [2]. - Primary authentication with mutual authentication between UE and DCS. AUSF is involved. DCS can be AAA server, in that case NSSAAF is involved. DCS can also be external entity using AUSF/UDM. ********************************************************************* Changes in rev XX to align with the approved S3-214446: - Clause 5.30.2.10.2.2: addition of an architecture figure for the case where DCS is not involved during primary authentication, with related text. - Clause 5.30.2.10.2.6: clarification on AUSF selection for the case where DCS is not involved during primary authentication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editor's note removed and the ON-SNPN architecture with DCS updated. Updated NSSAAF description. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Architecture for DCS is not completely described |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.10.2.2; 5.30.2.10.2.7; 5.30.2.10.2.6; 6.2.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes * * * *

5.30.2.10.2.2	Architecture

Figure 5.30.2.10.2.2-1 and 5.30.2.10.2.2-2 depict the architecture for Onboarding of UEs in an ON-SNPN.

Figure 5.30.2.10.2.2-1: Architecture for UE Onboarding in ON-SNPN in case DCS includes an AUSF and a UDM

Figure 5.30.2.10.2.2-2: Architecture for UE Onboarding in ON-SNPN in case DCS includes a AAA server used for primary authentication

Figure 5.30.2.10.2.2-3: Architecture for UE Onboarding in ON-SNPN when DCS is not involved during primary authentication

NOTE 1:	AUSF in the ON-SNPN interfaces with the DCS owned by an entity that is internal or external to the ON-SNPN.

NOTE 2:	The functionality with respect to exchange information between PVS and SO-SNPN to provision SNPN credentials and other data from the SO-SNPN in the UE is out of 3GPP scope.

NOTE 3:	The dotted lines in Figure 5.30.2.10.2.2-1 indicate that domains (e.g. DCS domain, PVS domain, and SO-SNPN) may not be separated depending on the deployment scenario.

NOTE 4:	See TS 33.501 [29] for the functionality beyond AUSF, and other interfaces required for security.

NOTE X:	The use of UDM in Figure 5.30.2.10.2.2-3 to store non-UE specific information for primary authentication is optional.

When the DCS is involved during mutual primary authentication during the Onboarding procedure (as in Figure 5.30.10.2.2-1 and  Figure 5.30.10.2.2-2), the following apply:

-	When the DCS includes an AUSF and a UDM functionality, then the AMF selects AUSF and UDM in the DCS domain. The ON-SNPN and DCS domain are connected via N32 and SEPP which are not shown in the Figure 5.30.2.10.2.2-1.

-	When the DCS includes a AAA server functionality, then the AMF selects AUSF in the ON-SNPN. Based on local configuration the AUSF skips the UDM selection and directly performs primary authentication towards DCS with AAA server functionality. The AUSF uses an NSSAAF (and the NSSAAF may use a AAA-P which is not shown in the figure 5.30.2.10.2.2-2) to relay EAP messages towards the DCS including a AAA Server.

NOTE 5:	If a given DCS supports both AUSF/UDM functionality as depicted in Figure 5.30.2.10.2.2-1 and AAA server functionality as depicted in Figure 5.30.2.10.2.2-2, the DCS needs to use separate Home Network Identifier for DCS with AUSF/UDM and for DCS with AAA server functionality to ensure correct selection of NFs.

When the DCS is not involved during primary authentication (as in Figure 5.30.10.2.2-3), the following applies:

-	The AMF selects a local AUSF as described in clause 5.30.2.10.2.6 and performs primary authentication towards the local AUSF using UE default credentials as described in TS 33.501 [29].

-	upon establishment of the restricted PDU Session the ON-SNPN may trigger secondary authentication procedure with the DCS using UE default credentials as described in clause 11.1 of TS 33.501 [29].

NOTE X:	If the secondary authentication fails, the SMF rejects the PDU Session for onboarding. Based on local policy the AMF can deregister the UE as described in clause 5.30.2.10.2.7.

NOTE 6:	The DCS and PVS can be owned by an administrative entity that can be different from either the ON-SNPN or SO-SNPN. The ownership of DCS and PVS is outside the scope of 3GPP.

# * * * * Next change * * * *

5.30.2.10.2.6	Registration for UE onboarding

When the user or UE has selected an ON-SNPN according to clause 5.30.2.10.2.5, the UE establishes an RRC connection towards the NG-RAN node of the ON-SNPN. The UE provides an indication in RRC Connection Establishment that the RRC connection is for onboarding as defined in TS 38.331 [28]. This indication allows the NG-RAN node to select an appropriate AMF that supports the UE onboarding procedures. The UE indicates the ON-SNPN as the selected network, and the NG-RAN node shall indicate the selected PLMN ID and NID of the ON-SNPN to the AMF.

NOTE:	As the configuration information in the UE does not include any S-NSSAI and DNN used for onboarding, the UE does not include S-NSSAI and DNN in RRC when it registers for UE onboarding purposes to the ONN.

The UE shall initiate the NAS registration procedure by sending a NAS Registration Request message with the following characteristics:

-	The UE shall set the 5GS Registration Type to the value "SNPN Onboarding" indicating that the registration request is for onboarding.

-	The UE shall provide a SUCI derived from a SUPI as specified in TS 23.003 [19] and TS 33.501 [29]. The SUPI shall uniquely identify the UE and shall be derived from the Default UE Credentials. The ON-SNPN may determine the corresponding DCS identity or address/domain, based on the SUCI (i.e. based on the Home Network Identifier of the SUCI).

The UE does not include a Requested NSSAI in NAS signalling when it registers for UE onboarding purposes to the ONN.

The AMF supporting UE onboarding is configured with AMF Onboarding Configuration Data that may include e.g.:

-	S-NSSAI and DNN to be used for UE onboarding or a configured SMF for the DNN and S-NSSAI used for UE onboarding;

-	Information to enable User Plane Remote Provisioning of UEs in SNPNs, see clause 5.30.2.10.4.

-	Information to use a local AUSF(s) within the ON-SNPN for onboarding of UEs with a SUCI for a DCS with AAA server or for onboarding of UEs in the case where the DCS is not involved during primary authentication.

When the AMF receives a NAS Registration Request with a 5GS Registration Type set to "SNPN Onboarding", the AMF:

-	starts an authentication procedure towards the AUSF, the authentication procedure is specified in TS 33.501 [29]. The AMF selects an appropriate AUSF as described in clause 6.3.4 based on the Home Network Identifier of the SUCI used during onboarding or based on local configuration in the AMF.

-	applies the AMF Onboarding Configuration Data e.g. used to restrict UE network usage to only onboarding for user plane remote provisioning of UE as described in clause 5.30.2.10.4.3.

-	stores in the UE context in AMF an indication that the UE is registered for SNPN onboarding.

Upon successful authentication from AUSF, the AMF informs the UE about the result of the registration. If the UE is not successfully authenticated, the AMF shall reject the registration procedure for onboarding, and the UE may select a different ON-SNPN to attempt to register.

# * * * * Next change * * * *

5.30.2.10.2.7	Deregistration from the ON-SNPN for onboarding registered UE

Once remote provisioning of SO-SNPN credentials is completed, the UE should initiate deregistration from the ON-SNPN.

Based on ON-SNPN policies, the AMF may start an implementation specific timer once the UE has registered to the ON-SNPN for the purpose of onboarding. Expiry of this timer triggers the AMF to deregister the onboarding registered UE from the ON-SNPN.

The AMF may also deregister the UE when it determines that the restricted PDU Session has been released by the SMF.

When AMF re-allocation occurs for a UE registered for SNPN onboarding during mobility registration update procedure as described in TS 23.502 [3] in clause 4.2.2.2.2 or during N2 based handover as described in TS 23.502 [3] clause 4.9.1.3, the new AMF supporting UE SNPN onboarding should be selected as described in clause 6.3.5. If the new AMF receives in UE context the indication that the UE is registered for SNPN onboarding, the new AMF may start an implementation specific timer for when to deregister the UE when the new AMF completes the Registration procedure (i.e. sends Registration Accept to the UE) or completes the N2 based handover procedure.

NOTE 1:	This specific timer is used to prevent onboarding registered UEs from staying at the ON-SNPN indefinitely.

NOTE 2:	When Onboarding network is a PLMN and the UE's subscription only allows for Remote Provisioning, then based on PLMN policies, the AMF can start an implementation specific timer once the UE has registered to the PLMN. Expiry of this timer triggers the AMF to deregister the UE from the PLMN. This specific timer is used to prevent registered UEs that only allow for Remote Provisioning from staying at the PLMN indefinitely.

# * * * * Next change * * * *

### 6.2.23	NSSAAF

The Network Slice-specific and SNPN Authentication and Authorization Function (NSSAAF) supports the following functionality:

-	Support for Network Slice-Specific Authentication and Authorization as specified in TS 23.502 [3] with a AAA Server (AAA-S). If the AAA-S belongs to a third party, the NSSAAF may contact the AAA-S via a AAA proxy (AAA-P).

-	Support for access to SNPN using credentials from Credentials Holder using AAA server (AAA-S) as specified in clause 5.30.2.9.2 or using credentials from Default Credentials Server using AAA server (AAA-S) as specified in clause 5.30.2.10.2. If the Credentials Holder or Default Credentials Server belongs to a third party, the NSSAAF may contact the AAA server via a AAA proxy (AAA-P).

NOTE:	When the NSSAAF is deployed in a PLMN, it supports Network Slice-Specific Authentication and Authorization, while when the NSSAAF is deployed in a SNPN may support Network Slice-Specific Authentication and Authorization and/or access to SNPN using credentials from Credentials Holder

# * * * * End of changes * * * *