3GPP TSG-SA WG6 Meeting #47-e	S6-220276

e-meeting, 14th – 22nd February 2022	(revision of S6-220067)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0307 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional model reference points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CR introduces the necessary reference points and related descriptions for the functional model when using an MC gateway UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new diagram is added, which shows the the reference points and how they are used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functional model remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 11.2.0, 11.2.1 (new), 11.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Accuracy: Reflects the uncertainty of the location at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Active MC service user profile: The MC service user profile that is currently used by an MC service client of an MC service user while receiving MC service.

Altitude: Third dimension for the geographical coordinates at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Bearing: Direction at the moment of location measurement, e.g. see 3GPP TS 25.305 [30].

ECGI: E-UTRAN Cell Global Identifier, which is used to identify cells globally, where the ECGI is constructed from the Mobile Country Code (MCC), Mobile Network Code (MNC) and the E-UTRAN Cell Identifier (ECI).

Interconnection: A means of communication between MC systems whereby MC service users obtaining MC service from one MC system can communicate with MC service users who are obtaining MC service from one or more other MC systems.

Interconnection group: An MC service group that is configured to allow inclusion of MC service group members who are MC service users from partner MC system(s).

LCS network: The 3GPP network that provides location service as defined in 3GPP TS 23.271 [29].

Location: The current physical location of the MC service UE.

MBMS SAI: Multimedia Broadcast Multicast Service Area Identity which is mapped to the MBMS service area.

MC gateway server: A server providing topology hiding for MC service interconnection with a partner MC system, where that partner MC system is in a different trust domain.

MC service: A generic name for any one of the three mission critical services: either MCPTT, or MCVideo, or MCData.

MC service affiliated group member: An MC service user who has indicated an interest in a particular MC service group and has been accepted to participate in MC service group communication for that MC service group.

MC service client: A generic name for the client application function of a specific MC service. MC service client could be replaced by MCPTT client, or MCVideo client, or MCData client depending on the context.

MC service group: A defined set of MC service users with associated communication dispositions (e.g. media restrictions, default priority and commencement directions) configured for the use with one or more MC services.

MC service group affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is determined.

MC service group call: A mechanism by which an MC service user can make a one-to-many MC service(s) transmission to other users that are members of MC service group(s).

MC service group de-affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is removed.

MC service group home system: The MC system where the MC service group is defined.

MC service group host MC service server: The MC service server within an MC system which provides centralised support for a particular MC service of an MC service group defined in a MC service group home system.

MC service group member: An MC service user, whose MC service ID is listed in a particular MC service group.

MC service ID: A generic name for the user ID of a mission critical user within a specific MC service. MC service ID could be replaced by MCPTT ID, or MCVideo ID, or MCData ID depending on the context.

MC service server: A generic name for the server application function of a specific MC service. MC service server could be replaced by MCPTT server, MCVideo server, or MCData server depending on the context.

MC service user: An authorized user, who can use an MC service UE to participate in one or more MC services.

MC service user profile: The set of information associated to an MC service user that allows that user to employ one or more MC services in a given role and from a given MC service UE.

MC service UE: A UE that can be used to participate in one or more MC services.

MC service UE label: A generic name for identification of a specific MC service UE.

MC system: The collection of applications, services, and enabling capabilities required to provide a single mission critical service or multiple mission critical services to one or more mission critical organizations.

MC user: A user, identified by an MC ID, who, after authorization, obtains mission critical service(s).

Migration: A means for an MC Service user to obtain MC service directly from a partner MC system.

Partner MC system: Allied MC system that provides MC services to an MC service user based on the MC service user profiles that are defined in the primary MC system of that MC service user.

Preconfigured MC service group: an MC service group used only for regrouping that has been configured in advance of a group or user regrouping operation to serve as the source of regroup group configuration.

Pre-selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile through configuration, and applicable for an authenticated MC service user upon MC service authorization.

Primary MC system: MC system where the MC service user profiles of an MC service user are defined.

Requested Priority: A value for use in a MC service group or MC private communication that, if accepted, is used by the MCX service server to temporarily replace the priority level that is predefined in the MC service group or MC service user profile. This value is used in combination with other factors to determine the application priority for the requested communication.

Selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile for an MC service upon request by an MC service user.

Serving MC service server: The MC service server which is providing MC service to an MC service client.

NOTE 1:	There is one serving MC service server for each MC service, which can be the primary MC service server of the MC service user of the MC service client, or can be a partner MC service server to which the MC service user has migrated.

Serving MC system: The MC system which is providing MC service to an MC user.

NOTE 2:	The MC system can be the primary MC system of the MC service user, or can be a partner MC system to which the MC service user has migrated.

Speed: Movement at the moment of location measurement, e.g. see 3GPP TS 25.305 [X] and 3GPP TS 23.032 [Y].

Time of measurement: Date and time expressed with a certain precision to reflect the moment of the location measurement.

For the purposes of the present document, the following terms given in 3GPP TS 22.280 [3] apply

Mission Critical

Mission Critical Applications

Mission Critical Organization

Mission Critical Service

Functional alias

For the purposes of the present document, the following terms given in 3GPP TS 22.179 [2] apply

Multi-talker control

For the purposes of the present document, the following terms related to a MC gateway UE function apply

MC gateway UE:  A functional entity that enables simultaneous access to the MC system for multiple MC clients.

MC client: Aggregates a set of clients (i.e. Group management client, Configuration management client, Identity management client, Key management client, Location management client and MC service client) operated on a non-3GPP device.

Non-3GPP device: A device that enables connectivity towards an MC gateway UE using an access method not specified by 3GPP. A subset of these devices can host an MC client specified by 3GPP.

MC gateway client: A client that enables the authorized binding with one or more MC GW UEs in order to be able to handle MC services (only one MC gateway UE per MC service).

MC gateway UE server: A server on an MC gateway UE that controls authorized binding with multiple MC gateway clients.

MC gateway UE function: Functional block as part of the MC service server that authorises and manages the association between MC client and MC gateway UE.

* * * * * * * NEXT CHANGE * * * * * * *

### 11.2.0	General

The MC gateway UE offers access to the MC server for several MC clients as shown in Figure 11.2.1-1). The MC clients can be either located in the MC gateway UE or in non-3GPP devices connected to the MC gateway UE via non-3GPP access.

For non-3GPP devices which can host an MC client, the MC gateway UE enables connectivity to the MC server. For non-3GPP devices which cannot host the MC client, the MC gateway UE hosts the instantiation of the MC client for the non-3GPP device.

Figure 11.2.0-1: Functional architecture

The MC gateway UE provides MC service capabilities and 3GPP access capabilities using 3GPP network credentials for authorized access with an MC server.

For non-3GPP devices which cannot host MC clients, the MC gateway UE shall control the access and manage the communication between the non-3GPP devices and the MC server. Upon reception of connection authorization request from a non-3GPP device, the MC gateway UE instantiates an MC client, acting on behalf of the non-3GPP device, to provide the requested services (e.g. emergency call, group calls, short data messages services, etc.). The communication interworking and the definition of associated procedures between the MC client (initiated at the MC gateway UE) and the non-3GPP devices is out of scope of this document.

NOTE 1:	MC clients residing on a non-3GPP device cannot use UICC credentials to perform authorisation with the 3GPP transport system.

For MC clients getting MC service access via the MC gateway UE, the MC gateway UE forwards (unmodified) signalling and media from the individual MC clients to the MC server and vice versa.

If the MC service user on the non-3GPP device utilizes multiple MC services simultaneously, the MC service access may also be provided by one or multiple MC gateway UEs as shown in figure 11.2.1-2 while restricting each MC service to one MC gateway UE (e.g. MCPTT via MC gateway UE1, MCData via MC gateway UE2).

Figure 11.2.0-2: Simultaneous multiple MC gateway UE use by a single non-3GPP device

NOTE 2:	Even not shown in the above figure, the same principle of simultaneous use of multiple MC gateway UEs is applied for non-3GPP devices which cannot host an MC client.

* * * * * * * NEXT CHANGE * * * * * * *

### 11.2.1	Functional model

For the application-level signalling necessary for the association between the MC client on the non-3GPP device and the MC gateway UE reference point Gateway-local (GW-local) is used. The MC gateway UE uses the Gateway-Core (GW-Core) reference point towards the MC service server to announce the association between MC gateway client and MC gateway UE server. Once the use of the MC gateway UE has been authenticated and authorized by the corresponding MC service server, the MC service user is authorized MC gateway user.

The MC gateway UE sever enables the following functions:

- Authentication of the MC gateway clients including the MC gateway UE function of the corresponding MC service server

- Relay of signalling between MC client and MC service server/CSC server

- Relay of signaling between the signaling user agent residing on the non-3GPP device and the SIP core

- Relay of signaling between the HTTP client residing on the non-3GPP device and the HTTP proxy

- Media plane forwarding between the MC servive server and the MC clients

The MC gateway UE function as part of the corresponding MC service server enables the use of an MC gateway UE with a corresponding MC gateway UE function that authorizes the association between the MC gateway client/MC client and the corresponding MC gateway UE to enable forwarding of the media plane to MC clients via the MC gateway UE.

Editor’s Note:	The association between MC client and CSC, signalling user agent and SIP core as well as http client and HTTP proxy are FFS.

Figure 11.2.1-1: Functional model of MC gateway UE signalling plane

### 11.2.2	Reference points

#### 11.2.2.1	General

The reference points for the use of the MC gateway UE are described in the following subclauses.

#### 11.2.2.2	Reference point GW-local (between the MC gateway client and the MC gateway UE)

The GW-local reference point between the MC gateway client and the MC gateway UE is used:

- To authorise the use of the MC gateway UE,

- To manage (request/release) forwarding of the signalling, media from the MC gateway UE to the MC gateway client and its associated MC clients for unicast, multicast/broadcast communication using corresponding identifiers, e.g., TMGI.

Editor’s Note:	Whether SIP or HTTP is used on this reference point is FFS.

#### 11.2.2.3	Reference point GW-Core (between the MC gateway UE and the MC service server)

The GW-Core reference point between the MC gateway UE and the MC service server shall be used:

- To authorise the use of the MC gateway UE by the corresponding MC service server.

- To manage (request/release) transport resources between the MC gateway UE and the MC service server signalling purposes are associated with the MC clients residing on a non-3GPP device.

NOTE 1:	Signalling of the MC clients utilizes the allocated resources (default bearer for EPS or corresponding QoS flow in 5GS) and if necessary, MC gateway UE may request additional resources.

NOTE 2:	The authorized use of necessary connectivity between the non-3GPP device and the MC gateway UE is outside 3GPP specification.

Editor’s Note:	Whether SIP or HTTP is used on this reference point is FFS.

#### 11.2.2.4	Reference points CSC-n, SIP-1 and HTTP-1

The reference points CSC-n belonging to the application plane and the reference points SIP-1 and HTTP-1 belonging to the signaling control plane are relayed by the MC gateway UE between MC client on the non-3GPP device towards the MC gateway UE and the corresponding MC service server.

CSC-n, SIP-1 and HTTP-1 reference points are specified in the present specification. The MCX-n reference points are specified in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12] and 3GPP TS 23.282 [13].

* * * * * * * END OF CHANGE * * * * * * *