3GPP TSG-SA WG6 Meeting #47-e	S6-220342

e-meeting, 14th – 22nd February 2022	(revision of S6-220142)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0029 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minor corrections to the procedures related to MBS session creation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the figure 7.3.3.1.2-1 step 5a it is specified that MC UE send a UE session join acknowledgement to the server . It should be UE session join notification instead of UE session join acknowledgement. In the procedure 7.3.3.1.3 Use of Dynamic MBS session it would be better to shows the MBS listening status report and the establishment of group communication over MBS bearer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrects the step 5a in figure 7.3.3.1.2-1 replacing UE session join acknowledgement with UE session join notification - Procedure 7.3.3.1.3 is updated to show the MBS listening status report and session via established bearer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.1.2, 7.3.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 7.3.3.1.2	Procedure for pre-created MBS session and service announcement

Pre-conditions:

-	The MC service server has decided to use an MBS session for MC service group communications associated to a certain MC service group based on transport only mode.

-	The MC service server has performed MB-SMF discovery and selection either directly or indirectly via NEF/MBSF, unless the corresponding information is locally configured.

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC service server is aware whether to request the creation of the MBS service server with or without dynamic PCC rule.

Figure 7.3.3.1.2-1: Use of pre-created MBS session.

1.	The MC service server initiates an MBS session creation procedure towards the 5GC as described in 3GPP TS 23.247 [15]. The procedure starts once the MC service server initiates a TMGI allocation request (either directly to MB-SMF or indirectly via NEF). Upon the reception of the TMGI allocation response, the MC service server sends an MBS session creation request, including further information related to the MBS session, e.g., MBS session ID, MBS session mode and the QoS requirements if dynamic PCC rule is not considered. However, if dynamic PCC rule is considered, the MC service server defines these requirements at a later step, namely it sends an MBS authorization/policy create request towards PCF (either directly or to the NEF) indicating the QoS requirements.

NOTE 1:	In case of LTE eMBMS and 5G MBS co-existence, the MC service server may trigger the establishment of eMBMS bearers as described in 3GPP TS 23.280 [3] (or it may establish a unicast bearer) based on the RAT capabilities supported by the affiliated members in the MC service group X. If MBSF and BM-SC are co-located, TMGI and 4G eMBMS can be the same as the MBS session ID.

Editor's note: it is FFS how the MC service server can retrieve the RAT capabilities of the MC service clients.

2.	The MC service server provides the MC service clients affiliated to MC service group X with the information related to the created MBS session via the MBS service announcement. As described in table 7.3.2.2-1, the MBS service announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type) and SDP information related to the MBS session under consideration.

NOTE 2:	The MC service server may send an MBS service announcement at an earlier step during the MBS session creation procedure towards the MC service clients once the MC group associated to the MBS session is known.

Optionally, the MC service server includes the information elements related to the established eMBMS bearer once the MC service server has determined the need, as indicated in table 7.3.2.2-1. The MC service clients which camp on LTE will subsequently react to the information elements related to the eMBMS bearer as described in 3GPP TS 23.280 [3].3.	MC service clients store and process the received MBS session information.

4.	MC service clients may provide an MBS service announcement acknowledgment to the MC service server to indicate the reception of the corresponding MBS service announcement.

5.	Based on the MBS session mode; either multicast or broadcast, the following actions take place.

5a.	For multicast MBS sessions, MC service clients initiate a UE session join request towards the 5GC using the information provided via the MBS service announcement. Hence, upon the first successful UE session join request, the multicast is then established, and the radio resources are reserved. The established session can either be in active or inactive state as indicated in 3GPP TS 23.247 [15]. The MC service clients may send a UE session join notification towards the server.

NOTE 3:	For broadcast MBS sessions, the session is established as part of the session configuration procedures, hence the network resources are reserved both in 5GC and NG-RAN.

5b.	For broadcast MBS sessions, the MC service clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS service announcement information, MC service clients report the monitoring state back to the MC service server.

NOTE 4:		It is implementation specific whether the broadcast reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g., measurements of radio level signaling such as the reference signals from the NG-RAN node(s), packet loss.

6.	The MC service clients provide a notification related to the announced session in the form on an MBS listening status report.

7.	An MC service group communication setup takes place as specified in TS 23.379 [6], TS 23.281 [4], or TS 23.282 [5]. The MC service server determines to use the pre-created MBS session for this group communication.

# * * * * Second change * * * *

##### 7.3.3.1.3	Procedure for dynamic MBS sessions

In this scenario, the group communication is already taken place and a unicast PDU session is utilized for MC DL transmission. When the MC service server decides to use an MBS session for the transmission under consideration, the MC service server interacts with 5GC to reserve the necessary network resources.

NOTE 1:	The MC service server logic for determining when to create a dynamic MBS session is implementation specific.

The procedure in figure 7.3.3.1.3-1 shows one MC service client receiving the DL media. There might also be MC service clients in the same MC group communication session that receive the communication on an MBS session.

Pre-conditions:

-	MC service client is attached to the 5GS, registered and affiliated to a certain MC service group X.

-	The MC service server is aware whether to request the creation of the MBS service server with or without dynamic PCC rule.

-	The MC service server has performed MB-SMF discovery and selection either directly or indirectly via NEF/MBSF, unless the corresponding information is locally configured.

-	No MBS session exists, or the existing multicast MBS session fails to satisfy the QoS requirements.

Figure 7.3.3.1.3-1: Use of dynamic MBS session.

1.	An MC service group communication session is established as specified in TS 23.379 [6], TS 23.281 [4], or TS 23.282 [5].

2.	The MC service server decides to create an MBS session. The MBS session creation procedure takes place as described in clause 7.3.3.1.

NOTE 1:	In case of LTE eMBMS and 5G MBS co-existence, the MC service server may trigger the establishment of eMBMS bearers as described in 3GPP TS 23.280 [3] (or it may establish a unicast bearer) based on the RAT capabilities supported by the affiliated members in the MC service group X. If MBSF and BM-SC are co-located, TMGI and 4G eMBMS can be the same as the MBS session ID.

3.	The MC service server provides the MC service client with the information related to the created MBS session via an MBS service announcement. As described in table 7.3.2.2-1, the service announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type), and SDP information related to the MBS session.

Optionally, the MC service server includes the information elements related to the established eMBMS bearer once the MC service server has determined the need, as indicated in table 7.3.2.2-1. The MC service clients which camp on LTE will subsequently react to the information elements related to the eMBMS bearer as described in 3GPP TS 23.280 [3].

4.	The MC service client stores the MBS session ID and other associated information.

5.	The MC service client may send an MBS service announcement ack back to the MC service server.

6.	Based on the MBS session mode; either multicast or broadcast, the following actions take place.

6a.	For multicast MBS sessions, MC service client initiates a UE session join request towards the 5GC using the information provided via the MBS service announcement. Hence, upon the first successful UE session join request, the multicast is then established, and the radio resources are reserved. The established session can either be in active or inactive state as indicated in 3GPP TS 23.247 [15]. The MC service client may send a UE session join notification towards the server.

6b.	For broadcast MBS sessions, if MC service client is accessing over 5G, the MC service clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS service announcement information, MC service clients report the monitoring state back to the MC service server.

7.	The MC service clients provide a notification related to the announced session in the form on an MBS listening status report.

8.	An MC service group communication via dynamic MBS session is established.

# * * * * End changes * * * *