3GPP TSG-SA WG6 Meeting #47-e	S6- 220214

e-meeting, 14th – 22nd February 2022	(was S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0032 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network notifications for EPS interworking related events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the case of MC services provided over a 5GS supporting EPS interworking, the MC service server may not be able to identify whether an MC service UE is registered on 5GS or EPS. This may be relevant to MC systems in order to monitor whether MC service UEs are registered on 5GS or EPS. Therefore, the MC service server can subscribe/unsubscribe to receive notifications of specific events provided by the network. These notifications can be related to PDU sessions/PDN connections to which MC service sessions are bounded or related to a specific MC service UE or group of MC service UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A subscription/notification procedure between the MC service server and the network is introduced for EPS interworking related events |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MC system may not be able to identify whether an MC service UE is registered on 5GS or EPS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 7.x.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

### 7.x.3	Network notifications of EPS interworking related events

In the case of MC services provided over a 5GS supporting EPS interworking, the MC service server may not be able to identify whether an MC service UE is registered on 5GS or EPS. Therefore, the MC service server can subscribe/unsubscribe to receive notifications of specific events from the network. These notifications can be related to PDU sessions/PDN connections to which MC service sessions are bounded or related to a specific MC service UE or group of MC service UEs.

Such notifications can be provided from the PCF (via N5 or Rx) for specific events, e.g. access network information notification and change of access type. If NEF is deployed, notifications can also be provided from NEF (via N33) for specific events, e.g. core network (CN) type change. The specification of these event procedures are described in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10], and 3GPP TS 23.503 [9].

The procedure in figure 7.x.3-1 describes a high level subscription/notification procedure between the MC service server and the network.

Pre-conditions:

-	MC service UE is a 5GS/EPS capable UE.

-	MC service UE is registered to a 5GS supporting EPS interworking for the support of MC services.

Figure 7.x.3-1: Network notifications of EPS interworking related events

1.	MC service UE is registered to the MC service server over the 5GS. MC service UE has established a PDU session(s) associated to MC service sessions.

2.	MC service server subscribes to receive notifications from the network associated to MC service UE related to EPS interworking events. For instance, the MC service server may subscribe to notifications of events from the PCF associated to the MC service UE, e.g. access network information notification and change of access type, as specified in 3GPP TS 23.503 [9].

If NEF is deployed, the MC service server may also subscribe to notifications of events from NEF associated to the MC service UE, e.g. CN type change, as specified in 3GPP TS 23.501 [7] and 3GPP TS 23.502 [10].

3.	The 5GS supporting EPS interworking decides that the MC service UE should be handed over to EPS, e.g. due to radio conditions. 5GS handles associated PDU sessions of the MC service UE to establish corresponding PDN connections in EPS, i.e. required default and dedicated EPS bearers associated to MC service sessions.

NOTE 1:	The interworking procedures with EPS are described in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10], and 3GPP TS 23.503 [9].

4.	MC service UE is registered on EPS. MC services continue being provided to the MC service UE over EPS via the established PDN connections (default and dedicated bearers associated to corresponding MC service sessions) in step 2.

5.	The network provides notifications to the MC service server of subscribed events associated to the MC service UE.

NOTE 2:	Step 5 can occur before or in parallel to step 4.