3GPP TSG-SA WG6 Meeting #47-e	S6-220393

e-meeting, 14th – 22nd February 2022	(revision of S6-220230)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS23.289 | CR | 0037 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information flows for media distribution over 5MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Lacking of signalling flows related with procedures of usage of 5GMBS media distribution in current TS, which requires related description. Some signalling flows without modification can be directly refered to TS 23.280. The signalling flows which requires modification specific for 5GMBS will be added to TS 23.289. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding an generic claification about those signalling flows (Media distribution release) which can be refered to TS 23.280. - The signalling flows which requires modification specific for 5GMBS(media distribution request, media distribution reponse) are added to TS 23.289. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The media distribution over 5MBS will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.x (new), 7.3.2.y (new), 7.3.2.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********************************The first change********************************

#### 7.3.2.x	Media distribution request

Table 7.3.2.x-1 describes the information flow media distribution request from the MC Service server to an MC service server (5G MBS Session control role) that has a desired 5G MBS session.

The difference compared with the information flow defined in 3GPP TS 23.280 [3] is that in 5G MBS, the information element for QoS is 5QI.

Table 7.3.2.x-1: Media distribution request

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID | M | MBS session identifier, which can also be TMGI. |
| Bandwidth | M | Maximum bandwidth required |
| Separate floor control | M | Whether or not a separate session is required for floor control |
| SDP information | M | SDP with media and floor control information applicable to groups that can use this MBS session (e.g. codec, protocol id) |
| 5QI | O | Desired 5QI |
| MC Group ID | O | The MC group id for when the request is sent for a specific group call. |

#### 7.3.2.y	Media distribution response

Table 7.3.2.y-1 describes the information flow media distribution response from an MC Service server (5G MBS session control role) that has a desired 5G MBS session to the MC service server.

Table 7.3.2.y-1: Media distribution response

| Information element | Status | Description |
| --- | --- | --- |
| TMGI | M | TMGI information |
| Bandwidth | M | Maximum bandwidth required |
| SDP information | M | SDP with media and floor control information applicable to groups that can use this session (e.g. codec, protocol id) |
| 5QI | O | Actual 5QI |
| Media stream identifier | O | This element identifies the media stream of the SDP used for the group call |
| Media distribution indicator | O | Indicates to the MC service server whether the media in the ongoing group communication should be sent or not |

#### 7.3.2.Z	Media distribution release

Information flow for Media distribution release is reused without modification as specified in clause 10.7.2.10 of 3GPP TS 23.280 [3].