3GPP TSG-SA5 Meeting #141-e 	S5-221671

# e-meeting, 17 -26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.240 | CR | 0439 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Charging architecture for Local Breakout |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHROAM |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of charging principles for Edge Computing in Local Breakout and MVNO charging |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new reference point between vSMF and hCHF Addition of charging principles for local breakout scenario Addition of charging principles wih MVNO involved Referencing Edge Computing charging principles |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging for LBO services is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 4.4.3, 5.5.3.x (new), 5.5.3.y (new), 6.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] - [9]	Void.

[10]	3GPP TS 32.250: "Telecommunication management; Charging management; Circuit Switched (CS) domain charging".

[11]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[12] 	 Void.

[13]	3GPP TS 32.253: "Telecommunication management; Charging management; Control Plane (CP) data transfer domain charging".

[14]	3GPP TS 32.254: "Telecommunication management; Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[15]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2".

[16]	3GPP TS 32.256: "Telecommunication management; Charging management; 5G connection and mobility domain charging; stage 2".

[17] 	3GPP TS 32.257: “Telecommunication management; Charging management; Edge Computing domain charging”.

[18] - [19]	Void.

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21] - [29]	Void.

[30]	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

[31] 	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[32] 	3GPP TS 32.272: "Telecommunication management; Charging management; Push-to-talk over Cellular (PoC) charging".

[33]	3GPP TS 32.273: "Telecommunication management; Charging management; Multimedia Broadcast and Multicast Service (MBMS) charging".

[34]	3GPP TS 32.274: "Telecommunication management; Charging management; Short Message Service (SMS) charging".

[35]	3GPP TS 32.275: "Telecommunication management; Charging management; MultiMedia Telephony (MMTel) charging".

[36] 	3GPP TS 32.276: "Telecommunication management; Charging management; Voice Call Service Charging".

[37] 	3GPP TS 32.277: "Telecommunication management; Charging management; Proximity-based Services (ProSe) Charging".

[38] 	3GPP TS 32.278: "Telecommunication management; Charging management; Monitoring Event charging".

[39]	Void.

[40]	3GPP TS 32.280: "Telecommunication management; Charging management; Advice of Charge (AoC) service".

[41]	3GPP TS 32.281: "Telecommunication management; Charging management; Announcement service".

[42] - [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55] 	Void.

[56]	3GPP TS 32.293: "Telecommunication management; Charging management; Proxy Function".

[57]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[58]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[59] - [69]	Void.

[70]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2".

[71]	3GPP TS 28.202: "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[72] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102] - [199]	Void.

[200] - [206]	Void.

[207]	3GPP TS 23.078: "Customized Applications for Mobile network Enhanced Logic (CAMEL); Stage 2".

[208]	3GPP TS 23.203: "Policy and charging control architecture".

[209]	3GPP TS 23.228:  "IP Multimedia Subsystem (IMS); Stage 2".

[210]	 Void.

[211]	3GPP TS 24.229: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[212]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[213] 	3GPP TS 24.002: "GSM - UMTS Public Land Mobile Network (PLMN) access reference configuration".

[214]	3GPP TS 23.502:"Procedures for the 5G System".

[215]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[216]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[217] 	3GPP TS 23.548: “5G System Enhancements for Edge Computing”.

[218]	3GPP TS 23.558: “Architecture for enabling Edge Applications”.

[219] - [297]	Void.

[298]	EU Roaming regulation III; "Structural Solutions; High Level Technical Specifications".

[299]	EU Roaming regulation III; "Interface & Protocol; Detailed Technical Specifications".

[300]	ITU-T Recommendation D.93: "Charging and accounting in the international land mobile telephone service (provided via cellular radio systems)".

[301] - [399]	Void.

[400] - [401]	Void.

[402]	IETF RFC 4006 (2005): "Diameter Credit-Control Application".

[403] - [499]	Void.

[500]	GSMA PRD BA.27: "Charging Principles".

Next change

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3G	3rd Generation

3GPP	3rd Generation Partnership Project

5GC	5G Core Network

5GS	5G System

5G DDNMF	5G Direct Discovery Name Management Function

ABMF	Account Balance Management Function

AF	Application Function

AMF	Access and Mobility Management Function

AoC	Advice of Charge

API	Application Program Interfaces

APN	Access Point Name

AS	Application Server

BD	Billing Domain

BGCF	Breakout Gateway Control Function

BS	Bearer Services

BSC	Base Station Controller

BSS	Base Station Subsystem

BTS	Base Transceiver Station

CAMEL	Customized Applications for Mobile network Enhanced Logic

CAP	CAMEL Application Part

CCS	Converged Charging System

CDF	Charging Data Function

CDR	Charging Data Record

CG	Charging Gateway

CGF	Charging Gateway Function

CHF	Charging Function

CN	Core Network

CP	Control Plane

CS	Circuit Switched

CSCF	Call Session Control Function (I-Interrogating; E-Emergency; P-Proxy; and S-Serving)

CTF	Charging Trigger Function

EATF	Emergency Access Transfer Function

EBCF	Event Based Charging Function

ECUR	Event Charging with Unit Reservation

EIR	Equipment Identity Register

EPC	Evolved Packet Core

ePDG	Evolved Packet Data Gateway

EPS	Evolved Packet System

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FQPC	Fully Qualified Partial CDR

GGSN	Gateway GPRS Support Node

GMLC	Gateway MLC

GMSC	Gateway MSC

GPRS	General Packet Radio Service

GSM	Global System for Mobile communication

gsmSCF	GSM Service Control Function

gsmSSF	GSM Service Switching Function

GSN	GPRS Support Node (either SGSN or GGSN)

HLR	Home Location Register

HPLMN	Home PLMN

HSCSD	High Speed Circuit Switched Data

IBCF	Interconnect Border Control Function

ICS	IMS Centralized Services

IE	Information Element

IEC	Immediate Event Charging

IETF	Internet Engineering Task Force

IMEI	International Mobile Equipment Identity

IMS GWF	IMS GateWay Function

IMS	IP Multimedia Subsystem

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

ISC	IMS Service Control

ISDN	Integrated Services Digital Network

ITU-T	International Telecommunication Union - Telecommunications standardization sector

LAC	Location Area Code

LAN	Local Area Network

LCS	Location Services

LRF	Location Retrieval Function

LTE	Long Term Evolution

MAP	Mobile Application Part

MBMS	Multimedia Broadcast and Multicast Service

ME	Mobile Equipment

MGCF	Media Gateway Control Function

MGW	Media GateWay

MLC	Mobile Location Center

MME	Mobility Management Entity

MMI	Man-Machine Interface

MMS	Multimedia Messaging Service

MMSE	Multimedia Messaging Service Environment

MMTel	MultiMedia Telephony

MNO	Mobile Network Operator

MnS	Management Service

MO	Mobile Originated

MOC	MO Call

MRF	Media Resource Function

MRFC	MRF Controller

MS	Mobile Station

MSC	Mobile Services Switching Centre

MSISDN	Mobile Station ISDN number

MT	Mobile Terminated

MTC	MT Call

MVNO	Mobile Virtual Netork Operator

NE	Network Element

NF	Network Function

NWDAF	Network Data Analytics Function

OCF	Online Charging Function

OCS	Online Charging System

OFCS	Offline Charging System

OMR	Optimal Media Routing

PCEF	Policy and Charging Enforcement Function

PCF	Policy Control Function

PCRF	Policy and Charging Rules Function

PDG	Packet Data Gateway

PDN	Packet Data Network

PDP	Packet Data Protocol, e.g. IP

PLMN	Public Land Mobile Network

PoC	Push-to-talk over Cellular

ProSe	Proximity-based Services

PS	Packet-Switched

PSPDN	Packet-Switched Public Data Network

QoS	Quality of Service

RF	Rating Function

RNC	Radio Network Controller

RNS	Radio Network Subsystem

RPC	Reduced Partial CDR

SBCF	Session Based Charging Function

SCCP	Signalling Connection Control Part

SCEF	Service Capability Exposure Function

SCF	Service Control FunctionSCS	Services Capability Server

SCUR	Session Charging with Unit Reservation

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMS	Short Message Service

SMF	Session Management Function

SSF	Service Switching Function

TAP	Transferred Account Procedure

TDF	Traffic Detection Function

TR	Technical Report

TRF	Transit and Roaming Function

TS	Technical Specification

TWAG	Trusted WLAN Access Gateway

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

UPF	User Plane Function

USIM	Universal SIM

VAS	Value Added Service

VLR	Visitor Location Register

VMSC	Visited MSC

VPLMN	Visited PLMN

WLAN	Wireless LAN

Next change

### 4.4.3	Charging services Reference point

The common charging architectures are mapped into the specific domain/subsystem/service charging architectures in the respective middle tier TSs, which contain in their reference point representation, the following reference points:

N28:	Reference point between PCF and CHF defined in TS 23.501[215].

N40:	Reference point between SMF and the CHF in the same PLMN defined in clause 4.2 of TS 32.255 [15].

N41:	Reference point between AMF and CHF in HPLMN defined in clause 4.2.2 of TS 32.256 [16].

N42:	Reference point between AMF and CHF in VPLMN defined in clause 4.2.2 of TS 32.256 [16].

N44:	Reference point between NEF and CHF defined in clause 4.4 of TS 32.254 [14].

N45:	Reference point between IMS Node and CHF defined in clause 4.4 of TS 32.260 [20].

N46:	Reference point between SMS Node and CHF defined in clause 4.4 of TS 32.274 [34].

N47: 	Reference point between SMF and the CHF in different PLMNs defined in clause x.y of TS 32.255 [15].

Next change

#### 5.5.3.x	Charging Principles for 5G Roaming architecture with Local Breakout

The 5G System roaming architecture with local breakout is specified in TS 23.501 [215]. The breakout point for both the control plane signalling and user plane traffic is in the VPLMN, i.e. the vSMF and vUPF respectively.

The VPLMN charging mechanism collects charging information related to the 5G data connectivity usage for each UE detected as in-bound roamer. The information collected include details of the services used by the visiting subscriber and it is conveyed to both the CHF in VPLMN and to the CHF in the HPLMN.

The CHF in the VPLMN uses the collected charging information for wholesale charging including service aware towards the HPLMN

The CHF in the HPLMN uses the collected charging information for retail charging towards the home subscriber while roaming.

Charging for Roaming with Local Breakout is covered by the 5G data connectivity domain converged charging architecture specified in TS 32.255 [15], using the SMF embedding the CTF.

Next change

#### 5.5.3.y	Charging Principles for 5G non roaming Mobile Virtual Network Operators charging

For scenarios in which subscribers have a subscription with an MVNO which allows usage of 5G data connectivity while in the home MNO, the MNO shall be able to collect charging information related to 5G data connectivity usage for each MVNO, for wholesale. The MVNO deploys their own billing and charging (CHF), but no other NFs.

The charging mechanism in the MNO collects charging information related to the 5G data connectivity usage for each UE and conveys this charging information to the MVNO for each UE.

The MVNO uses the charging information collected for retail charging (MVNO to subscriber). Charging for MVNO scenario is covered by the 5G data connectivity domain converged charging architecture specified in TS 32.255 [15].

N47 reference point is also used when an additional actor (i.e. MVNO) performs retail charging for its own subscribers. In such a case N47 is the reference point between SMF in the MNO and CHF in the MVNO.

Next change

## 6.z	5G Edge Computing services charging

Edge Computing support in 5GS is defined in TS 23.501[215], TS 23.502[214] and TS 23.548[217]. The architecture for enabling Edge Applications is specified in TS 23.558[218].

The charging principles for the Edge Computing domain are specified in TS 32.257[17] and TS 32.255[15].

The architecture of Edge Computing in Local Breakout roaming scenario and charging for Edge Computing in local breakout follows the principles in subclause 5.5.3.9 are specified in TS.23.501 [215].

End of changes