3GPP TSG-SA5 Meeting #142-e 	S5-222433

# e-meeting, 4 - 12 April 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0424 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Trigger for PLMN change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-03-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The trigger for PLMN change is not described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the a SIP message may be used to report a PLMN change and removal of duplicated text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how PLMN change may be reported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.4.3	Applicable triggers for IMS charging

The IMS nodes for which SIP method a charging Data Request is sent shall be operator configurable. The tables below describe all possible Charging Data Requests that might be sent from IMS nodes.

The generation of a Charging Data Request message by the IMS nodes in response to a particular "triggering SIP method" shall be operator configurable (enable or disable).

Table 5.4.3.1: Default Trigger conditions in IMS Node

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| SIP INVITE | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial] |
| SIP NOTIFY | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Initial] IEC: Charging Data Request [Event] |
| SIP MESSAGE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REGISTER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP SUBSCRIBE | - | Immediate | Not Applicable | Not Applicable |  |
| SIP REFER | - | Immediate | Not Applicable | Not Applicable |  |
| SIP PUBLISH | - | Immediate | Not Applicable | Not Applicable |  |
| Change of charging conditions |  |  |  |  | SCUR: Charging Data Request [Update] |
| SIP RE-INVITE or UPDATE (e.g. change in media components, terminating identity change) | - | Immediate | No | No |  |
| SIP 2xx acknowledging a SIP INVITE, RE-INVITE or UPDATE (e.g. change in media components) | - | Immediate | No | No |  |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body | - | Immediate | No | No |  |
| SIP 4xx, 5xx or 6xx response, indicating an unsuccessful SIP RE-INVITE or UPDATE | - | Immediate | No | No |  |
| Any other SIP message during a SIP session, that allows the SIP session to continue | - | Immediate | Yes | No |  |
| CHF limit |  |  |  |  |  |
| Expiry of time limit | - | Immediate | No | Yes |  |
| Expiry of limit of number of charging condition changes | - | Immediate | No | Yes |  |
| Quota management |  |  |  |  |  |
| Time threshold reached | - | Immediate | No | Yes |  |
| Time quota exhausted | - | Immediate | No | Yes |  |
| Unit quota exhausted | - | Immediate | No | Yes |  |
| Expiry of quota validity time | - | Immediate | No | Yes |  |
| Expiry of quota holding time | - | Immediate | No | Yes |  |
| Re-authorization request by CHF | - | Immediate | No | Yes |  |
| Other |  |  |  |  |  |
| Management intervention | - | Immediate | No | No |  |
| SIP BYE message | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] |
| SIP 2xx acknowledging a SIP BYE | - | Immediate | Not Applicable | Not Applicable |  |
| Aborting a SIP session set-up procedure, using an internal trigger or a SIP CANCEL | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 3xx final or redirection response | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 2xx acknowledging non-session related SIP messages | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| Deregistration | - | Immediate | Not Applicable | Not Applicable |  |
| SIP 4xx, 5xx or 6xx final response, indicating an unsuccessful procedure | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] PEC: Charging Data Request [Event] |

For converged charging, the following details of chargeable events and corresponding actions in the IMS nodes are defined in Table 5.4.3.2:

Table 5.4.3.2: Chargeable events and their related actions in IMS nodes

| Chargeable event | Conditions | IMS node action |
| --- | --- | --- |
| SIP INVITE |  | SCUR: Charging Data Request [Initial] with a possible request quota for later use |
| SIP NOTIFY |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP MESSAGE (e.g., PLMN change) |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP REGISTER |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP SUBSCRIBE |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP REFER |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP PUBLISH |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use IEC: Charging Data Request [Event] |
| SIP RE-INVITE or UPDATE (e.g. change in media components, terminating identity change) |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP 2xx acknowledging a SIP INVITE, RE-INVITE or UPDATE (e.g. change in media components) |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP 1xx provisional response, mid-dialog requests, mid-dialog responses and SIP INFO embedding RTTI XML body |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| SIP 4xx, 5xx or 6xx response, indicating an unsuccessful SIP RE-INVITE or UPDATE |  | SCUR: Charging Data Request [Update] with a request quota with a possible amount of quota |
| Any other SIP message during a SIP session, that allows the SIP session to continue |  | Start new counts with time stamps for all active service data flows. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | SCUR: Charging Data Request [Update] with a possible request quota. |
| Expiry of time limit | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of limit of number of charging condition changes | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Time threshold reached | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Time quota exhausted | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Unit quota exhausted | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of quota validity time | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of quota holding time | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Re-authorization request by CHF | If the corresponding trigger is enabled | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Management intervention |  | SCUR: Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| SIP BYE message, both normal and abnormal session termination cases |  | SCUR: Charging Data Request [Termination] Close the counts with time stamps |
| SIP 2xx acknowledging a SIP BYE | If last user location information of originating/ terminating party is required | SCUR: Charging Data Request [Termination] Close the counts with time stamps |
| SIP 2xx acknowledging non-session related SIP messages |  | ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| Aborting a SIP session set-up procedure, using an internal trigger or a SIP CANCEL |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| Deregistration |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| SIP 3xx final or redirection response |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps |
| SIP 4xx, 5xx or 6xx final response, indicating an unsuccessful procedure |  | SCUR: Charging Data Request [Termination] ECUR: Charging Data Request [Termination] Close the counts with time stamps IEC: Charging Data Request [Event] PEC: Charging Data Request [Event] |

End of changes