3GPP TSG-SA/CT/RAN Meeting #76	SP-220590

Budapest, Hungary, 6 – 10 June 2022

Title:	Summary for Dynamically Changing AM Policies in the 5GC

Type:	Work Item Summary

Agenda Item:	20.5 (SA)

Source:	Ericsson/Belen Pancorbo

Contact:	belen.pancorbo@ericsson.com

This document is a contribution to TR 21.917 ("Release 17 Description; Summary of Rel-17 Work Items"). It is meant to be endorsed by the plenary.

Summary based on the input provided by Ericsson in SP-220590.

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 870005 | Stage 2 for TEI17_DCAMP | TEI17_DCAMP | S2 | SP-200446 | Pancorbo, Belen (Ericsson) |

Introduction

The TEI17_DCAMP Work Item provides the capability for an Application Function (i.e., AF) to request for a UE either service area coverage or the indication that high throughput is desired or both.

Description

The TEI17_DCAMP WID specifies two methods:

- The AF requests a service area coverage for the UE and/or indicates that high throughput is desired, knowing that certain conditions are met, i.e., the application traffic needs a change of service area coverage or high throughput.

- The AF provides the service area coverage and/or the indication that high throughput is desired for one or multiple UEs that may or may not already be registered or fulfil certain conditions related to application traffic.

In method a) the AF contacts the NEF then the NEF contacts the BSF to retrieve the identity of the PCF serving the UE. In method b) the AF contacts the NEF then the NEF stores the AF request in the UDR as Data Set "Application Data" and Data Subset "AM influence information".

In both methods, the AF provides the service area coverage that the PCF takes as input for policy decisions then to determine whether the service area restrictions need to be updated. The AF provides the indication that high throughput is desired that is considered by the PCF to decide if the RFSP value index for a UE needs to be changed. The PCF reports to the AF that the request was executed. The specification work applies for non-roaming scenarios.

References

List of related CRs: select "TSG Status = Approved" in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=870005

[1]	TS 23.503: "Policy and charging control framework for the 5G System (5GS)"

[2]	TS 23.502: "Procedures for the 5G System (5GS)"