3GPP TSG-SA5 Meeting #145-e 	S5-225369

# e-meeting, 15 - 24 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0004 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct procedures for intent management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | DN of intent MOI is not included in the intent fulfilment information and intent FulfilmentInfo does not only include fulfilmentStatus, the step description of intent fulfilment information notification in clause 6.3 is unclear. The description of procedures for intent management in clause 6.3 has some syntax errors and the step description of feasibility check performing need further elaborated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the step description of intent fulfilment information notification and corresponding figures in clause 6.3, update the PlantUML source code in Annex A.1. Correct the syntax errors in clause 6.3. Elaborate the step description of performing feasibility check. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The steps of intent fulfilment information notification in clause 6.3 is unclear. Some syntax errors remain in clause 6.3. The step description of performing feasibility check may lead misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, Annex A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.3	Procedures for intent management

6.3.1	Introduction

This clause describes the procedures for intent management.

6.3.2	Create an intent

Figure 6.3.2-1 illustrates the procedure for create a new intent.

Figure 6.3.2-1: Procedure for create an intent

1.	MnS Consumer sends a request to create an intent instance to MnS Producer with 'objectClass' and list of [Attribute,Value] for the new intent to be created. The detailed [Attribute,Value] see the concrete intent IOC defined in clause 6.2. 'objectClass' is the name for the concrete intent IOC.

2.	Based on the received request, MnS Producer performs the feasibility check of the intent instance. MnS Producer can perform the feasibility check and get the results based on latest statistics of network or service performance metrics, historical experience (e.g. experience based feasible value range or threshold of performance gain), current operating status including network resource utilization and availability, prediction results based on network simulation system, and predefined checking rules or policies.

NOTE:	Whether to perform the feasibility check can be determined according to the feasibility check enabling policy (e.g. enforce to perform feasibility check in any case, enforce to perform feasibility check in specific cases, not to perform feasibility check in specific cases, not to perform feasibility check in any case). And the feasibility check enabling policy can be predefined/configured in the MnS Producer or sent with the intent creation request from the MnS Consumer.

If the feasibility check result is 'feasible':

3.	Based on the request, the MnS Producer creates the concrete intent MOI (i.e. instance of intent IOC) with value for attribute 'objectInstance' allocated, and configure the new created intent MOI with the received list of [Attribute, Value]. 'objectInstance' is the identifier (DN) for the concrete intent MOI.

4.	MnS Producer sends a response to the MnS Consumer with status (OperationSucceeded or OperationFailed) and 'objectInstance' of the created intent MOI. The response information may also include the possible reasons for the unsuccessful executions (e.g. conflicting with existing intents).

5.	Based on the intent, MnS Producer identifies the MOI for managed entities (e.g. ManagedElement, ManagedFunction) and derives one or more executable management tasks (including deployment and configuration requirements) for these managed entities, then MnS producer deploys or configures corresponding managed entities to satisfy the intent.

6.	During the execution of the intent, MnS Producer continuously monitors intent fulfilment status.

7.	MnS Producer analyses and adjusts the managed entities to ensure the intent is continuously satisfied.

8.	MnS Producer may notify MnS Consumer about DN of intent MOI, and the intent fulfilment information.

If the feasibility check result is 'infeasible', MnS Producer does not create an intent MOI and feedback the feasibility check result information to MnS Consumer.

6.3.3	Modify an intent

Figure 6.3.3-1 illustrates the procedure for modify an existing intent.

Figure 6.3.3-1: Procedure for modify an intent

1.	MnS Consumer sends a request to modify an intent instnace to MnS Producer with 'objectInstance' of the intent MOI and List of ['Attrribute', 'newValue'] to be modified. The detailed [Attribute,Value] see the concrete intent IOC defined in clause 6.2.

2.	Based on the received request, MnS Producer performs the feasibility check of the modified intent instance. Whether to perform the feasibility check can be determined according to the feasibility check enabling policy.

If the feasibility check result is 'feasible':

3.	Based on the request, MnS Producer configure the intent MOI with list of 'Attribute' = 'newValue' which is required to be modified.

4.	MnS Producer sends a response to the MnS consumer with status (OperationSucceeded or OperationFailed), and'objectInstance' of the modified intent MOI, and list of ['Attribute', 'newValue'] which is modified. The response information may also include possible reasons for the unsuccessful executions (e.g. conflicting with other intents).

5.	MnS Producer derives one or more executable management tasks for these managed entities, then MnS producer deploys or configures corresponding managed entities to satisfy the intent.

6.	During the execution of the intent, MnS producer continuously tracks intent fulfilment status.

7.	MnS producer analyses and adjusts the managed entities to ensure the intent is continuously satisfied.

8.	MnS Producer may notify MnS Consumer about DN of intent MOI, and the intent fulfilment information via notification or intent reporting.

If the feasibility check result is 'infeasible', MnS Producer does not modify the intent MOI and feedback the feasibility check result information to MnS Consumer.

6.3.4	Delete an intent

Figure 6.3.4-1 illustrates the procedure for modify an existing intent.

Figure 6.3.4-1: Procedure for delete an intent

1.	MnS Consumer sends a request to delete an intent to MnS Producer with 'objectInstance' of the intent MOI.

2.	Based on the request, MnS Producer deletes the intent MOI.

3.	MnS Producer sends response to the MnS consumer with status (OperationSucceeded or OperationFailed), 'objectInstance' of the deleted intent MOI.

2nd Change

A.1	Procedures for intent management

A.1.1	Create an intent

@startuml

title "[Create an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

Collections "ManagedEntity" as ManagedEntity

MnS_Consumer -> MnS_Producer: 1. Request to create an intent instance ('objectClass',list of \n[Attribute,Value] of intent IOC)

MnS_Producer -> MnS_Producer: 2. Perform the feasibility check of the intent instance

alt feasibility check result is "Feasible"

MnS_Producer -> MnS_Producer: 3. Create and configure intent MOI

MnS_Producer -> MnS_Consumer: 4. Response for create an intent instance \n (status, DN of intent MOI, reason)

alt status is OperationSucceeded

Ref over MnS_Producer, ManagedEntity: 5. Perform service or network management tasks

loop

Ref over MnS_Producer, ManagedEntity: 6. Evaluate intent fulfilment

opt

Ref over MnS_Producer, ManagedEntity: 7. Adjust to fulfil the intent requirement

end

end

MnS_Producer -> MnS_Consumer:8. Notify of feedback\n (DN of intent MOI, FulfilmentInfo)

end

End

hide footbox

@enduml

A.1.2	Modify an intent

@startuml

title "[Modify an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

MnS_Consumer -> MnS_Producer: 1.Request to modify an intent\n('objectInstance' of intent MOI,List of ['Attribute','newValue'])

MnS_Producer -> MnS_Producer: 2. Perform the feasibility check of \n the modified intent

alt feasibility check result is "Feasible"

MnS_Producer -> MnS_Producer: 3.Configure the intent MOI with \n List of ['Attribute' = 'newValue']

MnS_Producer -> MnS_Consumer: 4.Response for modify an intent\n ('objectInstance' of intent MOI)

Ref over MnS_Producer, ManagedEntity: 5. modify service or network management tasks

loop

Ref over MnS_Producer, ManagedEntity: 6. Evaluate intent fulfilment

opt

Ref over MnS_Producer, ManagedEntity: 7. Adjust to fulfil the intent requirement

End

End

MnS_Producer -> MnS_Consumer:8. Notify of feedback\n (DN of intent MOI, FulfilmentInfo)

End

hide footbox

@enduml

End of Change