3GPP TSG-SA5 Meeting #145-e 	S5-225854

# e-meeting, 15 - 24 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0118 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update procedures for modification of Network Slice and Network Slice Subnet MOIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2022-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The sequence diagrams for Network Slice Instance Modification and Network Slice Subnet Instance Modification are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The sequence diagrams in figure 7.6-1 and figure 7.7-1 are corrected to reflect the correct APIs. Abbreviations for Network Slice Subnet Management Service Provider and Network Slice Subnet Management Service Consumer corrected in clause 7.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification leads to confusion and incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6, 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - S5-225086 is CR converted from draftCR S5-224279 that was approved in #144e meeting. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - Cover page updated to correct the category of the CR and to reflect in other comments that the CR is converted from draftCR from #144e to CR in #145e. |  |  |  |  |  |  |  |  |

1st Change

## 7.6	Procedure of Network Slice Instance Modification

The Figure 7.6-1 illustrates the procedure of modifying an existing NSI.

Figure 7.6-1: Network Slice Instance Modification Request procedure

1)	Network Slice Management Service Provider (NSMS_P) receives a modifyMOIAttributes operation defined in TS 28.532 [8] from Network Slice Management Service Consumer (NSMS_C) with the DN of NetworkSlice MOI and the new network slice related requirements (see ServiceProfile defined in clause 6.3.3 in TS 28.541[6]).

2)	Based on the new network slice related requirements, NSMS_P invokes the feasibility check procedure. If the modification requirements can be satisfied, go to step 3), else go to step 5).

3) NSMS_P decomposes the NetworkSlice MOI modification request into NSSI modification request(s), i.e., generating the new network slice subnet related requirements for each NSSI if needed.

4) NSMS_P, as the role of Network Slice Subnet Management Service Consumer (NSSMS_C), invokes the NSSI modification procedure.

5) NSMS_P sends NSI modification result (see modifyMOIAttributes operation defined in TS 28.532 [8]) to NSMS_C.

## 7.7	Procedure of Network Slice Subnet Instance Modification

The Figure 7.7-1 illustrates the procedure of modifying an existing NSSI.

Figure 7.7-1: Network Slice Subnet Instance Modification Request procedure

1)	Network Slice Subnet Management Service Provider (NSSMS_P) receives a modifyMOIAttributes operation defined in TS 28.532 [8] from Network Slice Subnet Management Service Consumer (NSSMS_C) with the DN of NetworkSliceSubnet MOI and the new network slice subnet related requirements (see SliceProfile defined in clause 6.3.3 in TS 28.541[6]).

2)	Based on the new network slice subnet related requirements, NSSMS_P invokes the feasibility check procedure. If the modification requirements can be satisfied, go to step 3), else go to step 5).

3) NSSMS_P decomposes the NetworkSliceSubnet MOI modification request into modification requests for each NSSI constituent.

4a) If the requested NSSI constituent is constituent NSSI, NSSMS_P invokes NSSI modification procedure as described in clause 7.7.

4b) If the requested NSSI constituent is NF instance, NSSMS_P invokes NF creation procedure as described in clause 7.10 or NF modification procedure as described in clause 7.11.

4c)	If the NSSI contains the virtualized part, NSSMS_P invokes the NS instance scaling and/or NS instance updating and/or NS instance instantiation procedure as described in TS 28.526 [7].

4d) If the NSSI contains the TN part, NSSMS_P invokes the TN related coordination procedure as described in clause 7.9.

5) NSSMS_P sends NSSI modification results (see modifyMOIAttributes operation defined in TS 28.532 [8]) to NSSMS_C.

End of change