3GPP TSG SA WG2 Meeting #152E	S2-2207000

# Elbonia, August 17 – 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3542 | rev | 1 | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of EAP Re-authentication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA2 has included the use of EAP Re-authentication (ERP) in order to enable a UE to move from a source TNAP to a target TNAP within the area of the same TNGF without a need to perform a full authentication. There is also a note (since rel-16) indicating further work needed in SA3: NOTE: It is expected that SA WG3 will confirm the above steps and will define how the re-authentication root key (rRK) is created. However, even though SA3 has discussed the topic, there has been no progress, and they have not specified any support for ERP in Rel-16 or Rel-17. Reasons for that are e.g. that it creates a parallel key hierarchy and that it only works for EAP-AKA’, not for 5G-AKA. CT1 has also not implemented support for ERP in TS 24.502. It should thus be clarified that full authentication is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove support for ERP and clarify that full authentication is needed at any inter TNAP mobility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete feature remaining in Rel-16/17 specifications in SA2 which is not supported in SA3 and CT1 and thus not implementable. SA2 specification not aligned with SA3 and CT1 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12a.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.12a.9	Support of mobility from source to target TNAP

In this release, if the UE moves from a source TNAP to a target TNAP, the UE shall perform a full authentication via the target TNAP to re-connect to the 5G system.

**** End of Changes ****