3GPP TSG-SA WG6 Meeting #50e	S6-222458

# E-Meeting, 22nd – 31st August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0112 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update_Unicast_QoS_Monitoring_Subscription operation in the SS_NetworkResourceMonitoring API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The update of the unicast QoS monitoring subscription is needed to provide flexibility in the QoS monitoring functionality. However, the possibility of the unicast QoS monitoring subscription update is not defined in the current version of 3GPP TS 23.434. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Update_Unicast_QoS_Monitoring operation is added to the SS_NetworkResourceMonitoring API. Minor editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Impossibility to update the unicast QoS monitoring subscription in the SS_NetworkResourceMonitoring API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.20; 14.3.2.xx(new); 14.3.2.yy(new); 14.3.3.4.x(new), 14.4.1; 14.4.4.x (new); |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# This new feature will support the following use cases:

- The monitoring subscription defines both the transmission quality parameters to report and the conditions when to report (frequency/periodicity, threshold etc.). With an increasing number of connections to be monitored the VAL server load spent on monitoring is also increasing. Thus, the VAL server may need to adjust the reporting thresholds/reporting frequency for the measured data, e.g., too frequent reports from the NRM server.

- Adjustments in measurement requirements based on the history of measurement data, like the aggregation window or list of the measurement indexes.

- Update of the notification URI for the VAL server application e.g. in cloud/microservice environment with CI/CD, where ‘frequent’ VAL server upgrade/relocation is normal. The downtime at each version update or loading rebalancing while re-establishing each subscription for each connection may be critical for MC applications.

# Proposed changes:

* * * First Change * * * *

#### 14.3.2.20	Unicast QoS monitoring subscription request

Table 14.3.2.20-1 describes the information flow from the VAL server to the network resource management server for unicast QoS monitoring subscription request.

Table 14.3.2.20-1: Unicast QoS monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs | O (see NOTE 1, NOTE 4) | List of VAL UEs whose QoS monitoring data is requested. |
| VAL group ID | O (see NOTE 1, NOTE 4) | The group ID used for the VAL group for which QoS monitoring data is requested. |
| List of VAL stream IDs | O (see NOTE 1, NOTE 4) | List of VAL streams for which QoS monitoring data is requested. |
| Measurement requirements | O | It describes the requirements on the QoS measurements. |
| > type of data | M | At least one of the following measurement index shall be provided. |
| >> packet delay | O | UL/DL/RT packet delay. For VAL stream, the packet delay means measurement from a source UE to a target UE. |
| >> packet loss rate | O | Average packet loss rate. |
| >> data rate | O | Average data rate and/or maximum data rate. |
| >> traffic volume | O | Average traffic volume for UL and/or DL |
| > measurement time period | O | The measurement time period indicates a current time period. If absent, current time and 5 minutes duration are used as default setting. |
| > aggregation granularity window | O | Indicates how long the QoS monitoring is performed to compute the average value and gather the maximum value. If absent, 1 minute is used as default setting. |
| Reporting requirements | O | It describes the requirements for QoS monitoring reporting. |
| > Frequency of reporting | O | It indicates the requested frequency of reporting. The reporting frequency may be periodic or event triggered (i.e. threshold reached). If absent in the request, default event triggered reporting is used. |
| > Reporting threshold | O | If the Frequency of reporting is event triggered (threshold reached), the reporting threshold corresponding to the measurement index shall be provided. |
| > Threshold reaching direction | O | It indicates the reaching direction (i.e. ascending, descending or crossed) for the reporting threshold. It shall be provided if reporting threshold is present. |
| > Reporting periodicity | O | If the Frequency of reporting is periodic, the reporting periodicity shall be provided. |
| > Termination of reporting | O | It indicates when the reporting of QoS monitoring shall stop. It can be event triggered (i.e. either number of reports reached or threshold reached) or when reporting duration expires or when explicit termination is requested. If absent in the request, it defaults to explicit termination. Once the termination condition is met, the monitoring subscription ceases to exist. |
| > Max number of reports | O (NOTE 3) | It indicates the maximum number of reports, it shall be provided when Termination of reporting is set to event triggered (number of reports reached). |
| > Termination threshold | O (see NOTE 2, NOTE 3) | It indicates the reporting termination threshold corresponding to the measurement index. Each measurement index has only one termination threshold. |
| > Termination condition for termination threshold | O (see NOTE 2) | It indicates threshold-based termination condition (e.g. all reached or any of them reached) if multiple measurement indexes are provided. |
| > Reporting duration | O (NOTE 3) | It indicates the reporting duration, it shall be provided when Termination of reporting is set to duration expiration. |
| NOTE 1: Only one of these information elements shall be present. NOTE 2: It shall be provided when Termination of reporting is set to event triggered (threshold reached). NOTE 3: Only one of these information elements may be present. NOTE 4: When used by the Update_Unicast_QoS_Monitoring_Subscription operation, the same information element with identical content shall be present as in the associated Subscribe_Unicast_QoS_Monitoring operation. |  |  |

* * * Next Change * * * *

#### 14.3.2.xx	Unicast QoS monitoring subscription update request

The information flow from the VAL server to the network resource management server for unicast QoS monitoring subscription update request has the same content as unicast QoS monitoring subscription request specified in clause 14.3.2.20.

* * * Next Change * * * *

#### 14.3.2.yy	Unicast QoS monitoring subscription update response

Table 14.3.2.yy-1 describes the information flow from the network resource management server to the VAL server for unicast QoS montoring subscription update response.

Table 14.3.2.yy-1: Unicast QoS monitoring subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Update status | M | It indicates the subscription update result |

* * * Next Change * * * *

##### 14.3.3.4.x	Unicast QoS monitoring subscription update procedure

###### 14.3.3.4.x.1	General

This subclause describes the high-level procedure to update unicast QoS monitoring subscription. This procedure updates the parameters to be monitored of an already existing unicast QoS monitoring subscription.

###### 14.3.3.4.x.2	Procedure

Figure 14.3.3.4.x.2-1 illustrates the high level procedure for unicast QoS monitoring subscription update.

Pre-conditions:

-	The VAL UE has an established connection in the 5GS

-	Unicast QoS monitoring subscription has successfully been invoked established for the VAL UE

Figure 14.3.3.4.x.2-1: Unicast QoS monitoring subscription update

1.	The VAL server sends a unicast QoS monitoring subscription update request to the NRM server. It shall refer to an existing subscription, identified by the list of VAL UEs, VAL UE group or VAL stream IDs included into the message.

2.	The NRM server interacts with the NEF to update the relevant QoS monitoring subscription parameters and reporting frequency as specified in step 2 of clause 14.3.3.4.1.2 of this document.

3.	The NRM server responds with a unicast QoS monitoring subscription update response indicating the subscription status.

* * * Next Change * * * *

### 14.4.1	General

Table 14.4.1-1 illustrates the SEAL APIs for configuration management.

Table 14.4.1-1: List of SEAL APIs for network resource management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NetworkResourceAdaptation | Reserve_Network_Resource | VAL server | Request /Response |
|  | Request_Unicast_Resource | VAL server | Request /Response |
|  | Update_Unicast_Resource | VAL server | Request /Response |
|  | Request_Multicast_Resource | VAL server | Request /Response |
|  | Notify_UP_Delivery_Mode | VAL server | Subscribe/Notify |
|  | TSC_Stream_Availability_Discovery | VAL server | Request /Response |
|  | TSC_Stream_Creation | VAL server | Request /Response |
|  | TSC_Stream_Deletion | VAL server | Request /Response |
| SS_EventsMonitoring | Subscribe_Monitoring_Events | VAL server | Subscribe/Notify |
|  | Notify_Monitoring_Events | VAL server |  |
| SS_NetworkResourceMonitoring | Subscribe_Unicast_QoS_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Unicast_QoS_Monitoring | VAL server |  |
|  | Unsubscribe_Unicast_QoS_Monitoring | VAL server |  |
|  | Obtain_Unicast_QoS_Monitoring_Data | VAL server | Request /Response |
|  | Update_Unicast_QoS_Monitoring_Subscription | VAL server |  |

* * * Next Change * * * *

#### 14.4.4.x	Update_Unicast_QoS_Monitoring_Subscription operation

API operation name: Update_Unicast_QoS_Monitoring_Subscription

Description: Updates the QoS monitoring subscription of a unicast resource.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.xx

Outputs: See subclause 14.3.2.yy

See subclause 14.3.3.4.x for the details of usage of this API operation.

* * * End of changes * * * *