3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221845

e-meeting, 22nd June – 1st July 2022	(revision of S6-221604)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0164 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating MCVideo private call end request information elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2022-06-22 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to support migration procedure during a private MCVideo call, and to inform the MCVideo private call participant that the other party needs to release the ongoing call due to the need to migrate to another MC system, the MCVideo private call end request message is updated to include the justification of call release, e.g., due to migration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding “private call end justification” information element to the MCVideo private call end request to indicate the reason, e.g., due to migration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MCVideo private call participant is not aware of the reason to release the MCVideo private call. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 7.2.2.2.5	MCVideo call end request

Table 7.2.2.2.5-1 describes the information flow MCVideo call end request from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.5-1: MCVideo call end request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| MCVideo call end reason | O | Indicate that the reason to end the call, e.g., due to migration, or due to user request. |

# * * * * End of changes * * * *