| 3GPP TR 23.700-05 V1.0.0 (2022-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on architecture enhancements for vehicle-mounted relays (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	9

2	References	9

3	Definitions of terms and abbreviations	10

3.1	Terms	10

3.2	Abbreviations	10

4	Architecture assumptions and requirements	10

4.1	Architecture assumptions	10

4.2	Architecture requirements	11

5	Key Issues	11

5.1	Key Issue #1: Mobile base station relay configuration support in 5GC	11

5.1.1	Description	11

5.2	Key Issue #2: Efficient mobility for UEs connecting to/disconnecting from mobile base station relay	12

5.2.1	Description	12

5.3	Key Issue #3: Efficient mobility and service continuity when served by mobile base station relay	12

5.3.1	General description	12

5.4	Key Issue #4: Support of roaming of mobile base station relays	13

5.5	Key Issue #5: Support of location services for UEs accessing via a mobile base station relay	13

5.6	Key Issue #6: Provide cell ID/TAC of mobile base station relay for services	14

5.7	Key Issue #7: Control of UE's access to 5GS via a mobile base station relay	14

6	Solutions	14

6.0	Mapping of solutions to key issues	14

6.1	Solution #1: Subscription data and O&M based mobile IAB-node configuration	15

6.1.1	Introduction	15

6.1.2	Functional Description	15

6.1.3	Procedures	15

6.1.4	Impacts on services, entities, and interfaces	16

6.2	Solution #2: Solution for provisioning and policy control of mobile base station relays	16

6.2.1	General	16

6.2.2	Functional descriptions	16

6.2.2.1	Policy provisioning architecture	16

6.2.2.2	Vehicle Mounted Relay Policy contents	17

6.2.3	Procedures	17

6.2.4	Impacts on services, entities, and interfaces	18

6.3	Solution #3: Reusing existing procedures for UE mobility	18

6.3.1	General	18

6.3.2	Functional descriptions	18

6.3.3	Procedures	18

6.3.3.1	UE mobility between mobile base station relays	18

6.3.3.2	UE mobility between mobile base station relay and macro base station	19

6.3.4	Impacts on services, entities, and interfaces	19

6.4	Solution #4: Stop mobility registration when moving with mobile base station relay	19

6.4.1	General	19

6.4.2	Functional descriptions	20

6.4.3	Procedures	21

6.4.3.1	UE performs Service Request or Registration Request via Mobile Base Station Relay	21

6.4.3.2	Mobile Base Station Relay mobility registration procedure with UEs	23

6.4.3.3	Mobile Base Station Relay handover with UEs	24

6.4.3.3.1	General	24

6.4.3.3.2	N2 handover	24

6.4.3.3.3	Xn handover	25

6.4.4	Impacts on services, entities and interfaces.	26

6.5	Solution #5: Mobile termination procedure due to mobility	26

6.5.1	General	26

6.5.2	Functional descriptions	26

6.5.3	Procedures	27

6.5.3.1	UE performs Service Request or Registration Request via Mobile Base Station Relay	27

6.5.3.2	UE handover to connect/disconnect to/from Mobile Base Station Relay with AMF change	29

6.5.3.2.0	General	29

6.5.3.2.1	N2 handover	30

6.5.3.2.2	Xn handover	31

6.5.3.3	Mobile Base Station Relay mobility registration procedure with AMF change	31

6.5.3.4	Mobile Base Station Relay handover with UEs with AMF change	32

6.5.3.5	Mobile termination when UE moves with Mobile Base Station Relay	34

6.5.4	Impacts on services, entities and interfaces.	35

6.6	Solution #6: Enabling roaming of mobile base station relays	35

6.6.1	General	35

6.6.2	Functional description	35

6.6.3	Signalling flows	37

6.6.3.1	Signalling flow for roaming of mobile base station relay using IAB node integration procedure	37

6.6.3.2	Signalling flow for roaming of mobile base station relay using inter-IAB-donor gNB mobility procedure	38

6.6.4	Impacts on services, entities and interfaces	38

6.7	Solution #7: Network based Positioning of mobile base station relay for location services	39

6.7.1	General	39

6.7.2	Functional descriptions	40

6.7.3	Procedures	40

6.7.3.0	General	40

6.7.3.1	MT_LR for UE served by a mobile base station relay	40

6.7.4	Impacts on services, entities, and interfaces	43

6.8	Solution #8: Support of positioning for UE accessing via mobile base station relay	43

6.8.1	General	43

6.8.2	Functional Descriptions	44

6.8.3	Positioning procedure for UE accessing via mobile base station relay	44

6.8.4	Impacts on services, entities, and interfaces	46

6.9	Solution #9: User location information based on IAB-UE	46

6.9.1	Introduction	46

6.9.2	Functional Description	46

6.9.3	Procedures	46

6.9.3.1	User Location Information on NGAP	46

6.9.4	Impacts on services, entities and interfaces	47

6.10	Solution #10: configuration of Mobile Base Station Relays in roaming	47

6.10.1	Introduction	47

6.10.2	Functional Description	47

6.10.3	Procedures	49

6.10.4	Impacts on services, entities and interfaces	49

6.11	Solution #11: Service time of Mobile Base Station Relay	50

6.11.1	General	50

6.11.2	Functional descriptions	50

6.11.3	Procedures	50

6.11.3.1	Mobile Base Station Relay broadcast service time	50

6.11.3.2	Conditional handover according to service time of Mobile Base Station Relay	51

6.11.4	Impacts on services, entities and interfaces.	51

6.12	Solution #12: IAB-node mobility with connected UEs	52

6.12.1	General	52

6.12.2	Functional descriptions	52

6.12.3	Procedures	52

6.12.3.1	IAB-node mobility with change of IAB-donor gNB via Xn	52

6.12.3.2	IAB-node mobility with change of IAB-donor gNB via N2	54

6.12.4	Impacts on services, entities, and interfaces	57

6.13	Solution #13: IAB optimization for Mobile Base Station Relays support	57

6.13.1	Introduction	57

6.13.2	Functional Description	57

6.13.3	Procedures	59

6.13.4	Impacts on services, entities and interfaces	61

6.14	Solution #14: Support of location services for UEs accessing via a mobile IAB node	61

6.14.1	Introduction	61

6.14.2	Functional Description	61

6.14.3	Procedures	62

6.14.4	Impacts on services, entities and interfaces	63

6.15	Solution #15: Support of location services for MBSR access with provision of MBSR Location and RAT dependent positioning	63

6.15.1	Introduction	63

6.15.2	Functional Description	64

6.15.3	Positioning Procedure	64

6.15.4	Impacts on services, entities, and interfaces	67

6.16	Solution #16: UE mobility management in case of IAB-node mobility with dynamic TAC provisioning	67

6.16.1	Introduction	67

6.16.2	Functional Description	68

6.16.3	Procedures	69

6.16.4	Impacts on services, entities, and interfaces	69

6.17	Solution #17: IAB-node mobility with dedicated TAC	70

6.17.1	Introduction	70

6.17.2	Functional Description	70

6.17.2.1	TAC broadcasted by the MBSR is unchanged during mobility	70

6.17.2.2	Using same TAC within a certain service area	72

6.17.3	Procedures	74

6.17.3.1	Procedures if the TAC broadcasted by the MBSR is unchanged during mobility	74

6.17.3.1.1	Procedure of intra UE-AMF mobility	74

6.17.3.1.2	Procedure of inter UE-AMF mobility	76

6.17.3.2	Procedures using same TAC within a certain service area	77

6.17.4	Impacts on services, entities, and interfaces	77

6.18	Solution #18: Solution for UE location service via mobile IAB-node	78

6.18.1	General	78

6.18.2	Functional descriptions	78

6.18.3	Procedures	78

6.18.3.1	Location service for UE served by a mobile IAB-node	78

6.18.4	Impacts on services, entities, and interfaces	79

6.19	Solution #19: Solution for location privacy check for Mobile Base Station Relay	80

6.19.1	General	80

6.19.2	Functional descriptions	80

6.19.3	Procedures	81

6.19.4	Impacts on services, entities, and interfaces	81

6.20	Solution #20: CAG based UE access control via MBSR	81

6.20.1	Introduction	81

6.20.2	Functional Description	81

6.20.3	Procedures	82

6.20.3.1	General	82

6.20.4	Impacts on services, entities and interfaces	82

7	Evaluation	83

7.1	Evaluations for KI#1	83

7.2	Evaluations for KI#2	83

7.3	Evaluations for KI#3	83

7.4	Evaluations for KI#4	84

7.5	Evaluations for KI#5	85

7.6	Evaluations for KI#6	86

8	Conclusions	87

8.1	Conclusions for KI#1	87

8.2	Conclusions for KI#2	87

8.3	Conclusions for KI#3	88

8.4	Conclusions for KI#4	88

8.5	Conclusions for KI#5	88

8.6	Conclusions for KI#6	89

Annex A: Change history	90

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The scope of this Technical Report is to study and identify potential architecture and system level enhancements for the 5G system to support the operation of base station relays mounted on vehicles, using NR for wireless access toward the UE and for wireless access through an IAB-donor toward the 5GC, i.e. in this release only IAB type relays based on the IAB architecture and functionalities specified in TS 23.501 [2] are studied. The study addresses the service requirements documented in TS 22.261 [3] for the mobile base station relays, and focuses on the following aspects:

-	efficient mobility and service continuity for UE or a group of UEs to efficiently deliver data during different mobility scenarios (including mobility of the mobile base station relays);

-	provisioning, policies and mechanisms to e.g. manage relay configuration, geographic restrictions, QoS, authorization and control of UEs' access via the mobile base station relay, etc.;

-	support for roaming of mobile base station relay (including the roaming of the IAB-UE and gNB-DU in IAB-node), support for regulatory requirements (e.g. emergency, priority services, public safety), and support for location services for UEs accessing a mobile base station relay.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[6]	3GPP TS 38.401: "NG-RAN; Architecture description".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[8]	3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[9]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[10]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[11]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[12]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[13]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[14]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[15]	3GPP TR 23.700-86: "Study on Architecture Enhancement to support Ranging based services and sidelink positioning".

[16]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[17]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[18]	3GPP TS 38.473: "F1 application protocol (F1AP)".

[19]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[20]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[21]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode ".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

mobile base station relay: A mobile base station acts as a relay between a UE and the 5G network, i.e. providing a NR access link to UEs and connected wirelessly (using NR) through a IAB-donor to the 5G Core. Such mobile base station relay is assumed to be mounted on a moving vehicle and serve UEs that can be located inside or outside the vehicle (or entering/leaving the vehicle).

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

MBSR	Mobile Base Station Relay

TRP	Transmission-Reception Point

VMR	Vehicle Mounted Relay

# 4	Architecture assumptions and requirements

## 4.1	Architecture assumptions

The study should be based on the following architecture assumptions:

-	the mobile base station relays uses the IAB architecture as defined in clause 5.35 of TS 23.501 [2];

NOTE 1:	Any other alternative of base station relay architecture depends on the RAN study output if any and should be discussed with relevant RAN WGs.

NOTE 2:	The mobile base station relay (i.e. IAB-node) is not applicable to NR satellite access in this release.

-	the mobile base station relay has a single hop to the IAB-donor node;

-	the mobile base station relay may serve UEs located inside or outside the vehicle mounted with the relay;

-	NR Uu is used for the radio link between a mobile base station relay and served UEs, and between mobile base station relay and IAB-donor node. However, optimization solutions can also consider combining other functionalities, e.g. using PC5 for discovery and mobility assistance/control amongst relays;

-	LCS framework as defined in TS 23.273 [4] is used for providing the location service to the served UEs;

-	the mobile base station may connect to an IAB-donor node of a PLMN or an SNPN.

## 4.2	Architecture requirements

Solutions of the study should provide architecture and system level enhancements to the 5G system to support the operation of base station relays to satisfy the normative requirements in TS 22.261 [3]. Specifically, the enhanced system needs to:

-	support legacy UE(s) to connect via base station relays, but can consider optimizations involving UE changes;

-	support end-to-end service continuity, in the presence of mobile base station relays, including when the mobile base station relays change the IAB-donor nodes;

-	support the mobile network operator to configure, provision and control the operation of a mobile base station relay;

NOTE 1:	Charging support will be coordinated with SA5, and if needed later handled as alignment work in SA WG2.

-	support of regulatory requirements (e.g. for support of emergency services, priority services) when UEs access 5GS via a mobile base station relay;

-	support roaming of the mobile base station relay from its HPLMN into a VPLMN.

NOTE 2:  In this Release, NR satellite access is not applicable.

# 5	Key Issues

## 5.1	Key Issue #1: Mobile base station relay configuration support in 5GC

### 5.1.1	Description

Mobile base station relays (i.e. IAB-node) may attach or detach to a mobile network depending on its location and movement. When a mobile base station relay just attaches to the network, it may lack the authorization policy and configuration data to serve as a mobile relay. Therefore dynamic provisioning mechanism is needed to configure the mobile base station relay in such cases.

This key issue should investigate the enhancements needed in 5GC for the configuration of the mobile base station relay to be operational and for the access management of the mobile based station relay.

The following aspects shall be studied:

-	What authorization policies and configuration data are needed for the mobile base station relay's operation.

-	Whether and what is needed in 5GC to support the configuration of a mobile base station relay, besides the existing configuration methods for IAB-node.

-	Whether and what is needed in 5GC for managing mobile base station relay's access to 5GS, besides the existing IAB-UE management.

NOTE:	Aspects of RAN specific configuration need to be coordinated with RAN WGs.

## 5.2	Key Issue #2: Efficient mobility for UEs connecting to/disconnecting from mobile base station relay

### 5.2.1	Description

During the mobility of the mobile base station relay (i.e. IAB-node), a UE or a group of UEs served by the mobile base station relay will move together with the mobile base station relay, and when the mobile base station relay is, e.g. out of service area, or out of service time, the group of UEs will be served by macro base station or another mobile base station relay nearby.

In addition, when a UE is connected to a macro base station and the vehicle is moving to an area in the vicinity of the UE, the surrounding UEs (i.e. outside the vehicle) may profit of a better communication link through the mobile base station relay requiring efficient mobility from the macro base station to the mobile base station relay and back to the macro base station.

Hence, following aspects need to be studied for UEs moving to/from mobile base station relay:

-	How to support efficient mobility for a UE or a group of UEs between mobile base station relays.

-	How to support efficient mobility for a UE or a group of UEs between mobile base station relay and macro base stations.

NOTE:	This KI has RAN dependency and should align with the progress of RAN WGs.

## 5.3	Key Issue #3: Efficient mobility and service continuity when served by mobile base station relay

### 5.3.1	General description

When the moving vehicles are equipped with mobile base station relays (i.e. IAB-node), the mobile base station relays can provide 5G coverage and communication to UEs (inside the vehicle and/or in its vicinity), and connected wirelessly to the 5G network via IAB-donor gNB. When one or a group of UEs are already served by the mobile base station relay, there are two mobility scenarios to be studied as the following:

-	Scenario A (mobility within the same IAB-donor gNB): When the UEs are continuously served by a mobile base station relay (e.g. inside the vehicle and/or in its vicinity), this mobile base station relay within the vehicle is moving around within a limited geographical area while keeping connecting with the same IAB-donor gNB. In this case, the UE keeps the connection with the same mobile base station relay (i.e. IAB-node), and there is no change of the IAB-donor gNB as in figure 5.3.1-1.

-	Scenario B (mobility between different IAB-donor gNBs): When the UEs are continuously served by a mobile base station relay (e.g. inside the vehicle and/or in its vicinity), this mobile base station relay within the vehicle is moving around over a long distance. The mobile base station relay node connects with a different IAB-donor gNB if the vehicle keeps moving. In this case, the UE keeps the connection with the same mobile base station relay (i.e. IAB-node), but there is change of the IAB-donor gNB.

NOTE 1:	For the above scenarios, whether the cell information in the System Information Broadcast (e.g. Cell ID, TAC) changes has RAN dependency.

NOTE 2:	For the above scenarios, whether UE needs to perform the legacy handover procedures has RAN dependency.

Figure 5.3.1-1: Scenarios for efficient mobility and service continuity

The following aspects need to be studied for UEs served by the mobile base station relay in the case of mobility within the same IAB-donor gNB and mobility between different IAB-donor gNBs:

-	Whether and how to enhance current procedures of mobility and service continuity for a UE or a group of UEs, to efficiently deliver the data. Following aspects need to be considered in potential solutions:

-	If the TAC in the System Information Broadcast changes, whether and how to enhance the NAS mobility procedure.

-	If the handover is needed due to cell changes, whether and how to enhance the mobility for group UEs (e.g. during handover).

NOTE 3:	Mechanisms related to mobility management and service continuity have RAN dependency and should align with the progress of RAN WGs.

## 5.4	Key Issue #4: Support of roaming of mobile base station relays

Based on the requirements of TS 22.261 [3], the 5G system shall support the mobile base station relay (i.e. IAB-node) having a subscription with a HPLMN and roaming from its HPLMN into a VPLMN.

When using the IAB architecture, a mobile base station relay (i.e. IAB-node) consists of a gNB-DU and an IAB-UE, as defined in clause 5.35 of TS 23.501 [2]. For roaming, the IAB-UE behaves as a UE, and thus may be able to access a VPLMN reusing UE procedures. However, in order to support the gNB-DU component of the mobile base station to operate properly in the VPLMN, additional support may need to be defined. This key issue addresses the following aspects of the mobile base station relay when it connects to a VPLMN:

-	Study the PLMN (i.e. HPLMN or VPLMN) to be presented to UE by mobile base station relay in case of roaming. And how to support access and communication from/to UEs via mobile base station relays.

-	Whether and how to support the controlling of a mobile base station relay, e.g. enable/disable mobile relay operation if the relay is roaming in a VPLMN.

NOTE:	Coordination between the RAN WGs and SA WG2 is needed.

## 5.5	Key Issue #5: Support of location services for UEs accessing via a mobile base station relay

Based on the requirements of TS 22.261 [3], the 5G system shall be able to support location services for the UEs accessing 5GS via a mobile base station relay. However, when the mobile base station relay (i.e. IAB-node) is present, the location of the cell/IAB-node serving the UEs may change, and this may affect not only positioning procedures but also regulatory services needs UE location. Therefore, this key issue needs to address:

-	How to support location services for the UEs served by a mobile base station relay that moves (with or without changing IAB-donor gNBs); or roams to VPLMN;

NOTE:	For this key issue, this study should not seek to change the LCS framework, and the coordination with the study on FS_eLCS_PH3 may be needed.

## 5.6	Key Issue #6: Provide cell ID/TAC of mobile base station relay for services

When the mobile base station relay (i.e. IAB-node) is present, the physical location of the cell/IAB-node serving the UEs may change even without changing its IAB-donor node. Therefore, the serving cell ID/TAC of the UE may not always accurately reflect the location of the UE. This may affect services, e.g. regulatory services, tariff notifications, and etc. that rely on cell ID/TAC as location reference. Therefore, this key issue needs to address:

-	How to provide appropriate cell ID/TACs information (and possible its corresponding geographic area) of mobile base station relay that moves when such cell ID/TACs are needed, e.g. to assist route an emergency call to the correct PSAP, to assist to handle Lawful Interception, to assist the Public Warning System, etc.

NOTE 1:	For mobile base station relay that moves, whether its cell information in the System Information Broadcast (e.g. Cell ID, TAC) changes or not due to its movement has RAN dependency.

NOTE 2:	For mobile base station relay that moves, even the cell information in the System Information Broadcast (e.g. Cell ID, TAC) does not change, it may represents different geographic area due to the movement.

## 5.7	Key Issue #7: Control of UE's access to 5GS via a mobile base station relay

Based on the requirements in TS 22.261 [3], it shall be possible to support provisioning and configuration mechanisms to control UEs' selection of and access to a mobile base station relay (i.e. IAB-node), e.g. based on geographic or temporary restrictions, etc.

This key issue is to address those requirements, and potential solutions providing granular and efficient control of both IDLE or CONNECTED mode UE access to MBSR e.g. during the connection or mobility of the UE. In particular, the following aspects should be addressed:

-	Whether and how to manage access control for the UE's access to IAB-node;

-	whether and how to manage the UE's access (e.g. permitted/restricted) to IAB-node for specific geographical location(s), time window(s), or other conditions/criteria;

-	whether and how to improve the control of new UEs.

NOTE 1:	Support of legacy UE(s) shall be considered.

NOTE 2:	Aspects related to RAN need to be coordinated with RAN WGs.

# 6	Solutions

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | x |  |  |  |  |  |  |
| 2 | x |  |  |  |  |  |  |
| 3 |  | x |  |  |  |  |  |
| 4 |  | x | x |  |  |  |  |
| 5 |  | x | x |  |  |  |  |
| 6 |  |  |  | x |  |  |  |
| 7 |  |  |  |  | x |  |  |
| 8 |  |  |  |  | x |  |  |
| 9 |  |  |  |  |  | x |  |
| 10 | x |  |  | x |  |  |  |
| 11 |  | x |  |  |  |  |  |
| 12 |  |  | x |  |  |  |  |
| 13 |  |  | x |  |  |  |  |
| 14 |  |  |  |  | x |  |  |
| 15 |  |  |  |  | x |  |  |
| 16 |  |  | x |  |  |  |  |
| 17 |  |  | x |  |  |  |  |
| 18 |  |  |  |  | x |  |  |
| 19 |  |  |  |  | x |  |  |
| 20 |  |  |  |  |  |  | x |

## 6.1	Solution #1: Subscription data and O&M based mobile IAB-node configuration

### 6.1.1	Introduction

The solution provides methods for mobile IAB-node configuration and 5GC access control based on O&M and subscription. This includes roaming scenario.

### 6.1.2	Functional Description

The solution is based on the support of existing IAB architecture as specified in TS 23.501 [2] with the following high-level description:

-	For the IAB-node connects to 5GS, the access control and mobility handling is in 5GC based on subscription data in the network for the IAB-UE (i.e. IAB-MT). In case of roaming, the local policy in serving AMF may further enforce it based on mobility restriction functionality.

-	For the IAB-node configuration, it is supported by the O&M using either backhaul IP layer by implementation or using IP provided by the IAB-UE's (or IAB-MT's) PDU session via 5G network as described in TS 38.401 [6]. In the case of roaming, it's the serving network O&M provides the configuration information to the IAB-node.

NOTE:	Coordination with RAN WGs is needed to decide if there is extra configuration support needed for IAB-node due to mobility.

Editor's note:	It is FFS if extra O&M procedures impact is needed.

### 6.1.3	Procedures

The registration request procedure in clause 4.2 of TS 23.502 [5] in Rel-17 can be used by the IAB-UE to access the 5GS.

Optionally the PDU session establishment procedure in clause 4.3 of TS 23.502 [5] may be used to provide configuration information. In case of roaming, the serving network provides the configuration information to the IAB-node.

### 6.1.4	Impacts on services, entities, and interfaces

AMF:

-	None.

gNB:

-	None.

## 6.2	Solution #2: Solution for provisioning and policy control of mobile base station relays

### 6.2.1	General

This solution addresses Key Issue#1, as defined in clause 5.1.

The UE Policy control and provisioning framework as defined in TS 23.503 [7] is used to support the provisioning and policy control for the mobile base station relays. Similar to the V2XP and ProSeP handling, a Vehicle Mounted Relay Policy (VMRP) can be delivered to the IAB-UE component of the mobile base station relay from PCF via N1 signalling.

The VMRP will be used by the mobile base station relay to govern it operation to satisfy the service requirements, e.g. when to activate/deactivate the relay based on location or time.

### 6.2.2	Functional descriptions

#### 6.2.2.1	Policy provisioning architecture

Figure 6.2.2-1 shows a reference architecture for the mobile base station relay policy provisioning control.

Figure 6.2.2-1: Policy provisioning framework for mobile base station relays

When the mobile base station relay is owned by the mobile network operator, the relay obtains the policies stored in the UDR via the PCF, similar to the case for URSP.

In certain deployment cases, the mobile base station relay may be owned by a third party, e.g. a fleet operator that has a service agreement with the PLMN, the relay may be controlled by the VMR Application Server. In this case, some additional policy may be provided by the VMR Application Server via the V1' reference point or via NEF-UDR-PCF.

The V1' reference point is similar to the V1 reference point as defined in TS 23.287 [11].

Editor's note:	It is FFS for the case that mobile base station relay is owned/controlled by a third party and connect to PLMN network.

#### 6.2.2.2	Vehicle Mounted Relay Policy contents

To satisfy the service requirements as defined in clause 5.2, the VMRP may contain the following elements:

-	Operating restrictions:

-	Geographic restrictions:

-	Relay operation can be allowed within a set of geographic areas / disallowed within a set of geographic areas.

NOTE 1:	Geographic area format used for V2X policy in TS 24.588 [8] can be re-used. A geographic area is encoded as a list of coordinates, each corresponding to a summit of a polygon.

Editor's note:	It is FFS how the IAB-UE determines the geographic areas that IAB-UE is located during mobility.

-	Subscription-based restrictions:

-	List of PLMN IDs where IABoperation can be allowed.

Editor's note:	How IAB-UE performs PLMN selection using the list of PLMN IDs where IAB operation can be allowed is FFS.

-	Time-of-day-based restrictions:

-	time window when relay operation can be allowed /disallowed.

NOTE 2:	This time window allows operator to have relays provide supplemental capacity only during peak hours.

-	Speed-based restrictions:

-	Speed thresholds for when relay operation can be allowed/disallowed.

NOTE 3:	Speed limit can be set to zero, in which case the relay can only operate when the vehicle is not moving.

Editor's note:	It is FFS if the Speed-based restrictions can be used only in certain cases, e.g. MBSR can reliably identify its own speed.

-	PDU session parameters, e.g. specific DNN & S-NSSAI to use for the PDU session established toward the OAM server for relay configurations.

Editor's note:	It is FFS that whether the other parameters can be contained in VMRP.

Editor's note:	It is FFS if some of the police and parameters should be provided by OAM.

### 6.2.3	Procedures

The procedures for service authorization and provisioning as defined in clause 6.2 of TS 23.304 [9] can be reused for the provisioning of VMRP to the mobile base station relay.

In order to support the operation, a new UE Policy part type for VMRP needs to be defined in clause D.6.2.7 of TS 24.501 [10], besides URSP, ANDSP, V2XP, and ProSeP.

Editor's note:	After the VMRP is provisioned to the mobile base station relays, it is FFS how the network enforces the operation of mobile base station relays, such as geographic areas and speed.

### 6.2.4	Impacts on services, entities, and interfaces

Impacts on existing entities:

PCF: Support the provisioning of VMRP.

Functions required at new entities:

Mobile base station relay: support a new type of UE Policy (i.e. VMRP) provisioning procedures.

## 6.3	Solution #3: Reusing existing procedures for UE mobility

### 6.3.1	General

This is a solution to Key Issue#2, which addresses the following:

-	How to support efficient mobility for a UE between mobile base station relays.

-	How to support efficient mobility for a UE between mobile base station relay and macro base station.

### 6.3.2	Functional descriptions

The IAB architecture for 5GS as defined in clause 5.35 of TS 23.501 [2] is reused. The gNB-DU in the IAB-node is responsible for providing NR Uu access to UEs and child IAB-nodes. The corresponding gNB-CU function resides on the IAB-donor gNB, which controls IAB-node gNB-DU via the F1 interface. IAB-node appears as a normal gNB to UEs and other IAB-nodes and allows them to connect to the 5GC.

Figure 6.3.2-1: IAB architecture for 5GS

### 6.3.3	Procedures

#### 6.3.3.1	UE mobility between mobile base station relays

In case of UE mobility between mobile base station relays (i.e. from one mobile base station relay to another mobile base station relay), e.g. due to the old mobile base station relay out of service, the following applies:

-	If the mobile base station relays connect to the same macro base station (i.e. same IAB-donor node), the legacy procedure of Inter-gNB-DU Mobility as defined in the TS 38.401 [6] is used.

-	If the mobile base station relays connect to different macro base stations (i.e. different IAB-donor nodes), the legacy handover procedure using the Xn or N2 reference points as defined in the TS 23.502 [5] is used.

Editor's note:	The details of UE mobility between mobile base station relays will be coordinated with RAN WGs.

#### 6.3.3.2	UE mobility between mobile base station relay and macro base station

In case of UE mobility between mobile base station relay and macro base station, e.g. due to mobile base station relay out of service, the following applies:

-	If this mobile base station relay's IAB-donor node is this macro base station, the legacy procedure of Inter-gNB-DU Mobility as defined in the TS 38.401 [6] is used.

-	If this mobile base station relay's IAB-donor node is another macro base station, the legacy handover procedure using the Xn or N2 reference points as defined in the TS 23.502 [5] is used.

Editor's note:	The details of UE mobility between mobile base station relay and macro base station will be coordinated with RAN WGs.

### 6.3.4	Impacts on services, entities, and interfaces

There is no impacts identified in this solution.

Editor's note:	The impacts will be coordinated with RAN WGs, if any.

## 6.4	Solution #4: Stop mobility registration when moving with mobile base station relay

### 6.4.1	General

This solution addresses Key Issue #2 and Key Issue #3 for mobility.

This solution assumes that the TACs allowed to be broadcasted by the Mobile Base Station Relay (MBSR) are pre-configured, and the gNB-CU is able to instruct the MBSR to broadcast one of them, which can be changed when the gNB-CU changed or determined by the gNB-CU. The pre-configuration of TACs for a MBSR is able to be obtained by the AMF serving the UE if the UE camps on the MBSR, and the Registration Area (RA) configured for the UE includes those TACs so that the UE will not perform mobility registration.

Following figure shows an example of the moving scenario:

Figure 6.4.1-1: Example of UE stopping mobility registration when moving with MBSR

1.	The UE is served by AMF1, and the RA configured in UE does not include any TAC that is allowed to be broadcasted by any MBSR.

2.	The UE onboards a vehicle equipped with a MBSR, and the MBSR is serving by AMF2. The MBSR is pre-configured with allowed TAC list that includes TAC1, TAC2, and TAC3, which is not included in the current RA of the UE, and the NG-RAN selects TAC1 for the MBSR to broadcast. The UE performs mobility registration via the MBSR due to RA change, and the AMF1 provides new RA to the UE, which includes TAC1, TAC2, and TAC3, so that the UE will not perform mobility registration again until the UE leaves the MBSR.

3.	The MBSR moves together with the UE, and the MBSR performs mobility registration and is instructed to broadcast TAC2. The serving NG-RAN decides to change the serving AMF for the UEs served by AMF1, e.g. NG-RAN does not have connection with the AMF1, and the serving NG-RAN sends context transfer request to AMF3 in order to transfer context for bulk UEs under the MBSR from AMF1.  The AMF3 requests UE Context transfer from AMF1 without temporary UE ID (5G-GUTI) change, i.e. no NAS signalling. After the UE Context transfer, the NG-RAN is able to send message to the AMF3 for the UEs. The AMF3 invokes Nudm_UECM_Registration with "move together" indication to UDM, the UDM will suspend the deregistration notification to the AMF1.

4.	The MBSR continues to move and performs mobility registration, and the serving AMF for the MBSR is changed to AMF4. The NG-RAN selects TAC3 for the MBSR to broadcast (the NG-RAN may also select TAC1 or TAC2).

5.	The UE gets off the vehicle, camps on a macro cell, and the TAC broadcasted by the cell is not in the current RA of the UE, so the UE performs mobility registration, which changes the serving AMF to AMF5. The AMF5 requests UE context from AMF1 according to the 5G-GUTI and invokes Nudm_UECM_Registration without "move together" indication, and UDM will deregister the AMF3 and resume the deregistration notification to the AMF1.

### 6.4.2	Functional descriptions

In this solution, the AMF serving the UE manages the association between the UE and the MBSR. The MBSR may be managed by a different AMF, so the AMF serving the UE allocates a Link ID to link to the MBSR and associates the UE with the Link ID. If multiple UEs under a MBSR are served by the same AMF, those UEs are associated with the same Link ID. The Link ID consists of AMF ID of the AMF that allocates it and a unique ID within the AMF.

Considering that the NG-RAN does not have the UE context when the UE is in CM-IDLE state, the NG-RAN is provided with the Link IDs from AMFs serving UEs and the NG-RAN manages the association between the MBSR and the Link IDs. When a MBSR moves, the NG-RAN may trigger the UE context transfer for the UEs under the MBSR according to the AMF ID in the Link ID (e.g. the NG-RAN does not have a connection with the AMF), i.e. the NG-RAN sends the Link ID to the new AMF serving the UEs, and the new AMF serving the UEs requests bulk UE context transfer from the AMF indicated by the AMF ID in the Link ID.

The following figure shows the data model for the mobility management:

Figure 6.4.2-1: Data model for mobility management

Each UE is served by an AMF (AMF serving UE, A.K.A. AMF-UE), which holds the UE Context. When the UE camps on a MBSR, the UE Context will include a Link ID linking to the MBSR that is dynamically assigned by the AMF. The Link ID includes AMF ID indicating the AMF assigning the Link ID, and includes a unique number assigned by the AMF indicated by the AMF ID. The UEs served by the same AMF and camps on the same MBSR will be assigned with the same Link ID. The 5G-GUTI included in the UE Context may be assigned by another AMF-UE if the UE does not perform mobility registration when moving with the MBSR (see clauses 6.4.3.1, 6.4.3.2 and 6.4.3.3).

The NG-RAN serving a MBSR holds the MBSR Context that includes zero or more Link IDs so that even a UE under the MBSR is in CM-IDLE state, the NG-RAN is able to trigger UE context transfer during MBSR mobility (see clauses 6.4.3.1 and 6.4.3.2). If a UE is in CM-CONNECTED state and is relayed by a MBSR, the UE context in the NG-RAN also includes a Link ID associated with the MBSR, so that when the UE performs handover (move together with MBSR or not), even the serving AMF for the UE is changed while 5G-GUTI of the UE is unchanged, the NG-RAN is able to know the serving AMF for the UE (see clause 6.4.3.3), and if multiple UEs relayed by the MBSR are served by the same AMF, they will associate the same Link ID even if the AMF IDs in the UE 5G-GUTIs are different, so that when the handovers of those UEs are triggered by the moving of the MBSR (move together), the NG-RAN is able to send bunch of handover requests for those UEs to the AMF (see clauses 6.4.3.3.2 and 6.4.3.3.3).

### 6.4.3	Procedures

#### 6.4.3.1	UE performs Service Request or Registration Request via Mobile Base Station Relay

As described in clause 6.4.1, the UE will keep silent as possible as it can when moving with MBSR, i.e. no mobility registration. As described in step 3 of clause 6.4.1, the AMF serving the UE may need to be changed later without NAS signalling exchange.

The Link ID is introduced to map to the MBSR in the procedure, which does not map to the UE, and is sent to NG-RAN via N2 message by the AMF serving the UE, and the Link ID includes the AMF ID of the AMF serving the UE, so that when the AMF serving the needs to be changed during MBRS mobility (step 3 of clause 6.4.1), even the UE is in CM-IDLE state, the NG-RAN can use the Link ID to ask the new AMF to interact with the old serving AMF indicated by the AMF ID in the Link ID to transfer the UE context (described in clause 6.4.3.2.

Figure 6.4.3.1-1: UE performs Service Request or Registration Request via Mobile Base Station Relay

1.	The UE camps on a MBSR.

2.	The UE initiates Service Request or Registration Request towards 5GC. The MBSR forwards the N1 message to the NG-RAN.

3.	The NG-RAN forwards the N1 message and includes the MBSR ID, e.g. 5G-GUTI of the MBSR, and may include the TAI list configured for the MBSR in the underlayer N2 message. In the case of registration procedure, the NG-RAN may select a new AMF-UE.

4.	In case new AMF-UE is selected, the AMF-UE requests UE Context transfer from the old AMF-UE. The new AMF-UE invokes Nudm_UECM_Registration to the UDM, the UDM resumes the deregistration notification that was suspended in step 6 of clause 6.4.3.2.

5.	The new AMF-UE sends a N2 message with the Link ID obtained in step 5, the N2 message may include a N1 message. In case of registration procedure, if the TAI list configured for the MBSR does not change, for the sake of saving mobility registration for UEs behind the MBSR, the AMF-UE may send Registration Accept to the UE with TAI List that only includes the TAI list received in step 3. The NG-RAN associates the Link ID with the MBSR and stores the Link ID into the UE context.

6.	The NG-RAN associates the Link ID with the MBSR ID and the UE ID. The NG-RAN sends an RRC message to the UE, the RRC message may include the N1 message. The usage of Link ID is referred to clauses 6.4.3.2, and 6.4.3.3.

7.	If the UE camps on a MBSR, the UE may suspend the mobile registration if the UE keeps camping on the MBSR (e.g. the TAI/TAC broadcasted by the MBSR does not change and the NG-RAN sends the TAI to the AMF-UE).

8.	The UE camps on another MBSR or NG-RAN.

9.	When the mobile registration is suspended, the UE resumes the mobile registration (e.g. the TAI/TAC broadcasted by a MBSR is not provisioned to the UE before the UE camps on the MBSR and when UE camps on a MBSR, only the TAI/TAC of the MBSR is provisioned to the UE). If the UE camps on another MBSR, the steps 2-9 are performed with a different Link ID, different MBSR ID, and a possibly different AMF-MBSR, if the UE camps on NG-RAN directly, the registration procedure described in clause 4.2.2 of TS 23.502 [5] is performed, and when Nudm_UECM_Registration without "move together" indication is invoked by a new AMF-UE, the UDM resumes the deregistration notification that was suspended in step 6 of clause 6.4.3.2.

#### 6.4.3.2	Mobile Base Station Relay mobility registration procedure with UEs

As described in clause 6.4.1, the UE will keep silent as possible as it can when moving with MBSR, i.e. no mobility registration. As described in step 3 of clause 6.4.1, the AMF serving the UE may need to be changed later without NAS signalling exchange.

According to procedure in clause 6.4.3.1, during the mobility of MBSR, even when the UE is in CM-IDLE state, the NG-RAN is able to determine whether the AMF serving the UE needs to be changed according to the Link ID associated with the MBSR. The NG-RAN sends the Link ID to the new AMF serving the UE and the new AMF sends the Link ID to the old AMF serving the UE for requesting bulk UE context transfer. The old AMF serving the UE stores the UE context with the Link ID, so bulk UE context transfer can be done.

Figure 6.4.3.2-1: Mobile Base Station Relay mobility registration procedure with UEs

1.	The MBSR sends Registration Request towards the 5GC via the NG-RAN, or the MBSR performs handover procedure.

2.	In case Registration procedure is performed, the NG-RAN may select an AMF-MBSR different from the old AMF-MBSR, the AMF-MBSR queries UE Context for the MBSR from the old AMF-MBSR and the old AMF-MBSR returns MBSR Context with the Link IDs (see clause 6.5.3.1 for how AMF-MBSR knows the Link IDs).

3.	In case Registration procedure is performed, the AMF-MBSR sends Registration Accept to the MBSR, whose underlayer N2 message includes the Link IDs, the NG-RAN updates the Link IDs for the MBSR.

4.	The (target) NG-RAN decides whether to change the AMF serving the UEs associated with a Link ID according to the Link ID.

NOTE 1:	If the UE decides to change slice requirement, the UE will initiate mobility registration update procedure without changing to a new TA outside the UE's RA according to clause 4.2.2.2.1 of TS 23.502 [5].

5.	If new AMF-UE is selected, the NG-RAN sends AMF Relocation Request (Link ID) to the new AMF-UE.

6.	The new AMF-UE invokes Bulk UE Context Transfer Request (Link ID) towards the old AMF-UE according to the Link ID. The old AMF-UE responds the context of UEs associated with the Link ID. The UE context includes the Link ID and the associated AMF ID indicating the AMF-MBSR that the old AMF-UE has subscribed event to.

The new AMF-UE invokes Nudm_UECM_Registration with "move together" indication to the UDM, and the UDM suspends deregistration notification to the old AMF-UE who did not indicate "move together".

NOTE 2:	The UDM still notifies deregistration to any other old AMF-UE, i.e., notifies deregistration to those old AMF-UE who indicated "move together".

7.	The new AMF-UE allocate a new Link ID, associates the new Link ID with the UEs, and responds to the NG-RAN with the new Link ID. The NG-RAN replace the Link ID with the new Link ID, i.e., replaces the Link ID in the context of MBSR with the new Link ID, and replaces the Link ID in context of UEs with the new Link ID.

#### 6.4.3.3	Mobile Base Station Relay handover with UEs

##### 6.4.3.3.1	General

As described in clause 6.4.1, the UE will keep silent as possible as it can when moving with MBSR, i.e. no mobility registration. As described in step 3 of clause 6.4.1, the AMF serving the UE may need to be changed during the MBSR handover without UE NAS signalling exchange.

The NG-RAN has the context of the UE that is in CM-CONNECTED state, which includes the Link ID as described in step 5 of clause 6.4.3.1 and step 7 of clause 6.4.3.2. As described in step 3 of clause 6.4.1, the 5G-GUTI of the UE is unchanged, hence the AMF ID in the 5G-GUTI may not indicate to the AMF that stores the UE context, in this procedure, the NG-RAN sends handover required message to the AMF according to the Link ID in the UE context instead of the 5G-GUTI of the UE. For N2 handover, if the AMF serving UE is changed, the AMF ID in the Link ID indicates the old AMF serving the UE, the new AMF serving the UE needs to sends a new Link ID to the target NG-RAN.

##### 6.4.3.3.2	N2 handover

Figure 6.4.3.3.2-1: Mobile Base Station Relay N2 handover with UEs

1.	The source NG-RAN determines to perform handover for the MBSR and sends the Handover Required (MBSR info) message to the AMF-MBSR. The AMF-MBSR may select a new AMF-MBSR for the handover procedure.

2.	The handover procedure for the MBSR continues. If the AMF-MBSR is not changed, the AMF-MBSR sends the Link IDs associated with the MBSR to the target NG-RAN; if the AMF-MBSR is changed, the new AMF-MBSR sends the Link IDs associated with the MBSR to the target NG-RAN (see clause 6.5.3.1 for how AMF-MBSR knows the Link IDs). The target NG-RAN updates the Link IDs for the MBSR.

3.	The AMF-MBSR sends the Handover Command (MBSR info) to the source NG-RAN.

4.	The source NG-RAN sends Handover Command to the MBSR with MBSR info.

The following steps 5-9 are performed per UE.

NOTE:	Following steps 5-9 can be executed simultaneously with steps 1-3. Whether the MBSR handover is together with UEs' handover or before UEs' handover is determined by RAN WGs, but there's no impact to the following procedures.

5.	The source NG-RAN also sends Handover Required (UE info (MBSR info), "move together") per the UE associated with the MBSR towards the AMF-UE according to the Link ID in the UE context. The UE info (MBSR info) is included in the transparent container. The source NG-RAN can combine multiple Handover Required messages in a bulk way sending to the same destination. In order to differentiate the case that UE handover due to moving together with the MBSR from the case that UE handover to leave the MBSR, the "move together" is included to indicate that the UE is moving together with the MBSR.

6.	The AMF-UE may determine to select a new AMF-UE for the specific UE, and invoke Create UE Context Request (UE Context (Link ID), UE info, "move together") towards the new AMF-UE. The UE Context includes the Link ID when "move together" is received.

7.	The handover procedure for the UE continues. In case new AMF-UE is not selected and the UE moves together with the MBSR, the AMF-UE sends the UE info to the target NG-RAN. In case new AMF-UE is selected and the UE moves together with the MBSR, if the new Link ID related to the MBSR associated with the Link ID has not been allocated, the new AMF-UE allocates a new Link ID. The new AMF-UE sends the new Link ID and the UE info to the target NG-RAN. The target NG-RAN associates the MBSR with the Link ID, or the new Link ID if received.

8.	In case step 3 is performed, the new AMF-UE responds to the AMF-UE.

9.	The AMF-UE sends Handover Command (UE info) to the source NG-RAN. The UE info includes the associated MBSR info.

10.	The resources used between the UE and the MBSR may not change, and the source NG-RAN may or may not send Handover Command to the UE depends on RAN procedures.

##### 6.4.3.3.3	Xn handover

Figure 6.4.3.3.3-1: Mobile Base Station Relay Xn handover with UEs

1.	The UE moves together with the MBSR and both the UE and the MBSR performs Xn handover procedure.

The following steps 2-3 are performed per UE.

2.	The target NG-RAN sends Path Switch (UE info, "move together") per the UE associated with the MBSR towards the AMF-UE according to the Link ID in the UE context. The target NG-RAN can combine multiple Path Switch messages in a bulk way sending to the same destination. In order to differentiate the case that UE handover due to moving together with the MBSR from the case that UE handover to leave the MBSR, the "move together" is included to indicate that the UE is moving together with the MBSR.

3.	The Xn handover procedure for the UE continues.

4.	The target NG-RAN also sends Path Switch (MBSR info) to the AMF-MBSR.

5.	The Xn handover procedure for the MBSR continues. The AMF-MBSR sends the Link IDs associated with the MBSR to the target NG-RAN (See clause 6.5.3.1 for how AMF-MBSR knows the Link IDs). The target NG-RAN updates the Link IDs for the MBSR.

### 6.4.4	Impacts on services, entities and interfaces.

AMF (serving UE):

-	Support mapping UE context with Link ID.

-	Support receiving MBSR information as well as sending and receiving Link ID along with the UE information to/from NG-RAN.

-	Support sending Registration Area information to UE according to the TAI list configuration of MBSR.

-	Support bulk UE context transfer operation according to Link ID.

-	Support indicating "move together" to UDM in case.

NG-RAN:

-	Support sending MBSR information along with the UE information to AMF.

-	Support mapping MBSR context with Link ID.

-	Support sending and receiving Link ID along with the UE information to/from AMF.

-	Support determining AMF for UE based on Link ID.

UDM:

-	Support suspending/resuming deregistration notification to the old AMF-UE who indicated "move together" according to whether the new AMF-UE indicates "move together" or not.

## 6.5	Solution #5: Mobile termination procedure due to mobility

### 6.5.1	General

This solution addresses Key Issue #2 and Key Issue #3 for mobility. This solution is based on solution#4, i.e. the UE will perform mobility registration when camps on a MBSR or leaves a MBSR, and after the UE performs mobility registration when camps on a MBSR, the UE will not perform mobility registration again until the UE leaves the MBSR. In the concept of UE will not perform successive mobile registration when moving with MBSR, the AMF serving the UE may not know the place of the UE for paging.

This solution addresses the mobile termination when UE moves together with MBSR without performing successive mobile registration.

### 6.5.2	Functional descriptions

In this solution, the AMF serving a UE knows the AMF serving the MBSR, and when the UE needs to be paged, the AMF serving the UE can forward the paging to the AMF serving the MBSR, and the AMF serving the MBSR pages the UE. In order to make the AMF serving the UE know the AMF serving the MBSR, the AMF serving the UE needs to manage the association between the UE and MBSR as well as between the MBSR and the AMF ID indicating the AMF serving the MBSR.

Considering the 5G-GUTI of MBSR changes frequently, the AMF serving the UE needs to allocates a Link ID for the MBSR, and sends the association of Link ID and MBSR to the AMF serving the MBSR, so that the AMF serving the MBSR can associate the MBSR with the Link ID. The Link ID consists of AMF ID of the AMF that allocates it and a unique ID within the AMF. When the AMF serving the MBSR is changed, the new AMF serving the MBSR can notify the AMF change event to the AMF serving the UE according to the Link ID.

The following figure shows the data model for the mobile termination when UE moves together with MBSR without performing successive mobile registration:

Figure 6.5.2-1: Data model

Each UE is served by an AMF (AMF serving UE, A.K.A. AMF-UE), which holds the UE Context. When the UE camps on by a MBSR, the UE Context will include a Link ID linking to the MBSR that is dynamically assigned by the AMF. The Link ID includes AMF ID indicating the AMF assigning the Link ID, and includes a unique number assigned by the AMF indicated by the AMF ID. The UEs served by the same AMF and camps on the same MBSR will be assigned with same Link ID. The UE Context also includes an AMF ID associating with the Link ID, the AMF ID indicates the AMF serving the MBSR. The 5G-GUTI included in the UE Context may be assigned by another AMF-UE if the UE does not perform mobility registration when moving with the MBSR (see clauses 6.4.3.1, 6.4.3.2 and 6.4.3.3).

Each Mobile Base Station Relay (MBSR) is served by an AMF (AMF serving MBSR, A.K.A. AMF-MBSR), which holds the MBSR Context. The MBSR Context includes the dynamic MBSR ID (i.e. 5G-GUTI) that known by NG-RAN, and includes multiple Link IDs that dynamically assigned by AMFs serving the UEs that camp on the MBSR. Different Link ID in the MBSR Context indicates different AMF. A UE count associated with each Link ID is also included in the MBSR context, which is used to indicate the number of UEs camp on the MBSR served by the same AMF that is indicated by the AMF ID in the Link ID.

### 6.5.3	Procedures

#### 6.5.3.1	UE performs Service Request or Registration Request via Mobile Base Station Relay

As described in clause 6.4.1 of solution #4, the UE will keep silent as possible as it can when moving with MBSR, i.e. no mobility registration, so the AMF serving the UE may not know where the UE is, but the AMF knows the UE is with MBSR without knowing where MBSR is.

The Link ID is introduced to map to the MBSR in the AMF of UE in this procedure because the 5G-GUTI of the MBSR will be changed without notifying to the AMF of UE. In order to make the serving AMF of UE know the serving AMF of the MBSR, whenever the serving AMF of MBSR is changed, the serving AMF of UE need to be notified according to the Link ID.

Figure 6.5.3.1-1: UE performs Service Request or Registration Request via Mobile Base Station Relay

1.	The UE camps on a MBSR. The UE initiates Service Request or Registration Request towards 5GC. The MBSR forwards the N1 message to the NG-RAN. The NG-RAN forwards the N1 message and includes the MBSR ID in the underlayer N2 message, e.g. 5G-GUTI of the MBSR.

2.	When the UE Context includes a Link ID and associated AMF ID (i.e. the UE may leave a MBSR and camps on a new MBSR), if the associated AMF ID is different from the AMF ID in the received MBSR ID (i.e. the UE leaves a MBSR and camps on the MBSR), the AMF-UE unsubscribes to the AMF-MBSR indicated by the associated AMF ID with the Link ID, meanwhile allocates a Link ID, which includes the AMF ID of the AMF-UE, associates the UE with the allocated Link ID, maps the allocated Link ID to the AMF ID in the received MBSR ID, and subscribes to the AMF-MBSR indicated by the received MBSR ID with the allocated Link ID and the received MBSR ID, the AMF-MBSR associates the MBSR with the Link ID.

If the associated AMF ID is same as the AMF ID in the received MBSR ID, the AMF-UE allocates a Link ID and performs the following:

-	If AMF-UE does not change, the AMF-UE update the subscription to the AMF-MBSR with the Link ID, the allocated Link ID, and the received MBSR ID (conditional update). If the MBSR indicated by the received MBSR ID has been subscribed with the Link ID (i.e. the UE still camps on the same MBSR), the AMF-MBSR returns an indication to the AMF-UE and keeps the subscription with the Link ID and the association between the Link ID and the MBSR, otherwise (i.e. the UE leave a MBSR and camps on the MBSR) the AMF-MBSR decreases the count for the Link ID, associates the MBSR with the allocated Link ID, and increase the count for the allocated Link ID. If the indication is received, the AMF-UE releases the allocated Link ID and keeps the association between the Link ID and the UE, otherwise, the AMF-UE associates the UE with the allocated Link ID.

-	If AMF-UE changed (i.e. no matter whether the MBSR is same, new Link ID allocated by the new AMF-UE is needed), the AMF-UE unsubscribed to the AMF-MBSR with the Link ID, meanwhile the AMF-UE subscribes to the AMF-MBSR with the allocated Link ID and the received MBSR ID, associates the UE with the allocated Link ID, and associates the allocated Link ID with the AMF ID in the received MBSR ID. The AMF-MBSR maps the allocated Link ID to the MBSR.

When the UE Context does not include a Link ID, the AMF-UE allocates a Link ID, which includes the AMF ID of the AMF-UE, for the received MBSR ID, and subscribe to the AMF-MBSR with the allocated Link ID and the received MBSR ID. The AMF-UE also associates the Link ID with the AMF ID indicated by the MBSR ID.

When subscribe request with a Link ID is received, the AMF-MBSR increase the count for the Link ID, and when unsubscribe request with a Link ID, the AMF-MBSR decrease the count of the Link ID.

3.	The new AMF-UE sends a N2 message, and the N2 message may include a N1 message. In case of registration procedure, if the TAI of the MBSR does not change, for the sake of saving mobility registration for UEs behind the MBSR, the AMF-UE may send Registration Accept to the UE with TAI List that only includes the TAI received in step 1.

4.	The NG-RAN sends an RRC message to the UE, the RRC message may include the N1 message.

5.	If the UE camps on a MBSR, the UE may suspend the mobile registration if the UE keeps camping on the MBSR (e.g. the TAI/TAC broadcasted by the MBSR does not change and the NG-RAN sends the TAI to the AMF-UE in step 1).

6.	When the event met, the AMF-MBSR notifies the AMF-UE with the Link ID and associated information, see clauses 6.5.3.3 and 6.5.3.4.

7.	The UE camps on another MBSR or NG-RAN. When the mobile registration is suspended, the UE resumes the mobile registration (e.g. the TAI/TAC received is not included in the current RA of the UE). The steps 1-5 are performed. If the UE camps on another MBSR, the NG-RAN indicates a different MBSR ID to the AMF-UE. If the UE camps on NG-RAN directly, the NG-RAN does not indicate MBSR ID to the AMF-UE.

8.	If no MBSR ID is received and a Link ID and associated AMF ID exist in the UE Context, the AMF-UE unsubscribes towards the AMF-MBSR indicated by the associated AMF ID with the Link ID.

#### 6.5.3.2	UE handover to connect/disconnect to/from Mobile Base Station Relay with AMF change

##### 6.5.3.2.0	General

Same as described in clause 6.5.3.1, the Link ID is introduced to map to the MBSR in the AMF of UE in the handover procedure During the handover, whenever the serving AMF of UE is changed, the serving AMF of MBSR also needs to be informed, so that the notification can be sent to the correct AMF of UE.

For the UE handover from one MBSR to another MBSR, the serving AMF of UE may change, the AMF of MBSR needs to know one UE is leaving the old MBSR and one UE (the same one) is joining the new MBSR by the means of received Link ID.

##### 6.5.3.2.1	N2 handover

Figure 6.5.3.2.1-1: UE N2 handover to connect/disconnect to/from Mobile Base Station Relay

1.	The UE is in CM-CONNECTED state and may be relayed by a MBSR.

2.	The source NG-RAN determines to perform handover for the UE and sends the Handover Required (UE info) message to the AMF-UE. The AMF-UE may select a new AMF-UE for the handover procedure. The UE info is included in the transparent container. The difference from step 2 in 6.4.3.3.2 of solution #4 is that the "move together" is not included to indicate that the UE handover from a macro cell or to leave a MBSR.

3.	In case new AMF-UE is selected, the AMF-UE invokes Create UE Context Request (UE Context, UE info) towards the new AMF-UE. The UE Context in the request does not include the Link ID and the associated AMF ID if the UE was relayed by a MBSR and "move together" is not received in the step 2.

4.	The handover procedure for the UE continues. If the UE is handed over to the target NG-RAN via another MBSR, the target NG-RAN indicates the new MBSR ID to the (new) AMF-UE.

5.	In case step 3 is performed, the new AMF-UE responds to the AMF-UE.

6.	In case step 3 is performed and Link ID is included in the stored UE Context (i.e. the UE handover to leave a MBSR), the AMF-UE unsubscribes to the AMF-MBSR indicated by the associated AMF ID with the Link ID. The AMF-MBSR decreases the count for the Link ID.

7.	The AMF-UE sends Handover Command (UE info) to the source NG-RAN.

8.	The source NG-RAN sends the Handover Command to the UE.

9.	The UE tunes to the target NG-RAN, which may be via another MBSR.

10.	The target NG-RAN sends Handover Confirm to the (new) AMF-UE.

11.	The handover procedure for the UE continues.

12.	If MBSR ID is received and the UE will not perform mobility registration, the AMF-UE allocates a Link ID for the received MBSR ID, and subscribe to the AMF-MBSR according to the MBSR ID with the allocated Link ID and the received MBSR ID. The AMF-MBSR maps the allocated Link ID to the MBSR and increases the count for the Link ID,

When the event met, the AMF-MBSR notifies the AMF-UE with the Link ID and associated information, see clauses 6.5.3.3 and 6.5.3.4.

##### 6.5.3.2.2	Xn handover

Figure 6.5.3.2.2-1: UE Xn handover to connect/disconnect to/from Mobile Base Station Relay

1.	The UE performs Xn handover to connect to a MBSR from direct NG-RAN connection or from another MBSR, or to connect to macro cell from a MBSR.

2.	The target NG-RAN sends the Path Switch ([MBSR ID], UE info) message to the AMF-UE. If the UE handover to directly connect to NG-RAN, the MBSR ID is not included. If the UE handover to a MBSR, the MBSR ID is included.

3.	Considering that UE will not handover to the same MBSR, the new AMF-UE may perform conditional subscription update as described in step 2 of clause 6.5.3.1, or unsubscribe and then subscribe to the AMF-MBSR as described in step 2 of clause 6.5.3.1.

When the event met, the AMF-MBSR notifies the AMF-UE with the Link ID and associated information, see clauses 6.5.3.3 and 6.5.3.4.

4.	The handover procedure for the UE continues.

#### 6.5.3.3	Mobile Base Station Relay mobility registration procedure with AMF change

Same as described in clause 6.5.3.1, the Link ID is introduced to map to the MBSR in the AMF of UE in the mobility registration procedure.

For the mobility registration of the MBSR, the AMF of the MBSR may change, the new AMF of MBSR needs to notify the AMF of UE with Link ID, which indicates the MBSR in the AMF of UE. If bulk UE context transfer as described in clause 6.4.3.3 of solution #4 is performed, the AMF of UE may change after the mobility registration of the MBSR, the new AMF of UE also needs to inform the AMF of MBSR the change of the AMF with the Link ID.

Figure 6.5.3.3-1: Mobile Base Station Relay mobility registration procedure

1.	The MBSR sends Registration Request towards the 5GC via the NG-RAN.

2.	The NG-RAN may select an AMF-MBSR different from the old AMF-MBSR, the AMF-MBSR queries UE Context for the MBSR from the old AMF-MBSR and the old AMF-MBSR returns MBSR Context with the Link IDs.

3.	If AMF-MBSR is changed, the AMF-MBSR notifies the AMF change event with the Link ID to the AMF-UE. The AMF-UE replaces the AMF ID associated with the Link ID with the AMF ID of the AMF-MBSR.

4.	The AMF-MBSR sends Registration Accept to the MBSR.

5.	Steps 4-7 in clause 6.4.3.2 of solution#4 may be performed.

6.	If step 5 is performed, the AMF-UE is changed, the new AMF-UE updates the subscription with the Link ID and the new Link ID (unconditional update) towards the AMF-MBSR associated with the Link ID. The AMF-MBSR replace the Link ID with the new Link ID.

#### 6.5.3.4	Mobile Base Station Relay handover with UEs with AMF change

Same as described in clause 6.5.3.1, the Link ID is introduced to map to the MBSR in the AMF of UE in the handover procedure.

For the N2 handover of MBSR together with UEs, both the AMF of the MBSR and the AMF of UE may change together during the N2 handover procedure, the new AMF of MBSR needs to notify the AMF of UE with Link ID, which indicates the MBSR in the AMF of UE. The new AMF of UE also needs to inform the AMF of MBSR the change of the AMF with the Link ID.

The following procedure is for N2 handover, no extension needed for Xn handover because AMF will not change during Xn handover.

Figure 6.5.3.4-1: Mobile Base Station Relay N2 handover with UEs

1.	The source NG-RAN determines to perform handover for the MBSR and sends the Handover Required (MBSR info) message to the AMF-MBSR. The AMF-MBSR may select a new AMF-MBSR for the handover procedure.

2.	The handover procedure for the MBSR continues.

3.	The AMF-MBSR sends Handover Command (MBSR info) to the source NG-RAN.

4.	The source NG-RAN sends Handover Command to the MBSR.

5.	If the new AMF-MBSR is selected, the new AMF-MBSR gets the MBSR context during the handover procedure, which includes the Link ID. The Link ID may be updated to a new Link ID by the new AMF-UE, e.g. as described in steps 10-11. The new AMF-MBSR notifies the AMF change event with the Link ID to the AMF according to the AMF ID in the Link ID.

The following steps 6-7 are performed per UE.

NOTE:	Following steps 6-10 can be executed simultaneously with steps 1-3. Whether the MBSR handover is together with UEs' handover or before UEs' handover is determined by RAN WGs, but there's no impact to the following procedures.

6.	The source NG-RAN also sends Handover Required (UE info (MBSR info), "move together") per the UE associated with the MBSR towards the AMF-UE. The UE info (MBSR info) is included in the transparent container. The source NG-RAN can combine multiple Handover Required messages in a bulk way sending to the same destination. In order to differentiate the case that UE handover due to moving together with the MBSR from the case that UE handover to leave the MBSR (e.g., described in clause 6.5.3.2.1), the "move together" is included to indicate that the UE is moving together with the MBSR.

7.	The AMF-UE may determine to select a new AMF-UE for the specific UE, and invokes Create UE Context Request (UE Context (Link ID, AMF ID), UE info, "move together") towards the new AMF-UE. The UE Context includes the Link ID and the associated AMF ID indicating the AMF-MBSR when "move together" is received.

8.	The handover procedure for the UE continues.

9.	If step 7 is performed, the new AMF-UE responds to the AMF-UE.

10.	The AMF-UE sends Handover Command (UE info) to the source NG-RAN. The UE info includes the associated MBSR info.

11.	The resources used between the UE and the MBSR may not change, and the source NG-RAN may or may not send Handover Command to the UE depends on RAN procedures.

12.	If step 7 is performed and "move together" is received, the new AMF-UE updates the subscription towards the AMF-MBSR with the Link ID and the new Link ID (unconditional update).

13.	If the AMF-MBSR is in handover procedure, when the AMF-MBSR does not select a new AMF-MBSR, the AMF-MBSR decreases the count for the Link ID, increases the count for the new Link ID, and associates the MBSR indicated by the Link ID with the new Link ID. When the AMF-MBSR selects a new AMF-MBSR in step 2, the AMF-MBSR forwards the subscription update towards the new AMF-MBSR, and the new AMF-MBSR decreases the count for the Link ID, increases the count for the new Link ID, and associates the MBSR indicated by the Link ID with the new Link ID.

14.	The AMF-MBSR responds to the subscription update.

#### 6.5.3.5	Mobile termination when UE moves with Mobile Base Station Relay

As described in clause 6.4.1, the UE will keep silent as possible as it can when moving with MBSR, i.e. no mobility registration, so the AMF serving the UE may not know where the UE is, but the AMF knows the UE is with MBSR without knowing where MBSR is.

According to the procedure from clause 6.5.3.1 to 6.5.3.4, the AMF of UE knows the AMF of MBSR, and Link ID is used to indicate the MBSR, the AMF of UE can relay paging message with the Link ID to the AMF of MBSR, and AMF of MBSR will page the UE via the MBSR indicated by the Link ID.

Figure 6.5.3.5-1: Mobile termination when UE moves with Mobile Base Station Relay

1.	UE moves with the MBSR, the AMF-UE is triggered to send N1 message or N2 message to UE.

2a.	If the UE is in CM-CONNECTED state, the AMF-UE sends the N1/N2 message towards the NG-RAN. If N1 message is received, the NG-RAN sends the N1 message to the UE via the MBSR.

2b.	If the UE is in CM-IDLE state and the UE context includes a Link ID, as well as the AMF-UE knows the serving RAN node for the MBSR related to the Link ID (e.g. the procedure described in clause 6.4.3.2 of solution#4 is just performed), the AMF-UE may send Paging request (UE ID, Link ID) to the NG-RAN. The NG-RAN maps the Link ID to the MBSR.

2c.	The AMF-UE may invoke Page Relay (Link ID, UE ID) towards the AMF-MBSR.

3.	If step 2c is performed, the AMF-MBSR maps the Link ID to the MBSR. The AMF-MBSR sends the Paging request (UE ID) to the NG-RAN.

4.	If step 2b or step 3 is performed, the NG-RAN sends the Paging request (UE ID) to the MBSR, and the MBSR pages the UE.

### 6.5.4	Impacts on services, entities and interfaces.

AMF (serving UE):

-	Support mapping UE context with Link ID and associate AMF ID indicating AMF serving MBSR with the Link ID.

-	Support invoking (un)subscribe operation with Link ID and MBSR ID for AMF change event towards the AMF serving MBSR, and receiving notification of AMF change from AMF serving MBSR.

-	Support paging UE with Link ID and/or redirecting the Paging message to AMF serving MBSR.

AMF (serving MBSR):

-	Support mapping MBSR context with Link ID.

-	Support (un)subscribe operation with Link ID and MBSR ID for AMF change event, and sending notification of AMF change to subscriber.

-	Support redirecting the Paging message from AMF serving UE to the NG-RAN serving the MBSR.

NG-RAN:

-	Support sending MBSR information along with the UE information to AMF.

## 6.6	Solution #6: Enabling roaming of mobile base station relays

### 6.6.1	General

This solution addresses Key Issue#4 as defined in clause 5.4.

### 6.6.2	Functional description

The high-level points of the proposed solution are:

1.	The mobile base station relay (i.e. a mobile IAB-node) may be pre-configured with one or more of the following:

a)	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a mobile base station relay;

NOTE 1:	The geographic area format used for V2X policy in TS 24.588 [8] can be re-used. A geographic area is encoded as a list of coordinates, each corresponding to a summit of a polygon.

b)	a list of PDU session parameters, e.g. specific DNN and S-NSSAI, and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session or using backhaul IP layer by implementation toward a regional OAM server for mobile base station relay configuration;

2a.	When the mobile base station relay moves out of coverage of the HPLMN and into VPLMN, IAB-node integration procedure may be used:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a mobile base station relay as described in step 1a), the relay attempts to select the PLMNs in the list, in priority order;

-	Steering of Roaming can be used to provision this information dynamically by the HPLMN;

-	otherwise, the mobile base station relay performs PLMN selection as specified in TS 23.122 [12].

2b.	Alternatively, when the mobile base station relay moves out of coverage of the HPLMN and into VPLMN in connected mode, inter-IAB-donor gNB mobility procedure may be used:

-	the inter-IAB-donor gNB mobility procedure is the outcome of KI#3 with enhancements to handle roaming.

NOTE 2:	The inter-IAB-donor gNB mobility procedure depends on RAN Rel-18 work.

NOTE 3:	The PDU sessions of UE camped on the mobile base station relay may be transferred from the HPLMN to the VPLMN. This assumes that inter-PLMN handover between the HPLMN and the VPLMN is supported.

3.	Upon selecting a VPLMN as described in step 2, the mobile base station relay initiates a registration procedure or performs handover procedure as requested by network and indicates its intention to operate as a Mobile Base Station Relay in RRC connection to the NG-RAN. The intention to operate as a Mobile Base Station Relay is included in the N2 parameters to the AMF by the NG-RAN.

4.	The AMF in the VPLMN:

a)	accepts the Registration Request or handover of the mobile base station relay, if the UE is allowed to act as MBSR;

b)	if determines that the mobile base station relay is not authorized to operate as a MBSR but allowed to receive normal services, Registration accept is sent to the UE by including an indication that it is not allowed to act as MBSR. The UE shall not act as MBSR in this case. Additionally, the AMF may include the list of TAIs where the UE is not allowed to act as MBSR. The UE adds this TAIs in list of IAB operation restriction; or

c)	rejects the Registration Request of the mobile base station relay with a 5GMM cause value, The UE will add current TAI in list of IAB operation restriction for MBSR. If the UE is allowed to receive normal service, it may trigger initial registration procedure without indicating support for MBSR (or IAB indication); or

d)	rejects the handover indicating that the mobile base station relay is not allowed to operate as a relay in the VPLMN.

NOTE 4:	In the case of 2b, the acceptance of the mobile base station relay is part of the inter-IAB-donor gNB procedure.

5a.	If the Registration Request of the mobile base station relay is accepted, in the case of 2a, the mobile base station relay:

-	performs an IAB node integration procedure with the VPLMN as specified in clause 8.12.1 of TS 38.401 [6]. Using the pre-configured information as described in step 1b) and solution provided for KI#1.

NOTE 4:	When performing IAB-node integration procedure, the PDU sessions of any UE camped on the mobile base station relay will be interrupted.

5b.	If the handover of the mobile base station relay is accepted, in the case of 2b, the mobile base station relay together with the IAB-donor gNB:

-	performs inter-IAB-donor gNB mobility procedure and becomes part of the VPLMN during the procedure.

NOTE 5:	When performing inter-IAB-donor gNB mobility procedure, the traffic on PDU sessions of any UE camped on the mobile base station relay may be maintained.

6.	The IAB-UE will not indicate support for MBSR (i.e. will not act as MBSR) when its camped on a TAI which is part of list of IAB operation restriction. The granularity of this lists can be at TAI level or it can be at PLMN level.

### 6.6.3	Signalling flows

#### 6.6.3.1	Signalling flow for roaming of mobile base station relay using IAB node integration procedure

Figure 6.6.3.1-1: Signalling flow for roaming of mobile base station relay operating using IAB node integration procedure

0.	The mobile base station relay may be pre-configured with:

-	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a mobile base station relay;

-	a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session or using backhaul IP layer by implementation toward a regional OAM server for relay configuration.

1.	The mobile base station relay moves out of coverage of its HPLMN and selects a VPLMN as follows:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a mobile base station relay, the IAB-UE attempts to select the PLMNs in the list, in priority order as described in step 2a of clause 6.2.2;

-	otherwise, the mobile base station relay performs PLMN selection as specified in TS 23.122 [12].

2.	The mobile base station relay (i.e. IAB-UE) provides an MBSR-indication to the NG-RAN when the RRC connection is established as defined in clause 5.35.2 of the TS 23.501 [2]. The mobile base station relay sends a Registration Request message to the AMF via NG-RAN.

3.	RAN may use this MBSR-indication for AMF selection that supports mobile base station relay operation and forwards MBSR-indication in N2 message with Registration Request.

4.	The VPLMN AMF performs UE authentication and registration and further checks with UDM . The UDM provides the MBSR-Operation allowed indication and list of IAB operation restriction to AMF as part of the Access and Mobility Subscription data as defined in clause 4.2.2.2.2 of TS 23.502 [5].

5.	If the AMF determines that the mobile base station relay is authorized to operate as a MBSR in current TAI, the AMF sends MBSR authorized indication to NG-RAN and a Registration Accept message to the mobile base station relay.

6.	The mobile base station relay performs an IAB node integration procedure with the VPLMN as specified in clause 8.12.1 of TS 38.401 [6], using the pre-configured information as described in step 0.

#### 6.6.3.2	Signalling flow for roaming of mobile base station relay using inter-IAB-donor gNB mobility procedure

Figure 6.6.3.2-1: Signalling flow for roaming of mobile base station relay operating using IAB inter-donor full migration procedure

0.	The mobile base station relay may be pre-configured with:

-	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a mobile base station relay;

-	a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session or using backhaul IP layer by implementation toward a regional OAM server for relay configuration.

1.	The mobile base station relay moves out of coverage of its HPLMN and selects a VPLMN.

The mobile base station relay performs an inter-IAB-donor gNB mobility procedure as described in step 2b in clause 6.6.2.

NOTE 1:	The inter-IAB-donor gNB mobility procedure for mobile base station relay, including the reconfiguration during mobility, depends on RAN Rel-18 work.

2-3.	The mobile base station relay sends a Registration Request message to the NG-RAN. NG-RAN may use this MBSR-indication for AMF selection that supports mobile base station relay operation and forwards MBSR-indication in N2 message with Registration Request.

4.	The VPLMN AMF performs UE registration and further checks with UDM whether the mobile base station relay is authorized to operate as a mobile base station relay. The UDM provides the MBSR-Operation allowed indication and list of IAB operation restriction to AMF as part of the Access and Mobility Subscription data as defined in clause 4.2.2.2.2 of the TS 23.502 [5].

5.	If the AMF determines that the mobile base station relay is authorized to operate as a mobile base station relay in current TAI, the AMF sends MBSR authorized indication to NG-RAN and a Registration Accept message to the mobile base station relay.

### 6.6.4	Impacts on services, entities and interfaces

The following impacts have been identified:

-	UE:

-	No impact.

-	Mobile base station relay:

-	Support indication in AS layer parameters of an MBSR-indication.

-	Support pre-configuration with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as a mobile base station relay.

Editor's note:	It needs to be evaluated if existing UE PLMN selection operation defined in TS 23.122 [12] is sufficient.

-	Support pre-configuration with a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for relay configuration.

-	Support for PLMN selection based on the prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node.

-	Support inter-IAB-donor gNB mobility procedure.

-	The MBSR (i.e., IAB-UE) maintains the list of IAB operation restriction for MBSR. The list can be at a granularity of TAI or PLMN. If IAB-UE is camped on this TAI/PLMN, the IAB-UE will not attempt to act as MBSR.

Editor's note:	The support of inter-IAB-donor gNB mobility procedure needs to be coordinated with RAN WGs.

-	NG-RAN:

-	Support for AMF selection based on AS layer MBSR-indication from mobile base station relay and forward MBSR-indication in N2 parameters to AMF.

Editor's note:	The support of mobile base station relay needs to be coordinated with RAN WGs.

-	AMF:

-	Support for determining whether a mobile base station relay attempting to register is allowed to operate as an IAB node and accepting/rejecting the Registration Request accordingly. Additionally, the AMF may include the list of TAIs where the IAB-UE is not allowed to act as MBSR.

## 6.7	Solution #7: Network based Positioning of mobile base station relay for location services

### 6.7.1	General

This solution relates to KI#5 "Support of location services for UEs accessing via a mobile base station relay". It proposes a solution where the UE and the mobile base station relay (MBSR) uses the legacy location procedures and positioning protocols described in the specifications TS 23.273 [4], TS 37.355 [13] and NRPPa in TS 38.455 [14].

There are three different cases for a UE location estimation served by a mobile base station relay:

-	A UE performing network assisted GNSS location.

-	A UE performing positioning measurements based on signals from the mobile base station relay and other base stations gNBs.

-	A UE performing positioning measurements only on the mobile base station relay, i.e. no neighbouring gNBs to measure on. The UE may use the Sidelink positioning procedures investigated in the FS_Ranging_SL captured in TR 23.700-86 [15]

This solution does not address the GNSS based positioning estimation case as the GNSS location is not dependent on the exact location of the mobile base station relay. Therefore, GNSS positioning can use the legacy positioning protocol.

### 6.7.2	Functional descriptions

A network node on which UE performs its positioning measurements against needs a known position. In case the network node is a mobile base station relay it may change its location, and therefore it is important that the position/location of the mobile base station relay node is known for the time period when the UE performs measurements on the node.

Thus, there are different cases that needs to be taken into account:

-	When a UE performs positioning measurements based on signals from the mobile base station relay and possibly other base stations gNBs the current position of the mobile base station relay needs to be known by the LMF and therefore the LMF may need to request positioning measurements of the mobile base station relay.

-	When a UE only can perform measurements on the mobile base station relay i.e. no or too few neighbouring gNBs to measure on, it may use the sidelink positioning procedures as investigated in FS_Ranging_SL TR 23.700-86 [15]. This solution will not go into details of this solution as it is handled within FS_Ranging_SL.

To support the UEs location procedures, the mobile base station relay needs to get a good location estimate. The solution is based on the following principles:

-	Reuse of the LPP TS 37.355 [13] for DL/UL positioning messages to the UE served by the mobile base station relay.

-	It is assumed that the mobile base station relay does not always transmit a Positioning Reference Signal (PRS). Therefore, the donor-IAB RAN node will need to send a control message to the mobile base station relay to turn on the PRS.

-	The donor-IAB RAN node will need to inform the LMF that it has an IAB-DU node that is a mobile base station relay. In Rel-17 the IAB-DU node was assumed to be static at a known location.

-	The LMF will need to perform location determination of the mobile base station relay in case the UE reports positioning data from the mobile base station relay, that the donor-IAB RAN node reported as a mobile base station relay.

-	The Mobile base station relay will then act like a UE using the LPP protocol.

### 6.7.3	Procedures

#### 6.7.3.0	General

Only the MT-LR UE positioning procedure is described. The MO-LR positioning procedure is a straightforward extension, where the trigger is MO-LR.

#### 6.7.3.1	MT_LR for UE served by a mobile base station relay

This procedure on MT-LR positioning consists of two parts, first the procedure for UE positioning from TS 23.273 [4] utilizing the mobile base station relay as a RAN node and then the IAB-UE positioning using the UE positioning procedures in TS 23.273 [4].

Figure 6.7.3.1-1: Procedure for MT_LR for UE served by a MBSR using TRP information request procedure

0.	TRP Information Exchange procedure is triggered by OAM, when the MBSR is integrated or fully migrated in the RAN at a gNB. The LMF requests gNBs to provide detailed information (e.g. the geographical coordinates for the TRP) for TRPs hosted by the gNBs. During this procedure, the MBSR IAB-DU informs the Donor gNB-CU that its TRP(s) is mobile, which is further forwarded to the LMF. The MBSR cell ID and a permanent MBSR IAB-UE identity (MBSR ID, which may be a GPSI) which is also used to identify the MBSR UE in the UDM, is also passed to the LMF.

NOTE 1:	This procedure is typically performed before LMF initiating a position procedure for a UE as this is UE independent.

1.	The AMF serving UE is triggered to initiate location reporting procedure for UE as specified in TS 23.273 [4].

NOTE 2:	This step may be same as the procedure for a UE served by normal gNB and does not require the MBSR to be register at the same AMF as the UE.

2.	The LMF, selected by the AMF serving UE, is triggered to initiate location reporting procedure for the UE. The AMF provides the LCS correlation ID and the cell-ID of the serving cell, i.e. the serving cell of the MBSR.

3. 	The LMF performs the positioning procedures, e.g. step 8 in clause 6.1.1 of TS 23.273 [4] 5GC-MT-LR procedure for the regulatory location service, or step 12 in clause 6.1.2 of TS 23.273 [4] 5GC-MT-LR Procedure for the commercial location service. The LMF configures the gNBs within the area of interest based on the provided serving cell-ID to start broadcast PRS, in the response the gNB provides the list of TRPs that are activated with an indication if the TRP is a MBSR. The UE send the UL positioning message including the positioning data captured by the UE. The data set may include one or more cell-ID(s) with positioning data associated with one or more mobile base station relay (which are known from step 0 or by the updated list received in this step). If no MBSR cell is included, the steps 4-6 are skipped.

4.	Since the LMF knows the TRP hosted by MBSR(s) is mobile (in step 0), the LMF initiate a TRP Information Exchange procedure to request the updated location information of the TRP(s) associated with MBSRs by directing the TRP information exchange messages to the gNB(s) associated with the Cell ID(s), for example, by setting the NRPPa TRP Information Type Item IE to "geo-coordinates". Donor-CU further send a F1 TRP INFORMATION REQUEST message to any MBSR IAB-DU.

5. 	The MBSR IAB-UE initiate a 5GC-MO-LR procedure (as defined in clause 6.2 of TS 23.273 [4]) to get its location information. The MBSR IAB-UE's location info is used by the co-located MBSR IAB-DU to determine the updated location for its TRP.

6.  The MBSR IAB-DU report its updated TRP location, e.g. TRP's geo-coordinate, to the Donor-CU, which is further forwarded to LMF which uses the updated location of MBSR's TRP for calculation the UE's position.

7.	The LMF provides the UE location to the AMF serving the UE.

There is another variation on how LMF know the updated location of TRP. In current F1/NRPPa, the DU can report its TRP location as a relative location to a reference point. Current reference point is fixed and its location info is configured by OAM.  This can be enhanced for MBSR by using the MBSR IAB-UE as the reference point. By knowing the reference point is a MBSR IAB-UE (as from step 0 TRP Information exchange), the LMF can first initiate MT-LR to determine the location of the Reference Point (i.e. MBSR IAB-UE), then determine the TRP's location. An example call flow is shown below.

Figure 6.7.3.1-2: Procedure for MT_LR for UE served by a MBSR using another MT-LR procedure

0.	TRP Information Exchange procedure is triggered by OAM. when the MBSR is integrated or fully migrated in the RAN at a new gNB. The LMF requests gNBs to provide detailed information (e.g. the geographical coordinates for the TRP) for TRPs hosted by the gNBs. During this procedure, the MBSR IAB-DU informs the Donor gNB-CU that its TRP(s) is mobile, which is further forwarded to the LMF. The MBSR cell ID and a permanent MBSR IAB-UE identity (MBSR ID, which may be a GPSI) which is also used to identify the MBSR UE in the UDM, is also passed to the LMF.

NOTE 1:	This procedure is typically performed before LMF initiating a position procedure for a UE as this is UE independent.

1.	The AMF serving UE is triggered to initiate location reporting procedure for UE as specified in TS 23.273 [4].

NOTE 2:	This step may be same as the procedure for a UE served by a normal gNB.

2.	The LMF, selected by the AMF serving UE, is triggered to initiate location reporting procedure for the UE. The AMF provides the LCS correlation ID and the cell-ID of the serving cell, i.e. the serving cell of the MBSR.

3.	The LMF performs the positioning procedures, e.g. Step 8 in  clause 6.1.1 of TS 23.273 [4] 5GC-MT-LR procedure for the regulatory location service, or step 12 in clause 6.1.2 of TS 23.273 [4] 5GC-MT-LR Procedure for the commercial location service. The LMF configures the gNBs within the area of interest based on the provided serving cell-ID to start broadcast PRS, in the response the gNB provides the list of TRPs that are activated with an indication if the TRP is a MBSR. The UE send the UL positioning message including the positioning data captured by the UE. The data set may include one or more cell-ID(s) with positioning data associated with one or more mobile base station relay (which are known from step 0 or by the updated list received in this step). If no MBSR cell is included, the steps 4-5 are skipped.

4.	Since the LMF know the TRP hosted by the MBSR is mobile (in step 0), the LMF initiate a 5GC-MT-LR procedure for the Reference Point (i.e. MBSR IAB-UE) by using the MBSR ID as UE identity. The serving AMF is retrieved from UDM using the MBSR ID.

5.	The LMF determines the location of the MBSR's TRP by using the updated location of the Reference Point (i.e. the updated MBSR-MT's position).

6.	The LMF provides the UE location to the AMF serving the UE.

### 6.7.4	Impacts on services, entities, and interfaces

LMF:

-	Support for receiving the indication that a TRP is mobile, and (optionally) the identity of the Reference Point which is the identity of the MBSR IAB-UE. If an identity is received MT-LR is used to retrieve the location or the UE.

- 	When there is a need to use the MBSR's TRP for positing a UE, LMF initiates TRP information exchange procedure to get the updated location information of the MBSR IAB-DU, or initiate a position procedure for the MBSR IAB-UE and determine the TRP location based on the MBSR IAB-UE's location.

RAN:

-	Support providing the indication that a TRP is mobile, and (optionally) the identity of the Reference Point which is the identity of the MBSR IAB-UE.

-	When there is a request for the location of the MBSR's TRP, initiate a MO-LR procedure for the MBSR IAB UE and determine the location of the TRP based on the location of the IAB-UE.

## 6.8	Solution #8: Support of positioning for UE accessing via mobile base station relay

### 6.8.1	General

This solution addresses the KI#5: Support of location services for UEs accessing via a mobile base station relay.

### 6.8.2	Functional Descriptions

For UEs accessing the network via a mobile base station relay (MBSR), when UE positioning is performed, in order to improve the accuracy of the UE location, the LMF obtains the location of the mobile base station relay which is used to calculate the UE location as follows:

-	For UL positioning, the mobile base station relay provides its location to LMF via NRPPa message. When the location is received, the LMF decides that the UE is accessing the network via a mobile base station relay and uses the location provided by the mobile base station relay to calculate UE location. In this case, the mobile base station relay initiates the 5GC-MO-LR procedure to obtain its location.

-	For DL positioning, the mobile base station relay provides UE ID (i.e. GPSI) in the NRPPa message to LMF. The LMF acts as an AF to request the location of mobile base station relay from GMLC.

The LMF determines that the UE connects to MBSR based on the MBSR indication received from AMF. When AMF receives positioning request for a UE, if the UE connects to MBSR, e.g. the AMF makes the decision based on the additional ULI received from NG-RAN as specified in solution#9, the AMF sends the MBSR indication to LMF.

### 6.8.3	Positioning procedure for UE accessing via mobile base station relay

Figure 6.8.3-1: Positioning procedure for UE accessing via mobile base station relay

0.	Triggers for UE positioning happen, e.g. the AMF receives Namf_Location_ProvidePositioningInfo Request from GMLC or receives MO-LR Request from UE.

1.	The AMF invokes a Nlmf_Location_DetermineLocation Request service operation towards the LMF. The service operation includes MBSR indication when AMF determines that the UE connects to MBSR, e.g. when additional ULI is received from NG-RAN as specified in solution#9.

Based on the MBSR indication received in step 1, the LMF determines that UE connects to MBSR. Then the LMF obtains MBSR location to calculate UE location as shown in box (A) or box (B).

The steps in box (A) (i.e. steps 2-8) are performed when UL positioning is used to obtain UE location.

2.	The LMF sends NRPPa message to NG-RAN.

3.	The NG-RAN sends NRPPa message to the mobile base station relay connected by UE.

4.	The mobile base station relay initiates the 5GC-MO-LR procedure in clause 6.2 in TS 23.273 [4] to obtain its location.

5.	The mobile base station relay obtains measurements.

6.	The mobile base station relay sends NRPPa message including mobile base station relay location and time of the location to NG-RAN.

7.	The NG-RAN sends NRPPa message including mobile base station relay location and time of the location to LMF.

8.	The LMF decides that the UE connects to mobile base station relay based on the location in the received NRPPa message and uses the location to calculate UE location.

The steps in box (B) (i.e. steps 9-16) are performed when DL positioning is used to obtain UE location.

9.	The LMF sends NRPPa message to NG-RAN.

10.	The NG-RAN sends NRPPa message to mobile base station relay.

11.	The mobile base station relay returns the NRPPa message including GPSI of the mobile base station relay to NG-RAN.

12.	The NG-RAN sends NRPPa message including GPSI of the mobile base station relay to LMF.

13.	The LMF sends the Ngmlc_Location_ProvideLocation Request including the GPSI to GMLC to obtain the location of the mobile base station relay. The LMF also requests UE velocity.

NOTE 1:	If the serving LMF of UE selects DL positioning, in order to reduce the time difference error, it is recommended that the serving LMF of UE defers the UE positioning procedure (i.e. step 15) some time later after the step 13 is performed. The duration deferred depends on the implementation.

13a.	[Conditional] If MBSR is roaming in VPLMN, the (V)GMLC sends Ngmlc_Location_ProvideLocation Request including the GPSI to HGMLC to obtain the location of the mobile base station relay.

14.	The steps 2-24b in 5GC-MT-LR procedure in clause 6.1.2 in TS 23.273 [4] are performed. After this step, the serving LMF of UE obtains the MBSR location, MBSR velocity and MBSR location time.

NOTE 2:	The step 14 reuses the existing procedure in TS 23.273, no enhancement is introduced.

15.	The LPP procedure is performed for LMF to obtain measurements from UE.

16.	The LMF calculate UE location using the location of the mobile base station relay obtained in step 14 and the measurements obtained in step 15.

NOTE 3:	Assume that the positioning measurements of UE are obtained at time T1, the MBSR location time at step 14 is time T2. In order to reduce the time difference error, it is recommended that the serving LMF of UE calculates MBSR location at the time T1 based on the MBSR location, MBSR velocity and time T2 firstly, and calculates UE location based on the positioning measurements of UE at time T1 and the MBSR location at time T1.

17.	The LMF sends the Nlmf_Location_DetermineLocation Response including UE location to AMF.

### 6.8.4	Impacts on services, entities, and interfaces

AMF: determines the UE connects to MBSR and sends Nlmf_Location_DetermineLocation Request including MBSR indication to LMF.

LMF: determines the UE connects to MBSR when MBSR indication is received from AMF and obtains MBSR location which is used to calculate UE location.

Mobile base station relay: provides its location or UE ID to LMF.

## 6.9	Solution #9: User location information based on IAB-UE

### 6.9.1	Introduction

The solution provides a way to represent the User Location Information, ULI, of UEs connected to the mobile IAB-node based on IAB-UE (i.e. IAB-MT).

### 6.9.2	Functional Description

When mobile IAB-mode moves to a new IAB-donor gNB or new geographic area covered by the same IAB-donor gNB, the cell ID/TAC may change as shown in figure 6.9.2-1.

During the mobility of the IAB-UE part, the new ULI for the IAB-UE is based on a cell/TAC managed by the same or new IAB-donor gNB and it is provided to CN. The ULI of the IAB-UE is always corresponding to a geographic area. And also Warning Area List for the Public Warning System corresponding to the cell/TAC is managed by IAB-donor gNB and CN.

When UEs connected to the IAB-node may be reconfigured by the same or new IAB-donor gNB, with the same or different cell/TAC which is covered by the IAB-node and managed by the same or new IAB-donor gNB, the ULI of the IAB-UE can be used to represent the ULI of the connected UEs, in addition to the ULI of the connected UE, e.g. the cell y is considered as coverage extension of the cell n in figure 6.9.2-1.

NOTE 1:	The cell ID/TAC and Warning Area List handling during IAB-node mobility needs to the coordinated with the work in RAN WGs.

NOTE 2:	No change on Warning Area List is expected in this study to support the Public Warning System.

Figure 6.9.2-1: Cell/TAC for IAB-UE and UE connected to IAB-node

### 6.9.3	Procedures

#### 6.9.3.1	User Location Information on NGAP

There is no new procedure for the additional ULI.

The current ULI information on NGAP and user location information on CN related interfaces are extended to support the additional ULI related to the IAB-MT for UEs connected to IAB-node.

During F1 setup procedure, the gNB-DU includes mobile IAB indication in the F1 SETUP REQUEST message to the IAB-donor gNB, so that the IAB-donor gNB will include additional ULI to the AMF for UEs connected to this IAB-node.

Editor's note:	How the IAB-donor gNB determines whether an IAB node is a mobile IAB will be coordinated with RAN WGs.

In order to distinguish between the ULI of UE and the ULI of IAB-UE in the AMF, the IAB-donor gNB sends the ULI of IAB-UE as a separated User Location Information on NGAP.

Editor's note:	The details of the ULI information of IAB-UE will be coordinated with RAN WGs.

After receiving the additional ULI, the AMF determines the UE location by taking the ULI of IAB-UE into account, such as Registration Area management, UE presence in Area of Interest and providing UE location to other NFs, e.g. SMF during PDU Session Establishment.

When applying mobility management (e.g. Mobility Restrictions) in CN for connected UEs, NF considers the ULI of the connected UE if it's applicable (i.e. the ULI matches the related mobility restriction information), otherwise the additional ULI can be applied.

Editor's note:	Whether NFs need to be aware of the additional ULI information is FFS.

### 6.9.4	Impacts on services, entities and interfaces

AMF:

-	Supports additional ULI besides current ULI and provide the additional ULI to other NFs.

gNB:

-	Supports the determination of mobile IAB during F1 setup procedure.

-	Supports the trigger to provide additional ULI.

-	Supports the formulation of additional ULI for UE connected to IAB-node besides the current ULI.

## 6.10	Solution #10: configuration of Mobile Base Station Relays in roaming

### 6.10.1	Introduction

This solution addresses KI#1 and KI#4 in that it covers both the configuration aspects and the enforcement of roaming policies.

NOTE:	While this solution is defined in this clause to address mainly the roaming case, it can apply to non-roaming also.

### 6.10.2	Functional Description

A Mobile Base Station Relay (MBSR) is an IAB node. An IAB-Nodes consist of two parts, the IAB-UE (which is basically a UE) and IAB-DU. While for the IAB- UE, existing roaming procedures can be re-used to a large extent (i.e. the donor IAB node is authorized to serve the MBSR as such if there is some subscription to authorize it, as detected by the serving AMF), the roaming IAB-DU will appear as a base station in the visited network and will thus interfere with the visited operators' network unless it is properly configured to be compatible with the existing deployments and regulations in the visited country.

Pre-condition: IAB Roaming Agreement

As a pre-condition, we assume an IAB-specific Roaming Agreement needs to be in place before IAB-Node moves into a VPLMN and obtain service in the VPLMN. Roaming agreements for a standard UE are of course quite well understood, but roaming agreements for an IAB-Node are not yet known and require extensions to a standard roaming agreement as the operation of the IAB-DU part needs to be considered (i.e. access node-related configurations including parameters like max transmission power, maximum bandwidth, antenna configurations).

This is particularly important as we will need really to ensure compatibility between the MBSRs visiting a PLMN and the VPLMN OAM. The MBSR may also be preconfigured with the settings that are necessary to operate in VPLMN(s) (with some additional tuning done using the F1 interface once the MBSR is in the VPLMN(s)).  Several options are possible and most likely a discussion will need to take place on which way to go as this will impact the MBSR implementation and deployment requirements.

High level description

The MBSR can be configured by the HPLMN per roaming partner PLMN ID and can be further reconfigured or updated by the VPLMN in such a way that the IAB-DU can be operated in the roaming partner operator's network. This includes reconfiguration of radio resources, e.g. used frequencies, power levels. This configuration by the VPLMN is subject to the MBSR to be an "mobile IAB Node allowed" in the INITIAL UE MESSAGE (in other words the UE context in the RAN includes the authorization of the MBSR to Mobile IAB node).

The MBSR IAB UE can obtain the IP address of an OAM configuration server and e.g. one time authentication credentials to access the local OAM in the Registration Accept message received from the AMF.  The address of the OAM server may be based on the MBSR PEI (e.g., for OAM compatibility reasons).

Editor's note:	The security aspects of any solution, including this one, need SA WG3 evaluation.

### 6.10.3	Procedures

Figure 6.10.3-1: Configuration and authorization of MBSR in a PLMN

1-2.	The IAB-UE initiates a registration of the MBSR and obtains in the registration accept the necessary information on what configuration to activate in the PLMN out of the ones available (if more than one could be possible) and/or the necessary information to be configured by the appropriate OAM server for the MBSR including its IP address (this information is confidentiality and integrity protected so it can include security material for access to the OAM server, e.g. a One Time Password type of credentials). The RAN also obtains the authorization to handle the UE as a MBSR based on subscription checking and Serving PLMN policy (e.g. based on roaming agreements).

3.	The IAB-UE connects via a PDU session to the OAM server, using the provided information by the AMF.

4.	The MBSR IAB-DU is integrated in the serving PLMN RAN and F1 is established.

### 6.10.4	Impacts on services, entities and interfaces

The solution has the following impacts:

MBSR (new): it should be identifiable as a MBSR so it can be authorized in the VPLMN and provided with any indication of which configuration to use or with updated/new configuration by the OAM of the VPLMN.

IAB donor: integration of the MBSR

AMF: authorize the MBSR in the PLMN and provide the OAM server address and the one time authentication material needed to access it.

## 6.11	Solution #11: Service time of Mobile Base Station Relay

### 6.11.1	General

This solution addresses Key Issue #2 for mobility.

Editor's note:	RAN related aspects e.g. SIB message, cell reselection based on the service time, and how the IAB-DU receives the service time information from IAB-UE need RAN coordination.

### 6.11.2	Functional descriptions

For a Mobile Base Station Relay on, e.g., bus, the service time is deterministic for the Mobile Base Station Relay. When it is close to the time that the bus is out of service, the UEs connect to the Mobile Base Station Relay on the bus needs to be aware and ready to reselect cell to continue the service.

### 6.11.3	Procedures

#### 6.11.3.1	Mobile Base Station Relay broadcast service time

Figure 6.11.3.1-1: Mobile Base Station Relay broadcast service time

1.	The subscription of MBSR (UE) has service time information. The MBSR (UE) receives service time information via registration procedure or UE configuration update procedure, i.e., the UDM sends the Subscription Data with additional MBSR service time information to the AMF, and AMF includes the MBSR service time information to the MBSR over NAS message.

2.	The MBSR (DU) setup F1 link with NG-RAN to start providing service based on the service time information.

3.	When the MBSR (DU) is commanded to broadcast SIBs, it includes the service time information in the SIB.

4.	When close to timeout of the service time for the MBSR, the UEs start cell reselection and performs re-establishment procedure for continuing the services.

5.	When timeout, the MBSR (DU) releases the F1 link with NG-RAN to stop the service.

#### 6.11.3.2	Conditional handover according to service time of Mobile Base Station Relay

Figure 6.11.3.2-1: Conditional handover according to service time of Mobile Base Station Relay

1.	The MBSR (UE) sends Registration Request to AMF-MBSR.

2.	The AMF queries the subscription information of the MBSR (UE) from the UDM, which includes service time information, i.e., the Subscription Data retrieved from UDM includes additional MBSR service time information.

3.	The AMF, after receiving the MBSR service time information in the Subscription Data, sends the MBSR service time information to the MBSR (UE) and the NG-RAN (CU). The NG-RAN stores the MBSR service time information in the UE context of the MBSR (UE).

If the MBSR service time information has been updated after registration, the UDM notifies the new MBSR service time information to the AMF, and AMF sends NAS message to the MBSR and N2 message to the NG-RAN (CU) with the updated MBSR service time.

4.	The MBSR (DU) setup F1 link with NG-RAN to start providing service based on the service time information.

5.	When close to timeout of the service time for the MBSR (DU), the NG-RAN (CU) commands the UEs under the MBSR (DU) to perform conditional handover based on time event (see clause 5.3.5.13.4 of TS 38.331 [16]).

6.	When time event is satisfied, the UEs handover to the target cell.

Editor's note:	How the NG-RAN performs conditional HO according to the MBSR service time information is FFS.

### 6.11.4	Impacts on services, entities and interfaces.

UDM:

-	Support subscription data with MBSR service time information.

AMF (serving MBSR):

-	Support sending service time information to UE that supports MBSR over NAS.

-	Support sending service time information to NG-RAN serving the MBSR.

NG-RAN (CU):

-	Support UE context with MBSR service time information.

MBSR:

-	Support broadcasting service time information over the air received via NAS.

-	Support starting and stopping the MBSR feature based on the service time information received via NAS.

## 6.12	Solution #12: IAB-node mobility with connected UEs

### 6.12.1	General

The solution mainly addresses KI#3 to support IAB-node mobility between IAB-donor-CUs (i.e. scenario B) together with all the UEs that still connected to the IAB-node and moves together with the IAB-node (i.e. "Connected UEs").

For scenario A in KI#3, there is no IAB-donor-CU change, there is no visible mobility to 5GC. The main issue is how to provide the new Cell/TAI to 5GC. This is addressed in KI#6.

### 6.12.2	Functional descriptions

The solution covers the two aspects:

-	Mobility of the IAB-UE (i.e., IAB-MT).

-	Mobility of the UEs connected to the IAB-DU.

The resource for Connected UEs is prepared at the target side together with the handover preparation of the IAB-UE. The move (e.g. the reconfiguration) of the Connected UEs is triggered after the target side accepts the IAB-UE move and configured the path at the target side. The Connected UEs may be registered in different AMFs and moves via different AMFs.

NOTE 1:	The details of signalling at Xn/RRC level for IAB-UE and Connected UEs depends on the mobile IAB work in RAN WGs.

During MO traffic for the UE in CM-IDLE mode but was connected via the IAB-node before the UE entered CM-IDLE mode, the UEs applies the normal NAS procedures (e.g. Service Request procedure if UE is still in the current Registration Area, or Registration Request procedure if UE is outside the current Registration Area) to move to CM-CONNECTED mode via the moved IAB-node. For MT traffic, the AMF initiates the paging based on UE's Registration Area as normal.

### 6.12.3	Procedures

#### 6.12.3.1	IAB-node mobility with change of IAB-donor gNB via Xn

In RAN Rel-17 IAB work, the inter-IAB-donor-CU change of IAB-UE is supported due to topology adaptation for a single-connected IAB-node. The IAB-UE switches connection from an old path (e.g. via the old IAB donor node) to a new path (e.g. via new IAB donor node), while the IAB-DU part of the IAB-node remains under the control of the old IAB-donor gNB.

The extra support in this Rel-18 study is to support the switch of the IAB-DU part also from the old IAB-donor-CU to the new IAB-donor-CU. In addition, the Connected UEs also switches from the old path to the new path via the new IAB-donor gNB.

Figure 6.12.3.1-1 below shows an example of the IAB-node moves from old IAB-donor gNB to the new IAB-donor gNB.

Figure 6.12.3.1-1: IAB-node mobility for inter IAB-donor-CU with Xn interface

1.	The source IAB-donor-CU triggers the move of IAB-UE from source path to target path as specified in TS 38.401 [6] for Rel-17. The resource for Connected UEs is also prepared during the handover preparation for the IAB-UE.

Editor's note:	The detailed handling depends on the work in RAN WGs and is FFS.

2.	The target IAB-donor-CU send NGAP Path Switch Request to AMF for the IAB-UE.

3.	Steps in clauses 4.9.1.2.2 and 4.9.1.2.3 of TS 23.502 [5] between AMF/SMF/UPF(s) are performed for IAB-UE.

4.	The AMF sends the NGAP Path Switch Request Acknowledge message to target IAB-donor-CU.

5.	Target IAB-donor-CU trigger the UE context release towards the source IAB-donor-CU which in turn triggers furth release of resource on the source path related to IAB-UE.

6.	The F1-C connection between migration IAB-node and target IAB-donor-CU is established based on new TNL addresses information exchanged in step 1. The target IAB-donor-CU configures BH RLC channels, BAP-sublayer routing entries and mapping rules on the target path.

NOTE 2:	The details of path and configuration management depend on the work in RAN WGs and coordination is needed.

7.	The Xn based move of the group of UEs connected to IAB-node is triggered to switch to the target IAB-donor-CU via the source IAB-donor-CU.

NOTE 3:	The details of the move of UEs connected to IAB-node depend on the work in RAN WGs and coordination is needed.

8.	When UE connects to the target IAB-donor-CU via RRC signalling, the target IAB-donor-CU triggers the path switch procedure towards the 5GC for all the UEs connected to IAB-node. If Connected UEs are served by different AMFs, the IAB-donor gNB may trigger one path switch procedure towards each involved AMF (i.e. one path switch request per AMF). The message may include list of Connected UEs registered in this AMF.

Editor's note:	It is FFS how the target IAB-donor-CU aggregates the information of all the UEs connected to IAB-node in a single path switch request message and coordination with RAN WGs is needed.

9.	The steps in clauses 4.9.1.2.2 and 4.9.1.2.3 of TS 23.502 [5] between AMF/SMF/UPF(s) are performed per UE/PDU session.

Editor's note:	It is FFS if the steps need to be optimised per group.

10.	The AMF sends the NGAP Path Switch Request Acknowledge message to target IAB-donor gNB.

11.	Target IAB-donor-CU triggers the UE context release towards the source IAB-donor-CU which in turn triggers furth release of resource on the source path related to IAB-UE.

#### 6.12.3.2	IAB-node mobility with change of IAB-donor gNB via N2

In the case of IAB-node mobility between IAB-donor gNB without Xn interface, the move of the IAB-UE and Connected UEs can be performed via 5GC using N2 interface.

Figure 6.12.3.2-1 below shows an example of the IAB-node moves from old IAB-donor gNB to the new IAB-donor gNB.

Figure 6.12.3.2-1: IAB-node mobility for inter IAB-donor-CU with N2 interface

1.	The source IAB-donor-CU sends a HANDOVER REQUIRED message for the IAB-UE to the target IAB-donor-CU over the N2 interface. This message may also include a list of UE information for Connected UEs registered in the same AMF as the IAB-UE. This message includes the request of migrating IAB-node's TNL address information in the RRC container. The IAB-donor-CU may also send HANDOVER REQUIRED message to another AMF where one of the Connected UEs registered and include a list of information for other Connected UEs registered in the same AMF.

Editor's note:	It is FFS how source IAB-donor-CU determines the Target ID for the Connected UEs and formulates the detailed information of the Connected UE in HANDOVER REQUIRED message.

2.	The CN prepares the handover for IAB-UE and Connected UEs indicated in step 1 as specified in steps 2-8 of clause 4.9.1.3.2 of TS 23.502 [5].

Editor's note:	It is FFS if the steps need to be optimised per group.

3.	The CN sends the HANDOVER REQUEST message to target IAB-donor-CU, including the transparent container from source IAB-donor-CU and the resource information prepared in CN for IAB-UE and the Connected UEs. If multiple AMFs are involved in step 1, different AMFs will send different HANDOVER Request.

4.	The target IAB-donor-CU performs admission control, allocates resources for IAB-UE and Connected UEs involving target IAB-donor-DU, based on possible multiple messages from different AMFs.

NOTE 1:	The detailed handling of resources for Connected UEs depends on the work in RAN WGs.

5.	The target IAB-donor-CU provides the new RRC configuration for the IAB-UE as part of the HANDOVER REQUEST ACKNOWLEDGE message. The RRC configuration includes a BAP address of the IAB-donor-DU in the target IAB-donor-CU's topology, default BH RLC channel and a default BAP routing ID configuration for UL F1-C/non-F1 traffic mapping on the target path. The RRC configuration may include the new TNL address(es) anchored at the target IAB-donor-DU for the migrating node as requested in step 1. The target IAB-donor-CU also provides the resource information allocated for Connected UEs. The target IAB-donor-CU does not provide RRC related information for Connected UEs in this step.

6.	The CN performs steps 11-12 in clause 4.9.1.3.2 of TS 23.502 [5] for IAB-UE and all connected UEs.

7.	The CN sends the HANDOVER COMMAND to source IAB-donor-CU including the RRC configuration information received in step 5 for the IAB-UE. No RRC related information is included for the Connected UEs.

8.	The source IAB-donor-CU performs step 2-3 in clause 4.9.1.3.3 of TS 23.502 [5] for IAB-UE part of the migrating/mobile IAB-node. The IAB-UE of the migrating/mobile IAB-node performs step 4 of clause 4.9.1.3.3 towards the target IAB-donor-CU. This step is not performed for the Connected UEs.

9.	The target IAB-donor-CU triggers the NGAP HANDOVER NOTIFY message towards the 5GC for the migrating/mobile IAB-UE.

10.	The CN and the IAB-UE of the IAB-node perform steps 6-12 in clause 4.9.1.3.3 of TS 23.502 [5] for the IAB-UE.

11.	The CN triggers NGAP UE CONTEXT RELEASE COMMAND message towards the source IAB-donor-CU which in turn triggers furth release of resource on the source path related to IAB-UE.

12.	The F1-C connection between migration IAB-node (e.g. via a virtual IAB-DU) and target IAB-donor-CU is established based on new TNL addresses information for IAB-node exchanged in step 5. The target IAB-donor-CU configures BH RLC channels, BAP-sublayer routing entries and mapping rules on the target path.

NOTE 2:	The details of path and configuration management depend on the work in RAN WGs.

13.	The target IAB-donor-CU triggers the move of the group of UEs connected to IAB-node to switch to the target IAB-donor-CU via CN and source IAB-donor-CU.

13.1:	The target IAB-donor-CU sends a NGAP message, e.g. Connected UE Reconfiguration Request message to CN (e.g. target AMF) to request the RRC reconfiguration for Connected UEs to the migrating/mobile IAB-node. The message includes RRC Reconfiguration information for Connected UEs. The message includes source IAB-donor-CU as the receiving node. The target IAB-donor-CU may send multiple messages via different AMFs in synch with the number of messages provided in step 5 above.

13.2:	If there is AMF change, the target AMF sends a message to source AMF, e.g. the Namf_COMMUNICATIN_N2infoNotify including the RRC Reconfiguration information for Connected UEs. This step may be executed in different AMFs.

13.3:	The CN (e.g. the source AMF) sends NGAP message, e.g. Connected UE Reconfiguration Command message to the source IAB-donor-CU including the RRC reconfiguration information for Connected UEs. This step may be executed in different AMFs.

The source IAB-donor-CU, via migrating IAB-node, sends the RRCReconfiguration message to each connected UE.

Editor's note:	It is FFS if the communication is linked to one of the specific UE or a separate group-based message is used.

14.	The UEs connect to the migrating IAB-node perform step 4 of clause 4.9.1.3.3 of TS 23.502 [5] towards the target IAB-donor-CU as a response to the RRC reconfiguration in step 13.

15.	The target IAB-donor-CU triggers the HANDOVER NOTIFY message towards 5GC including the list of UEs connected to the IAB-node. The target IAB-donor-CU may perform this step towards different AMFs if connected UEs are handled by different AMFs as in step 13.

Editor's note:	It is FFS if the communication is linked to one of the specific UE or a separate group-based message is used.

16.	The CN performs steps 6-11 in clause 4.9.1.3.3 of TS 23.502 [5] for all connected UEs.

17.	The CN sends NGAP UE CONTEXT RELEASE COMMAND message to the source IAB-donor-CU including the list of UEs connected to the IAB-node. The source IAB-donor-CU triggers the release of resource on the source path. This step may be executed in by different AMFs.

### 6.12.4	Impacts on services, entities, and interfaces

gNB:

-	Supports the inclusion of UEs connected to IAB-node during HO.

-	Supports handover of group UEs connected to the IAB-node.

AMF:

-	Supports HO with group handling.

-	Supports HO without RRC container.

## 6.13	Solution #13: IAB optimization for Mobile Base Station Relays support

### 6.13.1	Introduction

This solution addresses KI#3 in that it proposes to mitigate the effect of IAB Donor change by introducing a mobile CU (m-CU) that acts as CU for Mobile Base Station Relays (MBSRs).

### 6.13.2	Functional Description

The mobility of a Mobile Base Station Relay (MBSR) over a wide area faces the challenge that when it changes IAB Donor, the PDCP and RRC connection of the UEs that it serves is impacted, so the UEs even if stationary inside the vehicle experience potentially nontrivial amount of signalling due to mobility in idle mode (due to need to adapt the TA values to the new gNB assigned values, change PDCP termination and security for user plane) and connected mode. See figure 6.13.2-1.

Figure 6.13.2-1: Mobility among donor IAB nodes: CU changes

The root cause of this mobility related signalling could be mitigated if MBSR, or more precisely DU of the MBSR, is served by a CU that is covering a much wider area. In short, it could be advantageous to offer the homing of MBSR to a dedicated mobile control unit (which can be e.g. called "m-CU") that would control the UEs connected to the MBSR. This would enable the MBSR to move across a much wider RAN coverage area without changing the m-CU. Hence, mobility of the MBSR between IAB Donors could be hidden from the UEs connected to the MBSR as long as the controller remains in the same m-CU like in figure 6.13.2-2.

Figure 6.13.2-2: mobility with m-CU: CU is stable

The analogy of the partial migration supported in Rel-17 whereby an IAB-DU can be homed to a previous IAB Donor CU and the proposed m-CU based solution is shown in Figure 6.13.2-3. The m-CU corresponds to the CU of the IAB Donor gNB on the left having Xn connection to the Donor gNB. According the Rel-18 VMR assumption, the intermediate IAB-node is not existing as the MBSR operation is limited to single-hop topology. Hence, the lower IAB-node corresponds to MBSR, and an intermediate IAB node could exist as long as it is not a MBSR.

Figure 6.13.2-3: Relationship between architecture for partial migration and m-CU

Rel-17 IAB specifications provide suitable basis for such solution without the need for major enhancements to existing procedures and signalling. This is possible due to the distributed nature of the RAN architecture. Referring to the architecture in figure 6.13.2-3, the m-CU corresponds to the source IAB-Donor CU maintaining F1 to the migrating IAB-node DU while the IAB-UE is handed over to the target topology.

During MBSR mobility between Donor nodes, IAB-UE part of the MBSR applies normal handover procedure the same way as in Rel-17 IAB migration. F1 between MBSR and m-CU is maintained during the mobility by moving the transport of the F1 from the source to the target Donor.

RAN WGs may elaborate the degree of required IAB capabilities in RAN (Donor) nodes and functional split between donor and m-CU considering the single-hop assumption.

### 6.13.3	Procedures

There are various options on how this can be supported. RAN groups have eventually to take the lead on specifying the details when they start working on this.

Possible options can imply that when the MBSR is integrated, the AMF validates and authorizes the MBSR to be served by the m-CU based on the MBSR's IAB-UE subscription information. The F1 between the MSBR and the m-CU can be routed via the IAB Donor to m-CU e.g. in a similar manner as in NR-DC or TNL routing in inter-donor partial migration specified for Rel.17 IAB. UE control and bearer termination will be at the m-CU eliminating UE handovers when the BH link/Donor is changed due to the MBSR mobility.

Editor's note:	The detailed procedures need to be discussed with RAN WG3.

Fig. 6.13.3-1 shows an example for the MBSR integration procedure. Alternative procedures may exist. The sub-procedures use either standard signalling or enhanced procedures for the interaction between RAN and m-CU (subject for RAN WGs to specify).

Figure 6.13.3-1: MBSR integration and mobility procedure

MBSR integration:

1.	IAB-UE of the MBSR establishes the connection via the selected RAN node (Donor). At this stage IAB-UE in MBSR behaves like a normal UE, the same way as in Rel-16/17 IAB, and registers with a serving network AMF (identified as AMF-mbsr). The IAB-UE may indicate to be a mobile IAB-node (MBSR) in the connection setup (msg5) and/or NAS Registration message to trigger this behaviour in this message flow, and the AMF confirms the IAB-UE is authorized to act as MBSR by checking the subscription information and local policy.

2.	F1 is setup between the m-CU and MBSR IAB-DU in coordination with the Donor. The BH connection, established between IAB-UE and the Donor, carries F1 between IAB-DU and m-CU. For the F1 the Donor and m-CU configure the transport (TNL) connection between Donor-DU and m-CU. TNL configuration can be similar to NR-DC or TNL configuration for inter-donor partial migration specified in Rel.17 IAB inter-donor topology adaptation.

3.	DU operation and cell(s) are configured with F1AP over established F1 using standard procedures. IAB-DU cells are activated for UEs to camp or connect to.

4.	UE can establish connection via the cells served by MBSR. IAB-DU and m-CU pair form the serving gNB for the UEs. Standard procedures are used without UE impacts. the UE can register with an AMF that in general needs not be the same as the AMF-mbsr, and we identify it as AMF-UE.

MBSR mobility:

5.	the MBSR moves towards the IAB Donor 2 coverage.

6.	the IAB UE of the MBSR sends measurements report to IAB Donor-1 based on the measurement and reporting configuration.

7.	Triggered by the measurement report, source IAB Donor (Donor-1) initiates HO preparation with the target IAB Donor (Donor-2) which indicates the IP address the MBSR will need to use in the target IAB Donor 2 to continue the F1 connectivity. Figure 6.13.3-1 refers (for illustration only) to Xn mobility, but NG mobility support is not excluded.

8.	Normal HO execution procedure is performed. IAB Donor-2 creates HO command and sends it to IAB Donor-1. RRC reconfiguration is sent to IAB UE by IAB Donor-1 to UE (including the IP address to be used for F1) followed by IAB-UE access and sending HO Complete message to the target cell to complete the procedure.

9.	F1 with the m-CU can be continued via the new IAB Donor 2, thus allowing to maintain the original cells served by MBSR.

NOTE:	This example is for the case where the m-CU remains the same (which we assume to be the relevant case in most cases covered by this solution). IAB full migration/mobility (including DU migration/mobility) is required when m-CU is changed (new F1 to be set up); this is an objective of the RAN Rel-18 WI.

10.	UEs served by the MBSR continue being served without the need to know about the MBSR mobility. No UE HOs are needed.

### 6.13.4	Impacts on services, entities and interfaces

The solution has the following impacts:

UE: None.

MBSR: It should be identifiable as a MBSR so it can be homed to the m-CU.

IAB donor: Provides access link to IAB-UE of the MBSR for wireless backhaul (for F1 to MBSR) and maintains it during the MBSR mobility (HO procedure). Interaction with m-CU to configure the transport for the F1 between m-CU and MBSR.

AMF: Authorize the IAB node is mobile IAB node and can be served in the country.

m-CU: new function: It acts as CU for the MBSR's IAB-DU.

## 6.14	Solution #14: Support of location services for UEs accessing via a mobile IAB node

### 6.14.1	Introduction

This solution addresses the KI#5: Support of location services for UEs accessing via a mobile IAB node.

### 6.14.2	Functional Description

This solution is based on the following principles:

-	The IAB-UE provides the serving Cell ID of IAB node to the core network. The NRF stores the association of IAB-UE ID, the identifier of AMF serving IAB-UE and Cell ID of IAB node.

-	The UE-AMF determines the UE is accessing via a mobile IAB node based on the additional ULI related to the IAB-UE included in the NGAP message as described in the Solution#9. When the location reporting procedure for UE is needed, the UE-AMF queries the IAB-UE ID and the identifier of AMF serving IAB-UE from NRF using Cell ID that UE is camping.

-	The UE-AMF sends Location Request to the LMF including the LCS Correlation identifier for UE, IAB-UE ID and identifier of AMF serving IAB-UE. The LMF additionally performs IAB-UE positioning procedure.

-	The LMF determines the location of UE by taking the location of the IAB-UE into account.

### 6.14.3	Procedures

Figure 6.14.3-1: Positioning procedure for UE accessing via mobile IAB node

1.	After the cell is configured and activated in the mobile IAB-node, the IAB-UE initiates Registration procedure and provides the Cell ID of IAB node to the 5GC. The AMF serving IAB-UE (i.e. IAB-AMF) sends the Cell ID of IAB node, IAB-UE ID (i.e. SUPI) and the identifier of AMF serving IAB-UE to the NRF.

If the Cell ID of IAB node is changed during mobility, the IAB-UE provides the new Cell ID to the IAB-AMF to trigger the cell information update in the NRF.

NOTE 1:	It is assumed the NRF is at PLMN level and it is accessible by any AMF within the whole Serving PLMN. The ID of PLMN level NRF is pre-configured in the AMFs.

2.	The UE is camped on the cell served by the mobile IAB-node and registered to the network.

3.	The AMF serving UE (i.e. UE-AMF) initiates the location reporting procedure for UE, as defined in the 5GC-MT-LR procedure or the 5GC-MO-LR procedure of TS 23.273 [4].

The UE-AMF determines the UE is accessing via mobile IAB-node based on the additional ULI related to the IAB-UE included in the NGAP message as described in the Solution#9.

4.	The UE-AMF sends Request message to the NRF including the Cell ID that UE is camping.

5.	The NRF returns the IAB-UE ID and the identifier of AMF serving IAB-UE corresponding to the Cell ID.

6.	The UE-AMF selects LMF and invokes Nlmf_Location_DetermineLocation Request service operation towards the LMF including the LCS Correlation identifier for UE, the IAB-UE ID and the identifier of AMF serving IAB-UE.

7.	The LMF performs one or more of the positioning procedures as described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [4].

8.	The LMF triggers the AMF serving IAB-UE to initiate the location reporting procedure for IAB-UE based on the identifier of AMF serving IAB-UE received in the step 6. The UE positioning procedure in step 7 and IAB-UE positioning procedure in step 8 are performed at the same time.

During this step the LMF receives the location of the IAB-UE.

9.	The LMF determines the location of UE by taking the location of the IAB-UE into account and provides the UE location to the AMF serving the UE.

NOTE 2:	How the LMF determines the location of UE by taking the location of the IAB-UE into account is up to LMF implementation.

### 6.14.4	Impacts on services, entities and interfaces

IAB-UE:

-	Supports providing current Cell ID of IAB node to the AMF.

IAB-AMF:

-	Supports sending the Cell ID of IAB node to the UDM.

PLMN level NRF:

-	Supports the storage of the mapping between a Cell ID and an IAB node.

-	Supports finding an IAB-UE ID and the identifier of AMF serving IAB-UE based on Cell ID.

UE-AMF:

-	Supports requesting the IAB-UE ID and the identifier of AMF serving IAB-UE from UDM using Cell ID of IAB node.

LMF:

-	Supports triggering the positioning procedure for IAB-UE based on the IAB-UE ID and the identifier of AMF serving IAB-UE.

-	Determines the location of UE by taking the location of the IAB-UE into account.

## 6.15	Solution #15: Support of location services for MBSR access with provision of MBSR Location and RAT dependent positioning

### 6.15.1	Introduction

The solution uses existing procedures defined in TS 23.273 [4] and TS 38.305 [17] to locate a UE with MBSR (e.g. VMR) access using RAT dependent positioning, where the MBSR location is used to help determine the UE location. A subset of the solution can be used with RAT independent position methods (e.g. A-GNSS or WLAN), though this is not described here.

Since the MBSR may be moving, a current (rather than previous) location of the MBSR may be critical to obtaining an accurate UE location. Preferably, the MBSR location is obtained at the same time as any location measurements obtained by the UE of the MBSR or any location measurements obtained by the MBSR of the UE. If the MBSR location is instead obtained some time before or after these location measurements are obtained, there may be an additional error in locating the UE. For example, if the UE is stationary but the MBSR (e.g. a VMR attached to a vehicle) is moving at 60kph and the MBSR location is obtained 3 seconds before or 3 seconds after the UE location measurements are obtained, then the MBSR location at the time the UE location is measured will differ from the obtained MBSR location by 50 meters. If instead the MBSR is travelling at 120kph (e.g. on a highway) and the time error is 5 seconds, the MBSR location error would be 167 meters. These errors may produce similar errors to the determined UE location - e.g. if the UE location is obtained based only on measurements for or by the MBSR.

### 6.15.2	Functional Description

Location procedures as currently defined in TS 23.273 [4] and TS 38.305 [17] are reused with the following additions:

(a)	It is assumed the position and velocity of MBSR can be determined in this solution.

NOTE 1:	The position and velocity of MBSR may be determined by the MBSR or the network. The supported methods should be defined by RAN.

(b)	The AMF or the UE informs the LMF that the UE is accessing an MBSR.

(c)	Based on the MBSR indication, the LMF requests and obtains the MBSR positioning capabilities when acting as a TRP using NRPPa [14] and F1AP [18].

NOTE 2:	The details above are within the scope of RAN3 who would need to evaluate and determine the precise support.

(d)	If the UE is positioned using measurements by the MBSR of UL SRS (or other signals) transmitted by the UE, the location and velocity of the MBSR at the time of these measurements is reported to the LMF along with the measurements.

NOTE 3:	The position and velocity of MBSR may be determined by the MBSR or the network. The supported methods should be defined by RAN.

(e)	If the UE is positioned using measurements by the UE of DL PRS (or other signals) transmitted by the MBSR, the UE indicates the time of measurement to the LMF. The LMF then determines the location of the MBSR at that time either from a location and velocity provided as in bullet (d) or by querying the MBSR for the MBSR location at that time.

NOTE 4:	While transmitting DL PRS, the MBSR might need to maintain and store a short location history to allow the LMF query in bullet (e). The support of indication of time of measurement will be defined by RAN.

### 6.15.3	Positioning Procedure

A generalized procedure to support positioning of a UE accessing an MBSR that is applicable to a 5GC-MT-LR, 5GC-MO-LR, 5GC-NI-LR and deferred 5GC-MT-LR as defined in TS 23.273 [4] is shown in Figure 6.15.3-1.

Figure 6.15.3-1: Positioning Procedure for a UE with MBSR access

1.	A trigger for either locating the UE or reporting a UE location event occurs at the AMF. The trigger can be a request for an 5GC-MT-LR received from a GMLC as at step 5 in clause 6.1.2 of TS 23.273 [4], a request for a 5GC-MO-LR received from the UE as at step 2 in clause 6.2 of TS 23.273 [4], a trigger for a 5GC-NI-LR as at step 1 in clause 6.10.1 of TS 23.273 [4], or an event report sent by the UE for a deferred 5GC-MT-LR for periodic or triggered location as at step 25 in clause 6.3.1 of TS 23.273 [4].

2.	The AMF selects an LMF. Except for a deferred 5GC-MT-LR for periodic or triggered location where the LMF may be preassigned, the AMF selects an LMF and may select an LMF able to support location for MBSR access if the AMF is aware (e.g. from the donor gNB) that the UE has access via an MBSR.

3.	For a 5GC-MT-LR, 5GC-MO-LR or 5GC-NI-LR, the AMF sends a location request to the LMF using a. Nlmf_Location_DetermineLocation Request service operation and may include parameters such as serving cell ID, LCS Correlation identifier and required QoS as defined in TS 23.273 [4]. For a deferred 5GC-MT-LR for periodic or triggered location, the AMF forwards the event report received form the UE to the LMF. For both cases, the AMF indicates MBSR access if the AMF is aware of this.

4.	The LMF obtains the UE positioning capabilities using LPP according to the procedure in clause 6.11.1 of TS 23.273 [4]. The UE, if aware, may indicate MBSR access to the LMF.

5.	If MBSR access is indicated at either step 3 or step 4, the LMF requests the positioning capabilities of the MBSR when functioning as a TRP using an NRPPa procedure (TRP Information exchange) defined in TS 38.455 [14] to the donor gNB, and a corresponding F1AP procedure defined in TS 38.473 [18] from donor gNB to MBSR. The procedure is modified as described in clause 6.15.2 bullet (c) to allow the LMF to indicate to the donor gNB that it applies to the MBSR. The MBSR returns its positioning capabilities which indicate, for example, a capability to act as a TRP that can configure and transmit DL PRS to the UE and/or configure and measure UL SRS transmitted by the UE. A TRP ID for the MBSR is also assigned by the donor gNB and returned to the LMF.

NOTE 1:	The TRP ID allows the LMF to indicate the MBSR to the donor gNB at steps 7, 8 and 15 and allows the MBSR to identify itself at step 14.

NOTE 2:	The details of LMF requesting the positioning capabilities of the MBSR should be decided by RAN.

6.	The LMF selects one or more position methods supported by the UE and the MBSR as indicated by the positioning capabilities received at steps 4 and 5. If position methods are selected that are not supported by the MBSR, the procedure here is no longer applicable and is not used.

7.	Depending on the position methods selected at step 6, the LMF may use existing procedures in TS 38.455 [14] and TS 38.473 [18] to configure transmission of DL PRS by the MBSR (and possibly from other nearby gNBs) and/or to configure the donor gNB, which in turn configures via RRC the transmission of UL SRS by the UE. These procedures make use of sending and received NRPPa messages to and from the donor gNB using the procedure in clause 6.11.2 of TS 23.273 [4].

8.	If the LMF configured transmission of UL SRS by the UE at step 7, the LMF uses existing procedures in TS 38.455 [14] and TS 38.473 [18] to request measurements of the UL SRS by the MBSR and optionally by other nearby gNBs. The LMF makes use of the procedure in clause 6.11.3 of TS 23.273 [4] to exchange NRPPa messages with the donor gNB (and other gNBs) for this purpose.

9.	If the LMF configured transmission of DL PRS by the MBSR at step 7, the LMF uses existing procedures in TS 37.355 [13] to request measurements of the DL PRS by the UE and optionally of DL PRS from other nearby gNBs. The LMF makes use of the procedure in clause 6.11.1 of TS 23.273 [4] to exchange LPP messages with the UE for this purpose.

10.	The UE transmits UL SRS if requested at step 7 and/or measures DL PRS from the MBSR (and other gNBs) if requested at step 9.

11.	The MBSR transmits DL PRS if requested at step 7 and/or measures UL SRS from the UE if requested at step 8.

12.	Other gNBs may transmit DL PRS if requested at step 7 and/or measure UL SRS from the UE if requested at step 8.

13.	If the UE measured DL PRS from the MBSR (and other gNBs) at step 10 as requested at step 9, the UE returns the measurements to the LMF according to the procedure in clause 6.11.1 of TS 23.273 [4] and includes the time (or times) of the measurements.

14.	If the MBSR (and other TRPs or gNBs) measured UL SRS from the UE at step 11 (and step 12) as requested at step 8, the MBSR (and other TRPs) return the measurements to the donor gNB, which (and other gNBs) in turn returns them to LMF according to the procedure in clause 6.11.3 of TS 23.273 [4]. The MBSR(s) participated in the measurement also includes the MBSR location at the time of measurement and the MBSR velocity.

NOTE 3:	The support of reporting location and velocity of the MBSR over F1AP will be defined by RAN WGs.

15.	If the LMF receives UE measurements of DL PRS transmitted by the MBSR at step 13 along with a time of measurement, the LMF may request the location of the MBSR from the MBSR for the time of measurement using a modified NRPPa procedure for obtaining the location of a TRP. This may not be needed, however, if the MBSR provides a location and velocity for the MBSR at step 14, because the LMF can then estimate the location of the MBSR at the time of measurement using this location and velocity.

NOTE 4:	The feasibility to allow LMF to obtain the location and velocity of MBSR for the time of measurement will be evaluated by RAN.

16.	The LMF determines the UE location using the measurements received at steps 13 and/or 14. The LMF also uses the location and velocity of the MBSR received at step 14 and/or the MBSR location received at step 15 to determine MBSR location(s) as input for the determination of the UE location.

17.	For a 5GC-MT-LR, 5GC-MO-LR or 5GC-NI-LR, the LMF returns the UE location to the AMF. For a deferred 5GC-MT-LR for periodic or triggered location, the LMF may forward the event report and the UE location to a GMLC to send on to an LCS Client or AF.

### 6.15.4	Impacts on services, entities, and interfaces

AMF:

-	receive an indication of an MBSR from the donor gNB and transfer to the LMF.

-	optionally select an LMF that supports location for MBSR access.

LMF:

-	support a request for the TRP capabilities of an MBSR.

-	receive and make use of a location and velocity of an MBSR reported in measurements of UL SRS.

-	request a location of an MBSR for a previous time.

Donor gNB:

-	enable a TRP capability request for an MBSR from the LMF and assign a TRP ID to the MBSR.

-	support of location and velocity report of the MBSR via F1AP.

MBSR:

-	support TRP functions as defined in TS 38.455 [14] and TS 38.473 [18] including configuration and transmission of DL PRS and/or configuration and measurement of UL SRS.

-	measure and provide a location and velocity of the MBSR when reporting measurements of UL SRS to the LMF.

-	provide a location of the MBSR for an indicated earlier time of transmitting DL PRS if requested by the LMF.

UE:

-	optionally indicate MBSR access to an LMF.

-	report a time of measurement of DL PRS from the MBSR to the LMF.

## 6.16	Solution #16: UE mobility management in case of IAB-node mobility with dynamic TAC provisioning

### 6.16.1	Introduction

The solution addresses the Key Issue#3 and provides methods for dynamic TAC provisioning during the mobility of the mobile base station relay (MBSR), in which the TAC broadcasted by the MBSR is the same as the TAC of the cell where the IAB-UE is located. In other words, the TAC broadcasted by the MBSR varies with the TAC where the IAB-UE is located.

The UEs inside the vehicle served by the MBSR perform Mobility Registration Update normally upon changing to a new TA outside the UE's Registration Area. The signaling overhead caused by the Mobility Registration Update for the UEs inside the vehicle depends on the velocity of vehicle.

When the MBSR is moving to an area in the vicinity of the surrounding UEs (outside the vehicle), the surrounding UEs camping on the cell of IAB-donor gNB will not initiate Mobility Registration Update because the MBSR currently broadcasts a TAC that is same as the TAC broadcasted by the IAB-donor gNB. Similarly, when the surrounding UEs disconnect from the MBSR and camp on the cell of IAB-donor gNB, no Mobility Registration Update will be initiated. Therefore, there is no signaling overhead caused by the Mobility Registration Update for the surrounding UEs regardless of connecting to or disconnecting from a MBSR.

### 6.16.2	Functional Description

The solution is based on the support of existing IAB architecture as specified in TS 23.501 [2] with the following high-level descriptions:

-	The IAB-donor gNB determines the TA where the MBSR (i.e IAB-UE) is located. During F1 Setup procedure as specified in the TS 38.401 [6], the gNB-DU includes a mobile IAB indication in the F1 SETUP REQUEST message to the IAB-donor gNB so that the IAB-donor gNB configures the MBSR's cell using the TAC where IAB-UE is located.

-	When the IAB-UE enters to a new TA, the cell broadcasting information is updated accordingly. The TAC configuration update is performed based on the following:

-	When the MBSR enters to a new TA supported by the same IAB-donor gNB, the IAB-donor gNB provisions the new TAC towards MBSR using GNB CU Configuration Update as specified in TS 38.401 [6].

-	When the MBSR enters to a new TA supported by a different IAB-donor gNB, the IAB-donor gNB configures the cell's TAC of MBSR during F1 Setup procedure.

Editor's note:	The details of dynamic TAC provisioning will be coordinated with RAN WGs.

The following Figure 6.16.2-1 shows an example of the dynamic TAC provisioning based on the TAC where the IAB-UE is located. When the MBSR is moving, the IAB-UE is camping on different cells (i.e. Cell#1, Cell#2, Cell#3 and Cell#4) supporting different TACs.

-	The IAB-UE is located in the Cell#1 and the MBSR starts to serve UEs by broadcasting the TAC#1.

-	When the IAB-UE moves from the Cell#1 to the Cell#2 within the same TAC, i.e. Intra-gNB-CU Mobility without TAC change, the TAC broadcasted by the MBSR is not changed. In order to accurately reflect the geographic location of the UE connected to MBSR, additional ULI (i.e. the ULI of the IAB-UE) in the Solution#9 can be used.

-	When the IAB-UE moves from the Cell#2 to the Cell#3, i.e. Intra-gNB-CU Mobility with TAC change, the IAB-donor gNB#1 updates the TAC configuration towards MBSR and the TAC broadcasted by the MBSR changes from the TAC#1 to the TAC#2.

-	When the IAB-UE moves from the Cell#3 to the Cell#4, i.e., from IAB-donor gNB#1 to the IAB-donor gNB#2, the TAC broadcasted by the MBSR is managed by the IAB-donor gNB#2.

Figure 6.16.2-1: Dynamic TAC provisioning based on the MBSR's location

### 6.16.3	Procedures

Figure 6.16.3-1: Dynamic TAC provisioning

1.	The MBSR (i.e. IAB-UE) camps in the normal cell and registers to the 5GC.

2.	The MBSR connects to the OAM for the IAB node configuration.

3.	The MBSR (i.e. IAB-DU) sends an F1 SETUP REQUEST message which includes a mobile IAB indication to the IAB-donor gNB. The IAB-donor gNB determines the TA where the IAB-UE is located and sends an F1 SETUP RESPONSE message including the TAC configuration for the IAB-DU.

The TAC included in the TAC configuration is same as the TAC where the IAB-UE is located.

4.	The MBSR broadcasts the TAC configured by the IAB-donor gNB.

5.	The UE registers to the 5GC via the MBSR.

6.	The MBSR (i.e. IAB-UE) enters to a new TA.

7.	If the new TA is supported by the same IAB-donor gNB, the IAB-donor gNB provisions the new TAC towards MBSR using GNB CU Configuration Update as specified in the TS 38.401 [6].

If the new TA is supported by a different IAB-donor gNB, the IAB-donor gNB provisions the TAC towards MBSR during F1 Setup, which is same as the step 3.

8.	The MBSR broadcasts the new TAC configured by the IAB-donor gNB.

9.	[Conditional] The UE performs the Mobility Registration Update upon changing to a new TA outside the UE's Registration Area.

### 6.16.4	Impacts on services, entities, and interfaces

MBSR:

-	Supports to include the mobile IAB indication to the IAB-donor gNB during F1 setup procedure.

-	Supports to receive the TAC configuration from the IAB-donor gNB.

-	Supports to broadcast the TAC provisioned by the IAB-donor gNB.

IAB-donor gNB:

-	Supports the determination of mobile IAB node during F1 setup procedure.

-	Supports to configure the cell information (i.e. TAC) of MBSR based on the TAC that MBSR is located.

## 6.17	Solution #17: IAB-node mobility with dedicated TAC

### 6.17.1	Introduction

The solution addresses the Key Issue#3 for UE mobility optimization when served by MBSR. This solution assumes that the MBSR operates with dedicated TAC(s). For the details, there are two options in this solution:

-	Option#1:it provides methods for UE mobility in case of the TAC broadcasted by the MBSR is unchanged during mobility. In other words, the MBSR is configured with a dedicated TAC which is unique per MBSR. Regardless of whether the IAB-donor gNB changes or not, this dedicated TAC broadcasted by the MBSR is fixed once the MBSR is operational.

-	Option#2: it is assumed that the MBSR uses the same TAC within a certain service area. The service area of dedicated TAC depends on the presence of N2 reference point between new IAB-donor gNB and AMF-UEs. If no N2 reference point can be established, the MBSR will change the TAC to broadcast during the full migration phase.

### 6.17.2	Functional Description

#### 6.17.2.1	TAC broadcasted by the MBSR is unchanged during mobility

The option is based on the support of existing IAB architecture as specified in TS 23.501 [2] with the following high-level description:

-	It is assumed that the OAM configures a dedicated TAC for the MBSR. The dedicated TAC is unique per MBSR and different from the TACs supported by the normal cells.

-	During F1 setup procedure, the gNB-DU includes a mobile IAB indication in the F1 SETUP REQUEST message to the IAB-donor gNB and also provides the dedicated TAC as the cell configuration information. The IAB-donor gNB shall perform gNB Configuration Update towards 5GC to add the dedicated TAI into the Supported TA List of donor gNB. In addition, IAB-donor gNB indicates the 5GC that the dedicated TAI is MBSR specific. Similarly, when the MBSR releases the F1 connection with IAB-donor gNB, the IAB-donor gNB shall perform gNB Configuration Update towards 5GC to delete the dedicated TAI from the Supported TA List of donor gNB. In other words, the dedicated TAC of MBSR enables the 5GC to detect whether UE is accessing via MBSR. Meanwhile, the IAB-donor gNB connecting with the MBSR can be tracked using the Supported TA List of donor gNB.

Figure 6.17.2.1-1: IAB-node mobility with dedicated TAC

-	When UE registers to the 5GC via the MBSR, the IAB-donor gNB selects a suitable AMF for UE. The UE-AMF knows that the UE is camping on the cell of MBSR based on the UE's TAI in the N2 message. The UE-AMF allocates Registration Area for UE connected to the MBSR which only includes UE's TAI (i.e the Registration Area includes a single TA). When UE disconnects from the MBSR and camps on a different cell, the UE initiates Mobility Registration Update upon changing to a new TA. The UEs in the vehicle connected to the MBSR will not initiate Mobility Registration Update, even when MBSR are served by cells of a new TA outside the UE's Registration Area during the mobility of MBSR. However, the UE still perform non-mobility related registration, e.g. periodic registration update, as defined in TS 23.501 [2].

-	When the MBSR moves from the source IAB-donor gNB to the target IAB-donor gNB, the dedicated TAI of MBSR is supported at the target IAB-donor gNB. There are two cases as the following:

-	Case#1: The target IAB-donor gNB has the N2 connection with the UE-AMF. In this option, the UE-AMF (i.e. AMF#1 in the Figure 6.17.2.1-2) can track the UE/MBSR location based on the Supported TA List of donor gNB via gNB Configuration Update procedure. The UE mobility management can be fulfilled within the same UE-AMF, such as the UE-AMF can decide whether to update UE configuration information such as Access and Mobility Management related parameters due to UE mobility. When the conditions to trigger paging are met, the UE-AMF sends the paging request message to the target IAB-donor gNB.

Figure 6.17.2.1-2: Intra UE-AMF mobility

-	Case#2: The target donor gNB has no N2 connection with the UE-AMF. In this option, the UE-AMF cannot track the dedicated TAI via gNB Configuration Update procedure due to the lack of N2 connection. In order to manage UE mobility and UE reachability, the 5GC needs some enhancements to trigger UE-AMF relocation (i.e. the UE-AMF changes from AMF#1 to the AMF#3 in the Figure 6.17.2.1-3). The details of UE-AMF relocation are described in the clause 6.17.3.1.2.

Figure 6.17.2.1-3: Inter UE-AMF mobility

#### 6.17.2.2	Using same TAC within a certain service area

Figure 6.17.2.2-1 shows an example operation of the option based on time sequence of 1 to 4. The UE1, MBSR, and AMF-UE 1 and AMF-UE 2 are the same entities at different time instances (1 to 4) in the figure.

Figure 6.17.2.2-1: Avoid mobility registration updates for UEs moving with the MBSR

This option follows the below general principles:

-	The MBSR is configured to use a specific TAC reserved for MBSR use in the serving PLMN. The TAC is different from that used by the fixed cells and does not change even when the MBSR migrates to different a NG-RAN.

-	When the MBSR registers to the serving PLMN, it may be provisioned with the TAC to use. The MBSR can inform the donor gNB-CU of the TAC via F1-AP signaling. Within the context for the MBSR, the donor gNB-CU allocates an MBSR-ID for the MBSR and stores the TAC.

Editor's note:	The necessity of MBSR-ID allocation by the donor gNB-CU and the details of the MBSR-ID are FFS.

-	Related to KI#2, when a UE performs registration via the MBSR, the donor gNB-CU includes the TAC in the Supported TA List of NG SETUP REQUEST message, or the NG CONFIGURATION UPDATE message, as defined in TS 38.413 [19] depending on whether a N2 reference point with the selected AMF-UE has already been established. The donor gNB-CU adds the AMF-UE ID (e.g. AMF Set ID and AMF Pointer) to the context of the MBSR. When other UEs perform registration and select the same AMF, this procedure can be skipped.

-	In the INITIAL UE MESSAGE, the donor gNB-CU provides the MBSR indication (this may be different from the IAB node Indication) and the MBSR-ID to the AMF-UE. The AMF-UE stores the information and ensures that the TAC for MBSR is included in the TAI list sent to the UE. When selecting the AMF-UE, the gNB-CU takes the MBSR indication into account, so that the AMF-UE can serve a larger area.

-	Related to KI#3, when the MBSR changes donor gNB-CU, it keeps announcing the same TAC. Therefore, the UEs will not perform mobility registrations.

-	During the MBSR mobility, the MBSR context will be forwarded to the new donor gNB-CU, if full migration is performed. The old donor gNB-CU may perform NG CONFIGURATION UPDATE to remove the TAC from the AMFs, if no other MBSRs context using the same TAC exist on this donor gNB-CU.

-	When the new donor gNB-CU receives the MBSR context in the HO preparation phase, it checks if the TAC for MBSR has already been reported to the AMF-UEs in the MBSR context. The donor gNB-CU may perform NG SETUP REQUEST or the NG CONFIGURATION UPDATE to inform the AMF-UEs of the TAC for MBSR, if needed.

-	If the new donor gNB-CU determines that it cannot establish the N2 reference point with the AMF-UEs, it indicates that to the MBSR. The MBSR will then change the TAC to broadcast during the full migration phase. This triggers the UEs camped on the MBSR to perform mobility registration and a new AMF-UE to be selected. A new 5G-GUTI will be allocated to the UEs as the result of this procedure.

Editor's note:	The details of TAC change during the full migration phase are FFS and should be coordinated with RAN WGs.

-	When a MT service for a UE is required, the AMF-UE is triggered to send a paging message for the UE. The Paging message will be sent to the latest donor gNB-CU that has registered the TAC of the MBSR with the AMF-UE. Optionally, the Paging message can carry also the MBSR-ID, so that the donor gNB-CU receives this Paging messages only triggers over-the-air paging in the corresponding MBSR cell.

NOTE 1:	If no MBSR-ID is provided, the gNB-CU will trigger paging in all cells using this MBSR TAC.

NOTE 2:	If there are multiple MBSRs using the MBSR TAC in the same PLMN, the AMF may trigger paging towards all the MBSRs registered to it with the MBSR TAC, to support UE mobility between the MBSRs using the same TAC without registration.

Editor's note:	When UE leaves the MBSR and camps on a normal cell whose TAI is also included in the TAI list, the idle UE will not initiate Mobility Registration Update.

-	Related to KI#2, when a UE leaves the MBSR and connects/camps on a fixed cell, it is triggered to perform mobility registration, due to the different TAC used by the fixed cell. Existing procedures as defined in TS 23.502 [5] can be used.

### 6.17.3	Procedures

#### 6.17.3.1	Procedures if the TAC broadcasted by the MBSR is unchanged during mobility

##### 6.17.3.1.1	Procedure of intra UE-AMF mobility

Figure 6.17.3-1: Intra UE-AMF mobility

It is assumed that the AMFs (i.e. AMF#1 and AMF#2) connecting with donor gNB#1 are the same as the AMFs connecting with donor gNB#2.

1.	The MBSR (i.e. IAB-UE) registers to the AMF#2 and receives the cell configuration from the OAM including the dedicated TAI. During the F1 Setup procedure, the gNB-DU includes mobile IAB indication in the F1 SETUP REQUEST message to the IAB-donor gNB#1 and also provides the dedicated TAI as the cell configuration information.

2.	The IAB-donor gNB#1 initiates gNB Configuration Update procedure towards 5GC (i.e. AMF#1 and AMF#2) to add the dedicated TAI of the MBSR into the Supported TA List of donor. In addition, IAB-donor gNB#1 indicates the 5GC that the dedicated TAI is MBSR specific. The involved AMF (i.e., AMF#1 and AMF#2) updates its profile (e.g., supported TAI) in the NRF as defined in TS 23.502 [5].

3.	The MBSR starts to serve UEs by broadcasting the dedicated TAI. The UE camps on the cell of MBSR and registers to the UE-AMF, i.e. AMF#1. The AMF#1 knows that the UE is accessing via MBSR based on the UE's TAI in the N2 message and allocates the Registration Area for the UE, which only includes the dedicated TAI of MBSR.

4.	The MBSR (i.e. IAB-UE) moves from the IAB-donor gNB#1 to the IAB-donor gNB#2. The UEs in CM-IDLE state served by the MBSR will not initiate Mobility Registration Update.

5.	This step is the same as the step 2. The MBSR performs F1 Setup with the IAB-donor gNB#2 by providing mobile IAB indication and the dedicated TAI to the donor gNB#2. The donor gNB#2 initiates gNB Configuration Update procedure towards 5GC to add the dedicated TAC into the Supported TA List. In addition, IAB-donor gNB#2 indicates the 5GC that the dedicated TAI is MBSR specific. Similar as Step 2, AMF further interacts with NRF to update its profile.

6.	The MBSR performs F1 Removal with IAB-donor gNB#1. The IAB-donor gNB#1 initiates gNB Configuration Update procedure towards 5GC to delete the dedicated TAI from the Supported TA List. In addition, the IAB-donor gNB#1 includes the IAB-donor gNB#2 ID to indicate 5GC that the IAB-donor gNB#2 supports the dedicated TAI. Similar as Step 2, AMF further interacts with NRF to update its profile.

NOTE:	It is assumed that the IAB-donor gNB#1 knows the IAB-donor gNB#2 information during the mobility of MBSR, such as based on the handover procedure of IAB-UE.

7.	The AMF#1 determines the UE is located in the IAB-donor gNB#2 based on the UE's last TAI included in the UE context stored in the AMF#1 and decides to continue serving UE.

8.	The AMF#1 determines the conditions to trigger paging are met.

9.	The AMF#1 sends the paging request message to the IAB-donor gNB#2 to trigger paging.

10.	The UE initiates the UE Triggered Service Request procedure.

##### 6.17.3.1.2	Procedure of inter UE-AMF mobility

Figure 6.17.3-2: Inter AMF mobility

It is assumed that the AMFs (i.e. AMF#3 and AMF#4) connecting with donor gNB#2 are different from the AMFs connecting with donor gNB#1(i.e. AMF#1 and AMF#2).

1-4.	The step 1 to step 4 are same as the step 1 to step 4 in the Figure 6.17.3-1.

5.	The MBSR performs F1 Setup with the IAB-donor gNB#2 by providing mobile IAB indication and the dedicated TAI to the donor gNB#2. The donor gNB#2 initiates gNB Configuration Update procedure towards 5GC (i.e. AMF#3 and AMF#4) to add the dedicated TAI into the Supported TA List. In addition, IAB-donor gNB#2 indicates the 5GC that the dedicated TAI is MBSR specific. The involved AMF (i.e., AMF#3 and AMF#4) updates its profile (e.g., supported TAI) in the NRF as defined in TS 23.502 [5].

6.	This step is the same as the step 6 in the in the Figure 6.17.3-1.

7.	The AMF#1 determines the UE is located in the IAB-donor gNB#2 based on the UE's last TAI included in the UE context stored in the AMF#1. However, there is no N2 connection between with AMF#1 and donor gNB#2 so that UE-AMF relocation is needed.

8.	The AMF#1 selects target AMF (e.g. AMF#3) via querying NRF using the dedicated TAI supported by IAB-donor gNB#2. The AMF relocation is performed to transfer UE context.

9.	The target AMF notifies the other NFs such as SMF, of the target AMF information.

10.	The target AMF allocates new 5G-GUTI for UE and registers the Serving AMF ID in the UDM.

11.	The target AMF initiates UE Configuration Update procedure to send the new 5G-GUTI and other Access and Mobility Management related parameters, if needed.

#### 6.17.3.2	Procedures using same TAC within a certain service area

The UE registration update uses the legacy procedure of clause 4.2 of TS 23.502 [5].

The IAB-donor updates the AMF with the list of supported TAs can use the existing NG Setup procedure of clause 8.7.1 or NGAP RAN Configuration Update procedure of clause 8.7.2 of TS 38.413 [19].

The MBSR reports its broadcast TAC(s) to the serving IAB-donor using the existing F1 Setup procedure of clause 8.2.3 of TS 38.473 [18].

The MBSR migration to a different IAB-donor via full migration procedure is to be defined by RAN WGs. The forwarding of MBSR context containing the AMF-UE ID lists between IAB-donors is to be defined by RAN WGs as part of the full migration and MBSR HO procedure.

### 6.17.4	Impacts on services, entities, and interfaces

For option#1:

MBSR:

-	Supports to include the mobile IAB indication to the IAB-donor gNB during F1 setup procedure.

-	Supports to broadcast a dedicated TAI, which is not changed during the mobility of MBSR.

IAB-donor gNB:

-	Supports the determination of mobile IAB node during F1 setup procedure.

-	Supports to indicate the MBSR specific TAI to the 5GC during gNB Configuration Update procedure.

AMF:

-	Supports to know the MBSR specific TAI during gNB Configuration Update procedure.

-	Supports to allocate a special Registration Area for UE, which only includes the dedicated TAI of MBSR.

-	Supports to trigger AMF relocation without Mobility Registration Update initiated by the UE.

For option#2:

AMF:

-	The AMF determines to which NG-RAN(s) it sends a paging request for a UE on a TAC based on the dynamic indications of TAC support/release by the different NG-RANs as defined in TS 38.413 [19].

Donor gNB-CU:

-	The donor gNB-CU maintains a MBSR context with the TAC and a list containing all AMF-UEs with whom UEs conducted NAS signalling via the MBSR within a time frame of the periodic registration update time.

-	Forwarding of the MBSR context to the donor gNB-CU during the full migration process of the MBSR.

-	Inform the MBSR of the result of establishing NG interface with the AMF-UEs in the MBSR context.

MBSR:

-	Support the configuration of the TAC to be used for broadcast.

UE:

-	None.

## 6.18	Solution #18: Solution for UE location service via mobile IAB-node

### 6.18.1	General

This solution relates to KI#5 "Support of location services for UEs accessing via a mobile base station relay".

### 6.18.2	Functional descriptions

This solution is based on the following principles:

-	NG-RAN holds the 5G-GUTI allocated by the AMF handling the mobile IAB-node (e.g. AMF provides the info to NG-RAN as part of the Registration procedure).

-	When a UE access network via a mobile IAB-node, the NG-RAN (i.e., IAB-donor gNB) provides the 5G-GUTI of the mobile IAB-node, together with ULI, as part of the NGAP message (e.g. NGAP Initial UE message) to AMF which handles the UE. The AMF stores the 5G-GUTI of the IAB-node as part of the UE context.

-	When AMF triggers location request towards LMF for the UE, the AMF provides also the 5G-GUTI of the IAB-node, together with the cell information from the additional ULI (as proposed in solution#9) if it's available.

-	When LMF determines the location of UE, it also retrieves the location of mobile IAB-node based on the 5G-GUTI/Cell information provided by the AMF.

### 6.18.3	Procedures

#### 6.18.3.1	Location service for UE served by a mobile IAB-node

Figure 6.18.3-1: Positioning procedure for UE accessing via mobile IAB node

1.	The IAB-node (the IAB-UE part) is registered to the network and properly configured to provide access service for UE. NG-RAN node receives the 5G-GUTI to IAB-UE from AMF serving the IAB-UE (i.e. the AMF-IAB) as part of the NGAP message (e.g. Initial context setup request or UE context modification request) where 5G-GUTI can be included (5G-S-TMSI/UE Identity Index value is already defined in TS 38.413 [19]). It shall also be possible for the AMF-IAB to provide only 5G-S-TMSI and NG-RAN formulates the 5G-GUTI based on GUAMI and the 5G-S-TMSI.

2.	The UE is camped on the cell served by the mobile IAB-node and initiates the registration procedure. For the UE accessing the mobile IAB-node, the NG-RAN includes the 5G-GUTI and ULI information of the mobile IAB-node as additional information in NGAP message for the UE. The AMF serving the UE (i.e. AMF-UE) store the information and finish the UE registration procedure.

NOTE 1:	In the case of 5G-GUTI update for the IAB-node (e.g. in case of IAB-node mobility with AMF change), the NG-RAN provides the updated IAB-node 5G-GUTI info to the AMF-UE for all UEs connected via the IAB-node (detailed messages needs to by synched with the solution for KI#2 and KI#3). using a NG-AP message associated to the UEs served by the IAB-UE. The 5G-GUTI is removed by AMF-UE when UE not accessing via IAB-node (related to KI#7).

3.	The AMF-UE initiates the location reporting procedure for the UE, as defined in the 5GC-MT-LR procedure or the 5GC-MO-LR procedure of TS 23.273 [4].

4.	The AMF-UE selects LMF for the UE and invokes Nlmf_Location_DetermineLocation Request service operation towards the LMF including the LCS Correlation identifier for UE, the 5G-GUTI and ULI information of the IAB-node.

5.	The LMF performs one or more of the positioning procedures towards the UE via the AMF-UE as described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [4].

6.	The LMF finds the AMF-IAB based on the GUAMI based on the 5G-GUTI received in step 4 via NRF, and initiates the location reporting procedure for IAB-node/IAB-UE based on the 5G-GUTI received in step 4. The UE positioning procedure in step 5 and IAB-UE positioning procedure in step 6 may can be performed at the same time.

7.	The LMF determines the location of UE considering the location information received in step 5 for the UE and the location information received in step 6 for the IAB-UE. Then the LMF provides the UE location to the AMF-UE serving the UE.

NOTE 2:	How the LMF determines the location of UE by taking the location of the IAB-UE into account is up to LMF implementation.

### 6.18.4	Impacts on services, entities, and interfaces

LMF:

-	Support receiving 5G-GUTI/cell information of IAB node from AMF when LCS for UE is requested.

-	Support using the 5G-GUTI to determine the serving AMF and trigger location estimation of the IAB-node.

-	Support determining the UE location based on the position information of the UE and the location estimated for the IAB-node.

AMF:

-	Support receiving and storing 5G-GUTI/cell information from NG-RAN when UE is accessing via mobile IAB-node.

-	Support providing the 5G-GUTI/cell information of the IAB-node to LMF when send LCS request for a UE accessing via mobile IAB-node.

-	Support LCS related message delivery from LMF based on 5G-GUTI.

NG-RAN:

-	Support providing the 5G-GUTI/cell information of the IAB-node to the AMF when UE is accessing via mobile IAB-node.

-	Retain in IAB-UE context the UEs' AMFs that need to be updated with the 5G-GUTI of the IAB-UE when this changes.

## 6.19	Solution #19: Solution for location privacy check for Mobile Base Station Relay

### 6.19.1	General

This solution address KI#5, Support of location services for UEs accessing via a mobile base station relay.

According to the existing 5GC-MT-LR Procedure (Figure 6.1.2-1) in TS 23.273 [4], steps 2, 7, 8, 9, 16, 17, 18, 19, 20, 21, 22, and 23 are related to the UE LCS privacy check. Based on the subscription data obtained in step 2, GMLC and AMF interact with the UE/User for UE/User LCS notification, which requires LCS related UE 5GMM signalling.

As described, UE LCS privacy check requires the interaction with UE/User, because the commercial location services which triggers this 5GC-MT-LR Procedure mainly target the end users.  Supposing that the existing location service framework and procedures are basically applied to the MBSR because the MBSR is also considered as a UE for the positioning, the UE privacy check procedure for the MBSR needs to be performed.

Thus, in case the positioning of the MBSR is additionally performed for the UE served by an MBSR as described in the solutions #7, #8, #14 and #15, the UE LCS privacy check procedures for MBSR obviously should not be invoked for every UEs' positioning procedure, as it causes delay.

This solution addresses the issue and avoids the unnecessary privacy check for the MBSR if MT-LR procedure is triggered by the UE-LMF to obtain the MBSR location.

Editor's note:	It is FFS how the privacy check is handled if UE-LMF triggers MT-LR procedure via the MBSR-AMF.

### 6.19.2	Functional descriptions

In order to solve the issue, there are several possible options:

1) 	MBRS-centric option:

- 	In this option, based on provisioning information, the MBSR is set to not indicate its LCS notification capability to AMF during registration procedure.

-	AMF does not invoke the UE LCS privacy check during 5GC-MT-LR Procedure accordingly.

2)	AMF/5GC and subscription data-centric option:

-	In this option, the AMF/5GC determines whether to keep or discard the LCS notification capability transferred from the UE based on the subscription data.

-	The subscription data includes the information that the UE is an MBSR.

-	The MBSR as a normal UE indicates its LCS notification capability to the AMF during e.g., registration procedure.

-	When the UE indicates its UE LCS notification capability, but the subscription data indicates that this UE is a MBSR, the AMF/5GC discards the UE LCS notification capability. The AMF/5GC does not invoke the UE LCS privacy check during 5GC-MT-LR Procedure accordingly.

3)	GLMC and subscription data (UE LCS privacy profile data)-centric option:

-	In this option, a new type of UE LCS privacy profile data is utilized to determine whether to proceed with the UE LCS privacy check by AMF/CN.

-	Existing UE LCS privacy profile data has the information on the location service requestor only. In this option, a new type of privacy profile data e.g., Target UE type has the information on the UE LCS privacy check (e.g. UE/user is/is not involved for privacy check).

-	The Target UE type in the UE LCS privacy profile data for MBSR includes the information that the UE does not require UE LCS privacy check.

-	The GMLC obtains the UE LCS privacy profile data of the MBSR from UDM during the 5GC-MT-LR Procedure as described in TS 23.273 [4]. The GMLC includes the information that UE LCS privacy check is not required for this UE to the location request message to AMF.

-	Based on the obtained information, the AMF/5GC does not invoke the UE LCS privacy check during 5GC-MT-LR Procedure accordingly.

### 6.19.3	Procedures

The registration request procedure in clause 4.2 of TS 23.502 [5] in Rel-17 can be used by the MBSR to access the 5GS. In addition, the location service procedure in clause 6.1.1 and 6.1.2 excepting for privacy check procedure in Rel-17 can be used for the location service.

In order to support the operation, information on MBSR needs to be defined in the subscription data, and a new type of UE LCS privacy profile data also needs to be defined in the clause 7 of TS 23.273 [4].

### 6.19.4	Impacts on services, entities, and interfaces

Impacts on existing entities

-	AMF: support to determine whether to invoke the UE LCS privacy check.

-	GMLC: support to include the information on UE LCS privacy check related information to location request message

Functions required at new entities

-	MBSR: To support change indication of LCS notification capability based on provisioned configuration.

## 6.20	Solution #20: CAG based UE access control via MBSR

### 6.20.1	Introduction

This solution relates to KI#7 "Control of UE's access to 5GS via a mobile base station relay". The solution proposes to use the existing CAG, Closed Access Group, concept as a base to manage the UE access control via MBSR.

There are certain simplifications on the existing CAG logic to fit into the IAB architecture.

Editor's note:	Whether this is a CAG or whether this will have to be an independent identifier supported from rel-18 or both the CAG and a new identifier could be considered is FFS.

### 6.20.2	Functional Description

When a UE tries to select or connect to 5GS via a MBSR (a mobile IAB-node), the access control of the UE is managed by CN, RAN and UE. During the CM-CONNECTED mobility, the access control via MBSR (mobile IAB-node) is managed by CN and RAN.

A new MBSR CAG identifier (or mobile-IAB CAG Identifier) shall be assigned for the MBSR node to use. The new MBSR CAG identifier is announced by the cells covered by the MBSR. Considering the roaming scenario, this new MBSR CAG Identifier in subscription data can be defined as a "common" CAG identifier or a flag which can be indicated so that different PLMNs can use it to determine whether the UE is allowed (or not allowed) to access the cell associated with the MBSR CAG Identifier.

RAN and CN supports the UE access control based on the new MBSR CAG identifier associated with the cell and the allowed CAG identifiers for the UE.

For UEs that support CAG, they may select the cells covered by a MBSR based on the configured MBSR CAG identifier using the existing CAG control logic.

For the UEs that do not support CAG (i.e. the UE is not able to understand the broadcasted CAG information by the cell) but supports the use of IAB architecture, it may select the cell as normal and rely on RAN and CN to control the IAB-node access using network side CAG control logic.

See more details of the CAG concept in clause 5.30.3 of TS 23.501 [2].

Considering the MBSR access use case, additional control (e.g., time duration, specific geographic location) may be linked to the MBSR CAG identifier management, e.g., when user purchase a buss/train ticket, the access to IAB-node/MBSR CAG identifier may be limited to certain time period and specific locations and granted to the user subscription data via O&M or AF. It may also be provisioned to the UE accordingly so that the UE can properly select the cell. Additionally, the range (number) of CAG identifiers may need to be increased, access to a CAG cell may be (de)prioritized, and roaming case of CAG use may need further control.

### 6.20.3	Procedures

#### 6.20.3.1	General

The CAG control is well described in different procedures in TS 23.502 [5]. Procedure details may be updated to support the additional control in case of MBSR CAG Identifier.

The CAG specific procedures, involving RAN/AS, are defined in TS 38.300 [20] clause 16.7. These procedures allow the control of:

-	whether new UEs (Rel-18 onward), or also legacy UEs, can access to a mobile-IAB CAG cell;

-	whether non-CAG capable UEs can access, or not, to a mobile-IAB CAG cell (as in TS 38.304 [21]).

In order to control the use of the CAG for MBSR use cases, the MBSR CAG configuration (in the UE and the IAB node) may be associated with the following:

-	validity criteria of the MBSR CAG ID, associated with time duration, geographic area;

-	priority of the MBSR CAG IDs.

A mobile-IAB CAG cell may also be configured with a human-readable network name, to be signalled to the UE for presentation to the user.

### 6.20.4	Impacts on services, entities and interfaces

AMF:

-	Supports baseline CAG control with possible additional controls (e.g. time duration, geographic location) based on a new MBSR CAG identifier or a flag.

UDM:

-	Supports new MBSR CAG Identifier with additional controls (e.g. time duration, geographic location) based on IAB CAG identifier or common flag.

gNB/IAB-node:

-	Supports CAG function.

UE:

-	Supports CAG function.

Editor's note:	It is FFS whether there are further enhancements needed for existing CAG procedures, e.g. extended range (max #) of CAG identifiers, specific CAG prioritization (for MBSR), roaming support of the CAG use, etc.

NOTE:	Some of the potential CAG extensions and additional control mechanisms may have RAN dependencies.

# 7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

## 7.1	Evaluations for KI#1

Solution 1 proposes to just reuse existing IAB provisioning methods. While this is for sure in general something the study should strive to maximize, the existing IAB configuration has some built in assumptions on the fact the IAB node is a rather trusted entity that also may be preconfigured with OAM sever address and any security credentials. However, the MBSRs are in many cases third party devices that need to be integrated in a PLMN. Solution#10 provides method for the serving PLMN to provide the MBSR with the address of the OAM server and the security credentials to access it. What security credentials are used is up to SA3 and SA5 to decide. The configuration of the MBSR itself can be based on procedures defined by SA5 and RAN WGs and may reuse the existing methods with any enhancements not in scope of SA2.

Solution#2 could be seen as complementary to solutions above and deal more directly with the configuration of the IAB-UE part of the MBSR to e.g. restrict the operation of the MBSR to specific areas (by e.g. restricting the areas where the backhaul connection can be provided and hence the advertisement of the associated PLMN ID(s)). This can reduce the unnecessary attempts from the MBSRs to the OAM servers.

## 7.2	Evaluations for KI#2

Editor's note:	Further evaluation need to be considered.

There are four solutions addressing Key Issue#2 Efficient mobility for UEs connecting to/disconnecting from mobile base station relay, i.e. solution# 3, solution#4, solution#5, and solution#11.

Among the solutions:

-	Solution #3 addresses only KI#2, and reuses the existing procedures for different scenarios of per UE mobility, including UE mobility between MBSRs, UE mobility between MBSR and macro base station. Solution#3 does not have any normative impact, although further verification by RAN WGs is needed. Solution#3 does not address any optimization for the mobility of a group of UEs.

-	Solution #11 addresses KI#2. It proposes to reuse the mechanism for NTN scenario that MBSR broadcast service time for influencing cell reselection and handover before MBSR is out of service in order to reduce service interruption. The use of such mechanism for MBSR instead of NTN operation needs verification by RAN WGs.

The solutions have RAN impact as following and need RAN coordination:

1)	Solution #11 requires NG-RAN to enable the MBSR (IAB-DU) broadcasting time event information that is received by the MBSR (IAB-UE) via NAS message.

## 7.3	Evaluations for KI#3

There are six solutions addressing Key Issue#3, Efficient mobility and service continuity when served by mobile base station relay, i.e. solution#4, solution#5, solution#12, solution#13, solution #16, and solution #17:

-	Solution #4 and #5 addresses both KI#2 and KI#3. Solution #4 assumes that the TACs potentially will be broadcasted by the MBSR are provided to the UE by the AMF of the UE served by the MBSR, so that the UEs do not need to perform mobility registration when the MBSR changes TACs. In order to support UE mobility management without the mobility registration, some special contexts are created on NG-RAN associated with the MBSR, and in the AMF serving the UEs. These contexts are connected using a new identifier "Link ID". With solution#4, the donor gNB is able to support AMF-UE relocation for all the UEs served by this AMF with one N2 signalling. i.e., when UE moves with MBSR, the NG-RAN proxies the UE idle mobility and NG-RAN triggers the UE handover due to MBSR mobility. Solution#4 works with legacy UEs when the UE is served by the MBSR, no mobility registration is performed and therefore 5G-GUTI is not updated even if the UE context transfer is triggered among AMFs. CT WGs and SA3 evaluation is required to confirm that the UE context management at AMF is not broken. Support of the new context at donor gNB and AMF and their maintenance during MBSR mobility requires verification by CT WGs and RAN3.

-	Solution #5 is based on solution #4 to enhance idle mobility and handover to support mobile terminated services. Solution#5 further creates a special context in AMF serving the MBSR, and associate it with the context in AMF-UE using the Link ID. Interactions between AMF-UE and AMF-MBSR are introduced to maintain the binding of the Link ID. and AMFs are enhanced to redirect the UE paging message from AMF-UE to AMF-MBSR. The AMF-MBSR forwards the paging message to NG-RAN serving the MBSR so that it can be broadcasted by MBSR. The paging forwarding by AMFs needs verification by CT WGs and RAN3.

-	Solution #12 address KI#3. It proposes to use a full migration approach with a 2-step handover procedure for reducing the service interruption, firstly MBSR handover is performed without F1 anchor change, then UE handover is triggered.  With this solution, the UEs served by MBSR would still experience mobility event even if they remain connected to the same MBSR. Other the other hand, Solution#12 does enhance the existing Xn and N2 handover procedures to support group UE handover and therefore reduces corresponding signalling over Xn and N2. Overall, the principle of this solution is to aggregate the information of group UEs in a single message to support bulk handling in 5GC. When group UEs are accessing the same IAB-node, the commonality is that group UEs share the same user location and AMF. The support of group mobility procedure requires verification of RAN WG3. The solution#12 does not address the IDLE mode mobility aspect when the UEs move together with the MBSR. Additionally, delivery of the RRC messages to the connected UEs requires F1-C connectivity between the source IAB-donor-CU and the migrating MBSR. This F1-C connectivity may have to be carried via the CN if the source IAB-donor-CU and the target IAB-donor do not have Xn and/or IP connectivity. This should be coordinated with RAN WGs.

-	Solution #13 address KI#3. It proposes to adopt a partial migration approach that besides the CU of gNB handles the MBSR mobility, a central CU that acts as F1 anchor for as long as practically possible to shield the MBSR mobility from the UEs it serves, which saves the AN signalling for mobility. This requires some discussion and adoption in RAN WGs. An interaction with RAN3 is required to consolidate this approach. The feasibility to support IAB mobility based on the Rel-17 IAB partial migration should be discussed in the RAN WGs. In addition, R18 NR_mobile_IAB in RAN WGs only focuses on full migration (RP-213601). Given that the m-CU cannot cover all possible MBSR operation locations, additional handling from other solutions may needed to complement this solution.

-	Solution #16 addressed KI#3. It proposes that the MBSR is configured by the donor gNB to broadcast the TAC of its serving cell. For a UE served by the MBSR, it would observe the TAC change when it moves together with the MBSR across different serving cells, even if it camps on the same MBSR cell. On the other hand, a UE would not be triggered to perform mobility registration when it camps on or leaves the MBSR cell temporarily.

-	Solution #17 addressed KI#3. It proposes that the MBSR uses a dedicated TAC, and the TAC remains the same when MBSR moves across different donor gNBs. The donor gNB will add or remove the TAC of the MBSR to UE's AMF(s) when the MBSR connects to leaves it. With this solution, the UE served by MBSR would not perform mobility registration, even when the MBSR changes donor gNBs.

The solutions have RAN impact as following and need RAN coordination:

1)	Solution #4 and #5 require NG-RAN to manage MBSR context with Link IDs sent by AMFs via NGAP messages, one Link ID identifies one AMF that serves a group of UEs camp on the MBSR, as well as require NG-RAN to initiate an NGAP message to new AMF with Link ID(s) for AMF relocation when needed.

2)	Solution #12 requires NG-RAN to support full migration approach with group handover for UEs. The potential F1-C connectivity via CN needs coordination with RAN WGs.

3)	Solution #13 requires NG-RAN to support a partial migration approach with a central CU (m-CU) together with a local CU (gNB-CU).

4) Solution #16 and #17 need RAN confirmation of the feasibility of updating the UE-AMF regarding the TAC supported by the MBSR via NGAP.

## 7.4	Evaluations for KI#4

There are two solutions address KI#4 Support of roaming of mobile base station relays, i.e. Soltuion#6 and Solution#10.

High level aspects of the solutions for KI#4:

-	Solution#6 allows the roaming of MBSR and provides means by which the IAB-UE is configured with PLMN ID list, geographic areas and OAM server FQDN. The roaming operation of IAB-UE is authorized by the 5GC based on the Access and Mobility Subscription data. When MBSR moves into VPLMN, it makes use of the information to either perform an IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to continue provide service via VPLMN NG-RAN. The feasibility to support the roaming of MBSR as an IAB-DU using IAB node integration procedure and inter-IAB-donor gNB mobility procedure in the VPLMN needs RAN WGs evaluation. Additionally, solution#6 also allows the VPLMN AMF to indicate to the UE that it is not allowed to act as MBSR as part of registration accept (if normal services are allowed) to the UE or as part of registration reject message with appropriate cause code. The UE will populate the PLMN ID/TAI in list of IAB operation restriction and will not attempt to access those PLMNs/TAIs for MBSR operations.

-	Solution#10 assumes the existence of a roaming agreement for the MBSR operation in VPLMN. The VPLMN 5GC authorizes the MBSR during mobility registration procedures and provides the OAM server address and the security information required to access the OAM during the registration. However, the solution does not specify how the VPLMN 5GC determines the OAM server address and the security information. In addition, the security aspects need SA3 evaluation.

## 7.5	Evaluations for KI#5

For KI#5, the solution #7, #8, #14, #15, solution x (from S2-2205653) and solution y (from S2-2206856) are related and have the following common and different designs:

-	Solution#7 (as updated at SA2#152) proposed that the LMF executes a TRP information procedure and gather information on which TRP is mobile when a MBSR is integrated in any RAN node (including at full migration time if applicable). this means that OAM ensures the LMF gathers the info it needs ahead of positioning procedure. then, when the LMF triggers a positioning event towards a RAN node, the LMF that one or more of the cells involved in UE positioning operation is a MBSR and the corresponding GPSI (with the serving cell ID) of the MBSR. The LMF uses the GPSI of the MBSR to identify the AMF serving the MBSR and trigger the positioning procedures for the MBSR via the AMF serving the MBSR. Alternately the LMF, for mobile TRP retrieves the TRP location information from the RAN of the MBSR(s)TRPs by causing the F1-AP to trigger a MO-LR procedure at MBSR IAB-UE (this can avoid contacting the UDM). The LMF uses this location together with the measurements data sets for the Target UE to estimate the location of the Target UE and provides the result to the Target UE's serving AMF. The LMF may instruct the MBSR which the UE is accessing and other cells to start PRS broadcasting. The Target UE may be instructed to perform DL positioning measurements on the cells and report the measurements back to the LMF. The LMF derive from the measurements/cell-ID which measurement data set that belongs to a MBSR.

The Target UE may also be instructed to perform DL positioning measurements on the cells and report the measurements back to the LMF. The LMF derive from the measurements/cell-ID which measurement data set that belongs to a MBSR and this triggers the MBRS TRP positioning.

-	Solution#8 proposed that the MBSR provides its location to the LMF via NRPPa as part of the UL positioning procedure of the UE, and the MBSR may trigger MO-LR to obtain its own position for this purpose; and for DL positioning of the UE, the MBSR provides its UE ID (e.g. GPSI) to LMF, which obtains the MBSR positioning and velocity information via MT-LR procedures. LMF perform calculation of UE location taking into account the MBSR location and velocity information.

-	Solution#14 proposed that the PLMN level NRF stores the UE ID, cell ID, and the serving AMF of the MBSRs. The AMF serving the UE determines that a UE is accessing via MBSR when received additional ULI, and obtains the AMF information and UE ID of the MBSR (e.g. SUPI) from the PLMN level NRF. The AMF provides the information to LMF, which triggers a positioning procedure for the MBSR. The LMF uses the location of the MBSR and the measurement data from the Target UE procedures to estimate the Target UE's location. The two location measurements shall be done at the same time T.

-	Solution#15 proposed that the AMF serving the UE indicates to the LMF the existence of the MBSR. The LMF either obtains the MBSR position information when receiving the measurement reports from MBSR or by querying the MBSR with a specific time information. The MBSR velocity information is taken into consideration to determine the MBSR location at the requested time. The LMF requests the MBSR positioning information via NRPPa procedure similar to obtaining location information of a TRP.

-	Solution #18 proposed that the NG-RAN keeps the 5G-GUTI of the MBSR, and provides that to the AMF-UE in the Initial UE message. When location request is sent to the LMF, AMF-UE provides also the 5G-GUTI of MBSR, so that additional location procedure for MBSR can be triggered by the LMF.

-	Solution #19 proposed to several options to avoid sending privacy check signaling toward MBSR nodes, in order to optimize the network operations.

It is clear from the above that all solutions, except solution#8, assume that the AMF serving the UE would need to be informed of the existence of the MBSR. This can be achieved via either an explicit indication from NG-RAN, or the additional ULI information. Solution#8 allows such additional indication of MBSR from NG-RAN to LMF via NRPPa directly.

The LMF decides that the target UE connects to MBSR based on information received from the AMF. The AMF provides the information to LMF when it determines that the UE connects to MBSR based on the additional ULI received from NG-RAN as described in solution#7, #8 and #14. However, it is not described in solution#15 that how the AMF makes the decision. Furthermore, solution#15 also includes another way for LMF to make the decision, i.e. based on UE positioning capability which is enhanced to include MBSR access provided by UE via LPP. But it is also not described that how the UE determines that it connects to MBSR.

All solutions believe that the Target UE shall behave as a legacy UE and no new UE features are needed to support LCS for UEs accessing the network via a MBSR, e.g. the Target UE reports the cell-ID in the DL positioning report to the LMF.

In most of the solutions, it is assumed that for the positioning procedure that involves MBSR measurements, the LMF obtains the MBSR location to calculate target UE location. How the LMF derives the location of the MBSR differs between the solutions. The LMF triggers the AMF that serves the MBSR directly (solution #7 and #14), or the LMF request the GMLC to derive the location (Solution #8). The GMLC will trigger the serving AMF in the next step. In both approaches the LMF needs to obtain the UE-ID of the MBSR. The third approach (in Solution #15) is that the LMF requests the location and velocity of the MBSR by using modified NRPPa procedure.

For Solution#15, the MBSR location is obtained by the UE's LMF using NRPPa query for TRP location, and no UE ID for MBSR is needed.

How the LMF resolves the cell-ID may differ slightly between the solutions. In Sol#7 The LMF correlates cell-ID with GPSI of MBSR received by the AMF and/or in TRP information exchanges. In Sol#8 the IAB-donor RAN node provides the GPSI to the LMF. In Sol#14 the AMF-UE queries the PLMN level NRF for the MBSR's SUPI and AMF-ID of the AMF that is serving the MBSR using the cell-ID. The AMF provides this info to the LMF. In Sol#15 the LMF receives the cell-ID with an indication that it is a MBSR from the AMF.

As indicated in both solution#8 and solution#15, the time of positioning measurements for UE and that for MBSR could be different. In order to reduce positioning error caused by the time difference, it is useful to take MBSR velocity into consideration. This is also applicable to other solutions. Solution#14 and solution#7 also touches on ensuring the positioning information to be measured at the same time or as close as possible.

## 7.6	Evaluations for KI#6

For KI#6, the KI definition in clause 5.6 addresses the problem of cell ID/TAC of MBSR not reflecting the location of the UE. The solution#9 (clause 6.9) proposes that the gNB (i.e. IAB-donor gNB) provides the additional ULI (IAB-UE ULI) to the AMF for the UEs when the UEs are connected via a MBSR. The AMF of the UE determines the UE's location by taking the IAB-UE ULI into account.

The additional ULI is the cell selected by the MBSR during the registration and managed by the IAB-donor gNB. The details of how to determine the cell that UE is located is a mobile IAB's cell and how to provide additional ULI of IAB-UE on NGAP have RAN dependency.

Considering the TAI/NR CGI in additional ULI is managed by gNB, it's static and related to a specific geographic area if the TAI/NR CGI in normal ULI does not reflect a specific geographic area.

The geographic area covered by the MBSR is normally also covered by the IAB-donor gNB. Using the additional ULI, together with the normal ULI, to manage mobility related restriction is feasible. However, as below figure 7.6-1 illustrates, there are cases where the additional ULI information of the IAB-UE itself does not provide sufficient information to determine the actual UE location.

Figure 7.6-1. Scenario for UE location when served by MBSR

As illustrated in figure 7.6-1, depending on the location of the MBSR 1, the UE 1 served by it can be at different locations, e.g. out of coverage of Cell ID 1, or in coverage of Cell ID2. However, according to solution#9, the same additional ULI (e.g. TAC1, Cell ID 1) would be provided to the 5GC.

Therefore, while providing the additional ULI based on MBSR's serving cell to 5GC can indicate the UE's rough area, it still does not provide accurate location information of the UE1 and could be misleading in some cases. For the 5GC operations that requires more accurate location of the UE (e.g. if Cell 1 has a range of 300m, and Cell 4 has a range of 200m, having two ULIs would only provide a granularity of 500m in UE location), the 5GC would need to obtain the location of the MBSR besides the ULI of MBSR.

Based on the solutions for KI#5, the LMF of the UE is able to obtain the location information of the MBSR. Therefore, if the AMF requires more accurate ULI information than the combined range, it can obtain the accurate MBSR location from the UE's LMF, which would trigger a NRPPa procedure to query the MBSR location information as a TRP. This should not require triggering the full location service procedure for the UE. Some new indication may be needed in the request to LMF to only request the MBSR location.

# 8	Conclusions

Editor's note:	This clause provides the conclusions for the study.

## 8.1	Conclusions for KI#1

Editor's note:	This clause includes interim conclusions.

Proceed with OAM configuration that is largely borrowing what was developed for IAB nodes in previous releases with the addition of new security measures to establish trusted access to the serving PLMN OAM.

Editor's note: 	This conclusion needs further discussion and is not final.

Editor's note:	It is FFS if the MBSR IAB-UE configuration using the V2X architecture framework can be used together with the OAM based configuration. If the answer is yes, the exact parameters to be configurable via V2X architecture framework can be determined in normative phase.

## 8.2	Conclusions for KI#2

Editor's note:	This clause includes interim conclusions, and needs further study.

## 8.3	Conclusions for KI#3

Editor's note:	This clause includes interim conclusions, and needs further study.

## 8.4	Conclusions for KI#4

Editor's note:	The interim conclusion is subject to feedback of RAN WGs and will be revisited as per the feedback.

For KI#4, the interim conclusions are as follows:

Editor's note:	How MBSR is made available with information related to its roaming operation in a VPLMN is FFS.

-	Depends on RAN WG confirmation, MBSR can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into VPLMN to provide service.

-	It is assumed that some roaming agreement for MBSR operation in VPLMN is in place in VPLMN, and the 5GC can make use of it for authorization of MBSR.

## 8.5	Conclusions for KI#5

For Key Issue #5 (Support of location services for UEs accessing via a mobile base station relay), the followings are taken as initial conclusion:

-	To aid the LMF to estimate the accuracy of the UE location estimation the MBSR velocity information and time for obtaining its location measurement data should be provided to the LMF when available.

-	Target UE performs location measurements and SRS transmission as a legacy Rel-17 UE.

-	The Target UE reports the cell-IDs of all the TRP/gNB/eNB the UE performed DL positioning measurements on.

-	The MBSR includes it's cell-ID and GPSI in the reported UL positioning measurement that it performed on the Target UE.

-	The AMF provides the cell-ID of the cell that the Target UE is connected to the LMF in the location request (legacy behaviour) and indicates if possible that the cell-ID belongs to a MBSR.

Editor's note:	It is FFS whether the AMF provides more parameters related to the MBSR to the LMF.

-	The LMF uses the Target UE reported cell-IDs to derive whether the cell-ID corresponds to a MBSR. There can be more than one MBSR in the measurement report.

-	The LMF can learn that a new integrated MBSR TRP at a gNB is mobile and its MBSR IAB UE ID (GPSI) via TRP information exchange towards the gNB with the cell Id of the TRP. this is triggered by OAM.

-	The UDM of MSBR holds MBSR subscription data stores an updated record that includes the MBSRs SUPI, MBSR GPSI serving AMF ID,

Editor's note:	It is FFS if there is any extra function in UDM is needed.

-	Options for the LMF to derive the location and velocity of the MBSR.

-	The LMF can derive the location of the MBSR by either triggering the AMF serving the MBSR, or the gNB serving the MBSR (as per solution#7 MO-LR case) or by requesting the GMLC to derive the location of the MBSR (UE).

-	When the LMF of the UE needs to obtain the MBSR location information, it can use NRPPa procedure for TRP location query that triggers the MBSR to perform MO-LR;

Editor's note:	It is FFS whether the NRPPa procedure for TRP location needs to be enhanced to include velocity estimation.

-	When MBSR is involved in providing the positioning measurements for the UE (and the LMF knows it from info obtained e.g. from TRP info exchanges), the MBSR location information may be provided by the MBSR via NG-RAN to the LMF via NRPPa;

Editor's note:	It is FFS whether all these options for LMF to derive the location of MBSR would be supported.

-	As the timing of the location estimations for the Target UE and MBSR(s) is important for the quality of the location estimation of the Target UE, means to secure minimal timing offset for when the positioning measurements is performed should be specified.

## 8.6	Conclusions for KI#6

For Key Issue #6 (Provide cell ID/TAC of mobile base station relay for services), the followings can be taken as initial conclusion:

-	the NG-RAN provides an additional ULI based on the MBSR's donor gNB information, together with the existing ULI, to the AMF when a UE connects to the 5GC via a MBSR. The additional ULI includes the TAI/NR CGI of selected by the IAB-Node when it's registered to the network.

-	AMF may use the additional ULI, together with the existing ULI, apply the mobility management (e.g. Mobility restriction).

NOTE:	Providing additional ULI needs further coordination with RAN WG and further updates may apply.

-	The Solution#9 is recommended for normative specifications as basis for providing serving cell ID/TAC of mobile base station relay to the AMF serving the UE.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-02 | SA2#149e | S2-2201574 | - | - | - | TR skeleton | 0.0.0 |
| 2022-02 | SA2#149e | S2-2201575 | - | - | - | Scope for FS_VMR | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201576 | - | - | - | Architecture assumptions and requirements for FS_VMR | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201577 | - | - | - | Key issue on "Mobile base station relay configuration support in 5GC" | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201579 | - | - | - | Key issue on "Efficient mobility for UEs connecting to/disconnecting from VMR" | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201580 | - | - | - | Key issue on "Efficient mobility and service continuity when served by VMR" | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201581 | - | - | - | Key issue on "Support of roaming of mobile base station relays" | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201582 | - | - | - | Key issue on "Support of location services for UEs accessing via a mobile base station relay" and "Provide cell ID/TAC of mobile base station relay for services" | 0.1.0 |
| 2022-04 | SA2#150e | S2-2203283 | - | - | - | Architecture Assumption: VMR not applicable to NR satellite access | 0.2.0 |
| 2022-04 | SA2#150e | S2-2202214 | - | - | - | Terminology updates | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203284 | - | - | - | Solution for configuration | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203285 | - | - | - | new solution proposal on provisioning and policy control for FS_VMR | 0.2.0 |
| 2022-04 | SA2#150e | S2-2202032 | - | - | - | Reusing existing procedures for UE mobility | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203286 | - | - | - | New Solution: Mobility procedures | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203287 | - | - | - | new solution on support of roaming of mobile base station relays | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203288 | - | - | - | Network based Positioning of VMR for location services. | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203289 | - | - | - | Support of positioning for UE accessing via mobile base station relay | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203290 | - | - | - | Solution for user location. | 0.2.0 |
| 2022-05 | SA2#151e | S2-2204132 | - | - | - | Update architecture requirements (about NR satellite access) | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204990 | - | - | - | Key Issue on UE permission and policies for accessing VMR | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204991 | - | - | - | Solution for configuration of Mobile Base Station Relays | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204918 | - | - | - | New solution on Service time of MBSR. | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204992 | - | - | - | New solution on IAB-node mobility with connected UEs | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204993 | - | - | - | Solution for IAB optimization for Mobile Base Station Relays | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204994 | - | - | - | New Solution on support of location services for UEs accessing via a mobile IAB node. | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204995 | - | - | - | New Solution on support of location services for an MBSR with provision of MBSR Location. | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204919 | - | - | - | Solution#3: Update on reusing the existing procedures. | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204996 | - | - | - | Solution#4 and #5 updates to remove ENs and clarify the proxy UE mobility. | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204997 | - | - | - | Solution#6 updates to clarify the procedures. | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204998 | - | - | - | Solution#7 updates to address EN | 0.3.0 |
| 2022-05 | SA2#151e | S2-2204999 | - | - | - | Solution#8 updates to remove ENs | 0.3.0 |
| 2022-05 | SA2#151e | S2-2205000 | - | - | - | Solution#9 updates to remove ENs | 0.3.0 |
| 2022-08 | SA2#152e | S2-2206092 | - | - | - | KI#3, New solution of UE mobility management in case of IAB-node mobility with dynamic TAC provisioning | 0.4.0 |
| 2022-08 | SA2#152e | S2-2207060 | - | - | - | KI#3, New solution of IAB-node mobility with dedicated TA | 0.4.0 |
| 2022-08 | SA2#152e | S2-2207061 | - | - | - | KI#5, New Sol, Solution for UE location service via mobile IAB-node | 0.4.0 |
| 2022-08 | SA2#152e | S2-2207062 | - | - | - | KI#5, New solution: Privacy check support for LCS via mobile base station relay | 0.4.0 |
| 2022-08 | SA2#152e | S2-2207063 | - | - | - | KI#7, New Sol, Solution for UE access via IAB-node control | 0.4.0 |
| 2022-08 | SA2#152e | S2-2206473 | - | - | - | KI#2&3, Sol#4: resolve Ens | 0.4.0 |
| 2022-08 | SA2#152e | S2-2207064 | - | - | - | KI#4, Sol#6, Update for mobility aspect | 0.4.0 |
| 2022-08 | SA2#152e | S2-2207065 | - | - | - | KI#5: updated Solution 7 | 0.4.0 |
| 2022-08 | SA2#152e | S2-2206622 | - | - | - | KI #5, Sol #8: Update solution #8 | 0.4.0 |
| 2022-08 | SA2#152e | S2-2206094 | - | - | - | KI#5, Update solution#14 to remove EN | 0.4.0 |
| 2022-08 | SA2#152e | S2-2207066 | - | - | - | KI#1,3,6 evaluations and conclusions | 0.4.0 |
| 2022-08 | SA2#152e | S2-2207067 | - | - | - | Evaluation for KI#1 to KI#5 | 0.4.0 |
| 2022-08 | SA2#152e | S2-2207068 | - | - | - | Evaluation and conclusion on KI#2 and KI#3 | 0.4.0 |
| 2022-08 | SA2#152e | S2-2207069 | - | - | - | KI#5 and KI#6, Initial evaluation and conclusion | 0.4.0 |
| 2022-09 | SP#97-e | SP-220817 | - | - | - | MCC editorial update for presentation to TSG SA for information | 1.0.0 |