3GPP TSG-SA3 Meeting #108-e	S3-222346

# e-meeting,  22 - 26 August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.210 | CR | 0073 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IPSec references to rfc8221 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCryptPr |  |  |  |  |  | Date: |  |  | 2022-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RFC 8221 has the following verbiage change from using deprecated MD5, SHA1, and DES-CBS algorithms. Rfc8221 prohibits the use of MD5 and changes the requirement of 3DES-CBC from optional from mandatory. MD5 has been deprecated by NIST and is no longer mentioned in publications such as [NISTSP800-131A-R2]. NIST formally deprecated the use of SHA-1 in 2011 [NISTSP800-131A-R2] and disallowed its use for digital signatures at the end of 2013, based on both the Wang et. al; attack and the potential for a brute-force attack. (https://csrc.nist.gov/CSRC/media/Publications/sp/800-131a/rev-2/draft/documents/sp800-131Ar2-draft.pdf). Also, according to draft guidance published by NIST on July 19, 2018, the Triple Data Encryption Algorithm (TDEA or 3DES) is officially being retired. The guidelines propose that 3DES is deprecated for all new applications after a period of public consultation and usage is disallowed after 2023. https://nvlpubs.nist.gov/nistpubs/SpecialPublications/NIST.SP.800-131Ar2.pdf |  |  |  |  |  |  |  |  |
| Summary of change: |  | CR proposes following 1) Algorithm implemetion and Usage guidence should follow the latest rfc 8221 for ESP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Obsolete algorithms persists in implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** Start of Changes********************

## 5.3	Profiling of IPsec

### 5.3.0	General

This section gives an overview of the features of IPsec that are used by NDS/IP. The overview given here defines a minimum set of features that shall be supported. In particular, this minimum set of features is required for interworking purposes and constitutes a well-defined set of simplifications.

The accumulated effect of the simplifications is quite significant in terms of reduced complexity. This is achieved without sacrificing security in any way. It shall be noted explicitly that the simplifications are specified for NDS/IP and that they may not necessarily be valid for other network constellations and usages.

Within their own network, operators are free to use IPsec features not described in this section although there should be no security or functional reason to do so.

NOTE:	Void

Clause 5.3 contains the general 3GPP IPsec ESP profile. Other 3GPP specifications (e.g. TS 33.203 [10], etc.) may point to clause 5.3. Thus parts of clause 5.3 may also apply to devices and network nodes as specified in other specifications. New specifications using ESP should refer to this profile with as few exceptions as possible. Unless explicitly stated otherwise, the 3GPP ESP profile apply for all uses of ESP to protect 3GPP interfaces

### 5.3.1	Support of ESP

When NDS/IP is applied, the ESP security protocol shall be used. IPsec ESP shall be supported according to RFC-4303 [31]. Extended sequence number may be supported. Usage guidance for the Implementation of Cryptographic Algorithm for ESP shall follow RFC-8221[70].

NOTE: Void

****************** End of Changes********************