3GPP TSG-SA6 Meeting #49-bis-e	S6-221518

# Online, , 22nd Jun 2022 - 1st Jul 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0089 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to API invoker onboarding/offboarding in TS 23.222 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, Uangel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF |  |  |  |  |  | Date: |  |  | 2022-06-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.222, CAPIF supports onboarding and offboarding of API invokers in the CAPIF core function via CAPIF-1 or CAPIF-1e. However, the current text provides the API invoker onboarding/offboarding capability in the API management function via CAPIF-5, which is misleading. While some additional features (e.g., granting or provisioning) for API invoker onboarding are addressed in the CAPIF-5 description, they are not implemented in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added notes to indicate that API invoker onboarding/offboarding in the API management function over CAPIF-5 is out of the scope of the current release |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may cause misleading in implementation of the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6, 6.4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

6.3.6	API management function

The API management function enables the API provider to perform administration of the service APIs. The API management function consists of the following capabilities:

-	Auditing the service API invocation logs received from the CAPIF core function;

-	Monitoring the events reported by the CAPIF core function;

-	Configuring the API provider policies to the CAPIF core function;

-	Monitoring the status of the service APIs;

-	Onboarding the new API invokers and offboarding API invokers; and

-	Registering and maintaining registration information of the API provider domain functions on the CAPIF core function.

NOTE: 	The API invoker onboarding/offboarding in the API management function is out of the scope of the current release.

NEXT CHANGE

6.4.8	Reference point CAPIF-5 (between the API management function and the CAPIF core function)

The CAPIF-5 reference point, which exists between the API management function and the CAPIF core function, is used for management of service API, API invoker and API provider domain function information.

The CAPIF-5 reference point supports:

-	Accessing the service API invocation logs by the API management function;

-	Enabling the API management function to monitor the events reported due to the service APIs invocations;

-	Onboarding new API invokers by provisioning the API invoker information at the CAPIF core function, requesting explicit grant of new API invokers onboarding and confirming onboarding success;

-	Offboarding API invokers;

-	Enabling the API management function to configure policies at the CAPIF core function e.g. service API invocation throttling, blocking API invocation for certain duration;

-	Enabling the API provider to monitor the status of service APIs (e.g. pilot or live status, start or stop status of service API);

-	Registering API provider domain functions on the CAPIF core function; and

-	Update of the registration information of API provider domain functions on the CAPIF core function.

NOTE 1:	The security aspects of CAPIF-5 are specified in subclause 6.6 of 3GPP TS 33.122 [12].

NOTE 2: 	The API invoker onboarding/offboarding over CAPIF-5 is out of the scope of the current release.

END OF CHANGES