3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221948

e-meeting, 22nd June – 1st July 2022	(revision of S6-221835)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0341 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC gateway UE association with one MC service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2022-06-16 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following two ENs in clause 11.5.4.1 need to be resolved: Editor´s Note: Further study is needed on how to prevent that an MC client selects another MC gateway UE if an active connection exists. Editor´s Note: The reconnection to the same MC server via another MC gateway UE for the same service and possible implications to the MC server needs further study. The MC gateway client enables the use of an MC gateway UE and informs the MC clients about the MC gateway UE connection status. Once a connection is indicated by the MC gateway client, the MC client(s) use(s) the MC gateway UE until the MC gateway client asks for disconnection from the MC gateway UE (or if the non-3GPP link towards the MC gateway UE breaks by some reasons). I.e., the MC client uses the previously determined MC gateway UE until the MC gateway client initiates a disconnection. The association between the MC client, the MC gateway UE and the MC server is kept until the MC gateway client cancels the association. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR clarifies that only one MC gateway UE to MC service binding at the time can be active and MC gateway UE reselection for a particular MC service is only applied when the MC gateway client is disconnected from the MC gateway UE. This rule is applicable to all MC clients associated with the same MC gateway client. Secondly, the CR details when a disconnection can take place, i.e., when there is no ongoing communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May allow multiple MC gateway UE connections for one MC service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1, 11.5.4.1, 11.5.4.2.3, 11.5.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-221632 |  |  |  |  |  |  |  |  |

********************************************1st Change*******************************************

## 11.1	General

An MC gateway UE enables MC service access for a MC service user residing on non-3GPP capable devices and for devices which are unable to host MC service clients. In order to be able to participate in a MC service, the MC gateway UE provides the following necessary functions to enable the MC service access for such MC clients:

-	MC gateway client to MC gateway association: Authorised binding and use of non-3GPP based access with an MC service client. Indicate associated MC gateway clients when the MC gateway UE cannot serve the connected MC service clients (any longer).

-	3GPP transport resources: Sharing of MC gateway UE 3GPP transport resources among the associated MC service clients.

-	MC client identification: Independent MC service client identification, MC service client authorisation and MC service profile association.

-	Location Management: Providing location management support for the associated MC clients to handle the location information and triggers related to 3GPP access network.

-	MC client communication integrity: Ensure that signalling, media content and traffic attributes, e.g. priority and QoS, of communications between the MC system and MC service client remains unaltered.

The following functions are provided by the MC gateway client:

-	MC gateway association: Allowing an MC gateway client to simultaneously bind to multiple MC gateway UEs.

NOTE 1:	An MC gateway client and its associated MC clients can only have one active binding per MC service with the associated MC gateway UE.

-	MC service selection: Choose between multiple MC gateway UEs for the required MC service capability.

-	MC service continuation: Enables reselection of the MC gateway UE association while MC service continues.

NOTE 2:	Further information about MC gateway UE selection are summarized in Annex D.

********************************************2nd Change*******************************************

##### 11.5.1.2.1	General

The solution is applied to non-3GPP devices which can host an MC client. The MC server performs authorization for the use of the MC gateway UE by the MC gateway client, i.e. the binding between the MC gateway UE and the MC gateway client is authorized and controlled by the MC server. The MC gateway client informs MC clients about the connection status.

For the period of association between MC server, MC gateway client and MC gateway UE, the MC server maintains the assignment between MC clients to the MC gateway UE used. This assignment is cancelled again with the disconnection.

11.5.1.2.2	Information flows

###### 11.5.1.2.2.1	Connection authorization request

Table 11.5.1.2.2.1-1 describes the information flow connection authorization request sent from the MC gateway client, which resides on a non-3GPP device, to the MC gateway UE, and from the MC gateway UE to the MC server.

Table 11.5.1.2.2.1-1: Connection authorization request

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| NOTE: The GW MC service ID indicates for which MC service the connection is to be authorised. |  |  |

NOTE:	The MC service ID used for MC service authorisation and the GW MC service ID used for connection authorization may have different values. Both identities are configured by the Mission Critical Organisation.

###### 11.5.1.2.2.2	Connection authorization response

Table 11.5.1.2.2.2-1 describes the information flow connection authorization response sent from the MC server to the MC gateway UE, and from the MC gateway UE to the MC gateway client residing on a non-3GPP device.

Table 11.5.1.2.2.2-1: Connection authorization response

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| Response | M | Result of the connection authorization request, service feasibility, and connection evaluation. |

##### 11.5.1.2.3	Connection authorisation procedure

The procedure for connection authorisation via an MC gateway UE towards an MC server is shown in figure 11.5.1.2.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device.

-	The MC gateway client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC gateway client has been provided with an appropriate GW MC service ID.

-	The MC gateway UE has performed service authorization for one or more MC services with the MC system.

-	The MC gateway client has selected an MC gateway UE or alternatively, the MC gateway client has performed a selection by internal criteria.

NOTE:	The internal criteria are outside the scope of the present document.

Figure 11.5.1.2.3-1: Connection authorisation with an MC server via an MC gateway UE

1.	The MC gateway client requests connection authorization via the MC gateway UE with an MC server. The MC gateway client provides the GW MC service ID.

2.	The MC gateway UE checks whether the requested MC service, as indicated by the GW MC service ID, is supported. The MC gateway UE may also check whether sufficient resources are available or if any other local criteria are met. If the MC service is supported, the procedure continues with step 3, otherwise the procedure proceeds with step 7.

NOTE:	Further information to the MC gateway UE selection is in Annex D.

3.	The MC gateway UE sends the connection authorization request to the MC server.

4.	The MC server performs an authorization check, to verify that access via the MC gateway UE is permitted. An MC server shall reject the connection authorization when the MC server receives connection authorization from a MC gateway client for a particular MC service for which the connection already exists with the same or different MC gateway UE.

5.	The MC server sends the connection authorization response to the MC gateway UE.

6.	The MC gateway UE marks the MC gateway client as authorized to have MC service access via the MC gateway UE.

7.	The MC gateway UE sends the connection authorization response to the MC gateway client.

After successful connection with the MC gateway UE, the MC clients have access to the MC server and may continue with user authentication and service authorization.

If the MC service user wishes to have access to another MC service, the above procedure is repeated. The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available.

#### 11.5.1.3	Connection authorisation for non-3GPP devices that do not host an MC client

##### 11.5.1.3.1	General

The solution is applied to non-3GPP devices which cannot host an MC client. The MC server performs authorization for the use of the MC gateway UE by the MC gateway client, i.e. the binding between the MC gateway UE and the MC client is authorized and controlled by the MC server.

NOTE:	The interworking between the MC gateway client hosted at the MC gateway UE and an MC service user is out of scope of the present document, nevertheless, the connection authorisation performed by the MC gateway UE shall enable the non-3GPP devices to get the access to MC services requested by the service user.

##### 11.5.1.3.2	Information flows

###### 11.5.1.3.2.1	Connection authorization request

Table 11.5.1.3.2.1-1 describes the information flow connection authorization request sent from the MC service client, which resides on a MC gateway UE, to the MC server.

Table 11.5.1.3.2.1-1: Connection authorization request

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| NOTE: The GW MC service ID indicates for which MC service the connection is to be authorised. |  |  |

NOTE:	The MC service ID used for MC service authorisation and the GW MC service ID used for connection authorization may have different values. Both identities are configured by the Mission Critical Organisation.

###### 11.5.1.3.2.2	Connection authorization response

Table 11.5.1.3.2.2-1 describes the information flow connection authorization response sent from the MC server to the MC gateway client residing on the MC gateway UE.

Table 11.5.1.3.2.2-1: Connection authorization response

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| Result | M | Success or failure of the connection authorization request (authorization successful/failed; service not supported). |

##### 11.5.1.3.3	Connection authorisation procedure

The procedure for connection authorisation of an MC gateway client hosted by the MC gateway UE towards an MC server is shown in figure 11.5.1.3.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device, where the MC gateway client and MC clients are hosted by the MC gateway UE.

-	The MC gateway client has selected an MC gateway UE or alternatively, the non-3GPP has performed a selection by internal criteria.

NOTE:	The internal criteria are outside the scope of the present document.

-	The MC gateway client, which is hosted by the MC gateway UE, has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC gateway UE has performed service authorization for one or more MC services with the MC system.

Figure 11.5.1.3.3-1: Connection authorisation of an MC gateway client hosted by an MC gateway UE

1.	The MC gateway client, hosted by the MC gateway UE, requests connection authorization with an MC server by providing the GW MC service ID. The MC gateway UE sends the connection authorization request to the MC server.

2.	The MC server performs an authorization check, to verify that access using the MC gateway UE is permitted. . An MC server shall reject the connection authorization when the MC server receives connection authorization from a MC gateway client for a particular MC service for which the connection already exists with the same or different MC gateway UE.

3.	The MC server sends the connection authorization response to the MC gateway client residing on the MC gateway UE.

The MC client has now access to the MC server and may continue with user authentication and service authorization.

If the MC service user wishes to have access to another MC service, the above procedure is repeated. The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available.

********************************************3rd Change*******************************************

### 11.5.4	Disconnection mechanism

#### 11.5.4.1	General

A connection using an MC gateway UE by the corresponding MC gateway client can be cancelled over time or re-established using same or another MC gateway UE. The connection/disconnection mechanism allows the MC gateway client to disconnect the use of the corresponding MC gateway UE considering the various MC client hosting scenarios.

Under certain circumstances, the connection with the corresponding MC gateway UE can change or has to be adjusted. The various reasons are detailed in the informative Annex D. For this purpose, the MC gateway UE can send a notification to the corresponding MC gateway client hosted on a non-3GPP device.

********************************************3rd Change*******************************************

##### 11.5.4.2.3	Disconnection procedure

The procedure for disconnection via an MC gateway UE towards an MC server is shown in figure 11.5.4.2.3-1.

Pre-conditions

-	The MC service user has an authorized connection via an MC gateway UE to an MC server.

-	The MC clients have no communication ongoing, e.g. group communication.

-	The MC gateway client wishes to disconnect the authorized connection.

Figure 11.5.4.2.3-1: Disconnection with an MC server via an MC gateway UE

1.	The MC gateway client requests disconnection via the MC gateway UE with an MC server. The MC gateway client of the MC service user provides the GW MC service ID.

2.	The MC gateway UE sends the disconnection request to the MC server to disconnect the authorized connection between the MC gateway client and the MC server.

3.	The MC server verifies if the connection is active and updates the connection status as disconnected.

4.	The MC server sends the disconnection response to the MC gateway UE.

5.	The MC gateway UE updates MC gateway client connection status as disconnected.

6.	The MC gateway UE sends the disconnection response to the MC gateway client.

##### 11.5.4.2.4	Connection status notification

The procedure for connection status notification initiated by an MC gateway UE towards an MC gateway client is shown in figure 11.5.4.2.4-1 informs about the status of connection status that may result into a disconnection.

Pre-conditions

-	The MC gateway client has an authorized connection via an MC gateway UE to an MC server.

-	The MC gateway UE is no longer able to provide the requested service depending on reasons further detailed in Annex D.

Figure 11.5.4.2.4-1: Connection status notification to an authorized MC gateway client

1.	The MC gateway UE wants to disconnect the connection with an MC server for the corresponding MC gateway client. The MC gateway UE sends connection status notification to the MC gateway client using the corresponding GW MC gateway ID.

2.	The connection status may result that the MC gateway client wants to disconnect the connection with the MC server (see disconnection in clause 11.5.4.2.3).

********************************************4th Change*******************************************

##### 11.5.4.3.3	Disconnection procedure

The procedure for disconnection of an MC gateway client hosted by the MC gateway UE towards an MC server is shown in figure 11.5.4.3.3-1.

Pre-conditions

-	The MC service user has an authorized connection using an MC gateway UE to an MC server.

-	The MC clients have no communication ongoing, e.g. group communication.

-	The MC gateway client hosted on a MC gateway UE wishes to disconnect the authorized connection.

Figure 11.5.4.3.3-1: Disconnection of an MC client hosted by an MC gateway UE

1.	The MC gateway client, hosted by the MC gateway UE, sends a disconnection request to the corresponding  MC server encompassing the GW MC service ID.

2.	The MC server verifies if the connection is active and updates the connection status as disconnected.

3.	The MC server sends the disconnection response to the MC gateway client residing on the MC gateway UE.

4.	The MC gateway UE updates MC gateway client connection status to disconnected.

##### 11.5.4.2.4	Connection status notification

The procedure for connection status notification initiated by an MC gateway UE towards an MC gateway client is shown in figure 11.5.4.2.4-1 informs about the status of connection status that may result into a disconnection.

Pre-conditions

-	The MC client has an authorized connection via an MC gateway UE to an MC server.

-	The MC gateway UE is no longer able to provide the requested service depending on reasons further detailed in Annex D.

Figure 11.5.4.2.4-1: Connection status notification to an authorized MC client

1.	The MC gateway UE wants to disconnect the connection with an MC server for the corresponding MC gateway client. The MC gateway UE sends connection status notification to the MC gateway client using the corresponding GW MC gateway ID.

2.	The connection status may result that the MC gateway client wants to disconnect the connection with the MC server (see disconnection in clause 11.5.4.2.3).

******************************************End of Changes*****************************************