3GPP TSG-SA3 Meeting #109	S3-223339

# Toulouse, France,  14 – 18 November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | 0037 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the threat references in TS 33.511 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  | Date: |  |  | 2022-10-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incorrect reference number in clauses 4.2.2.1.4, 4.2.2.1.5 and 4.2.2.1.14. Threat reference missing in clause 4.2.2.1.19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the reference numbers in clauses 4.2.2.1.4, 4.2.2.1.5 and 4.2.2.1.14. Add threat reference details to 4.2.2.1.19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Threat references incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.4, 4.2.2.1.5, 4.2.2.1.14, 4.2.2.1.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

##### 4.2.2.1.4	RRC integrity check failure

Requirement Name: RRC integrity check failure

Requirement Reference: TS 33.501 [2], clause 6.5.1

Requirement Description: "The RRC integrity checks shall be performed both in the ME and the gNB. In case failed integrity check (i.e. faulty or missing MAC-I) is detected after the start of integrity protection, the concerned message shall be discarded. This can happen on the gNB side or on the ME side." as specified in TS 33.501 [2], clause 6.5.1.

Threat References: TR 33.926 [5], clause D.2.2.2, Control plane data integrity protection

Test Case:

Test Name: TC-CP-DATA-RRC-INT-CHECK_gNB

Purpose:

Verify that RRC integrity check failure is handled correctly by the gNB.

Pre-Conditions:

Test environment with a UE. The UE may be simulated. RRC integrity protection is activated at the gNB.

Execution Steps

1a)	The UE sends a RRC message to the gNB without MAC-I; or

1b)	The UE sends a RRC message to the gNB with a wrong MAC-I.

2b)	The gNB verifies the integrity of the RRC message from the UE.

Expected Results:

The RRC message is discarded by the gNB after step 1a) or after step 2b).

Expected format of evidence:

Sample copies of the log files.

##### 4.2.2.1.5	UP integrity check failure

Requirement Name: UP integrity check failure

Requirement Reference: TS 33.501 [2], clause 6.6.4

Requirement Description: "If the gNB or the UE receives a PDCP PDU which fails integrity check with faulty or missing MAC-I after the start of integrity protection, the PDU shall be discarded." as specified in TS 33.501 [2], clause 6.6.4.

Threat References: TR 33.926 [5], clause D.2.2.4, User plane data integrity protection

Test Case:

Purpose:

Verify that UP integrity check failure is handled correctly by the gNB.

Pre-Conditions:

Test environment with a UE. The UE may be simulated. UP integrity protection is activated at the gNB.

Execution Steps

1a)	The UE sends a PDCP PDU to the gNB without MAC-I; or

1b)	The UE sends a PDCP PDU to the gNB with a wrong MAC-I.

2b)	The gNB verifies the integrity of the PDCP PDU from the UE.

Expected Results:

The PDCP PDU is discarded by the gNB after step 1a) or after step 2b).

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

**** NEXT CHANGE ****

##### 4.2.2.1.14	Bidding down prevention in Xn-handovers

Requirement Name: Bidding Down Prevention

Requirement Reference: TS 33.501 [2], clause 6.7.3.1

Requirement Description: "In the Path-Switch message, the target gNB shall send the UE's 5G security capabilities, UP security policy with corresponding PDU session ID received from the source gNB to the AMF." as specified in TS 33.501 [2], clause 6.7.3.1."

Threat References: TR 33.926 [5], clause D.2.2.6 Bidding Down on Xn-Handover

Test Case: the test case in subclause 4.2.2.1.10 of TS 33.216 [4]

**** NEXT CHANGE ****

##### 4.2.2.1.19	UP security activation in Inactive scenario

Requirement Name: UP security activation in Inactive scenario

Requirement Reference: TS 33.501 [2], clause 6.8.2.1.3.

Requirement Description: "If the UP security activation status can be supported in the target gNB/ng-eNB, the target gNB/ng-eNB shall use the UP security activations that the UE used at the last source cell. Otherwise, the target gNB/ng-eNB shall respond with an RRC Setup message to establish a new RRC connection with the UE." as specified in TS 33.501 [2], clause 6.8.2.1.3.

Threat Reference: TR 33.926 [5], clause D.2.2.9 State transition from inactive state to connected state

Test Name: TC_GNB_INACTIVE_TO_ACTIVE

Purpose:

Verify that the target gNB/ng-eNB uses the UP security activation status to activate the UP security.

Pre-Conditions:

-	The gNB network product shall be connected in emulated/real network environments.

-	The UE may be simulated.

Execution Steps

1. The tester shall complete a Registration Procedure and PDU Session establishment procedure to make sure the gNB configure the UP security, and get the UP security activation status.

2. The gNB sends RRC Release message with a suspend config to the UE.

3. The tester deletes the UP security activation status of the UE.

4. The tester triggers the UE to send RRC Resume message.

Expected Results:

The gNB sends RRC Setup message to the UE.

Expected format of evidence:

Screenshot containing the operational results.

**** END OF CHANGES ****