3GPP TSG-SA WG6 Meeting #51-e	S6-222852

e-meeting, 10th – 19th October 2022	(revision of S6-22abcd)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0094 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to reference point usage in switching from MBS session to unicast bearer for MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The switching from MBS session to unicast bearer or eMBMS bearer for MCData procedure uses both unicast and muticast reference points. In the current specification, the reference points specified for SDS service is for unicast delivery, FD service is for multicast delivery and DS service is for multicast delivery only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In addition to existing reference points used in the switching from MBS session to unicast bearer or eMBMS bearer for MCData procedure: - A reference point for multicast delivery for SDS service and - A reference point for unicast delivery for FD and DS services are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification creates confusion on whether specific reference points are used or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.3.3.12.3 	Switching from MBS session to unicast bearer or eMBMS bearer for MCData

The MCData service shall support the procedure for switching from MBS session to unicast bearer or eMBMS bearer as specified in clause 7.3.3.8 and clause 7.3.3.9 with the following clarifications:

-	The MC service client is the MCData client;

-	The MC service server is the MCData server; and

-	The MC service ID is the MCData ID.

The MCData service shall use the MCData-SDS-1, MCData-SDS-2, MCData-SDS-3, MCData-FD-1, MCData-FD-2, MCData-FD-3, MCData-DS-1, MCData-DS-2 and MCData-DS-3 reference points for this procedure.

* * * End of Changes * * * *