3GPP TSG-SA WG6 Meeting #51-e	S6-222853

e-meeting, 10th – 19th October 2022	(revision of S6-22abcd)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0095 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Title correction and use of group communication connect & disconnect clarification in MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The group communication connect and disconnect over MBS session procedures are defined but not referenced from any other subclauses, and which reference points are used for MBS delivery for the MCData service is not specified. The MCData clause title description is same as that of the subclause title description, and which is also not inline with the title description of the MCPTT and MCVideo clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed changes in the CR - Provides an additional information of group communication connect and disconnect over MBS session procedures usage. - Updates the clause title and is inline with description of MCPTT and MCVideo clause titles. - Some editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The group communication connect and disconnect over MBS session will not have any reference from other procedures and the MCData clause title description will remain sams as that of the subclause title description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.12, 7.3.3.12.1 and 7.3.3.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.3.3.12	MBS transmission in MCData

##### 7.3.3.12.1	General

The procedures and information flows for Short data service and file distribution of MC Data group are defined in 3GPP TS 23.282 [5].

When using 5G MBS, MCData server can determine to use either pre-created MBS session or dynamic MBS session as defined in clause 7.3.3.1 for the transmission of DL link media for different types of MCData service capabilities.

MCData may use the 5G MBS session for the MCData features short data service and file distribution for MCData group communication. The 5G MBS session can be used by any MCData group.

Both the media packets as well as application level control signalling (e.g. transmission control) to the receiving users may be sent on the MBS session. Optionally, a separate MBS Session could be used for the application level control signalling (e.g. transmission control).

When MBSF/MBSTF are deployed in 5GS, MCData server could also determine to use File distribution capabilities provided by MBSF/MBSTF for file distribution.

##### 7.3.3.12.2	Usage of MBS session for MCData

The MCData service shall support the procedure for using pre-created MBS session or dynamic MBS session as specified in Clause 7.3.3.1 with the following clarifications:

-	The MC service client is the MCData client;

-	The MC service server is the MCData server; and

-	The MC service ID is the MCData ID.

When using the procedures for pre-created or dynamic MBS session establishment for MCData, the MCData server performs the procedure of group communication connect and disconnect over MBS session at the group communication session establishment phase.

The MCData service shall use the MCData-SDS-1, MCData-SDS-3, MCData-FD-1, MCData-FD-3, MCData-DS-1 and MCData-DS-3 reference points for this procedure.

* * * End of Changes * * * *