# 3GPP TSG-SA3 Meeting #112													 	S3‑234233

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1692 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AUN3 device supporting 5G key hierarchy procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, CableLabs, Huawei, HiSilicon, Charter Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2_Sec |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As agreed in the TR conclusion for KI1 (TR 33.887), the AUN3 device supporting 5G key hierarchy should also be supported for authentication. Authentication of these devices are based on agreed solution 9 in the TR. As discussed in the solution9 and KI1, AUN3 and RG connections needs to be protected via 4-way handshake. For this, 4-way handshake, key is required at RG and UE. Therefore, new key needs to be derived and provided to RG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - AUN3 device supporting 5G key hierarchy are defined and its authentication procedure is described. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uncomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7B.7.X (new), A.9, Z.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Previous version S3-233649, S3‑233758 also merged into this CR |  |  |  |  |  |  |  |  |

*** 1st CHANGE ***

### 7B.7.X	Authentication for AUN3 devices supporting 5G key hierarchy

This clause specifies the how an AUN3 device supporting 5G key hierarchy behind 5G-RG shall be registered to the 5GC by the 5G-RG and shall be authenticated by 5GC using EAP-AKA’.

Figure 7B.7.X-1 Authentication Procedure for AUN3 devices supporting 5G key hierarchy using EAP-AKA’

Steps 1-7 are the same as steps 1-7 in clause 7B.7.2.

8. The UDM shall send to the AUSF a Nudm_UEAuthentication_Get Response message, including the EAP-AKA’ authentication vector (RAND, AUTN, XRES, CK´ and IK´), the SUPI.

Steps 9-15 are the same as steps 9-15 in clause 7B.7.2.

16. The AUSF shall verify the AKA’-Challenge message as described in RFC 5448[12]. If successful, the AUSF shall generate the KAUSF as defined in section 6.1.3.1.

17. The AUSF shall send to the AMF/SEAF an Nausf_UEAuthentication_Authenticate Response message including the EAP-Success, the Anchor key, and the SUPI.

18. In the final authentication message from the home network, if the AUSF has sent the anchor key KSEAF, the SEAF shall derive the KAMF from KSEAF and send it to the AMF.

19. The AMF shall derive KWAGF key.

NOTE: 	Whether the key KWAGF is used by the 5G-RG and W-AGF is out of the scope of 3GPP.

20. Step 20 is the same as step 19 in clause 7B.7.2.

21a-21b. If the layer 2 connection is over WLAN (IEEE 802.11), the AUN3 device and the 5G-RG use KWAGF as the PMK, from which the WLAN keys are derived.

22. Step 22 is the same as step 21 in clause 7B.7.2.

*** 2nd CHANGE ***

# A.9	KgNB, KWAGF, KTNGF, KTWIF and KN3IWF derivation function

When deriving the keys KgNB, KWAGF, KTNGF, KTWIF and KN3IWF from KAMF and the uplink NAS COUNT in the UE and the AMF the following parameters shall be used to form the input S to the KDF.

-	FC = 0x6E

-	P0 = Uplink NAS COUNT

-	L0 = length of uplink NAS COUNT (i.e. 0x00 0x04)

- 	P1 = Access type distinguisher

-	L1 = length of Access type distinguisher  (i.e. 0x00 0x01)

The values for the access type distinguisher are defined in table A.9-1. The values 0x00 and 0x03 to 0xf0 are reserved for future use, and the values 0xf1 to 0xff are reserved for private use.

The access type distinguisher shall be set to the value for 3GPP (0x01) when deriving KgNB. The access type distinguisher shall be set to the value for non-3GPP (0x02) when deriving KN3IWF, KWAGF, KTWIF or KTNGF..

Table A.9-1: Access type distinguishers

| Access type distinguisher | Value |
| --- | --- |
| 3GPP access | 0x01 |
| Non 3GPP access | 0x02 |

The input key KEY shall be the 256-bit KAMF.

This function is applied when cryptographically protected 5G radio bearers are established and when a key change on-the-fly is performed.

As N5CW devices do not support NAS over non-3GPP access, the Uplink NAS COUNT shall be set to 0 for KTWIF key generation, see clause 7A.2.4. Similarly, the AUN3 devices do not support NAS over non-3GPP access, the Uplink NAS COUNT shall be set to 0 for KWAGF key generation, see clause 7B.7.X.

*** 3rd CHANGE ***

# Z.2	Authentication of AUN3 devices by 5GC

Figure Z.2-1 Authentication Procedure for AUN3 devices by 5GC using key-generating EAP method

This authentication procedure is based on clause 7B.7.2 but differs in some steps.

Steps 1-6 are the same as steps 1-6 in clause 7B.7.2.

7.  Upon reception of the Nudm_UEAuthentication_Get Request, the UDM shall invoke the SIDF to map the SUCI to the SUPI and select an authentication method based on the SUPI and the AUN3 device indicator. When the "username" part of the SUPI is "anonymous" or omitted, the UDM may select an authentication method based on the “realm” part of the SUPI, the AUN3 device indicator, a combination of the "realm" part and the AUN3 device indicator, or the UDM local policy. When EAP-AKA´ authentication method is selected, the UDM/ARPF shall generate an authentication vector using the Access Network Identity as the KDF input parameter.

8. The UDM shall send to the AUSF a Nudm_UEAuthentication_Get Response message, including the SUPI and EAP-AKA’ authentication vector if EAP-AKA’ is selected or the selected authentication method if other key generating EAP method (e.g., EAP-TLS, EAP-TTLS, etc) is selected. According to the AUN3 subscription data, the UDM shall also send the MSK indicator to AUSF.

9. The AUN3 device and the AUSF perform the selected EAP authentication method.

Steps 10-15 are the same as steps 17-22 in clause 7B.7.2.

*** 4th CHANGE ***