3GPP TSG SA WG1 Meeting #103	S1-232618

# Gothenburg, Sweden, 21-25 August

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.837 | CR | 016 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding CPRs in the consolidated functional requirements section |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Sensing |  |  |  |  |  | Date: |  |  | 2023-08-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During discussions in S1-232433 additional CPRs are concluded and there is a need to add them into the consolidated functional requirements section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of new CPRs in the consolidated functional requirements section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consolidated functional requirements will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S1-232469 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 7.1	Consolidated functional requirements

### 7.1.1 General

Table 7.1.1-1	General Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.1-1 | The 5G system shall be able to provide 5G wireless sensing service in a sensing service area location using sensing transmitters and sensing receivers. | PR 5.2.6-1 PR 5.9.6-1 PR 5.11.6-1 P.R 5.14.6-1 PR 5.1.6-2 |  |
| CPR 7.1.1-2 | Subject to regulation and operator policy, the 5G network shall be able to activate, configure, and deactivate 5G wireless sensing based on parameters such as location and network conditions (e.g. network load). | P.R 5.14.6-3 P.R 5.13.6-7 PR 5.8.6-6 |  |
| CPR 7.1.1-3 | Subject to user consent, regulation, and operator’s policy, the 5G system shall be able to collect non-3GPP sensing data from authorized non-3GPP sensors and securely provide it to 5G network. | PR.5.4.6-1 PR 5.21.6-2 |  |
| CPR 7.1.1-4 | The 5G system shall support continuity for 5G wireless sensing service (e.g. for sensing a moving object). | PR 5.23.6 -1 PR 5.18.6.1 |  |
| CPR 7.1.1-5 | Subject to operator’s policy, the 5G System shall be able to provide the 5G wireless sensing service in case of roaming. | PR 5.24.6-1 |  |
| CPR 7.1.1-6 | Subject to user consent, regulation, and operator’s policy, the 5G system should support the combination of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result. | PR 5.21.6-3 PR 5.4.6-4 PR 5.27.6-2 |  |
| CPR 7.1.1-7 | Subject to regulation and operator’s policy, 5G network shall provide prioritization among 5G wireless sensing services (e.g. prioritizing between communication and sensing services). | PR 6.2.2-1 |  |
| CPR 7.1.1-8 | The 5G system shall be able to enable UEs without 5G coverage to use unlicensed spectrum to provide 5G wireless sensing service. | PR 5.25.6-3 PR 5.1.6-5 PR 5.25.6-2 |  |
| CPR 7.1.1-9 | Subject to regulation, the 5G system shall enable UEs supporting V2X application to perform 5G Wireless sensing when not served by RAN using the allowed ITS spectrum and unlicensed spectrum. | PR 5.30.6-2 |  |
| CPR 7.1.1-10 | The 5G system shall be able to provide sensing service to detect, identify and/or track one or more objects (e.g., UAVs, birds) and their environment. Note: There is a need to clarify “identify” during normative phase. | PR 5.12.6-2 PR 5.10.6-1 |  |
| CPR 7.1.1-11 | Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing. | PR 5.2.6-4 PR. 5.3.6-1 PR. 5.6.6-1 PR. 5.7.6-1 PR 5.8.6-2 PR.5.1.6-4 PR 5.9.6-3 PR 5.11.6-2 PR 5.13.6-3 PR 5.17.6-4 PR 5.2.6-5 PR. 5.3.6-2 PR 5.13.6-1 PR 5.21.6-1 |  |
| CPR 7.1.1-12 | Subject to operator’s policy, the 5G system may be able to use sensing assistance information to derive the sensing result. | PR 5.22.6-2 |  |

### 7.1.2 Configuration and authorization

Table 7.1.2-1	Configuration and authorization Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.2-1 | The 5G system shall be able to provide mechanisms for an MNO to configure UEs supporting V2X application for 5G Wireless sensing service when not served by RAN. | PR 5.30.6-1 |  |
| CPR 7.1.2-2 | Subject to regulation and operator’s policies, the 5G network shall be able to configure and/or authorize or revoke authorization of sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service. NOTE: Such configuration and authorization can be based on sensing transmitter or sensing receiver location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data, etc. | PR 5.2.6-3 PR 5.15.6-1 PR 5.9.6-2 PR 5.20.6-2 PR 5.8.6-1 PR 5.8.6-4 PR 5.28.6-1 PR 5.1.6-1 PR 5.5.6-3 PR 5.11.6-4 P.R 5.13.6-8 P.R 5.10.6-2 PR 5.17.6-3 PR 5.24.6-2 |  |
| CPR 7.1.2-3 | Based on location, the 5G network shall be able to ensure that sensing transmitters and sensing receivers use licensed spectrum only in network coverage and under the full control of the operator who provides the coverage. NOTE 1: The above requirement does not apply for public safety and V2X networks with dedicated spectrum, where 5G wireless sensing can be allowed out of coverage or in partial coverage as well. | PR 5.25.6-1 PR 5.20.6-1 |  |

### 7.1.3 Network exposure

Table 7.1.3-1	 Network exposure Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.3-1 | Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met. NOTE: These conditions could be e.g. the target object distance from the restricted area border or entering restricted area. | PR 5.13.6-6 |  |
| CPR 7.1.3-2 | Subject to operator’s policy, the 5G network shall be able to provide secure means to enable trusted third-party to request discovering a sensing group in the proximity of the UE that is requesting a 5G wireless sensing service from application server. | PR 5.19.6-2 |  |
| CPR 7.1.3-3 | Subject to operator’s policy and regulation, the 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g. refresh rate, period of time, sensing KPIs, geographical location) and to receive the corresponding sensing results. | PR 5.11.6-3 PR 5.13.6-4 PR 5.12.6-3 PR 5.12.6-5 PR 5.12.6-4 PR 5.5.6-1 PR 5.5.6-2 PR 5.25.6-4 PR 5.2.6-6 PR 5.9.6-4 PR 5.7.6-2 PR 5.15.6-2 PR 5.14.6-2 PR 5.10.6-3 PR 5.13.6-5 PR 5.22.6-1 PR 5.27.6-1 PR 5.1.6-3 PR 5.4.6-2 PR 5.4.6-3 |  |
| CPR 7.1.3-4 | Subject to operator’s policy, the 5G system shall be able to provide secure means for a trusted third-party to receive sensing results with contextual information. | PR 5.8.6-5 PR 5.3.6-3 |  |
| CPR 7.1.3-5 | Subject to user’s consent, regulation and operator’s policy, the 5G network may provide secure means to expose to a trusted third-party the combined sensing result derived from the joint processing of the 3GPP sensing data and non-3GPP sensing data. | PR 5.21.6-4 |  |
| CPR 7.1.3-6 | Subject to operator’s policy, the 5G network may provide secure means for the operator to expose information towards trusted third-party on whether a given sensing service is available and the estimated quality of the given service for a certain geographic area and time. | PR 5.2.6-8 PR 5.10.6-5 PR 5.18.6-3 |  |
| CPR 7.1.3-7 | Subject to operator’s policy, the 5G network may enable secure means for a trusted third party to provide sensing assistance information. | PR 5.32.6-1 |  |

### 7.1.4 Security

Table 7.1.4-1	Security Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.4-1 | The 5G system shall provide a mechanism to protect identifiable information that can be derived from the 3GPP sensing data from eavesdropping. | PR 5.16.6-1 |  |
| CPR 7.1.4-2 | The 5G system shall limit the exposure of the sensing results only to third party authorized to receive that sensing results. | PR 6.1.2-1 |  |
| CPR 7.1.4-3 | The 5G system shall support encryption, integrity protection, privacy of the 3GPP sensing data, non-3GPP sensing data and sensing results, to protect the data inside the 5G system. | PR 5.27.6-4 PR 5.27.6-3 PR 5.23.6 -2 PR 6.1.2-2 |  |
| CPR 7.1.4-5 | The 5G system shall support appropriate sensing KPIs of 5G wireless sensing for both situations where consent can be obtained from the sensing targets, and where it cannot. | PR 6.1.2-3 |  |
| CPR 7.1.4-6 | Subject to regulation and user’s consent, the 5G network may associate sensing results and identity of the user together for further processing for a sensing target that has a UE and the UE is subscribed in the same network. | PR 6.1.2-4 |  |

### 7.1.5 Charging

Table 7.1.5-1	Charging Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.5-1 | The 5G system shall be able to support charging for the 5G wireless sensing service (e.g. considering sensing KPIs, duration). | PR 5.20.6-4 PR 5.2.6-7 PR 5.28.6-2 |  |

* * * End of Changes * *