3GPP TSG-SA1 Meeting #103	S1-232010

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.843 | CR | 0001 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of Editor's Notes in TR 22.843 on 5GS to UTM exposure of location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_UAV_Ph3 |  |  |  |  |  | Date: |  |  | 2023-07-25 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are 2 Editors Notes in clause 5.5.6 which need to be resolved in order to consolidate the requirements and complete the TR. The controversial issue is to differentiate the location of a connected UE vs. the derived location of an object via Sensing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper proposes enhancements to the Potential Requirements in order to differentiate the UE location derived by 5GC Location Services or via UE methods such as GNSS, vs. the derivation of an object’s location in aerial space (and therefore of interest to the UTM) via 5G Wireless Sensing. This separation of the source of the positioning/location information should serve to be clear that (1) only connected UEs can generate and expose a UE location via 5GC Location Services or GNSS, and (2) 5G Wireless Sensing has the ability to detect connected or unconnected objects in the aerial space (subject to accuracy KPIs and object size/shape). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s notes remain. Use case not fulfilled by consolidated potential requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.6, 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- 1st CHANGE ---

### 5.5.6		Potential New Requirements needed to support the use case

[P.R 5.5.6-001] Based on operator policy and UTM’s request, the 5G system shall be able to provide UTM with a UEs’ location (specifically 5GC Location Services derived positioning or UE-derived positioning e.g. from GNSS) and identity.

[P.R 5.5.6-002] Subject to user consent and national or regional regulation, based on operator policy, and the UTM’s request, the 5G system may be able to provide aerial object location(s) derived by 5G Wireless Sensing to the UTM.

--- 2nd CHANGE ---

## 6.1	Network support and exposure for UAV usage

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.1-001 | Based on operator’s policy, the 5G system shall be able to support a method to predict, monitor network conditions and QoS (e.g. bitrate, latency, reliability) and report to 3rd party along a continuous geographic planned flight path of a UAV at specific times of its expected flight duration. | P.R 5.2.6-001 P.R 5.6.6-001 |  |
| CPR 6.1-002 | Based on operator’s policy, the 5G system shall be able to provide UTM with the information about geographic areas where UAV service requirements could or could not be met based on predicted network conditions and QoS (e.g. bitrate, latency, reliability). | P.R 5.2.6-002 |  |
| CPR 6.1-003 | Based on a 3rd party request, the 5G system shall be able to provide to the 3rd party alternative UAV flight paths, as long as UTM-provided required waypoints (e.g. start/end locations), required QoS, and exclusion zones are fulfilled. | P.R 5.2.6-003 |  |
| CPR 6.1-004 | Based on a 3rd party request and operator’s policy, the 5G system shall be able to reconfigure network resources to provide the required QoS along a UAV planned flight path, e.g. at particular geographical area(s) and time(s). | P.R 5.2.6-004 |  |
| CPR 6.1-005 | The 5G system shall be able to support a mechanism to enable a network operator to track a UAV which doesn’t have subscription with this network operator. | P.R 5.7.6-001 |  |
| CPR 6.1-006 | The 5G system shall be able to support service enablement layer exposure mechanisms for the UTM or other authorized 3rd party to provide the UAV application with configuration information to route and switch traffic between one active and one standby PLMN connection, e.g. for C2 communication reliability and redundancy purpose, or to route different traffic across different PLMN connections simultaneously, e.g. C2 traffic via one PLMN and other data via the second PLMN. NOTE 1: The above requirement can be extended to scenarios where one network is a PLMN and one is an NPN. NOTE 2: There is no impact on legacy network selection. NOTE 3: It is assumed that UAV traffic handling, over each PLMN, is subject to NW control mechanisms (e.g. in accordance with MNO routing priorities, available QoS/NW resources, etc.). | P.R 5.8.6-001 P.R 5.9.6-001 |  |
| CPR 6.1-007 | The 5G system shall be able to support mechanisms for the UTM to configure different aerial flight zones where UAV application settings and communication QoS may be different, and provide network and UAV with means to identify those flight zones. | P.R 5.10.6-001 |  |
| CPR 6.1-008 | The 5G system shall be able to support mechanisms for the UTM to provide network and UAV with policy information including UAV application settings and communication QoS to be applied in specific aerial flight zones, e.g. based on flight zone type indicated or available to the UAV in a certain geographical location. | P.R 5.10.6-002 |  |
| CPR 6.1-009 | Based on operator policy and UTM’s request, the 5G system shall be able to provide UTM with a UEs’ location and identity. | P.R 5.5.6-001 |  |
| CPR 6.1-010 | Subject to user consent and national or regional regulation, based on operator policy, and the UTM’s request, the 5G system may be able to provide aerial object location(s) derived by 5G Wireless Sensing to the UTM. | P.R 5.5.6-002 |  |

--- END OF CHANGES ---