3GPP TSG-SA2 Meeting #140-e	S2-2005808

# Online, , 19th Aug 2020 - 2nd Sep 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1235 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Error correction for imsSubscriber Data Management Service and acquistion of an 5GS IP address for |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eIMS5G_SBA |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. Annex Y states: To support IMS, the UE shall establish a PDU session with PDU session type set to IP for the corresponding DNN (see TS 23.501 [93], clause 5.6.1), and acquire an IP address according to TS 23.501 [93], clause 5.8.1.1. … But there is no clause 5.8.1.1 in 3GPP TS 23.501. The correct clause rather seems to be 5.8.2.2. 2. Table AA.2.2.2-1 only lists the AS as consumer of the imsSubscriber DataManagement Subscribe service operation. However, according to Note 6 of Table AA.2.2.2-1, this service operation " Corresponds to Cx-Pull when S-CSCF needs to fetch and subscribe to notification of changes in IMS User's Service Profile Data". Thus the S-CSCF should be added as consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In Annex Y, reference is corrected to clause 5.8.2.2 in 3GPP TS 23.501. - S-CSCF is added as service consumer of the imsSubscriber DataManagement Subscribe service operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect information in specification can cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Y.1.0, AA.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## Y.1.0	General

To support IMS, the UE shall establish a PDU session with PDU session type set to IP for the corresponding DNN (see TS 23.501 [93], clause 5.6.1), and acquire an IP address according to TS 23.501 [93], clause 5.8.2.2. If IMS Multimedia Telephony Service as specified in TS 22.173 [53] is to be used within the PDU session, SSC mode 1 shall be set for the PDU session. In this release of the specification, it is assumed that single PDU session with multiple PDU session anchors defined in TS 23.501 [93] clause 5.6.4 is not applicable for PDU sessions dedicated to IMS.

NOTE 1:	In the case of Dual Registration as defined in TS 23.501 [93], the UE will register (and if needed re-register) in IMS with the IP address of the IMS PDU session when the IMS PDU session is transferred between 5GS and EPS.

If the UE changes its IP address due to changes triggered by the 5GS procedures or due to, for example, PLMN change, then the UE shall re- register in the IMS.

If the UE acquires an additional IP address, then the UE may perform an IMS registration using this additional IP address as the contact address. If IMS registration is performed, this IMS registration may co-exist with the previous IMS registration from this UE and the UE shall be notified that this IMS registration results in multiple simultaneous registrations.

NOTE 2:	In this Release of the specification, the UE can acquire an additional IP address that can be used for registration to the IMS only outside of the 5G System.

In Dual Connectivity with 5GC case, the UE shall use the access network information based on the primary cell of the Master RAN node that is serving the UE for network location information when the UE interacts with IMS, regardless whether the IMS traffic is routed via the Master RAN node or the Secondary RAN node or both.

2nd change

### AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| imsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS |
| imsUE Context | Registration | Request/Response | S-CSCF |
| Management | DeregistrationNotification | Subscribe/Notify | S-CSCF |
| (_ImsUECM) | Deregistration | Request/Response | S-CSCF |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
| ImsUE Authentication | Get | Request/Response | S-CSCF |

End of changes