3GPP TSG-SA5 Meeting #132e 	S5-204484

# e-meeting  17th 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0257 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ADD EPS fallback redirection related Measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2020/8/3 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Under the constraints of UE and network coverage, EPS fallback is an important means to guarantee voice continuity.Restricted by the UE capability and network configuration, the EPS fallback supports the modes of handover and redirection. The measurement related to EPS fallback is helpful for evaluating voice continuity and for the operator's network planning and optimization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add EPS fallback redirection related Measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement of voice-related indicators is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.X, 5.1.1.6.X.1, A.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-204259 |  |  |  |  |  |  |  |  |

1st modified section

5.1.1.6.X	RRC redirection measurement

5.1.1.6.X.1	number of EPS fallback redirection

a)	This measurement provides the number of RRC release for EPS fallback redirection.

b)	SI

c)	Transmission of a “RRCRelease” message to UE, which contains “redirectedCarrierInfo” IE and “voiceFallbackIndication” IE indication EPS fallback for IMS voice. (see 3GPP TS 38.331).

d)	A single integer value.

e)	MM.Redirection.5gsToEpsFallback.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

Next modified section

A.x Use case of EPS fallback monitor

Under the constraints of UE and network coverage, EPS fallback is an important means to guarantee voice continuity. Restricted by the UE capability and network configuration, the EPS fallback supports the modes of handover and redirection.The measurement related to EPS fallback is helpful for evaluating voice continuity and for the operator's network planning and optimization.

End of modifications