3GPP TSG-WG SA2 Meeting #140E e-meeting  	S2-2006212

# Elbonia, August 19 – September 01, 2020	(revision of S2-2005513)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2372 | rev | 1 | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to interaction between the SMF and UPF for Availability after DDN failure event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In the information flow for Availability after DDN Failure with UPF buffering procedure, it defines if PCC is not used and there is no installed PDR with the exact same traffic descriptor, the SMF will provide a PDR with a buffer notification action within. However, for the Availability after DDN Failure event, the SMF may need to request the UPF to report the discarded packets matching the Traffic Descriptor when the SMF is aware that UE is not reachable and the has indicated to the UPF to discard packets (as specified in step 3c of clause 4.2.3.3) . - The incorrect service is used for DDN failure status notification between SMF and AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 4.15.3.2.9 to cover both of the following scenarios: - the SMF is aware that UE is not reachable,and has indicated to UPF to discard packets; - the SMF is not aware of UE’s reachability status. The update focus on the additions compare to clause 4.2.3.3 Network Triggered Service Request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Availability after DDN Failure event not working properly when the SMF is aware UE is not reachable and indicated to UPF to discard packets.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 4.15.3.2.9	Information flow for Availability after DDN Failure with UPF buffering

The procedure is used if the SMF requests the UPF to buffer packets. The procedure describes a mechanism for the Application Function to subscribe to notifications about availability after DDN failure.

Cancelling is done by sending Nnef_EventExposure_Unsubscribe request identifying the subscription to cancel with Subscription Correlation ID. Steps 2 to 7 are not applicable in the cancellation case.

Figure 4.15.3.2.9-1: Information flow for availability after DDN Failure event with UPF buffering

1.	AF interacts with NEF to subscribe availability after DDN failure subscription procedure as described in clause 4.15.3.2.7.

2.	 The SMF provides the Traffic Descriptor in the PDR and requests the UPF to report when there are corresponding downlink packets received in the UPF as described in clause 5.8.3 in 23.501 [2]. If PCC is not used and there is no installed PDR with the exact same traffic descriptor, the SMF copies the installed PDR that would have previously matched the incoming traffic described by the traffic descriptor in the notification subscription, but provides that traffic descriptor, a higher priority, and a buffer notification or drop notification action within depending on whether the SMF is aware of UE being unreachable and if so what action the SMF has indicated to the UPF as specified in step 3c of clause 4.2.3.3.

If PCC is used. the SMF checks if the Policy Control Trigger Request are set to report the reception of a subscription to DDN failure and then the SMF requests PCC Rules to the PCF before contacting the UPF; the PCF then provides PCC rule(s) taking into consideration the traffic descriptor for the subscribed DDN failure event.

3-4.	When downlink packet is received in the UPF, if in step 2 the SMF indicated drop notification to the UPF , the UPF notifies the SMF if the discarded DL data matches the traffic descriptor provided in step 2, and the SMF reports the DDN Failure status with NEF Correlation ID, by means of Nsmf_PDUSession_SMContextStatusNotify message, to the AMF indicated as notification endpoint.

When downlink packet is received in the UPF, if in step 2 the SMF indicated buffer notification to the UPF, the UPF notifies the SMF if the buffered packet matches the traffic descriptor provided in step 2, the SMF initiates Network Triggered Service Request as specified in clause 4.2.3.3. If the AMF responds Namf_Communication_N1N2MessageTransfer response with failure (e.g. due to UE not reachable, or paging no response), in addition to what is specified in clause 4.2.3.3, the SMF reports DDN Failure status with NEF Correlation ID, by means of Nsmf_PDUSession_SMContextStatusNotify message, to the AMF indicated as notification endpoint.

When the AMF receives  DDN Failure status from the SMF, the AMF shall set a Notify-on-available-after-DDN-failure flag corresponding to the Notification Correlation Id and the identifier of the UE if available.

5-6.	[Conditional] The AMF detects the UE is reachable and sends the event report(s) based on the Notify-on-available-after-DDN-failure flag, by means of Namf_EventExposure_Notify message(s), only to the NEF(s) indicated as notification endpoint(s) identified via the corresponding subscription in step 1. In this way, only the AF(s) for which DL traffic transmission failed are notified.

7.	The NEF sends Nnef_EventExposure_Notify message with the "Availability after DDN Failure" event to AF.

# * * * * End of changes * * * *