3GPP TSG-SA1 Meeting #91e	S1-203374

Electronic Meeting, 24 August - 2 September 2020	(revision of S1-203071)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 22.261 | CR | 0464 | rev | 2 | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Quality improvement ‒ enumeration of requirements in TS 22.261 (R16; this CR does not introduce any technical change) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Siemens, Sennheiser, EBU, BMWi, BBC, Bosch, ETRI, Volkswagen |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | cyberCAV |  |  |  |  |  | Date: |  |  | 2020-08-9-18 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Quality improvement of TS 22.261 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enumeration of all requirements in TS 22.261 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Currently, it is quite hard to refer to TS 22.261 requirements other than in a verbose style (“sub-clause 5.6.1, third and seventh paragraph”) or a verbatim style (namely a verbatim copy of the requirement). The lack of unique requirement identifiers makes it cumbersome to trace mentions of TS 22.261 requirements in down-stream documents (for instance by employing search macros on all SA2 Rel-16 documents). This issue does not only pertain to standardisation work in 3GPP. Examples for relevant “outside” work are feature-to-requirement gap analyses by Market Representation Partners, requirements enginnering for concrete deployments, etc. See S1-203073 for more information about the introduced numbering scheme. Release 16 is frozen, and according to table 4A in TR 21.900, category-D are thus not permissible. However, this CR does not seek to fix editorial issues, but it introduces a new, structural framework to this technical specification. 3GPP SA is thus kindly asked to waive this rule for the current CR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2, 6.1.2, 6.2.2, 6.2.3, 6.2.4, 6.3.2, 6.4.2, 6.5.2, 6.6.2, 6.7.2, 6.8, 6.9.2, 6.10.2, 6.11.2, 6.12.2, 6.13.2, 6.14.2, 6.15.2, 6.16.2, 6.17.2, 6.18.2, 6.19.2, 6.20.2, 6.21.2, 6.22.2, 6.32.2, 6.24.2, 6.25.2, 6.26.2, 6.27.2, 6.28.2, 6.29.2, 7.2.3, 7.3.2, 7.4.2, 8.2, 8.3, 8.4, 8.5, 8.6, 8.7, 8.8, 8.9, 9.1, 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF PROPOSED CHANGES

Start of Change 1

### 5.1.2	Requirements

#### 5.1.2.1	Interworking between 5G systems

[R-5.1.2.1-001] The 5G system shall support a UE with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the UE's 5G Home Mobile Network.

[R-5.1.2.1-002] The 5G system shall enable a Visited Mobile Network to provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

[R-5.1.2.1-003] The 5G system shall enable a Visited Mobile Network to provide support for services provided in the home network as well as provide services in the visited network. Whether a service is provided in the visited network or in the home network is determined on a service by service basis.

[R-5.1.2.1-004] The 5G system shall provide a mechanism for a network operator to limit access to its services for a roaming UE, (e.g. based on roaming agreement).

[R-5.1.2.1-005] The 5G system shall provide a mechanism for a network operator to direct a UE onto a partnership network for routing all or some of the UE user plane and associated control plane traffic over the partnership network, subject to an agreement between the operators.

#### 5.1.2.2	Legacy service support

[R-5.1.2.2-001] In principle, the 5G system shall support all EPS capabilities (e.g. from TSs 22.011, 22.101, 22.278, 22.185, 22.071, 22.115, 22.153, 22.173), however:

-	[R-5.1.2.2-002] Voice service continuity from NG-RAN to GERAN shall not be supported,

-	[R-5.1.2.2-003] Voice service continuity from NG-RAN to UTRAN CS should be supported (see Note),

-	[R-5.1.2.2-004] Voice service continuity from GERAN to NG-RAN shall not be supported,

-	[R-5.1.2.2-005] Voice service continuity from UTRAN to NG-RAN shall not be supported,

-	[R-5.1.2.2-006] CS fallback from NG-RAN to GERAN shall not be supported,

-	[R-5.1.2.2-007] CS fallback from NG-RAN to UTRAN shall not be supported,

-	[R-5.1.2.2-008] Seamless handover between NG-RAN and GERAN shall not be supported,

-	[R-5.1.2.2-009] Seamless handover between NG-RAN and UTRAN shall not be supported,

-	[R-5.1.2.2-010] Access to a 5G core network via GERAN or UTRAN shall not be supported,

-	[R-5.1.2.2-011] Video service continuity between 5GS and UMTS shall not be supported,

-	[R-5.1.2.2-012] IP address preservation for PS service when UE moves between 5GS and GSM/UMTS shall not be supported,

-	[R-5.1.2.2-013] Service continuity between 5GS and CDMA2000 shall not be supported.

NOTE:	Architectural or protocol changes needed to support voice service continuity from NG-RAN to UTRAN CS are expected to have minimum impact on architecture, specifications, or the development of the 5G New Core and New Radio.

#### 5.1.2.3	Interoperability with legacy 3GPP systems

[R-5.1.2.3-001] The 5G system shall support mobility procedures between a 5G core network and an EPC with minimum impact to the user experience(e.g. QoS, QoE).

End of Change 1

Start of Change 2

### 6.1.2	Requirements

#### 6.1.2.1	General

[R-6.1.2.1-001] The serving 5G network shall support providing connectivity to home and roaming users in the same network slice.

[R-6.1.2.1-002] In shared 5G network configuration, each operator shall be able to apply all the requirements from this clause to their allocated network resources.

[R-6.1.2.1-003] The 5G system shall be able to support IMS as part of a network slice.

[R-6.1.2.1-004] The 5G system shall be able to support IMS independent of network slices.

#### 6.1.2.2	Management

[R-6.1.2.2-001] The 5G system shall allow the operator to create, modify, and delete a network slice.

[R-6.1.2.2-002] The 5G system shall allow the operator to define and update the set of services and capabilities supported in a network slice.

[R-6.1.2.2-003] The 5G system shall allow the operator to configure the information which associates a UE to a network slice.

[R-6.1.2.2-004] The 5G system shall allow the operator to configure the information which associates a service to a network slice.

[R-6.1.2.2-005] The 5G system shall allow the operator to assign a UE to a network slice, to move a UE from one network slice to another, and to remove a UE from a network slice based on subscription, UE capabilities, the access technology being used by the UE, operator's policies and services provided by the network slice.

[R-6.1.2.2-006] The 5G system shall support a mechanism for the VPLMN, as authorized by the HPLMN, to assign a UE to a network slice with the needed services or to a default network slice.

[R-6.1.2.2-007] The 5G system shall enable a UE to be simultaneously assigned to and access services from more than one network slice of one operator.

[R-6.1.2.2-008] Traffic and services in one network slice shall have no impact on traffic and services in other network slices in the same network.

[R-6.1.2.2-009] Creation, modification, and deletion of a network slice shall have no or minimal impact on traffic and services in other network slices in the same network.

[R-6.1.2.2-010] The 5G system shall support scaling of a network slice, i.e. adaptation of its capacity.

[R-6.1.2.2-011] The 5G system shall enable the network operator to define a minimum available capacity for a network slice. Scaling of other network slices on the same network shall have no impact on the availability of the minimum capacity for that network slice.

[R-6.1.2.2-012] The 5G system shall enable the network operator to define a maximum capacity for a network slice.

[R-6.1.2.2-013] The 5G system shall enable the network operator to define a priority order between different network slices in case multiple network slices compete for resources on the same network.

[R-6.1.2.2-014] The 5G system shall support means by which the operator can differentiate policy control, functionality and performance provided in different network slices.

#### 6.1.2.3	Network slice constraints

[R-6.1.2.3-001] The 5G system shall support a mechanism to prevent an unauthorized UE from trying to access a radio resource dedicated to a specific private slice for any purpose other than that authorized by the associated third party.

NOTE 1: UEs that are not authorized to access a specific private slice will not be able to access it for emergency calls

[R-6.1.2.3-002] The 5G system shall support a mechanism to configure a specific geographic area in which a network slice is accessible, i.e. a UE shall be within the geographical area in order to access the network slice.

[R-6.1.2.3-003] The 5G system shall support a mechanism to limit a UE to only receiving service from an authorized slice.

#### 6.1.2.4	Cross-network slice coordination

[R-6.1.2.4-001] The 5G system shall support a mechanism to provide time stamps with a common time base at the monitoring API, for services that cross multiple network slices and 5G networks.

[R-6.1.2.4-002] The 5G system shall provide suitable APIs to coordinate network slices in multiple 5G networks so that the selected communication services of a non-public network can be extended through a PLMN (e.g. the service is supported by a slice in the non-public network and a slice in the PLMN).

[R-6.1.2.4-003] The 5G system shall provide a mechanism to enable an MNO to operate a hosted non-public network and private slice(s) of its PLMN associated with the hosted non-public network in a combined manner.

End of Change 2

Start of Change 3

### 6.2.2	General requirements

[R-6.2.2-001] The 5G network shall allow operators to optimize network behaviour (e.g. mobility management support) based on the mobility patterns (e.g. stationary, nomadic, spatially restricted mobility, full mobility) of a UE or group of UEs.

[R-6.2.2-002] The 5G system shall enable operators to specify and modify the types of mobility support provided for a UE or group of UEs.

[R-6.2.2-003] The 5G system shall optimize mobility management support for a UE or group of UEs that use only mobile originated communications.

[R-6.2.2-004] The 5G system shall support inter- and/or intra- access technology mobility procedures within 5GS with minimum impact to the user experience (e.g. QoS, QoE).

### 6.2.3	Service continuity requirements

[R-6.2.3-001] The 5G system shall enable packet loss to be minimized during inter- and/or intra- access technology changes for some or all connections associated with a UE.

[R-6.2.3-002] The 5G system shall minimize interruption time during inter- and/or intra- access technology mobility for some or all connections associated with a UE.

NOTE:	The interruption time includes all delays which have impact on service continuity.

[R-6.2.3-003] For applications that require the same IP address during the lifetime of the session, the 5G system shall enable maintaining the IP address assigned to a UE when moving across different cells and access technologies for connections associated with a UE.

[R-6.2.3-004] The 5G system shall enable minimizing impact to the user experience (e.g. minimization of interruption time) when changing the IP address and IP anchoring point for some or all connections associated with a UE.

[R-6.2.3-005] The 5G system shall support service continuity for a remote UE, when the remote UE changes from a direct network connection to an indirect network connection and vice-versa.

[R-6.2.3-006] The 5G system shall support service continuity for a remote UE, when the remote UE changes from one relay UE to another and both relay UEs use 3GPP access to the 5G core network.

For a 5G system with satellite access, the following requirements apply:

-	[R-6.2.3-007] The 5G system shall support service continuity between 5G terrestrial access network and 5G satellite access networks owned by the same operator or owned by 2 different operators having an agreement.

### 6.2.4	Roaming related requirements

For a 5G system with satellite access, the following requirements apply:

-	[R-6.2.4-001] A 5G system with satellite access shall enable roaming of UE supporting both satellite access and terrestrial access between 5G satellite networks and 5G terrestrial networks.

-	[R-6.2.4-002] UEs supporting satellite access shall support optimized network selection and reselection to PLMNs with satellite access, based on home operator policy.

End of Change 3

Start of Change 4

### 6.3.2	Requirements

#### 6.3.2.1	General

[R-6.3.2.1-001] Based on operator policy, the 5G system shall enable the UE to select, manage, and efficiently provision services over the 3GPP or non-3GPP access.

[R-6.3.2.1-002] Based on operator policy, the 5G system shall support steering a UE to select certain 3GPP access network(s).

[R-6.3.2.1-003] Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g. from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type.

[R-6.3.2.1-004] Based on operator policy, the 5G system shall be able to provide simultaneous data transmission via different access technologies, (e.g. NR, E-UTRA, non-3GPP), to access one or more 3GPP services.

[R-6.3.2.1-005] When a UE is using two or more access technologies simultaneously, the 5G system shall be able to optimally distribute user traffic over select between access technologies in use, taking into account e.g. service, traffic characteristics, radio characteristics, and UE's moving speed.

[R-6.3.2.1-006] The 5G system shall be able to support data transmissions optimized for different access technologies (e.g. 3GPP, non-3GPP) for UEs that are simultaneously connected to the network via different accesses.

[R-6.3.2.1-007] Based on operator policy, the 5G system shall be able to add or drop the various access connections for a UE during a session.

[R-6.3.2.1-008] The 5G system shall be able to support mobility between the supported access networks (e.g. NG-RAN, WLAN, fixed broadband access network, 5G satellite access network).

[R-6.3.2.1-009] The 5G system shall support UEs with multiple radio and single radio capabilities.

[R-6.3.2.1-010] The 5G system shall support dynamic and static network address allocation of a common network address to the UE over all supported access types.

[R-6.3.2.1-011] The 5G system shall support a set of identities for a single user in order to provide a consistent set of policies and a single set of services across 3GPP and non-3GPP access types.

[R-6.3.2.1-012] The 5G system shall support the capability to operate in licensed and/or unlicensed bands.

#### 6.3.2.2	E-UTRA access

[R-6.3.2.2-001] The 5G system shall be able to support seamless handover between NR and E-UTRA.

[R-6.3.2.2-002] The 5G system shall support UEs with dual radio capability (i.e. a UE that can transmit on NR and E-UTRA simultaneously) as well as UEs with single radio capability (i.e. a UE that cannot transmit on NR and E-UTRA simultaneously).

#### 6.3.2.3	Satellite access

[R-6.3.2.3-001] The 5G system shall be able to provide services using satellite access.

[R-6.3.2.3-002] A 5G system with satellite access shall support different configurations where the radio access network is either a satellite NG-RAN or a non-3GPP satellite access network, or both.

[R-6.3.2.3-003] A UE supporting satellite access shall be able to provide or assist in providing its location to the 5G network.

[R-6.3.2.3-004] A 5G system with satellite access shall be able to determine a UE’s location in order to provide service (e.g. route traffic, support emergency calls) in accordance with the governing national or regional regulatory requirements applicable to that UE.

#### 6.3.2.4	Fixed broadband access

[R-6.3.2.4-001] The 5G system shall be able to efficiently support connectivity using fixed broadband access.

NOTE:	The specification of fixed broadband access network is outside the scope of 3GPP.

[R-6.3.2.4-002] The 5G system shall support use of a relay UE that supports multiple access types (e.g. 5G RAT, WLAN access, fixed broadband access).

[R-6.3.2.4-003] The 5G system shall support use of a home base station that supports multiple access types (e.g. 5G RAT, WLAN access, fixed broadband access).

End of Change 4

Start of Change 5

### 6.4.2	Requirements

#### 6.4.2.1	General

[R-6.4.2.1-001] The 5G system shall minimize control and user plane resource usage for data transfer from send only UEs.

[R-6.4.2.1-002] The 5G system shall minimize control and user plane resource usage for stationary UEs (e.g. lower signalling to user data resource usage ratio).

[R-6.4.2.1-003] The 5G system shall minimize control and user plane resource usage for transfer of infrequent small data units.

[R-6.4.2.1-004] The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of small data units.

[R-6.4.2.1-005] The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of continuous uplink data that requires both high data rate (e.g. 10 Mbps) and very low end-to-end latency (e.g. 1-10 ms).

[R-6.4.2.1-006] The 5G network shall optimize the resource use of the control plane and/or user plane to support high density connections (e.g. 1 million connections per square kilometre) taking into account, for example, the following criteria:

-	type of mobility support;

-	communication pattern (e.g. send-only, frequent or infrequent);

-	characteristics of payload (e.g. small or large size data payload);

-	characteristics of application (e.g. provisioning operation, normal data transfer);

-	UE location;

-	timing pattern of data transfer (e.g. real time or non-delay sensitive).

[R-6.4.2.1-007] The 5G system shall efficiently support service discovery mechanisms where UEs can discover, subject to access rights:

-	status of other UEs (e.g. sound on/off);

-	capabilities of other UEs (e.g. the UE is a relay UE) and/or;

-	services provided by other UEs (e.g. the UE is a colour printer).

[R-6.4.2.1-008] The 5G system shall be able to minimise the amount of wireless backhaul traffic (e.g. consolidating data transmissions to 1 larger rather than many smaller), when applicable (e.g. providing service in an area subject to power outages).

[R-6.4.2.1-009] The 5G system shall support small form factor UEs with single antenna.

NOTE:	Small form factor UEs are typically expected to have the diagonal less than 1/5 of the lowest supported frequency wave length.

For a 5G system with satellite access, the following requirements apply:

-	[R-6.4.2.1-010] The 5G system with satellite access shall support the use of satellite links between the radio access network and core network, by enhancing the 3GPP system to handle the latencies introduced by satellite backhaul.

-	[R-6.4.2.1-011] A 5G system with satellite access shall be able to support meshed connectivity between satellites interconnected with intersatellite links.

#### 6.4.2.2	Efficient bulk operations for IoT

[R-6.4.2.2-001] The 5G system shall support a timely, efficient, and/or reliable mechanism to transmit the same information to multiple UEs.

#### 6.4.2.3	Efficient management for IoT

[R-6.4.2.3-001] The 5G network shall optimize the resource use of the control plane and/or user plane to manage (e.g. provide service parameters, activate, deactivate) a UE.

[R-6.4.2.3-002] The 5G network shall be able to provide policies for background data transfer to a UE so that the 5G system can optimally use the control plane and/or user plane resources.

#### 6.4.2.4	Efficient control plane

[R-6.4.2.4-001] The 5G system shall minimize the signalling that is required prior to user data transmission.

NOTE:	The amount of signalling overhead may vary based on the amount of data to be transmitted, even for the same UE.

End of Change 5

Start of Change 6

### 6.5.2	Requirements

[R-6.5.2-001] Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path between UEs attached to the same network, modifying the path as needed when the UE moves during an active communication.

[R-6.5.2-002] The 5G network shall enable a Service Hosting Environment provided by operator.

[R-6.5.2-003] Based on operator policy, the 5G network shall be able to support routing of data traffic between a UE attached to the network and an application in a Service Hosting Environment for specific services, modifying the path as needed when the UE moves during an active communication.

[R-6.5.2-004] Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path, modifying the path as needed when the UE moves or application changes location, between a UE in an active communication and:

-	an application in a Service Hosting Environment; or

-	an application server located outside the operator’s network.

[R-6.5.2-005] The 5G network shall maintain user experience (e.g. QoS, QoE) when a UE in an active communication moves from a location served by a Service Hosting Environment to:

-	another location served by a different Service Hosting Environment; or

-	another location served by an application server located outside the operator’s network, and vice versa.

[R-6.5.2-006] The 5G network shall maintain user experience (e.g. QoS, QoE) when an application for a UE moves as follows:

-	within a Service Hosting Environment; or

-	from a Service Hosting Environment to another Service Hosting Environment; or

-	from a Service Hosting Environment to an application server located place outside the operator’s network, and vice versa.

[R-6.5.2-007] The 5G network shall be able to interact with applications in a Service Hosting Environment for efficient network resource utilization and offloading data traffic to the Service Hosting Environment close to the UE's point of attachment to the access network.

[R-6.5.2-008] The 5G network shall support configurations of the Service Hosting Environment in the network (e.g. access network, core network), that provide application access close to the UE's point of attachment to the access network.

[R-6.5.2-009] The 5G system shall support mechanisms to enable a UE to access the closest Service Hosting Environment for a specific hosted application or service.

[R-6.5.2-010] The 5G network shall enable instantiation of applications for a UE in a Service Hosting Environment close to the UE's point of attachment to the access network.

[R-6.5.2-011] The 5G system shall be able to suspend or stop application instances in a Service Hosting Environment.

NOTE:	Not all applications will always be available in all Service Hosting Environments. Therefore, it may be needed to instantiate an application at a Service Hosting Environment nearby for serving a particular UE.

[R-6.5.2-012] Based on operator policy, the 5G system shall provide a mechanism such that one type of traffic (from a specific application or service) to/from a UE can be offloaded close to the UE's point of attachment to the access network, while not impacting other traffic type to/from that same UE.

For a 5G system with satellite access, the following requirements apply:

- [R-6.5.2-013] A 5G system with satellite access shall be able to select the communication link providing the UE with the connectivity that most closely fulfils the agreed QoS

- [R-6.5.2-014] A 5G system with satellite access shall be capable of supporting simultaneous use of 5G satellite access network and 5G terrestrial access networks.

- [R-6.5.2-015] A 5G system with satellite access shall be able to support both UEs supporting only satellite access and UEs supporting simultaneous connectivity to 5G satellite access network and 5G terrestrial access network.

End of Change 6

Start of Change 7

### 6.6.2	Requirements

[R-6.6.2-001] The 5G system shall enable efficient delivery of content from a content caching application under the control of the operator (e.g. a cache located close to the UE).

[R-6.6.2-002] The 5G system shall support a content caching application in a UE under the control of the operator.

[R-6.6.2-003] The 5G system shall support configurations of content caching applications in the network (e.g. access network, core network), that provide content close to the UE.

[R-6.6.2-004] Based on operator policy, the 5G system shall support an efficient mechanism for selection of a content caching application (e.g. minimize utilization of radio, backhaul resources and/or application resource) for delivery of the cached content to the UE.

[R-6.6.2-005] The 5G system shall support a mechanism for the operator to manage content distribution across content caching applications.

[R-6.6.2-006] The 5G system shall support delivery of cached content from a content caching application via the broadcast/multicast service.

For a 5G system with satellite access, the following requirements apply:

-	[R-6.6.2-007] A 5G system with satellite access shall be able to optimise the delivery of content from a content caching application by taking advantage of satellites in supporting ubiquitous service, as well as broadcasting/multicasting on very large to global coverages.

End of Change 7

Start of Change 8

### 6.7.2		Requirements

[R-6.7.2-001] The 5G system shall allow flexible mechanisms to establish and enforce priority policies among the different services (e.g. MPS, Emergency, medical, Public Safety) and users.

NOTE 1:	Priority between different services is subject to regional or national regulatory and operator policies.

[R-6.7.2-002] The 5G system shall be able to provide the required QoS (e.g. reliability, end-to-end latency, and bandwidth) for a service and support prioritization of resources when necessary for that service.

[R-6.7.2-003] The 5G system shall enable the network operator to define and statically configure a maximum resource assignment for a specific service that can be adjusted based on the network state (e.g. during congestion, disaster, emergency and DDoS events) subject to regional or national regulatory and operator policies.

[R-6.7.2-004] The 5G system shall allow decoupling of the priority of a particular communication from the associated QoS characteristics such as end-to-end latency and reliability.

[R-6.7.2-005] The 5G system shall be able to support a harmonised QoS and policy framework applicable to multiple accesses.

[R-6.7.2-006] The 5G system shall be able to support E2E (e.g. UE to UE) QoS for a service.

NOTE 2:	E2E QoS needs to consider QoS in the access networks, backhaul, core network, and network to network interconnect.

[R-6.7.2-007] The 5G system shall be able to support QoS for applications in a Service Hosting Environment.

[R-6.7.2-008] A 5G system with multiple access technologies shall be able to select the combination of access technologies to serve an UE on the basis of the targeted priority, pre-emption, QoS parameters and access technology availability

## 6.8	Dynamic policy control

[R-6.8-001] The 5G system shall support the creation and enforcement of prioritisation policy for users and traffic, during connection setup and when connected.

NOTE:	Prioritisation, pre-emption, and precedence of critical traffic associated with certain priority services (e.g. MPS and Emergency) are subject to regional/national regulatory and operator policies.

[R-6.8-002] The 5G system shall support optimised signalling for prioritised users and traffic where such signalling is prioritized over other signalling traffic.

[R-6.8-003] Based on operator policy, the 5G system shall allow flexible means for authorized entities to create and enforce priority among the different service flows.

[R-6.8-004] Based on operator policy, the 5G system shall support a real-time, dynamic, secure and efficient means for authorized entities (e.g. users, context aware network functionality) to modify the QoS and policy framework. Such modifications may have a variable duration.

[R-6.8-005] Based on operator policy, the 5G system shall maintain a session when prioritization of that session changes in real time, provided that the new priority is above the threshold for maintaining the session.

End of Change 8

Start of Change 9

### 6.9.2	Requirements

The following set of requirements complement the requirements listed in 3GPP TS 22.278 [5], clauses 7B and 7C.

[R-6.9.2-001] The connection between a remote UE and a relay UE shall be able to use 3GPP RAT or non-3GPP RAT and use licensed or unlicensed band.

[R-6.9.2-002] The connection between a remote UE and a relay UE shall be able to use fixed broadband technology.

[R-6.9.2-003] The 5G system shall support indirect network connection mode in a VPLMN when a remote UE and a relay UE subscribe to different PLMNs and both PLMNs have a roaming agreement with the VPLMN.

[R-6.9.2-004] The 5G system shall be able to support a UE using simultaneous indirect and direct network connection mode.

[R-6.9.2-005] The 5G system shall be able to support QoS for a user traffic session between the remote UE and the network using 3GPP access technology.

For a 5G system with satellite access, the following requirements apply:

-	[R-6.9.2-006] A 5G system with satellite access shall be able to support relay UE’s with satellite access.

NOTE:	The connection between a relay UE and a remote UE is the same regardless of whether the relay UE is using satellite access or not.

-	[R-6.9.2-007] A 5G system with satellite access shall support mobility management of relay UEs and the remote UEs connected to the relay UE between a 5G satellite access network and a5G terrestrial network, and between 5G satellite access networks.

-	[R-6.9.2-008] A 5G system with satellite access shall support joint roaming between different 5G networks of a relay UE and the remote UEs connected to that relay UE.

End of Change 9

Start of Change 10

### 6.10.2	Requirements

The following set of requirements complement the requirements listed in 3GPP TS 22.101 [6], clause 29.

[R-6.10.2-001] Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted third party to create, modify, and delete network slices used for the third party.

[R-6.10.2-002] Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third party to monitor the network slice used for the third party.

[R-6.10.2-003] Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third party to define and update the set of services and capabilities supported in a network slice used for the third party.

[R-6.10.2-004] Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third party to configure the information which associates a UE to a network slice used for the third party.

[R-6.10.2-005] Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third party to configure the information which associates a service to a network slice used for the third party.

[R-6.10.2-006] Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third party to assign a UE to a network slice used for the third party, to move a UE from one network slice used for the third party to another network slice used for the third party, and to remove a UE from a network slice used for the third party based on subscription, UE capabilities, and services provided by the network slice.

[R-6.10.2-007] The 5G network shall provide a mechanism to expose broadcasting capabilities to trusted third party broadcasters' management systems.

[R-6.10.2-008] Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted third party to manage this trusted third party owned application(s) in the operator's Service Hosting Environment.

[R-6.10.2-009] Based on operator policy, the 5G network shall provide suitable APIs to allow a third party to monitor this trusted third party owned application(s) in the operator's Service Hosting Environment.

[R-6.10.2-010] Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third party to scale a network slice used for the third party, i.e. to adapt its capacity.

[R-6.10.2-011] Based on operator policy, a 5G network shall provide suitable APIs to allow one type of traffic (from trusted third party owned applications in the operator's Service Hosting Environment) to/from a UE to be offloaded to a Service Hosting Environment close to the UE's location.

[R-6.10.2-012] Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third party application to request appropriate QoE from the network.

[R-6.10.2-013] Based on operator policy, the 5G network shall expose a suitable API to an authorized third party to provide the information regarding the availability status of a geographic location that is associated with that third party.

[R-6.10.2-014] Based on operator policy, the 5G network shall expose a suitable API to allow an authorized third party to monitor the resource utilisation of the network service (radio access point and the transport network (front, backhaul)) that are associated with the third party.

[R-6.10.2-015] Based on operator policy, the 5G network shall expose a suitable API to allow an authorized third party to define and reconfigure the properties of the communication services offered to the third party.

[R-6.10.2-016] Based on operator policy, the 5G network shall expose a suitable API to provide the security logging information of UEs, for example, the active 3GPP security mechanisms (e.g. data privacy, authentication, integrity protection) to an authorized third party.

[R-6.10.2-017] Based on operator policy, the 5G network shall be able to acknowledge within 100ms a communication service request from an authorized third party via a suitable API.

[R-6.10.2-018] The 5G network shall provide suitable APIs to allow a trusted third party to monitor the status (e.g. locations, lifecycle, registration status) of its own UEs.

NOTE: The number of UEs could be in the range from single digit to tens of thousands.

[R-6.10.2-019] The 5G network shall provide suitable APIs to allow a trusted third party to get the network status information of a private slice dedicated for the third party, e.g. the network communication status between the slice and a specific UE.

[R-6.10.2-020] The 5G system shall support APIs to allow the non-public network to be managed by the MNO’s Operations System.

[R-6.10.2-021] The 5G system shall provide suitable APIs to allow third party infrastructure (i.e. physical/virtual network entities at RAN/core level) to be used in a private slice.

[R-6.10.2-022] A 5G system shall provide suitable APIs to enable a third party to manage its own non-public network and its private slice(s) in the PLMN in a combined manner.

[R-6.10.2-023] The 5G system shall support suitable APIs to allow an MNO to offer automatic configuration services (for instance, interference management) to non-public networks deployed by third parties and connected to the MNO’s Operations System through standardized interfaces.

End of Change 10

Start of Change 11

### 6.11.2	Requirements

[R-6.11.2-001] The 5G system shall support network resource utilization efficiently and network optimization based on system information, including:

-	network conditions, such as network load and congestion information;

-	information on served UEs such as access information (e.g. 3GPP access, non-3GPP access), cell type (e.g. macro cell, small cell), user experienced data rate;

-	application's characteristics (e.g. expected traffic over time);

-	information on prioritized communication such as user subscription profile and priority level, priority services (e.g. MPS, Emergency, and Public Safety), application used for priority communications (e.g. voice, video, and data) and traffic associated with priority communications (signalling and media);

-	subject to user consent, enhanced traffic characteristic of UE (e.g. Mobility information (e.g. no mobility, nomadic, spatially restricted mobility, full mobility), location, sensor-level information (e.g. direction, speed, power status, display status, other sensor information installed in the UE), application-level information (e.g. foreground applications, running background application, and user settings).

[R-6.11.2-002] The 5G system shall support mechanisms to collect system information for network optimization within an operator configured time scale.

End of Change 11

Start of Change 12

### 6.12.2	Requirements

[R-6.12.2-001] The 5G network shall enable operators to support wireless self-backhaul using NR and E-UTRA.

[R-6.11.2-002] The 5G network shall support flexible and efficient wireless self-backhaul for both indoor and outdoor scenarios.

[R-6.11.2-003] The 5G network shall support flexible partitioning of radio resources between access and backhaul functions.

[R-6.11.2-004] The 5G network shall support autonomous configuration of access and wireless self-backhaul functions.

[R-6.11.2-005] The 5G network shall support multi-hop wireless self-backhauling.

NOTE 1: This is to enable flexible extension of range and coverage area.

[R-6.11.2-006] The 5G network shall support autonomous adaptation on wireless self-backhaul network topologies to minimize service disruptions.

[R-6.11.2-007] The 5G network shall support topologically redundant connectivity on the wireless self-backhaul.

NOTE 2: This is to enhance reliability and capacity and reduce end-to-end latency.

End of Change 12

Start of Change 13

### 6.13.2	Requirements

The following set of requirements complement the requirements listed in 3GPP TS 22.146 [7], TS 22.246 [8] and TS 22.101 [6], clause 32.

[R-6.13.2-001] The 5G system shall support operation of downlink only broadcast/multicast over a specific geographic area (e.g. a cell sector, a cell or a group of cells).

[R-6.13.2-002] The 5G system shall support operation of a downlink only broadcast/multicast system over a wide geographic area in a spectrally efficient manner for stationary and mobile UEs.

[R-6.13.2-003] The 5G system shall enable the operator to reserve 0% to 100% of radio resources of one or more radio carriers for the delivery of broadcast/multicast content.

[R-6.13.2-004] The 5G network shall allow the UE to receive content via a broadcast/multicast radio carrier while a concurrent data session is ongoing over another radio carrier.

[R-6.13.2-005] The 5G system shall be able to support broadcast/multicast of UHD streaming video (e.g. 4K/8K UHD).

NOTE:	Taking into account the bandwidth needs for different streaming video resolution.

[R-6.13.2-006] The 5G network shall allow the operator to configure and broadcast multiple quality levels (i.e. video resolutions) of broadcast/multicast content for the same user service in a stand-alone 3GPP based broadcast/multicast system.

[R-6.13.2-007] The 5G network shall support parallel transfer of multiple quality levels (i.e. video resolutions) of broadcast/multicast content for the same user service to the same UE taking into account e.g. UE capability, radio characteristics, application information.

[R-6.13.2-008] The 5G system shall support parallel transfer of multiple multicast/broadcast user services to a UE.

[R-6.13.2-009] The 5G system shall support a stand-alone multicast/broadcast network comprising of multiple cells with inter-site distances of up to 200 km.

[R-6.13.2-010] The 5G system shall support multicast/broadcast via a 5G satellite access network, or via a combination of a 5G satellite access network and other 5G access networks.

End of Change 13

Start of Change 14

### 6.14.2	Requirements

[R-16.4.2-001] An IoT device which is able to access a 5G PLMN in direct network connection mode using a 3GPP RAT shall have a 3GPP subscription.

[R-16.4.2-002] The 5G system shall allow the operator to identify a UE as an IoT device based on UE characteristics (e.g. identified by an equipment identifier or a range of equipment identifiers) or subscription or the combination of both.

[R-16.4.2-003] The 5G system shall be able to provide mechanisms to change the association between a subscription and address/number of an IoT device (e.g. changing the owner and subscription information associated with the IoT device) within the same operator and in between different operators in an automated or manual way.

[R-16.4.2-004] The 5G system shall be able to support identification of subscriptions independently of identification of IoT devices. Both identities shall be secure.

[R-16.4.2-005] An IoT device which is able to connect to a UE in direct device connection mode shall have a 3GPP subscription, if the IoT device needs to be identifiable by the core network (e.g. for IoT device management purposes or to use indirect network connection mode).

[R-16.4.2-006] Based on operator policy, the 5G system shall support a mechanism to provision on-demand connectivity (e.g. IP connectivity for remote provisioning). This on-demand mechanism should enable means for a user to request on-the-spot network connectivity while providing operators with identification and security tools for the provided connectivity.

[R-16.4.2-007] The 5G system shall support a secure mechanism for a home operator to remotely provision the 3GPP credentials of a uniquely identifiable and verifiably secure IoT device.

End of Change 14

Start of Change 15

### 6.15.2	Requirements

[R-6.15.2-001] The 5G access network shall support an energy saving mode with the following characteristics:

-	the energy saving mode can be activated/deactivated either manually or automatically;

-	service can be restricted to a group of users (e.g. public safety user, emergency callers).

NOTE:	When in energy saving mode the UE's and Access transmit power may be reduced or turned off (deep sleep mode), end-to-end latency and jitter may be increased with no impact on set of users or applications still allowed.

[R-6.15.2-002] The 5G system shall support mechanisms to improve battery life for a UE over what is possible in EPS.

[R-6.15.2-003] The 5G system shall optimize the battery consumption of a relay UE via which a UE is in indirect network connection mode.

[R-6.15.2-004] The 5G system shall support UEs using small rechargeable and single coin cell batteries (e.g. considering impact on maximum pulse and continuous current).

End of Change 15

Start of Change 16

### 6.16.2	Requirements

[R-6.16.2-001] In constrained circumstances (e.g. reduced power supply), the 5G system shall be able to support a minimal user experience (e.g. user experienced data rate of [100] kbps, E2E latency of 50 ms, lower availability of the network of 95%).

[R-6.16.2-002] The 5G system shall support centralized automation and management of the network in order to reduce local management tasks.

[R-6.16.2-003] The 5G system shall support a mechanism to reduce data transfer rate at the cell edge for very large coverage area (e.g. 100 kbps for more than 100 km cell coverage, 1 Mbps for 100 km cell coverage).

[R-6.16.2-004] The 5G system shall be able to give priority to services (e.g. e-Health) when resources are limited.

End of Change 16

Start of Change 17

### 6.17.2	Requirements

[R-6.17.2-001] The 5G system shall support the extreme long-range coverage (up to 100 km) in low density areas (up to 2 user/km2).

[R-6.17.2-002] The 5G system shall support a minimum user throughput of 1 Mbps on DL and 100 kbps on UL at the edge of coverage.

[R-6.17.2-003] The 5G system shall support a minimum cell throughput capacity of 10 Mbps/cell on DL (based on an assumption of 1 GB/month/sub).

[R-6.17.2-004] The 5G system shall support a maximum of [400] ms E2E latency for voice services at the edge of coverage.

End of Change 17

Start of Change 18

### 6.18.2	Requirements

[R-6.18.2-001] The 5G system shall enable users to obtain services from more than one network simultaneously on an on-demand basis.

[R-6.18.2-002] For a user with a single operator subscription, the use of multiple serving networks operated by different operators shall be under the control of the home operator.

[R-6.18.2-003] When a service is offered by multiple operators, the 5G system shall be able to maintain service continuity with minimum service interruption when the serving network is changed to a different serving network operated by a different operator.

NOTE 1: A business agreement is required between the network operators.

[R-6.18.2-004] In the event of the same service being offered by multiple operators, unless directed by the home operator's network, the UE shall be prioritized to receive subscribed services from the home operator's network.

NOTE 2: If the service is unavailable (e.g. due to lack of network coverage) from the home operator's network, the UE may be able to receive the service from another operator's network.

NOTE 3: QoS provided by the partner operator's network for the same service will be based on the agreement between the two operators and could be different than that provided by the home operator's network.

End of Change 18

Start of Change 19

### 6.19.2	Requirements

The following set of requirements complement the requirements listed in 3GPP TS 22.011 [3], clause 3.2.

[R-6.19.2-001] The 5G system shall support selection among any available PLMN/RAT combinations, identified through their respective PLMN identifier and Radio Access Technology identifier, in a prioritised order. The priority order may, subject to operator policies, be provisioned in an Operator Controlled PLMN Selector lists with associated RAT identifiers, stored in the 5G UE.

[R-6.19.2-002] The 5G system shall support, subject to operator policies, a User Controlled PLMN Selector list stored in the 5G UE, allowing the UE user to specify preferred PLMNs with associated RAT identifier in priority order.

End of Change 19

Start of Change 20

### 6.20.2	Requirements

The 3GPP system supports the transport of messages with different performance requirements to support V2X scenarios. The associated requirements are described in eV2X 3GPP TS 22.186 [9].

End of Change 20

Start of Change 21

### 6.21.2	Requirements

Requirements related to NG-RAN sharing are described in 3GPP TS 22.101 [6], clause 28.2.

[R-6.21.2-001] A 5G satellite access network shall support NG-RAN sharing.

End of Change 21

Start of Change 22

### 6.22.2	Requirements

#### 6.22.2.1	General

[R-6.22.2.1-001] Based on operator’s policy, the 5G system shall be able to prevent UEs from accessing the network using relevant barring parameters that vary depending on Access Identity and Access Category. Access Identities are configured at the UE as listed in Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

[R-6.22.2.1-002] The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in one or more areas of the RAN.

[R-6.22.2.1-003] The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the UE.

[R-6.22.2.1-004] In the case of multiple core networks sharing the same RAN, the RAN shall be able to apply access control for the different core networks individually.

[R-6.22.2.1-005] The unified access control framework shall be applicable both to UEs accessing the 5G CN using E-UTRA and to UEs accessing the 5G CN using NR.

[R-6.22.2.1-006] The unified access control framework shall be applicable to UEs in RRC Idle, RRC Inactive, and RRC Connected at the time of initiating a new access attempt (e.g. new session request).

NOTE 1:	"new session request" in RRC Connected refers to events, e.g. new MMTEL voice or video session, sending of SMS (SMS over IP, or SMS over NAS), sending of IMS registration related signalling, new PDU session establishment, existing PDU session modification, and service request to re-establish the user plane for an existing PDU session.

[R-6.22.2.1-007] The 5G system shall support means by which the operator can define operator-defined Access Categories to be mutually exclusive.

NOTE 2:	Examples of criterion of operator-defined Access Categories are network slicing, application, and application server.

[R-6.22.2.1-008] The unified access control framework shall be applicable to inbound roamers to a PLMN.

[R-6.22.2.1-009] The serving PLMN should be able to provide the definition of operator-defined Access Categories to the UE.

#### 6.22.2.2	Access identities

Table 6.22.2.2-1: Access Identities

| Access Identity number | UE configuration |
| --- | --- |
| 0 | UE is not configured with any parameters from this table |
| 1 (NOTE 1) | UE is configured for Multimedia Priority Service (MPS). |
| 2 (NOTE 2) | UE is configured for Mission Critical Service (MCS). |
| 3-10 | Reserved for future use |
| 11 (NOTE 3) | Access Class 11 is configured in the UE. |
| 12 (NOTE 3) | Access Class 12 is configured in the UE. |
| 13 (NOTE 3) | Access Class 13 is configured in the UE. |
| 14 (NOTE 3) | Access Class 14 is configured in the UE. |
| 15 (NOTE 3) | Access Class 15 is configured in the UE. |
| NOTE 1: Access Identity 1 is used by UEs configured for MPS, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN, PLMNs equivalent to HPLMN, and visited PLMNs of the home country. Access Identity 1 is also valid when the UE is explicitly authorized by the network based on specific configured PLMNs inside and outside the home country. NOTE 2: Access Identity 2 is used by UEs configured for MCS, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN or PLMNs equivalent to HPLMN and visited PLMNs of the home country. Access Identity 2 is also valid when the UE is explicitly authorized by the network based on specific configured PLMNs inside and outside the home country. NOTE 3: Access Identities 11 and 15 are valid in Home PLMN only if the EHPLMN list is not present or in any EHPLMN. Access Identities 12, 13 and 14 are valid in Home PLMN and visited PLMNs of home country only. For this purpose, the home country is defined as the country of the MCC part of the IMSI. |  |

[R-6.22.2.2-001] Any number of these Access Identities may be barred at any one time.

#### 6.22.2.3	Access categories

Table 6.22.2.3-1: Access Categories

| Access Category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 | All | MO signalling resulting from paging |
| 1 (NOTE 1) | UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN. | All except for Emergency, or MO exception data |
| 2 | All | Emergency |
| 3 | All except for the conditions in Access Category 1. | MO signalling on NAS level resulting from other than paging |
| 4 | All except for the conditions in Access Category 1. | MMTEL voice (NOTE 3) |
| 5 | All except for the conditions in Access Category 1. | MMTEL video |
| 6 | All except for the conditions in Access Category 1. | SMS |
| 7 | All except for the conditions in Access Category 1. | MO data that do not belong to any other Access Categories (NOTE 4) |
| 8 | All except for the conditions in Access Category 1 | MO signalling on RRC level resulting from other than paging |
| 9 | All except for the conditions in Access Category 1 | MO IMS registration related signalling (NOTE 5) |
| 10 (NOTE 6) | All | MO exception data |
| 11-31 |  | Reserved standardized Access Categories |
| 32-63 (NOTE 2) | All | Based on operator classification |
| NOTE 1: The barring parameter for Access Category 1 is accompanied with information that define whether Access Category applies to UEs within one of the following categories: a) UEs that are configured for delay tolerant service; b) UEs that are configured for delay tolerant service and are neither in their HPLMN nor in a PLMN that is equivalent to it; c) UEs that are configured for delay tolerant service and are neither in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list on the SIM/USIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN. When a UE is configured for EAB, the UE is also configured for delay tolerant service. In case a UE is configured both for EAB and for EAB override, when upper layer indicates to override Access Category 1, then Access Category 1 is not applicable. NOTE 2: When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is neither 0 nor 2, the UE applies the Access Category based on operator classification. When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is 0 or 2, the UE applies the standardized Access Category. NOTE 3: Includes Real-Time Text (RTT). NOTE 4: Includes IMS Messaging. NOTE 5: Includes IMS registration related signalling, e.g. IMS initial registration, re-registration, and subscription refresh. NOTE 6: Applies to access of a NB-IoT-capable UE to a NB-IOT cell connected to 5GC when the UE is authorized to send exception data. |  |  |

[R-6.22.2.3-001] Access Category 0 shall not be barred, irrespective of Access Identities.

NOTE:	The network can control the amount of access attempts relating to Access Category 0 by controlling whether to send paging or not.

End of Change 22

Start of Change 23

### 6.23.2	Requirements

[R-6.23.2-001] The 5G system shall provide a mechanism for supporting real time E2E QoS monitoring within a system.

[R-6.23.2-002] The 5G network shall provide an interface to application for QoS monitoring (e.g. to initiate QoS monitoring, request QoS parameters, events, logging information, etc.).

[R-6.23.2-003] The 5G system shall be able to provide real time QoS parameters and events information to an authorized application / network entity.

NOTE:	The QoS parameters to be monitored and reported can include latency (e.g. UL/DL or round trip), jitter, packet loss rate.

[R-6.23.2-004] The 5G system shall be able to log the history of the communication events. This includes for examples parts of the SLA that are not met, timestamp of the events, and event position (e.g. UEs and radio access points associated with the events).

[R-6.23.2-005] The 5G system shall support different levels of granularity for QoS monitoring (e.g. per flow or set of flows).

[R-6.23.2-006] The 5G system shall be able to provide event notification upon detecting error that the negotiated QoS level cannot be met/guaranteed.

[R-6.23.2-007] The 5G system shall be able to provide information that identifies the type and the location of a communication error. (e.g. cell id).

[R-6.23.2-008] The 5G system shall be able to provide notification of communication events to authorized entities per pre-defined patterns (e.g. every time the bandwidth drops below a pre-defined threshold for QoS parameters the authorized entity is notified and event is logged).

[R-6.23.2-009] The 5G system shall be able to respond to a request from an authorized entity to provide real-time QoS monitoring information within a specified time after receiving the request (e.g. within 5s).

[R-6.23.2-010] The 5G system shall support an update/ refresh rate for real time QoS monitoring within a specified time (e.g. at least 1 per second).

[R-6.23.2-011] The 5G system shall be able to provide statistical information of service parameters and error types while a communication service is in operation.

[R-6.23.2-012] The 5G system shall provide information on the current availability of a specific communication service in a particular area (e.g. cell id) upon request of an authorized entity.

End of Change 23

Start of Change 24

### 6.24.2	Requirements

[R-6.24.2-001] The 3GPP system shall be able to support an Ethernet transport service.

[R-6.24.2-002] The 5G network shall support the routing of non-IP packet (e.g. Ethernet frame) efficiently for private communication between UEs within a 5G LAN-type service.

[R-6.24.2-003] The 5G network shall be able to provide the required QoS (e.g. reliability, latency, and bandwidth) for non-IP packet (e.g. Ethernet frame) for private communication between UEs within a 5G LAN-type service.

[R-6.24.2-004] The Ethernet transport service shall support routing based on information extracted from Virtual LAN (VLAN) ID by the 3GPP system.

[R-6.24.2-005] The Ethernet transport service shall support the transport of Ethernet frames between UEs that Ethernet devices are connected to

[R-6.24.2-006] The Ethernet transport service shall support the transport of Ethernet frames between a UE that an Ethernet device is connected to and an Ethernet network in DN (Data Network).

NOTE:	If more than one Ethernet devices need to be connected to a UE, they can be connected using an Ethernet switch between the devices and the UE.

[R-6.24.2-007] The Ethernet transport service shall support the transport of Ethernet broadcast frames.

[R-6.24.2-008] The Ethernet transport service shall support traffic filtering and prioritization based on source and destination MAC addresses.

[R-6.24.2-009] The Ethernet transport service shall support traffic filtering and prioritization based on Ethertype (including multiple Ethertypes in double tagging)

[R-6.24.2-010] The Ethernet transport service shall support traffic filtering and prioritization based on 802.1Q VLAN tags (including double tagging).

[R-6.24.2-011] The Ethernet transport service shall support routing based on information extracted by the 3GPP system from the Bridge Protocol Data Units created in the Ethernet network based on a Spanning Tree Protocol (e.g. RSTP.)

End of Change 24

Start of Change 25

### 6.25.2 	Requirements

[R-6.25.2-001] The 5G system shall support non-public networks.

[R-6.25.2-002] The 5G system shall support non-public networks that provide coverage within a specific geographic area.

[R-6.25.2-003] The 5G system shall support both physical and virtual non-public networks.

[R-6.25.2-004] The 5G system shall support standalone operation of a non-public network, i.e. a non-public network may be able to operate without dependency on a PLMN.

[R-6.25.2-005] Subject to an agreement between the operators and service providers, operator policies and the regional or national regulatory requirements, the 5G system shall support for non-public network subscribers:

- access to subscribed PLMN services via the non-public network;

- seamless service continuity for subscribed PLMN services between a non-public network and a PLMN;

- access to selected non-public network services via a PLMN;

- seamless service continuity for non-public network services between a non-public network and a PLMN.

[R-6.25-2.006] A non-public network subscriber to access a PLMN service shall have a service subscription using 3GPP identifiers and credentials provided or accepted by a PLMN.

[R-6.25.2-007] The 5G system shall support a mechanism for a UE to identify and select a non-public network.

NOTE:	Different network selection mechanisms may be used for physical vs virtual non-public networks.

[R-6.25.2-008] The 5G system shall support identifiers for a large number of non-public networks to minimize collision likelihood between assigned identifiers.

[R-6.25.2-009] The 5G system shall support a mechanism to prevent a UE with a subscription to a non-public network from automatically selecting and attaching to a PLMN or non-public network it is not authorized to select.

[R-6.25.2-010] The 5G system shall support a mechanism to prevent a UE with a subscription to a PLMN from automatically selecting and attaching to a non-public network it is not authorized to select.

[R-6.25.2-011] The 5G system shall support a mechanism for a PLMN to control whether a user of a UE can manually select a non-public network hosted by this PLMN that the UE is not authorized to select automatically.

[R-6.25.2-012] The 5G system shall support a change of host of a non-public network from one PLMN to another PLMN without changing the network selection information stored in the UEs of the non-public network.

End of Change 25

Start of Change 26

### 6.26.2	Requirements

#### 6.26.2.1	General

[R-6.26.2.1-001] The 5G system shall support 5G LAN-type service in a shared RAN configuration.

[R-6.26.2.1-002] The 5G system shall support 5G LAN-type service over a wide area mobile network.

[R-6.26.2.1-003] The 5G network shall support service continuity for 5G LAN-type service, i.e. the private communication between UEs shall not be interrupted when one or more UEs of the private communication move within the same network that provides the 5G LAN-type service.

[R-6.26.2.1-004] The 5G system shall support use of unlicensed as well as licensed spectrum for 5G LAN-type services.

[R-6.26.2.1-005] The 5G system shall enable the network operator to provide the same 5G LAN-type service to any 5G UE, regardless of whether it is connected via public base stations, indoor small base stations connected via fixed access, or via relay UEs connected to either of these two types of base stations.

#### 6.26.2.2	5G LAN-virtual network (5G LAN-VN)

[R-6.26.2.2-001] A UE shall be able to select a 5G LAN-VN, that the UE is a member of, for private communications.

[R-6.26.2.2-002] A 5G system shall support 5G LAN-VNs with member UEs numbering between a few to tens of thousands.

[R-6.26.2.2-003] The 5G LAN-VN shall support member UEs that are subscribed to different PLMNs, e.g. a 5G LAN-VN may span multiple countries and have member UEs that have a subscription to a PLMN in their home country.

[R-6.26.2.2-004] The 5G system shall support on-demand establishment of UE to UE, multicast, and broadcast private communication between members UEs of the same 5G LAN-VN. Multiple types of data communication shall be supported, at least IP and Ethernet.

[R-6.26.2.2-005] The 5G network shall ensure that only member UEs of the same 5G LAN-VN are able to establish or maintain private communications among each other using 5G LAN-type service.

[R-6.26.2.2-006] The 5G system shall allow member UEs of a 5G LAN-VN to join an authorized multicast session over that 5G LAN-VN.

[R-6.26.2.2-007] The 5G system shall be able to restrict private communications within a 5G LAN-VN based on UE’s location (i.e. when the UE moves out of the area it can no longer communicate on the 5G LAN-VN).

[R-6.26.2.2-008] The 5G network shall enable member UEs of a 5G LAN-VN to use multicast/broadcast over a 5G LAN-type service to communicate with required latency (e.g. 180ms).

[R-6.26.2.2-009] The 5G system shall support a mechanism to provide consistent QoE to all the member UEs of the same 5G LAN-VN.

[R-6.26.2.2-010] The 5G system shall support routing based on a private addressing scheme within the 5G LAN-VN.

#### 6.26.2.3	Creation and management

[R-6.26.2.3-001] The 5G network shall enable the network operator to scale up/down a 5G LAN-VN, e.g. the coverage, capacity for efficient consumption of network resources.

[R-6.26.2.3-002] The 5G network shall enable the network operator to create, manage, and remove 5G LAN-VN including their related functionality (subscription data, routing and addressing functionality).

[R-6.26.2.3-003] The 5G network shall enable the network operator to add one or more authorized UEs to an existing 5G LAN-VN.

NOTE 1:	A UE needs to be authorized by the MNO to use 5G LAN-type service before it can be added to any 5G LAN-VN.

NOTE 2: 	some use cases will require user permission for a UE to be added to a 5G LAN-VN.

[R-6.26.2.3-004] The 5G system shall enable the network operator to add an authorized UE to multiple independent 5G LAN-VNs.

[R-6.26.2.3-005] The 5G network shall enable the network operator to remove one or more UEs from an existing 5G LAN-VN.

NOTE 3:	Removing a UE from a 5G LAN-VN does not have impact on other 5G LAN-VNs that the UE is a member of.

[R-6.26.2.3-006] The 5G system shall enable the network operator to configure a 5G LAN-VN that is available only within a geographical area.

#### 6.26.2.4	Privacy

[R-6.26.2.4-001] The 5G system shall be able to prevent the sharing of a UE’s identifying information (e.g. SUPI, MSISDN) on private communication among UEs using 5G LAN-type service.

#### 6.26.2.5	Traffic types

[R-6.26.2.5-001] A 5G system shall support all media types (e.g. voice, data, multimedia) for 5G LAN-type service.

[R-6.26.2.5-002] The 5G system shall support traffic scenarios typically found in a home setting (from sensors to video streaming, relatively low amount of UEs per group, many devices are used only occasionally) for 5G LAN-type service.

[R-6.26.2.5-003] The 5G system shall support traffic scenarios typically found in an office setting (from sensors to very high data rates e.g. for conferencing, medium amount of UEs per group) for 5G LAN-type service.

[R-6.26.2.5-004] The 5G system shall support traffic scenarios typically found in an industrial setting (from sensors to remote control, large amount of UEs per group) for 5G LAN-type service.

#### 6.26.2.6	Discovery

[R-6.26.2.6-001] The 5G system shall enable a member UE to discover other member UEs within the same 5G LAN-VN.

[R-6.26.2.6-002] The 5G LAN-type service shall be able to support existing non-3GPP service discovery mechanisms (e.g. mechanisms to discover printers).

#### 6.26.2.7	Void

#### 6.26.2.8	Indirect communication mode

[R-6.26.2.8-001] The 5G system shall support 5G LAN-type service for authorized UEs using indirect network connection or direct network connection.

[R-6.26.2.8-002] The 5G network shall be able to provide a remote UE using 5G LAN-type service with same level of service as if the remote UE would be using a direct network connection (i.e. provide required QoS for the Ethernet packets transferred between remote UE and relay UE if they using 3GPP access).

[R-6.26.2.8-003] The 5G network shall be able to support service continuity for the private communication between a remote UE with other member UEs of the same 5G LAN-VN, when the remote UE changes from one relay UE to another or when the UE changes between direct and indirect network connection.

#### 6.26.2.9	Service exposure

[R-6.26.2.9-001] Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third party to create/remove a 5G LAN-VN.

[R-6.26.2.9-002] Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third party to manage a 5G LAN-VN dedicated for the usage by the trusted third party, including the address allocation.

[R-6.26.2.9-003] Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third party to add/remove an authorized UE to/from a specific 5G LAN-VN managed by the trusted third party.

End of Change 26

Start of Change 27

### 6.27.2	Requirements

[R-6.27.2-001] The 5G system shall provide 5G positioning services in compliance with regulatory requirements.

NOTE 1: 	example of regulatory requirements encompasses requirements on emergency calls (e.g. e911), reliability and safety requirement (RAMS) applicable to some use cases and verticals, implementation of Priority, Precedence, Preemption (PPP) mechanisms to ensure sufficient reliability metrics are reached, etc.

[R-6.27.2-002] The 5G system shall provide different 5G positioning services, supported by different single and hybrid positioning methods to supply absolute and relative positioning.

NOTE 2: 	hybrid positioning methods include both the combination of 3GPP positioning technologies and the combination of 3GPP positioning technologies with non-3GPP positioning technologies such as, for instance, GNSS (e.g. Beidou, Galileo, GPS, Glonass), Network-based Assisted GNSS and High-Accuracy GNSS, Terrestrial Beacon Systems, dead-reckoning sensors (e.g. IMU, barometer), WLAN/Bluetooth-based positioning.

[R-6.27.2-003] The 5G system shall enable an MCX UE to use the 5G positioning services to determine its position with the associated uncertainty/confidence of the position, on request, triggered by an event or periodically.

[R-6.27.2-004] The 5G System shall be able to provide the 5G positioning services in case of roaming.

[R-6.27.2-005] The 5G system shall support mechanisms to determine the UE’s position-related data for period when the UE is outside the coverage of 3GPP RAT-dependent positioning technologies but within the 5G positioning service area (e.g. within the coverage of satellite access).

[R-6.27.2-006] The 5G system shall be able to make the position-related data available to an application or to an application server existing within the PLMN, external to the PLMN, or in the User Equipment.

NOTE 3: 	the position service latency can be tailored to the use cases.

[R-6.27.2-007] The 5G system shall be able to manage and log position-related data in compliance with applicable traceability, authentication and security regulatory requirements.

[R-6.27.2-008] The 5G system shall be able to request the UE to provide its position-related-data on request, triggered by an event or periodically and to request the UE to stop providing its position-related data periodically.

NOTE 4: 	This requirement does not preclude whether the position is computed in the UE or elsewhere in the 5G System (e.g. core network).

[R-6.27.2-009] The 5G system shall support mechanisms to configure dynamically the update rate of the position-related data to fulfil different performances (e.g. power consumption, position service latency, etc.) or different location modes.

NOTE 5: 	for example, the 5G System needs to be able to request the UE to provide its location periodically with an update rate ranging from one location every [1 s-10 s] in location normal mode to one location every [30 s-300 s, or more] in location power saving mode. The 5G System needs to allow UEs to sleep for extended periods (e.g. one week), without requiring the UE to update its position data.

[R-6.27.2-010] The 5G system shall allow the UE to trigger a different update rate of the position-related data based on whether the UE is moving or not.

[R-6.27.2-011] The 5G system shall be able to determine the position-related data of the 5G positioning services with any update rate ranging from one set of position-related data every 0.1 s to one set of position-related data every month.

NOTE 6: 	the position service latency can be tailored to the use cases.

[R-6.27.2-012] The 5G System shall be able to negotiate the positioning methods according to the operator’s policy or the application’s requirements or the user’s preferences and shall support mechanisms to allow the network or the UE to trigger this negotiation.

[R-6.27.2-013] The 5G system shall supply a method for the operator to configure and manage different positioning services for different users.

[R-6.27.2-014] The 5G system shall be able to determine the reliability, and the uncertainty or confidence level, of the position-related data.

[R-6.27.2-015] The 5G system shall be able to access to the positioning methods used for calculating the position-related data and to the associated uncertainty/confidence indicators.

End of Change 27

Start of Change 28

- 6.28.2	Requirements

[R-6.28.2-001] The 5G system supports the communication services for cyber-physical control applications in the vertical domains of factories of the future (smart manufacturing), electric power distribution, central power generation, and rail-bound mass transit. The associated requirements are described in 3GPP TS 22.104 [21].

End of Change 28

Start of Change 29

### 6.29.2	Requirements

[R-6.29.2-001] The 5G system supports the MSGin5G Service. The associated service level requirements of the MSGin5G Service are described in 3GPP TS 22.262 [22].

End of Change 29

Start of Change 30

Table 7.1-1 Performance requirements for high data rate and traffic density scenarios.

|  | Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Overall user density | Activity factor | UE speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Urban macro [R-7.1-1-001] | 50 Mbps | 25 Mbps | 100 Gbps/km2 (note 4) | 50 Gbps/km2 (note 4) | 10 000/km2 | 20% | Pedestrians and users in vehicles (up to 120 km/h | Full network (note 1) |
| 2 | Rural macro [R-7.1-1-002] | 50 Mbps | 25 Mbps | 1 Gbps/km2 (note 4) | 500 Mbps/km2 (note 4) | 100/km2 | 20% | Pedestrians and users in vehicles (up to 120 km/h | Full network (note 1) |
| 3 | Indoor hotspot [R-7.1-1-003] | 1 Gbps | 500 Mbps | 15 Tbps/km2 | 2 Tbps/km2 | 250 000/km2 | note 2 | Pedestrians | Office and residential (note 2) (note 3) |
| 4 | Broadband access in a crowd [R-7.1-1-004] | 25 Mbps | 50 Mbps | [3,75] Tbps/km2 | [7,5] Tbps/km2 | [500 000]/km2 | 30% | Pedestrians | Confined area |
| 5 | Dense urban [R-7.1-1-005] | 300 Mbps | 50 Mbps | 750 Gbps/km2 (note 4) | 125 Gbps/km2 (note 4) | 25 000/km2 | 10% | Pedestrians and users in vehicles (up to 60 km/h) | Downtown (note 1) |
| 6 | Broadcast-like services [R-7.1-1-006] | Maximum 200 Mbps (per TV channel) | N/A or modest (e.g. 500 kbps per user) | N/A | N/A | [15] TV channels of [20 Mbps] on one carrier | N/A | Stationary users, pedestrians and users in vehicles (up to 500 km/h) | Full network (note 1) |
| 7 | High-speed train [R-7.1-1-007] | 50 Mbps | 25 Mbps | 15 Gbps/train | 7,5 Gbps/train | 1 000/train | 30% | Users in trains (up to 500 km/h) | Along railways (note 1) |
| 8 | High-speed vehicle [R-7.1-1-008] | 50 Mbps | 25 Mbps | [100] Gbps/km2 | [50] Gbps/km2 | 4 000/km2 | 50% | Users in vehicles (up to 250 km/h) | Along roads (note 1) |
| 9 | Airplanes connectivity [R-7.1-1-009] | 15 Mbps | 7,5 Mbps | 1,2 Gbps/plane | 600 Mbps/plane | 400/plane | 20% | Users in airplanes (up to 1 000 km/h) | (note 1) |
| NOTE 1: For users in vehicles, the UE can be connected to the network directly, or via an on-board moving base station. NOTE 2: A certain traffic mix is assumed; only some users use services that require the highest data rates [2]. NOTE 3: For interactive audio and video services, for example, virtual meetings, the required two-way end-to-end latency (UL and DL) is 2-4 ms while the corresponding experienced data rate needs to be up to 8K 3D video [300 Mbps] in uplink and downlink. NOTE 4: These values are derived based on overall user density. Detailed information can be found in [10]. NOTE 5: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |  |  |  |  |

End of Change 30

Start of Change 31

### 7.2.3	Other requirements

#### 7.2.3.1	AR/VR

Audio-visual interaction is characterised by a human being interacting with the environment or people, or controlling a UE, and relying on audio-visual feedback. In the use cases like VR and interactive conversation the latency requirements include the latencies at the application layer (e.g. codecs), which could be specified outside of 3GPP.

[R-7.2.3.1-001] To support VR environments with low motion-to-photon capabilities, the 5G system shall support:

-	motion-to-photon latency in the range of 7-15ms while maintaining the required user data rate of [1Gbps] and

-	motion-to-sound delay of [<20ms].

NOTE: 	The motion-to-photon latency is defined as the latency between the physical movement of a user's head and the updated picture in the VR headset. The motion-to-sound latency is the latency between the physical movement of a user's head and updated sound waves from a head mounted speaker reaching their ears.

[R-7.2.3.1-002] To support interactive task completion during voice conversations the 5G system shall support low-delay speech coding for interactive conversational services (100 ms, one-way mouth-to-ear).

Due to the separate handling of the audio and video component, the 5G system will have to cater for the VR audio-video synchronisation in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronization).

[R-7.2.3.1-003] To support VR environments the 5G system shall support audio-video synchronisation thresholds:

- in the range of [125ms-5ms] for audio delayed and

-	in the range of [45ms-5ms] for audio advanced.

#### 7.2.3.2 Wireless road-side infrastructure backhaul

Intelligent Transport Systems embrace a wide variety of communications-related applications that are intended to increase travel safety, minimize environmental impact, improve traffic management, and maximize the benefits of transportation to both commercial users and the general public.

Road-side infrastructure such as traffic light controllers, roadside units, traffic monitoring in urban areas and along highways and streets is wirelessly connected to traffic control centres for management and control purposes. The backhaul communication between the road-side infrastructure and the traffic control centre requires low-latency, high communication service availability, and high-capacity connections for reliable distribution of data. Road-side infrastructure is deployed alongside streets in urban areas and alongside major roads and highways every 1-2 km.

[R-7.2.3.2-001] To support wireless road-side infrastructure backhaul the 5G system shall support the performance requirements in table 7.2.3.2-1.

Table 7.2.3.2-1 Performance requirements for wireless ITS infrastructure backhaul scenario

| Scenario | Max. allowed end-to-end latency (note 1) | Survival time | Communication service availability (note 2) | Reliability (note 2) | User experienced data rate | Payload size (note 3) | Traffic density (note 4) | Connection density (note 5) | Service area dimension (note 6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| wireless road-side infrastructure backhaul [R-7.2.3.2-1-001] | 30 ms | 100 ms | 99,9999% | 99,999% | 10 Mbps | Small to big | 10 Gbps/km2 | 1 000/km2 | 2 km along a road |
| NOTE 1: This is the maximum end-to-end latency allowed for the 5G system to deliver the service in the case the end-to-end latency is completely allocated to the 5G system from the UE to the Interface to Data Network. NOTE 2: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. NOTE 3: Small: payload typically ≤ 256 bytes NOTE 4: Based on the assumption that all connected applications within the service volume require the user experienced data rate. NOTE 5: Under the assumption of 100% 5G penetration. NOTE 6: Estimates of maximum dimensions; the last figure is the vertical dimension. NOTE 7: All the values in this table are example values and not strict requirements. Deployment configurations should be taken into account when considering service offerings that meet the targets. |  |  |  |  |  |  |  |  |  |

End of Change 31

Start of Change 32

### 7.3.2	Requirements

#### 7.3.2.1	General

[R-7.3.2.1-001] The 5G System shall provide different 5G positioning services with configurable performances working points (e.g. accuracy, positioning service availability, positioning service latency, energy consumption, update rate, TTFF) according to the needs of users, operators and third parties.

[R-7.3.2.1-002] The 5G system shall support the combination of 3GPP and non-3GPP positioning technologies to achieve performances of the 5G positioning services better than those achieved using only 3GPP positioning technologies.

NOTE 1:	For instance, the combination of 3GPP positioning technologies with non-3GPP positioning technologies such as for example GNSS (e.g. Beidou, Galileo, GLONASS, and GPS), Terrestrial Beacon Systems (TBS), sensors (e.g. barometer, IMU), WLAN/Bluetooth-based positioning, can support the improvement of accuracy, positioning service availability, reliability and/or confidence level, the reduction of positioning service latency, the increase of the update rate of the position-related data, increase the coverage (service area).

NOTE 2:	The combination can vary over time to optimise the performances, and can be the combination of multiple positioning technologies at the same epoch and/or the combination of multiple positioning technologies at different epochs.

[R-7.3.2.1-003] The corresponding positioning information shall be acquired in a timely fashion, be reliable, and be available (e.g. it is possible to determine the position).

[R-7.3.2.1-004] UEs shall be able to share positioning information between each other e.g. to a controller if the location information cannot be processed or used locally.

#### 7.3.2.2	Requirements for Horizontal and Vertical positioning service levels

[R-7.3.2.2-001] The 5G system shall be able to provide positioning services with the performances requirements reported in Table 7.3.2.2-1.

NOTE: 	The requirements do not preclude any type of UE, including specific UE such as for example V2X, MTC.

Table 7.3.2.2-1 Performance requirements for Horizontal and Vertical positioning service levels

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  | Require-ment # |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |  |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h | [R-7.3.2.2-1-001] |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h | [R-7.3.2.2-1-002] |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h | [R-7.3.2.2-1-003] |
| 4 | A | 1 m | 2 m | 99.9 % | 15 ms | NA | NA | Indoor - up to 30 km/h | [R-7.3.2.2-1-004] |
| 5 | A | 0.3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h | [R-7.3.2.2-1-005] |
| 6 | A | 0.3 m | 2 m | 99.9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h | [R-7.3.2.2-1-006] |
| 7 | R | 0.2 m | 0.2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each others (note 3) |  |  | [R-7.3.2.2-1-007] |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |  |

#### 7.3.2.3	Other performance requirements

[R-7.3.2.3-001] The 5G system shall be able to provide the 5G positioning services with a TTFF less than 30 s and, for some 5G positioning services, shall support mechanisms to provide a TTFF less than 10s.

NOTE 1: 	In some services, a TTFF of less than 10s may only be achievable at the expense of a relaxation of some other performances (e.g. horizontal accuracy may be 1 m or 3 m after 10 s TTFF, and reach a steady state accuracy of 0.3 m after 30s).

[R-7.3.2.3-002] The 5G system shall support a mechanism to determine the UE's velocity with a positioning service availability of 99%, an accuracy better than 0.5 m/s for the speed and an accuracy better than 5 degree for the 3-Dimension direction of travel.

[R-7.3.2.3-003] The 5G system shall support a mechanism to determine the UE's heading with an accuracy better than 30 degrees (0.54 rad) and a positioning service availability of 99.9 % for static users and with an accuracy better than 10 degrees (0.17 rad) and a positioning service availability of 99 % for users up to 10 km/h.

[R-7.3.2.3-004] The 5G system shall support positioning technologies that allow the UE to operate at Service Level 1 for at least 12 years using less than 1800 mWh of battery capacity, assuming multiple position updates per hour.

NOTE 2:	This requirement aims energy-efficient positioning technologies draining a minimal energy on the UE battery. It derives from use cases, such as asset tracking, with a small form-factor battery representative of an IoT device. This requirement may translate into an energy consumption for the UE’s positioning functions in the order of 20 mJ per fix.

NOTE 3:	This requirement does not preclude the use of higher energy consumption to fulfil higher position update rates than the one above, or other KPIs than those of Service Level 1 (e.g. more accurate service levels).

End of Change 32

Start of Change 33

### 7.4.2	Requirements

[R-7.4.2-001] A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

NOTE 1:	 5 ms network latency is assumed and added to satellite one-way delay.

[R-7.4.2-002] A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 95 ms end-to-end latency.

NOTE 2:	 5 ms network latency is assumed and added to satellite one-way delay.

[R-7.4.2-003] A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency.

NOTE 3:	 5 ms network latency is assumed and added to satellite one-way delay.

[R-7.4.2-004] A 5G system shall support negotiation on quality of service taking into account latency penalty to optimise the QoE for UE.

[R-7.4.2-005] The 5G system with satellite access shall support high uplink data rates for 5G satellite UEs.

[R-7.4.2-006] The 5G system with satellite access shall support high downlink data rates for 5G satellite UEs.

[R-7.4.2-007] The 5G system with satellite access shall support communication service availabilities of at least 99.99%.

End of Change 33

Start of Change 34

## 8.2	General

[R-8.2-001] The 5G system shall support a secure mechanism to store cached data.

[R-8.2-002] The 5G system shall support a secure mechanism to access a content caching application.

[R-8.2-003] The 5G system shall support a secure mechanism to access a service or an application in an operator's Service Hosting Environment.

[R-8.2-004] The 5G system shall enable support of an access independent security framework.

[R-8.2-005] The 5G system shall support a mechanism for the operator to authorize subscribers of other PLMNs to receive temporary service (e.g. mission critical services).

[R-8.2-006] The 5G system shall be able to provide temporary service for authorized users without access to their home network (e.g. IOPS, mission critical services).

[R-8.2-007] The 5G system shall allow the operator to authorize a third party to create, modify and delete network slices, subject to an agreement between the third party and the network operator.

[R-8.2-008] The 5G system shall support a secure mechanism to protect relayed data from being intercepted by a relay UE.

[R-8.2-009] Subject to HPLMN policy as well as its service and operational needs, any USIM able to access EPS instead of a 5G USIM may be used to authenticate a user in a 5G system to access supported services according to the user‘s subscription.

[R-8.2-010] The 5G system shall provide integrity protection and confidentiality for communications between authorized UEs using a 5G LAN-type service.

[R-8.2-011] The 5G LAN-VN shall be able to verify the identity of a UE requesting to join a specific private communication.

[R-8.2-012] The 5G system shall provide suitable APIs to allow use of a trusted third party provided encryption between any UE served by a private slice and a core network entity in that private slice.

## 8.3	Authentication

[R-8.3-001] The 5G system shall support an efficient means to authenticate a user to an IoT device (e.g. biometrics).

[R-8.3-002] The 5G system shall be able to support authentication over a non-3GPP access technology using 3GPP credentials.

[R-8.3-003] The 5G system shall support operator controlled alternative authentication methods (i.e. alternative to AKA) with different types of credentials for network access for IoT devices in isolated deployment scenarios (e.g. for industrial automation).

[R-8.3-004] The 5G system shall support a suitable framework (e.g. EAP) allowing alternative (e.g. to AKA) authentication methods with non-3GPP identities and credentials to be used for UE network access authentication in non-public networks.

NOTE 2:	Non-public networks can use 3GPP authentication methods, identities, and credentials for a UE to access network but are also allowed to utilize non-AKA based authentication methods such as provided by the EAP framework.

[R-8.3-005] Subject to an agreement between an MNO and a third party, the 5G system shall support a mechanism for the PLMN to authenticate and authorize UEs for access to both a hosted non-public network and private slice(s) of the PLMN associated with the hosted non-public network.

[R-8.3-006] The 5G network shall support a 3GPP supported mechanism to authenticate legacy non-3GPP devices for 5G LAN-VN access.

## 8.4	Authorization

[R-8.4-001] The 5G system shall allow the operator to authorize an IoT device to use one or more 5G system features that are restricted to IoT devices.

[R-8.4-002] The 5G system shall allow the operator to authorize /de-authorize UEs for using 5G LAN-type service.

NOTE:	when a UE is de-authorized from using 5G LAN-type service, it is removed from all 5G LAN-VNs.

[R-8.4-003] Based on operator policy, before establishing a direct device connection using a non-3GPP access technology, IoT devices may use 3GPP credentials to determine if they are authorized to engage in direct device connection.

[R-8.4-004] Based on operator policy, the 5G system shall provide a means to verify whether a UE is authorized to use prioritized network access for a specific service.

## 8.5	Identity management

[R-8.5-001] The 5G system shall provide a mechanism for an operator to allow access from a UE using a temporary identifier that hides its subscriber identity.

[R-8.5-002] The 5G system shall provide a mechanism for an operator to allow access from a UE connected in an indirect network connection using a temporary identifier that hides its subscriber identity.

[R-8.5-003] The HPLMN shall be able to associate a temporary identifier to a UE's subscriber identity.

[R-8.5-004] The 5G system shall be able to protect subscriber identity and other user identifying information from passive attacks.

[R-8.5-005] Subject to regional or national regulatory requirements, the 5G system shall be able to protect subscriber identity and other user identifying information from active attacks.

[R-8.5-006] The 5G system shall be able to allow the equipment identifier to be collected by legitimate entity regardless of UE's user interface, when required.

[R-8.5-007] The 5G system shall be able to support identification of subscriptions independently of identification of equipment.

[R-8.5-008] The 5G system shall support a secure mechanism to collect system information while ensuring end-user and application privacy (e.g. application level information is not to be related to an individual user identity or subscriber identity and UE information is not to be related to an individual subscriber identity).

[R-8.5-009] Subject to regional or national regulatory requirements, the 5G system shall be able to provide the 5G positioning services while ensuring the protection of the privacy of the UE’s user or owner, including the respect of his consent to the positioning services.

NOTE 1: 	this includes the ability for the 5G system to provide the positioning services on demand without having to track continuously the position of the involved UE.

NOTE 2: 	the respect of the user’s consent to some positioning services could abide by different rules in case of emergency (for example, rules that would also receive consent from the user, but well before the emergency occurs).

[R-8.5-010] For a private network using 5G technology, the 5G system shall support network access using identities, credentials, and authentication methods provided and managed by a third party and supported by 3GPP.

## 8.6	Regulatory

[R-8.6-001] The 5G system shall support regional or national regulatory requirements for all supported access networks.

[R-8.6-002] The 5G system shall support Lawful Intercept subject to regional or national regulatory requirements.

[R-8.6-003] A 5G satellite access network connected to 5G core networks in multiple countries shall be able to meet the corresponding regulatory requirements from these countries (e.g. LI).

[R-8.6-004] A 5G system shall support regulatory requirements for 5G LAN-type services.

## 8.7	Fraud protection

[R-8.7-001] Subject to regional or national regulatory requirements, the 5G system shall support a secure mechanism for allowing an authorized entity to disable from normal operation of a UE reported as stolen.

[R-8.7-002] Subject to regional or national regulatory requirements, the 5G system shall support a secure mechanism for allowing an authorized entity to re-enable a recovered stolen UE to normal operation.

[R-8.7-003] The 5G system shall be able to protect user location information from passive attacks.

[R-8.7-004] Subject to regional or national regulatory requirements, the 5G system shall be able to protect user location information from active attacks.

[R-8.7-005] Subject to regional or national regulatory requirements, the 5G system shall support mechanisms to protect the production of the user location information and user positioning-related data against tampering and spoofing.

[R-8.7-006] Subject to regional or national regulatory requirements, the 5G system shall support mechanisms to detect tampering and spoofing attempts on the production of the user location information and the user position-related data.

## 8.8	Resource efficiency

[R-8.8-001] The 5G system shall minimize security signalling overhead without compromising the security level of the 3GPP system.

[R-8.8-002] The 5G system shall support an efficient secure mechanism to transmit the same data (e.g. service provisioning multiple sensors) to multiple UEs.

## 8.9	Data security and privacy

[R-8.9-001] The 5G system shall support data integrity protection and confidentiality methods that serve URLLC and energy constrained devices.

[R-8.9-002] The 5G system shall support a mechanism to verify the integrity of a message as well as the authenticity of the sender of the message.

[R-8.9-003] The 5G system shall support encryption for URLLC services within the requested end-to-end latency.

# 9	Charging aspects

## 9.1	General

The following set of requirements complement the requirements listed in 3GPP TS 22.115 [11]. The requirements apply for both home and roaming cases.

[R-9.1-001] The 5G core network shall support collection of all charging information on either a network or a slice basis.

[R-9.1-002] The 5G core network shall support collection of charging information for alternative authentication mechanisms.

[R-9.1-003] The 5G core network shall support collection of charging information associated with each serving MNO when multi-network connectivity is used under the control of the home operator.

[R-9.1-004] The 5G core network shall support charging for services/applications in an operator’s Service Hosting Environment.

[R-9.1-005] The 5G core network shall support charging for content delivered from a content caching application.

[R-9.1-006] The 5G core network shall support collection of charging information based on the access type (e.g. 3GPP, non-3GPP, satellite access).

[R-9.1-007] The 5G core network shall support collection of charging information based on the slice that the UE accesses.

[R-9.1-008] The 5G core network shall support collection of charging information based on the capacity and performance metrics.

[R-9.1-009] In a 5G system with satellite access, charging call records associated with satellite access(es) shall include the location of the associated UE(s) with satellite access

NOTE: The precision of the location of the UE can be based on the capabilities of the UE or of the network.

## 9.2	5G LAN

[R-9.2-001] A 5G core network shall support collection of charging information for a 5G LAN-type service based on resource usage (e.g. licensed or unlicensed spectrum, QoS, applications).

[R-9.2-002] The 5G core network shall support collection of charging information for a 5G LAN-type service when a UE joins or leaves a specific private communication.

[R-9.2-003] The 5G core network shall support collection of charging information for a 5G LAN-type service for both home and roaming UEs based on the UE’s HPLMN.

End of Change 34

END OF PROPOSED CHANGES