3GPP TSG-SA WG6 Meeting #40-e	S6-202296

e-meeting, 16th – 24th November 2020	(revision of S6-202141)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0033 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for T8 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL |  |  |  |  |  | Date: |  |  | 2020-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The location accuracy requirement for V2X is addressed in TS 22.185 below: [R-5.1-015] The 3GPP network should make available any supported positional accuracy improvement techniques (e.g., DGPS and/or OTDOA) in a resource efficient way to a subscribed UE supporting V2X application. In the current version of TS 23.271, there is no procedure, function or interface defining how the location information from LCS of 4G system is delivered to SCEF. Hence the location management server is not able to acquire high accuracy location information (by techniques including OTDOA supported in E-UTRAN network) from 4G system via T8 interface. According to TS 23.682 and TS 29.122, the SCEF may acquire location information of 4G system from MME at the accuracy level of cell-Id, eNB-Id, TAI, MBMS SAI. Such location information can be acquired by location management server via T8 interface. Alternatively, to achieve high accuracy location from LCS, the location management server can use LCS service from 3GPP network via Le interface, which is defined outside of 3GPP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the architecture and accuracy for location acquired from SCEF via T8 from 4G system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification on whether the location information acquired by current interface meets the service requirements for location accuracy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2, 9.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.2.2	On-network functional model description

Figure 9.2.2-1 illustrates the generic on-network functional model for location management.

Figure 9.2.2-1: On-network functional model for location management

The location management client communicates with the location management server over the LM-UU reference point. The location management client provides the support for location management functions to the VAL client(s) over LM-C reference point. The VAL server(s) communicate with the location management server over the LM-S reference point.

The location management server communicates with the SCEF via T8 reference point to obtain location information from the underlying 3GPP network system.

NOTE:	Location information from LCS of 4G system is not exposed by SCEF.

* * * Next Change * * * *

9.2.4.3	Location management server

The location management server is a functional entity that receives and stores user location information and provides user location information to the vertical application server. The location management server may also acquire location information provided by PLMN operator via T8 reference point.The location management server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The location management server also supports interactions with the corresponding location management server in distributed SEAL deployments.

NOTE:	The accuracy of location information acquired from 4G system via T8 reference point is not higher than at cell level (ECGI) for E-UTRAN.