3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201784

e-meeting, 12th – 20th October 2020	(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0152 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCVideo pull and push corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2020-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MCVideo pull and push procedures describe that the request sent to the target MCVideo client indicates if the incoming MCVideo private call is for video pull or push. However, the corresponding push and pull indication elements are missing in the request sent to the target client. Also, in the MCVideo pull procedure it is described that the target MCVideo client includes an implicit transmit media request in the response. However, this information element is missing in the MCVideo private call response. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MCVideo pull and push indications are included in the MCVideo private call request to be sent from the MCVideo server to the MCVideo client. Also, the implicit transmit media request information element is included in the MCVideo private call response for the case of a video pull. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The target MCVideo client cannot identify that an MCVideo private call request is for video push or pull. Also, as required for the case of a video pull, the target MCVideo client does not implicitly request the transmission of media. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2.2, 7.2.2.2.3, 7.2.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

##### 7.2.2.2.2	MCVideo private call request

Table 7.2.2.2.2-1 describes the information flow MCVideo private call request from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.2-1: MCVideo private call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Transmit media request indication | M | This element indicates whether transmission control will be used for the private call. |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested commencement mode | O | An indication that is included if the user is requesting a particular commencement mode |
| Implicit transmit media request (NOTE) | O | An indication that the user is also requesting the permission to transmit video |
| Push indication | O | Indicates that the private call request is for a one-to-one push call |
| Pull indication | O | Indicates that the private call request is for a one-to-one pull call |
| NOTE: This element shall not be included when the request is sent from the MCVideo server to the MCVideo client. |  |  |

##### 7.2.2.2.3	MCVideo private call response (MCVideo client – MCVideo server)

Table 7.2.2.2.3-1 describes the information flow MCVideo private call response from the MCVideo client to the MCVideo server.

Table 7.2.2.2.3-1: MCVideo private call response (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | O | The MCVideo ID of the called party |
| SDP answer | M | Media parameters selected |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit transmit media request (NOTE) | O | An indication that the user is also requesting the permission to transmit video |
| NOTE: This element shall be included when the response is associated to a request including a pull indication. |  |  |

##### 7.2.2.2.4	MCVideo private call response

Table 7.2.2.2.4-1 describes the information flow MCVideo private call response from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.4-1: MCVideo private call response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | O | The MCVideo ID of the called party |
| Acceptance confirmation | O | An indication whether the user has positively accepted the call. |
| SDP answer | M | Media parameters selected |
| Implicit transmit media request (NOTE) | O | An indication that the user is also requesting the permission to transmit video |
| NOTE: This element shall not be included when the request is sent from the MCVideo server to the MCVideo client. |  |  |