TSG SA Meeting #SP-90E	SP-201032

08 - 14 December 2020, Electronic meeting	(Revision of SP-200446)

3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007119

12 – 23 October 2020	(revision of S2-200xxxx)

Source:	Ericsson, Telstra

Title:	Revised WID: TEI17_DCAMP - Dynamically Changing AM Policies in the 5GC

Document for:	Approval

Agenda Item:	9.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Dynamically Changing AM Policies in the 5GC

Acronym: TEI17_DCAMP

Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-17.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

3	Justification

The RFSP is a useful parameter that allows networks to steer certain services to preferred carriers in the RAN. This can be for reasons of capacity management, improving coverage, to access features that are only available on certain carriers, to avoid inter-modulation issues and more. In some circumstances, the appropriate RFSP index for a certain user will change dynamically.

Release 16 supports AM policies in the PCF, that enables to modify the RFSP index value assigned by subscription based on operator policies, but the options to request to dynamically triggering the modification of the RFSP index value triggered  by third parties or based on application detection have been overlooked. Three scenarios have been identified:

Scenario 1: The user manually activates a bolt-on pack (e.g. via self-care web portal) to improve experience for a particular application or a group of applications, or the user pays a fee to have access to a full range of coverage, not restricted to a subscribed list of TAIs. Both examples refer to temporary conditions with a time duration provided e.g. for a week. The new conditions are applicable before the service starts to enable a seamless experience.

Scenario 2: The application is automatically detected by a DPI type function; this triggers a change of RFSP while the application is active.

Scenario 3:  A third-party application such as an interactive video provider signals to the network that a call has commenced. This triggers a change of RFSP while the application is active.

4	Objective

The objective is to enable the dynamic change of AM policies, as follows:

- To address scenario 1: specify a new NEF service Nnef_AMInfluence that allows AF to influence PCF decisions for RFSP index value when specific services are in use and Service Area Restrictions(SAR) for a UE. The purpose of this new NEF service is to allow the AF to provide parameters without the need to have the UE registered in the network. These parameters are stored in the UDR and then provided to the PCF serving the AMF, when UE registers in the AMF.

The PCF serving the AMF may enforce the RFSP index change when the service or one of the services associated with the AF request starts/stops. This requires the need for the PCF to be informed when the service starts/stops.

In case the AF did not include a service in its request then the PCF serving the AM enforces the change of RFSP index immediately and the RFSP or SAR change lasts until the time conditions expire.

- To address scenario 2: The detection of the start and stop of an application in the UPF triggers the change of the RFSP by the PCF serving the AMF. This requires to be informed when such detection happens.

The PCF serving the SMF triggers the reporting of start and stop of the application using the existing PCC rules and PCRT s in the SMF.

- To address scenario 3: A new NEF service Nnef_AMPolicyAuthorization and corresponding Npcf_AMPolicyAuthorization allows the AF to provide an indication that a change of AM policies, such as RFSP value or SAR, may be required because a service has started or terminated.

In those scenarios where the PCF serving the AMF (PCF-AM) and SMF (PCF-SM)  are different ones, and the PCF-AM needs to be informed that a service requiring steering starts or stops, the PCF-AM subscribes to PCF-SM events on start or stop of an application. The subscription to this event triggers the installation or removal of PCC Rules related with the application in the SMF. SMF and UPF changes if any should be minimized.

The PCF-AM subscribes to the BSF to be notified when a new PCF-SM serving a particular (DNN, S-NSSAI) for a SUPI is registered / deregistered in the BSF.

To address scenario 3 the NEF needs to be able to discover the PCF-AM for a UE; the BSF is used for this purpose.

The expected time for completing this work is 1 TUs. 0.5 time slots in one SA2 meeting to define the functional description and 0.5 time slots in next SA2 meeting to define the procedures and services.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 TS 23.502 TS 23.503 | Definition of the new NEF service to offer the capability to influence Access and Mobility policy decisions, in particular RFSP index value or SAR. Definition of the new NEF and PCF service to offer the capability to influence Access and Mobility policy decisions, in particular RFSP index value or SAR. Update PCF discovery mechanism to allow PCF being serving the SMF discovered by PCF serving the AMF Definition of the new NEF service operations, and procedures to show how the AF influences AM policy decisions. Update of the UDR Application data to include Steering profile requirements and area of coverage data for a SUPI. Define how the PCF serving the AMF is informed that a service that requires steering is in use. Define how the PCF serving the AMF is informed that a new PDU session serving a DNN, S-NSSAI for a UE is established, that triggers the installation of PCC Rules to detect the service. This includes the update to BSF Services operations. Define the BSF impacts. Update Access and mobility related policy control functional description to enable the AF to influence the Access and Mobility decisions, i.e. RFSP index value and Service Area Restrictions for a UE Define how the PCF serving the AM is informed that a service that requires steering is in use. | TSG#91 (March 2021) |  |

6	Work item Rapporteur(s)

Pancorbo, Belen	Belen dot Pancorbo at ericsson dot com

7	Work item leadership

SA2.

8	Aspects that involve other WGs

Charging aspects will be studied by SA5.

9	Supporting Individual Members

Supporting IM name

Telstra

Orange

Charter Communications

AT&T

Verizon UK Ltd

Vodafone

Ericsson

Oracle

Nokia

Nokia Shanghai Bell

Telecom Italia

Matrixx

Samsung

Huawei

China Telecom