TSG SA Meeting #SP-92E	SP-210364

15 - 21 June 2021, Electronic meeting	(revision of SP-200455)

# Source:	SA WG2

# Title:	Revised WID on Dynamic management of group-based event monitoring

# Document for:	Approval

Agenda Item:	6.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Dynamic management of group-based event monitoring

## Acronym: TEI17_GEM

## Unique identifier: 	870007

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 770050 | (Small) Technical Enhancements and Improvements for Rel-16 | Enhanced Multiple Event Monitoring feature introduced in TS 23.682 by CR#0448 and CR#0459. |

## 3	Justification

In the case of a group-based monitoring event, the same event is monitored for all the group member UEs in the group.

For a UE belonging to a group which has an active group-based event configuration, the UE’s event monitoring can be cancelled due to reasons such as

- the UEs’ subscription is deleted from the HSS while monitoring is active, or

- the UE’s authorization for a given event or a set of events is revoked, or

- the UE’s event monitoring is cancelled (e.g. cancellation initiated by SCS/AS or cancellation because of UE moving to an MME not supporting event monitoring), or

- the UE is removed from the group.

If a UE’s monitoring event is cancelled for any of the above reasons, and if the Enhanced Multiple Event Monitoring feature is not supported, the event monitoring for all the other members in the group will have to be cancelled, that is, dynamic management of group-based event monitoring is not supported, and such dynamic management is necessary to avoid unexpected event cancellation for the other group members within the group. This is required to support those MTC providers which require these tasks to be as smooth as possible (e.g. a car hiring company changing cars from one group to another one, decommissioning cars, selling cars, etc.)

Even if the Enhanced Multiple Event Monitoring feature is supported, there is no clear mechanism how the group monitoring event is handled if one or more group members are removed.

This enhancement of dynamic group event handling is also applicable to 5GS.

## 4	Objective

The work item aims to clarify the handling of event cancellation for an individual member or a sub-set of members from a group, and to ensure that event cancellation of some member(s) does not affect other members in the group.

The detailed objectives are to capture the necessary updates in TS 23.682 and 23.502.

Note: This work will require 0.5 TU.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR may be transposed by Ops.}

{Note 2: The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.682 | Handlidng group member’s event monitoring cancellation | SA#92-e (June 2021) |  |
| 23.502 | Handlidng group member’s event monitoring cancellation | SA#92-e (June 2021) |  |

## 6	Work item Rapporteur(s)

Qian Chen XB (Qian dot XB dot Chen at ericsson dot com)

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

Ericsson

Convida Wireless LLC

AT&T

Samsung