3GPP TSG-SA WG6 Meeting #42-bis-e	S6-211037

e-meeting, 12th – 20th April 2021	(revision of S6-210926)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0055 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information flows and API definitions for session-oriented service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Information flows and API definition for Session-oriented service is to be completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Information flows and API definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.19.1, 9.19.2.1 (new), 9.19.2.2 (new), 9.19.2.3 (new), 9.19.2.4 (new), 9.19.2.5 (new), 9.19.2.6 (new), .9.19.3.1, 10.2.1, 10.2.x (new), 10.2.x.1 (new), 10.2.x.2 (new) , 10.2.x.3 (new), 10.2.x.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.19.1	General

The VAE layer provides support at the application layer for V2X application specific server and V2X UE to initiate session-oriented services by providing mechanisms for establishing, updating and terminating sessions.

* * * Next Change * * * *

#### 9.19.2.1	Session-oriented service trigger request

Table 9.19.2.1-1 describes the information flow session-oriented service trigger request from the V2X application specific server to the VAE server.

Table 9.19.2.1-1: Session-oriented service trigger request

| Information element | Status | Description |
| --- | --- | --- |
| Remote V2X UE ID | M | Identity of the V2X UE (VAE client) which is the remote vehicle. |
| V2X service ID | M | The V2X service ID for which application requirement corresponds to. |
| V2X application specific server identity information |  | M |  | Identity information of the V2X application specific server. |  |
| Session ID | O | The session identifier to be used for the session-oriented service. |
| V2X application QoS requirements for the session | O | The application QoS requirements (reliability, delay, jitter) for the session-oriented service. |

#### 9.19.2.2	Session-oriented service trigger response

Table 9.19.2.2-1 describes the information flow session-oriented service trigger response from the VAE server to the V2X application specific server.

Table 9.19.2.2-1: Session-oriented service trigger response

| Information element | Status | Description |
| --- | --- | --- |
| Acknowledgement | M | Acknowledgement for the request |

#### 9.19.2.3	Session-oriented service establishment notification

Table 9.19.2.3-1 describes the information flow session-oriented service establishment notification from the VAE server to the V2X application specific server.

Table 9.19.2.3-1: Session-oriented service establishment notification

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicating success or failure to establish session-oriented service |
| Remote V2X UE ID | M | Identity of the V2X UE (VAE client) which is the remote vehicle. |
| Session ID | O (NOTE) | The session identifier to be used for the session-oriented service. |
| NOTE: This IE is present when Result is success. It is generated by the VAE server if not provided by the requester. |  |  |

#### 9.19.2.4	Session-oriented change trigger request

Table 9.19.2.4-1 describes the information flow session-oriented change trigger request from the V2X application specific server to the VAE server.

Table 9.19.2.4-1: Session-oriented change trigger request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session-oriented service. |
| V2X application QoS requirements for the session | M | The application QoS requirements (reliability, delay, jitter) for the session-oriented service that is to be updated. |

#### 9.19.2.5	Session-oriented change trigger response

Table 9.19.2.5-1 describes the information flow session-oriented change trigger response from the VAE server to the V2X application specific server.

Table 9.19.2.5-1: Session-oriented change trigger response

| Information element | Status | Description |
| --- | --- | --- |
| Acknowledgement | M | Acknowledgement for the request |

#### 9.19.2.6	Session-oriented service change notification

Table 9.19.2.6-1 describes the information flow session-oriented service change notification from the VAE server to the V2X application specific server.

Table 9.19.2.6-1: Session-oriented service change notification

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicating success or failure to change session-oriented service |

#### 9.19.2.7	Session-oriented termination trigger request

Table 9.19.2.7-1 describes the information flow session-oriented termination trigger request from the V2X application specific server to the VAE server.

Table 9.19.2.7-1: Session-oriented termination trigger request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session-oriented service. |

#### 9.19.2.8	Session-oriented termination trigger response

Table 9.19.2.8-1 describes the information flow session-oriented termination trigger response from the VAE server to the V2X application specific server.

Table 9.19.2.8-1: Session-oriented termination trigger response

| Information element | Status | Description |
| --- | --- | --- |
| Acknowledgement | M | Acknowledgement for the request |

#### 9.19.2.9	Session-oriented service termination notification

Table 9.19.2.9-1 describes the information flow session-oriented service termination notification from the VAE server to the V2X application specific server.

Table 9.19.2.9-1: Session-oriented service termination notification

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicating success or failure to terminate session-oriented service |

* * * Next Change * * * *

#### 9.19.3.1	General

The VAE layer provides support at the application layer for V2X UE to initiate session-oriented services by providing mechanisms for establishing, updating and terminating sessions.

* * * Next Change * * * *

### 10.2.1	General

Table 10.2.1-1 illustrates the VAE server APIs.

Table 10.2.1-1: List of VAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| VAE_MessageDelivery API | Deliver_DL_Message | V2X application specific server | Request/ Response |
|  | Deliver_UL_Message | V2X application specific server | Subscribe/notify |
| VAE_FileDistribution API | Distribute_File | V2X application specific server | Request/ Response |
| VAE_ApplicationRequirement API | Reserve_NetworkResource | V2X application specific server | Subscribe/notify |
|  | Notify_NetworkResource |  |  |
| VAE_DynamicGroup API | Configure_DynamicGroup | V2X application specific server | Request/Response |
|  | Notify_DynamicGroup |  |  |
| VAE_ServiceContinuity API | Query_ServiceContinuity | VAE server | Request/Response |
| VAE_HDMapDynamicInfo API | Subscribe_HDMapDynamicInfo | V2X application specific server | Subscribe/notify |
|  | Notify_HDMapDynamicInfo |  |  |
| VAE_SessionOrientedService API | Establish_Session | V2X application specific server | Subscribe/notify |
|  | Notify_Establish_Session |  |  |
|  | Update_Session |  |  |
|  | Notify_Update_Session |  |  |
|  | Terminate_Session |  |  |
|  | Notify_Terminate_Session |  |  |

The SEAL server(s) APIs available via SEAL-S reference point as specified in 3GPP TS 23.434 [6] are consumed by the V2X application specific server via the VAE server (acting as a VAL server) over the Vs reference point. When V2X application specific server invokes a SEAL server API via the VAE server over the Vs reference point, the VAE server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response.

NOTE:	When V2X application specific server invokes a SEAL server API directly, the V2X application specific server acting as VAL server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response as specified in 3GPP TS 23.434 [6].

Editor's note:	Whether the VAE_DynamicGroup API is to be moved to SEAL is FFS.

* * * Next Change * * * *

### 10.2.X	VAE_SessionOrientedService API

#### 10.2.X.1	General

API description: This API enables the V2X application specific server to communicate with the VAE server to trigger establishment, update and termination of session-oriented service.

#### 10.2.X.2	Establish_Session operation

API operation name: Establish_Session

Description: Triggers the establishment of the session-oriented service by the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.19.2.1.

Outputs: Refer subclause 9.19.2.2.

See subclause 9.19.4.2 for the details of usage of this API operation.

#### 10.2.X.3	Notify_Establish_Session operation

API operation name: Notify_Establish_Session

Description: Notification about the establishment of the session-oriented service by the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.19.2.3.

Outputs: None.

See subclause 9.19.5.2 for the details of usage of this API operation.

#### 10.2.X.4	Update_Session operation

API operation name: Update_Session

Description: Triggers the update to the session-oriented service by the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.19.2.4.

Outputs: Refer subclause 9.19.2.5.

See subclause 9.19.4.3 for the details of usage of this API operation.

#### 10.2.X.5	Notify_Update_Session operation

API operation name: Notify_Update_Session

Description: Notification about the update to the session-oriented service by the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.19.2.6.

Outputs: None.

See subclause 9.19.5.3 for the details of usage of this API operation.

#### 10.2.X.6	Terminate_Session operation

API operation name: Terminate_Session

Description: Triggers the termination of the session-oriented service by the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.19.2.7.

Outputs: Refer subclause 9.19.2.8.

See subclause 9.19.5.4 for the details of usage of this API operation.

#### 10.2.X.7	Notify_Terminate_Session operation

API operation name: Notify_Terminate_Session

Description: Notification about the termination of the session-oriented service by the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.19.2.9.

Outputs: None.

See subclause 9.19.5.4 for the details of usage of this API operation.