3GPP TSG-SA WG6 Meeting #43	S6-211380

e-meeting, 24th May – 2nd June 2021	(revision of S6-211223)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0280 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to location management procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - 10.9.2.7 Location information notification : This information flow is shown as from LMS to MC service server, but it is also applicable from LMS to LMC. - 10.9.3.6.1 Step 2 mentions “location information report”, but it should be “location information notification” And changed to “singular” MC service user. - 10.9.3.8.1 : In pre-condition #4, it is the location management server subscribed to the functional alias controlling MC service server and not the MC service server. - 10.9.3.8.2: In pre-condition #4, it is the location management server subscribed to the functional alias controlling MC service server and not the MC service server. - 10.9.3.9.4.2 Cancel location history reporting procedure (LMS – LMC) : In the description the flow direction is written as LMC to LMS. It should be LMS to LMC - Figure 10.9.3.9.4.3-1: Cancel location history reporting procedure : Step 5 in the figure should be Location information history cancel response instead of Location information history cancel report - 10.9.3.9.4.3 : In step 4 we should refer to the existing procedure of cancelling the location history reporting procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes are made in the required sub-clauses as described in the “Reason for change” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.7,10.9.3.6.1, 10.9.3.8.1, 10.9.3.8.2, 10.9.3.9.4.2, 10.9.3.9.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * First Change * * *

#### 10.9.2.7	Location information notification

Table 10.9.2.7-1 and Table 10.9.2.7-2 describe the information flows from the location management server to the MC service server and from the location management server to the location management client.

Table 10.9.2.7-1: Location information notification (LMS to MC service server)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user whose location information is being notified |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information (see NOTE) | M | Location information |
| NOTE: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. |  |  |

Table 10.9.2.7-2: Location information notification (LMS to LMC)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user whose location information is being notified |
| MC service ID | M | Identity of the MC service user subscribed to location information |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information (see NOTE) | M | Location information |
| NOTE: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. |  |  |

* * Second Change * * *

##### 10.9.3.6.1	Event-trigger location information notification procedure

NOTE: This procedure is valid for single MC system operation only.

Figure 10.9.3.6.1-1 illustrates the high level procedure of event-trigger usage of location information.  The same procedure can be applied for location management client and other entities that would like to subscribe to location information of MC service user.

Pre-condition:

1.	The MC service server or location management client has subscribed to the location information of a target MC service user.

Figure 10.9.3.6.1-1: Event-trigger usage of location information procedure

1.	Based on configuration, e.g., subscription, periodical location information timer, the location management server is triggered to report the latest user location information of the target MC service user to the subscribed MC service server or subscribed location management client.

2.	The location management server sends the location information notification including the latest location information of one MC service user to the MC service server or to the location management client. The latest location information is derived from the location report procedure as described in clause 10.9.3.1, or from PLMN operator (e.g. LCS network).

* * Third Change * * *

##### 10.9.3.8.1	Client-triggered one-time location information report

Figure 10.9.3.8.1-1 illustrates the procedure when a location management client requests one-time location information from other location management clients for location information reporting using functional alias, which can be shared between several MC service users. Under this condition, the actual location of all MC service users sharing the same functional alias are contained in the one-time location report.

Pre-conditions:

1.	MC service client 2 and MC service client 3 share the same functional alias.

2.	MC service client 2 and MC service client 3 activated the functional alias.

3.	MC service client 1 may have an activated functional alias.

4.	The location management server has subscribed to the functional alias controlling MC service server within the MC system for functional alias activation/de-activation updates.

Figure 10.9.3.8.1-1: One-time location information report for shared functional alias

1.	Location management client 1 sends a location reporting trigger, limited to one MC service at the time, to the location management server to activate a one-time location report procedure which shall retrieve the location information of the MC service users that may share the contained functional alias. Location management client 1 may include its own activated functional alias.

2.	Location management server checks whether location management client 1 is authorized to send a location reporting trigger.

3.	The location management server uses on-demand location reporting procedures. The location information request contains the functional alias used by the location management client 1 to address location management client 2 and location management client 3.

4.	Upon receiving the report, the location management server updates location of the reporting location management clients. If the location management server does not have location information of a reporting location management client before, then just stores the reporting location information for that location management client.

5.	The location management server sends a location report response to location management client 1 containing the provided location information from location management client 2 and location management client 3. If not all location management clients immediately respond to the location management server, i.e. others report some time later, subsequent location reporting responses may be sent.

* * * Fourth Change * * *

##### 10.9.3.8.2	Client-triggered periodic location information report

Figure 10.9.3.8.2-1 illustrates the procedure when a location management client requests periodic location information from other location management clients for location information reporting using functional aliases which may be shared between several MC service users. Under this condition, the actual location of all MC service users sharing the same functional alias are contained in the periodic location report.

Pre-conditions:

1.	MC service client 2 and MC service client 3 share the same functional alias.

2.	MC service client 2 and MC service client 3 activated the functional alias.

3.	MC service client 1 may have an activated functional alias.

4.	The location management server has subscribed to the functional alias controlling MC service server within the MC system for functional alias activation/de-activation updates.

Figure 10.9.3.8.2-1: Periodic location information report for shared functional alias

1.	Location management client 1 sends a location reporting trigger, limited to one MC service at the time, to the location management server to activate a periodic location reporting procedure which shall retrieve the location information of the MC service users sharing the contained functional alias. Location management client 1 may include its own activated functional alias.

2.	Location management server checks whether location management client 1 is authorized to send a location reporting trigger.

3.	Depending on the information given by the location reporting trigger, the location management server uses event-triggered location information procedure and immediately send location information request to the location management clients that contains the functional alias requested by the location management client 1.

4.	Upon receiving the reports, the location management server updates location of the reporting location management clients.

5.	Based on the received location information reports, the location management server will periodically issue a location report response encompassing the MC service IDs, the associated functional aliases and the individual location information of the addressed MC service IDs.

NOTE 1:	If a functional alias is deactivated for an MC service client, the corresponding location management client stops sending periodic location information reports.

NOTE 2:	If a functional alias has been newly activated for an MC service client, the location management server activates location information reporting for this location management client.

NOTE 3:	If a functional alias is simultaneously shared between several MC service IDs, all location management clients with the associated MC service IDs will send the location report until the functional alias status change, e.g. take-over, for the individual MC service ID.

* * * Fifth Change * * *

###### 10.9.3.9.4.2	Cancel location history reporting procedure (LMS – LMC)

Figure 10.9.3.9.4.2-1 illustrates the procedure for the cancellation on location history reporting from the location management server to the location management client.

Pre-conditions:

1.	The location management client has returned to report location information and has stored location information reports.

Figure 10.9.3.9.4.2-1: Cancel location history reporting procedure (LMS – LMC)

1.	The location management client transmits stored location information while there is a communication link between the location management client and location management server.

2.	The location management server requests the cancellation of the location information history reporting at any time during the transmission.

3.	Location management client cancels the location information history reporting.

4.	Location management client confirms the executed cancellation request.

* * * Sixth Change * * *

###### 10.9.3.9.4.3	Cancel location history reporting procedure

Figure 10.9.3.9.4.3-1 illustrates the procedure for the cancellation on location history reporting from the MC service server or location management client to the location management server.

Pre-conditions:

1.	The location management client 1 has returned to report location information and has stored location information reports.

Figure 10.9.3.9.4.3-1: Cancel location history reporting procedure

1.	The location management client 1 transmits stored location information while there is a communication link between the location management client and location management server.

2.	The MC service server or location management client 2 requests the cancellation of the location information history reporting at any time during the transmission.

3.	The location management server checks the authorization of this request and decides either to cancel the transmission from the reporting location management client 1 as described in clause 10.9.3.9.4.2, to cancel the transmission from the location management server or to cancel the transmission from both.

4.	The location management server executes the requested cancellation.

5.	Location management server confirms the executed cancellation.

* * * End Changes * * *