TSG-SA Working Group 1 (Services) meeting #3	TSGS1#3(99)310

Hampton Court, Surrey, UK 10th-12th May 1999

Source:	T-Mobil

Title:	Requirements of IP based Multimedia-Applications over UMTS

Document for:	Information and Discussion

Agenda Item:	6.3.10

# Introduction

The Internet and it’s application the World Wide Web (WWW) is the most popular multimedia system of today. As a consequence it is broadly accepted that mobile access to Internet resources and its applications (video conferencing, video telephony, streaming audio and video, WWW, E-Mail, File Transfer, etc.) will bring a major push for the mobile data market. That has already been reflected in TS22.00 clearly stating that UMTS will interconnect to the IP-world.

Meeting QoS guarantees in such heterogeneous and distributed environment involving fixed and mobile networks and terminals is fundamentally an end – to – end issue, that is from application to application. IETF has already specified and tested mechanisms for interworking with other networks in order to guarantee end – to – end QoS, for UMTS shall adopt these mechanisms to establish interworking.

To guarantee end – to – end QoS within UMTS the three core areas: air – interface , access and core have to be aware of the requirements and support the end – to – end QoS context.

# Classes of Applications

Traffic over the Internet significantly changed since the early days of the Net. In the beginning of the Internet the exchange of ASCII files was the only communication form. Today multimedia e-mail, Internet Telephony and extensively web pages are common. Also Internet radio and video multicast are in use today.

Due to the wide range of application requirements three different classes of applications were established:

- Non-real time – like e-mail. There are nearly no requirements in the network.

- Streaming – With streaming a nearly real time transport of voice, music and video is possible. Depending on the network load a appropriate play out buffer is filled to overcome congestion.

- Real time – high requirements on the network.

## Real-time

For the real time class the requirements in delay and delay jitter are very high. More buffer lead to larger queuing delay. These delays are very disturbing even in interactive communication. Real-time is necessary for interactive communication like IP-Telephony and Videoconferencing. Real-time traffic is transmitted with UDP. Because of the real-time character a retransmission of lost packets is not useful, because packet retransmission would increase delay tremendous. Since real-time traffic is very sensitive against delay a Guaranteed Service will be the most desirable. A guaranteed delay and delivery is assured.

## Streaming

Delay jitter is compensated by playout buffer with an impact on the delay. Some streaming applications also detect the network status and buffers an appropriate amount of data. Streaming is particularly used for audio and video transmission even for stored data as live feed. This applications are adapted very well to today Internet conditions but are limited because they can only react.

The streaming information is normally transported via UDP. Some vendors include a retransmission strategy in the application layer which make it possible to retransmit video key frames and inserts them into the data stream again.

## Non Real-time

For non real-time applications there is no requirement in delay. Non real-time applications are the most used applications in the today used Internet (see also Traffic Patterns).

Non real-time traffic is transported with TCP. TCP is a reliable protocol which transmits each byte of information. This can take a longer time when retransmissions happen. In the today used IP networks these are the standard application and use only best effort. But in future a large number of scenarios are possible to favour important traffic to other (e.g. transmission of a X-ray image should be preferred to network games).

## Example Applications

In the table below example applications are listed for the different types of application.

| Class | Example application Bandwidth <64kbps | Bandwidth >64kbps |  | Bandwidth>2Mbps |  |
| --- | --- | --- | --- | --- | --- |
| Non-real-time | e-mail, http, news... |  | Urgent http |  |  |
| Streaming | Streaming Audio |  | Streaming Video |  | Video on Demand, Video broadcast, high quality video |
| Real-time | IP-Telephony |  | Video, Picture Telephony, Videoconferencing |  |  |

Table 1: Example applications

## Requirements of Applications to the Network

- The requirements of multimedia applications to the network can be characterised by:

- bandwidth

- delay

- delay jitter

- bit error rate

- availability

These requirements have different weights for specific applications. As listed in table 2 the real-time applications have high requirements in delay and delay jitter. A streaming application can deal with delay jitter but is limited by the trade-off between delay and jitter and by the buffer size. Non real-time application can handle this but need for a good service appropriate bandwidth.

Availability is a often forgotten requirement and only remembered by mobile telephone users. Availability is very important for the normal communication services like telephony and e-mail.

| Requirement | Application |
| --- | --- |
| Bandwidth | Important for all applications, especially for streaming and real-time applications |
| Delay | Video, Voice, IP Telephony (Real-time applications) |
| Delay jitter | Video, Voice, IP Telephony |
| Bit error rate | e-mail, fax, WWW, FTP,..(non real-time applications) |
| Availability | Important for all applications; especially for applications like IP Telephony, e-mail,... |

Table 2: Special requirements for specific applications

For the different classes of applications, the requirements of real-time IP traffic are very high, especially delay and the appropriate bandwidth are absolute critical. With a too high delay interactive communication is nearly impossible.

For streaming delay and delay jitter in certain boundaries are not the problem because of buffering. But when the bandwidth is too low the best buffering can not generate an acceptable result.

For non real-time traffic high BER is a great problem. As non real-time applications use TCP as transport protocol a reliable, which means without loss, transmission is very difficult. With a high BER, likelihood of errors in a TCP packet rises and more retransmissions take place.

## Traffic Patterns of single flows

As mentioned before audio and video transmissions are commonly used applications in today’s internet although the quality is in most cases very poor. UMTS supports bandwidth up to 2 Mbps (in-house) and 384 (outdoor). This is not enough to transmit high quality video, but sufficient for videoconferencing.

Generally, there are two observable trends: on behalf of general improvements in codec quality, applications tend to use less and less bandwidth for a/v transmissions, while on the other home and business usage increases dramatically. Given that and the trend towards provision of IP QoS mechanisms, the real-time patterns can be expected to increase.

In table 3 some example applications are listed. This list is not complete and gives only a relation between the parameters.

|  | Bandwidth kbps | Delay | Delay jitter | Bit error rate | Comment |
| --- | --- | --- | --- | --- | --- |
| Video M-JPEG, Single picture compression, 300 x 300 pixels, 24 Bit colour | 1000 700 | Low | Low | 10-4 | Picture with a high entropy, Picture with bureau surrounding |
| MPEG-1, low PAL-Quality | 1500 | Low | Low | 10-4 | Bandwidth with free decoder up to factor 4, max. bit error rate 10 –6, that means in 18 minutes video min. 1 error. |
| MPEG-2, good TV-Quality | 6000 | Low | Low | 10-4 | Online-coding problematic I, P, B Frames are added with different error protection |
| Picture telephone H.320 | 64 | <200 ms | low | 10-4 | Gateway functionality to the core network |
| Audio, Telephone | 32 | <200 ms | low | 10-4 | Low Delay (Echo) |
| e-mail, FAX | 10 - 64 | N/A | N/A | 10-6 | Delay independent |
| Internet, WWW | 500 min 32 | TCP time-out | TCP time-out | 10-6 | Delay independent, for good usability smaller delay is required |
| Voice over IP | 10 - 64 | <200 ms |  |  | Needs low bandwidth but allows only small delay, delay jitter sensitive |
| Teleworking, Application-sharing, | 2000 | <100 ms |  | 10-6 | Integration of different Components, Delay < 100 ms (ITU, G.114) |

Table 3: QoS requirements of typical applications

Of course it is possible that one application combines e.g. video, voice and application sharing. Because of the fundamental difference between the requirements on QoS there will be different flows of traffic with different QoS parameters. To synchronise this flows protocols like RTP/RTCP can be used and are already tested and used in a number of applications.

# Conclusion

Provision of QoS for multimedia applications is only meaningful with end – to – end negotiation, that is from application to application. This has to be considered for the complete UMTS system (Air- Interface, Access and Core) and interworking networks.

Network to network negotiation should be based on IETF mechanisms.

UMTS has to support three classes of applications: non-real time, streaming and real time.

Further requirements from an application perspective are:

- Usage of link layer features so UMTS, that guarantee

- bit error rate

- delay

- delay jitter

- availability

- Flexible Best Effort Service

- Adaptation of the TCP stack for the special requirements of wireless links

- Appropriate support for standard IP protocols like UDP, TCP etc.

It is important that these requirements are included in 22.05.