3GPP TSG-S1 	Tdoc S1-99373

Hampton Court, UK

10-12 May 1999

| CHANGE REQUEST No : |  |  |  |  | A019 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 22.100 |  | Version: |  |  | 3.3.0 |  |  |  |
| Submitted to SA | 4 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | x |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM | x | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | S1 | Date: |  |
| --- | --- | --- | --- |

| Subject: | Support of Emergency Call in UMTS Phase 1, R'99 |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | x |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | Adds a requirement to support an emergency call teleservice as defined in TS22.101 |
| --- | --- |

| Clauses affected: | 6 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 6.2. Bearer services

UMTS phase 1 shall support GSM phase 2+ Release '99 data bearer services :

Circuit switched data: Circuit switched data services and "real time" data services shall be provided for interworking with the PSTN/ISDN so that the user is unaware of the access network used (UMTS and GSM access network or handover between access networks). Both transparent (constant delay) and non-transparent (zero error with flow control) services shall be supported. These data services shall operate with minimum loss of data on handover between the GSM access network and the UTRAN.

Packet switched data: Packet switched data services shall be provided for interworking with packet networks such as IP-networks and LANs. The standard shall provide mechanisms which ensure the continuity of packet based services upon handover e.g. between GSM and UMTS.

## 6.3 Emergency Call

UMTS Phase 1 R'99 shall support an emergency call teleservice as defined in [1].

7	UTRAN capabilities

NOTE :	The term performance refers in this clause to the realisation of the QoS objectives inside the UTRAN.

UTRAN capabilities for UMTS are the complete set of bearer capabilities and bearer control specified in UMTS 22.05. The UTRAN shall have the following capabilities :

1)	A UTRAN shall be contained within only one UMTS network. (In the case of a network with a phase 1 UMTS core network consisting of an evolved GSM core network, it shall be possible to connect the UTRAN to the GSM NSS and GPRS backbone infrastructures or only one of them.)

2)	The UTRAN shall support the set-up, re-negotiation and clearing of connections with a range of traffic and performance characteristics. The re-negotiation may result from an upper layer request or a change in the radio conditions (handover, cell load modification,...) and may be mobile station or network initiated. It shall be possible for the UTRAN to apply the following traffic policing mechanisms such as :

. connection admission control (CAC) during connection set-up and re-negotiation,

. flow control (FC) on a connection during its lifetime,

.  usage parameter control (UPC) on a connection during its lifetime..

3)	The UTRAN shall support a range of traffic and performance characteristics for the connectionless traffic.

4)	The range of traffic and performance characteristics that shall be supported by UTRAN for connection oriented and connectionless traffic is indicated in TS 22.05 sections 5.2 to 5.4.

5)	The UTRAN shall allow one mobile termination to handle more than one bearer service simultaneously and to have bearer services of different connection modes. It is nevertheless expected that the terminal and network capabilities will put some limitations on the number of bearer services that can be handled simultaneously. It shall be possible for each connection to have independent traffic and performance characteristics. It shall be possible for each connectionless message to have independent traffic and performance characteristics.