TSG-SA Working Group 1 (Services) meeting #3	TSGS1#3(99)392

Hampton Court, Surrey, UK 10th-12th May 1999

| CHANGE REQUEST No : |  |  |  |  | A013r2 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.105 |  | Version: |  |  | 3.4.0 |  |  |  |
| Submitted to TSG_SA |  |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  |  |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | x |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | QoS |
| --- | --- |

| Source: | Nortel Networks | Date: | 14/05/1999 |
| --- | --- | --- | --- |

| Subject: | End-to-end QoS service requirements |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | UMTS 99 | x |
| marked with an X) |  |  |  |  |  |

| Reason for change: | End to end QoS is required for UMTS Release'99 to enable satisfactory operation of end user applications, which have specific requirements regarding bandwidth, delay, delay variation or information loss. |
| --- | --- |

| Clauses affected: | 4.2 , 4.2.1, 5.1, 5.2, 5.2.1, 5.2.2, 5.4, 5.5, |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

3G TS 22.105 3.4.0 (1999-04)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects

Service aspects;

Services and Service Capabilities

(3G TS 22.105 version 3.4.0)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTS/TSGS-0122105U

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

# Contents

Foreword	7

1	Scope	8

2	References	8

2.1	Normative references	8

2.2	Informative references	9

3	Definitions and abbreviations	9

3.1	Definitions	9

3.2	Abbreviations	10

4	Framework for the description of telecommunication services and applications	11

4.1	General	11

4.2	Basic telecommunication services	11

4.2.1	Bearer services	12

4.2.2	Teleservices	12

4.3	Supplementary services	12

4.4	Service features	13

5	Bearer Services	13

5.1	Definition of bearer services	13

5.2	Description of bearer services	13

5.2.1	Information transfer attributes	14

5.2.2	Information quality attributes	15

5.3	Supported bit rates	16

5.4	Supported QoS	16

5.5	Supported topologies	17

5.6	Radio Interface optimisation	22

5.7	Support of GSM general bearer services	22

6	Teleservices	22

6.1	Definition of teleservices	22

6.2	Description of teleservices	22

6.3	Support of teleservices in UMTS networks	22

6.4	Existing Teleservices supported by UMTS networks	24

6.4.1	Speech	24

6.4.2	Emergency Call	24

6.4.3	Short Message Service  - Point to Point (SMS-PP)	24

6.4.4	Short Message Service - Cell Broadcast (SMS-CB)	25

6.5	Internet Access	25

7	Supplementary Services	25

8	Service features	25

8.1	Security/Privacy features	26

8.2	Access Control  features	26

8.3	Address Translation Features	26

8.4	Call/Session/Bearer Control Features	26

8.5	Location Features	27

8.6	Messaging features	27

8.7	Service control features	28

8.8	User Interaction Features	28

9	Standardised Protocols and Capabilities	28

9.1	Access protocols	28

9.2	Execution Environment	28

10	Existing GSM System features	30

10.1	Network Identity and Time Zone (NITZ)	30

10.2	Support of Localised Service Area (SoLSA)	30

10.3	Mobile station Execution Environment (MExE)	30

10.4	Location Services (LCS)	30

10.5	Customised Application for Mobile network Enhanced Logic (CAMEL)	30

10.6	Unstructured Supplementary Service Data (USSD)	30

Annex A (informative): Examples of services built from service features	31

Annex B (informative) : Description and analysis of communication schemes	32

B.1	Communication schemes	32

B.1.1	Background traffic	32

B.1.2	Interactive traffic	32

B.1.3	Real time streams	33

B.1.4	Real time conversation	33

B.2	Adaptability and bearer service negotiation	34

Annex C (informative) : Change history	35

History	36

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TS, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version 3.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	Indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the specification;

# 1	Scope

Pre-UMTS systems have largely standardised the complete sets of bearer services, teleservices and supplementary services which they provide. One major difference between UMTS and pre-UMTS systems is that service capabilities rather than services are standardised for UMTS, allowing service differentiation and system continuity.  This Technical Specification (TS) describes how and what kind of services the UMTS user has access to.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

- A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

## 2.1	Normative references

[1]	GSM 02.02: "Digital cellular telecommunications system (Phase 2+); Bearer services supported by a GSM Public Land Mobile Network (PLMN)".

[2]	GSM 02.03: "Digital cellular telecommunications system (Phase 2+); Teleservices supported by a GSM Public Land Mobile Network (PLMN)".

[3]	GSM 02.04: "Digital cellular telecommunications system (Phase 2+); General on supplementary services".

[4]	GSM 02.42: "Digital cellular telecommunications system (Phase 2+); Network Identity and Timezone (NITZ); Service description; Stage 1".

[5]	GSM 02.43: "Digital cellular telecommunications system (Phase 2+); Support of Localised Service Area (SoLSA); Service description; Stage 1".

[6]	GSM 02.57: "Digital cellular telecommunications system (Phase 2+); Mobile Station Application Execution Environment (MExE); Service description; Stage 1".

[7]	GSM 02.71: "Digital cellular telecommunications system (Phase 2+); Location Services (LCS); Service definition - Stage 1".

[8]	GSM 02.78: "Digital cellular telecommunications system (Phase 2+); Customised Applications for Mobile network Enhanced Logic (CAMEL); Service definition - Stage 1".

[9]	GSM 02.90: "Digital cellular telecommunications system; Unstructured Supplementary Service Data (USSD) - Stage 1".

[10]	GSM 22.01: "Universal Mobile Telecommunications System (UMTS); Service aspects; Service principles".

[11]	GSM 22.20: "Universal Mobile Telecommunications System (UMTS); Virtual Home Environment (VHE), Stage 1".

[12]	GSM 23.10: "Universal Mobile Telecommunications System (UMTS); UMTS Access Stratum; Services and Functions".

## 2.2	Informative references

[1]	ITU-T recommendation F.700: "Framework recommendation for audio-visual/multimedia services".

[2]	GSM 02.01: "Digital cellular telecommunications system (Phase 2+); Principles of telecommunication services supported by a GSM Public Land Mobile Network (PLMN)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of this TS, the following definitions apply:

Basic telecommunication service : this term is used as a common reference to both bearer services and teleservices.

Bearer service : is a type of telecommunication service that provides the capability of transmission of signals between access points.

Call : a logical association between several users (this could be connection oriented or connection less).

Connection : is a communication channel between two or more end-points (e.g. terminal, server etc.).

Mobile termination : the mobile termination is the component of the mobile station which supports functions specific to management of the radio interface (Um).

Multimedia service : Multimedia services are services that handle several types of media. For some services, synchronisation between the media is necessary (e.g. synchronised audio and video). A multimedia service may involve multiple parties, multiple connections, and the addition or deletion of resources and users within a single call.

Nomadic Operating Mode : Mode of operation where the terminal is transportable but being operated while stationary and may in addition require user co-operation (e.g. close to open spaces, antenna setup...).

Quality of Service : the collective effect of service performances which determine the degree of satisfaction of a user of a service. It is characterised by the combined aspects of performance factors applicable to all services, such as;

service operability performance;

-	service accessibility performance;

-	service retainability performance;

-	service integrity performance; and

-	other factors specific to each service.

Service feature : Standardised building block used to create services.

Supplementary service : is a service which modifies or supplements a basic telecommunication service. Consequently, it cannot be offered to a user as a standalone service. It must be offered together with or in association with a basic telecommunication service. The same supplementary service may be common to a number of basic telecommunication services.

Teleservice; is a type of telecommunication service that provides the complete capability, including terminal equipment functions, for communication between users according to standardised protocols and transmission capabilities established by agreement between operators.

## 3.2	Abbreviations

For the purposes of this TS, the following abbreviations apply;

BER	Bit Error Rate

B-ISDN	Broadband ISDN

CAMEL	Customised Application for Mobile network Enhanced Logic

DTMF	Dual Tone Multiple Frequency

TR	Technical Report

TS	Technical Specification

ETSI	European Telecommunications Standards Institute

FAX	Facsimile

FER	Frame Erasure Rate

GSM	Global System for Mobile Communications

HE	Home Environment

IMUN	International Mobile User Number

IN	Intelligent Network

ISDN	Integrated Services Digital Network

ISO	International Organisation for Standardisation

ITU	International Telecommunication Union

LCS	Location Services

MExE	Mobile station Execution Environment

MMI	Man Machine Interface

MO	Mobile Origination

MS	Mobile Station

MT	Mobile Termination

O&M	Operations and Maintenance

PBX	Private Branch eXchange

PC	Personal Computer

PCMCIA	Personal Computer Memory Card International Association

PIN	Personal Identity Number

PNP	Private Numbering Plan

POTS	Plain Old Telephony Service

QoS	Quality of Service

USIM	User Service Identity Module

SMS	Short Message Service

SAT	SIM Application Toolkit

SN	Serving Network

SoLSA	Support of Localised Service Area

UMTS	Universal Mobile Telecommunications System

# 4	Framework for the description of telecommunication services and applications

## 4.1	General

Telecommunication services supported by UMTS are the communication capabilities made available to users by home environment and serving network. A UMTS network provides, in co-operation with other networks, a set of network capabilities which are defined by standardised protocols and functions and enable telecommunication services to be offered to users.

A service provision by a HE/SN to a UMTS user may cover the whole or only part of the means required to fully support the service.

The service classification and description which follow are independent of different possible arrangements for the ownership and provision to the user of the means required to support a service.

## 4.2	Basic telecommunication services

Basic telecommunication services are divided in two broad categories;

-	bearer services, which are telecommunication services providing the capability of transmission of signals between access points

- teleservices, which are telecommunication services providing the complete capability, including terminal equipment functions, for communication between users according to protocols established by agreement between network operators.

The communication link between the access points consists of  UMTS, one or more transit networks and a terminating network. The networks between the two access points typically use different means for bearer control.

Figure 1 illustrates these definitions.

NOTE 1:	In order to limit the complexity of the figure, only one transit network is shown.

NOTE 2:	The terminating network type may include a UMTS network, either the originating one or another one.

NOTE 3:  The bearer service terminates in the mobile station.

- NOTE 4:   The terminating network may be a non UMTS network such as: PSTN, N-ISDN, GSM, IP networks/LANs and X.25

- Figure 1; Basic telecommunication services supported by a UMTS network

### 4.2.1	Bearer services

The characterisation of a bearer service is made by using a set of charactristics that distinguishes it from other bearer services. Particular values are assigned to each characteristic  when a given bearer service is described and defined.

The service characteristics as they apply at a given reference point where the user accesses the bearer service.

In the general case networks between the two access points use different control mechanisms. In this case the bearer services of each network throughout the communication link have to be translated at the network interfaces to realize an end to end bearer service.

A list of definitions of attributes and values used for bearer services is contained in clause 5.

The bearer services are negotiable and can be used flexibly by applications.

### 4.2.2	Teleservices

Chapter 6 defines both standardised and non-standardised teleservices. Some teleservices are standardised because that interworking with other systems have been recognised as a requirement. Other teleservices shall not be standardised. A decoupling between lower layer (i.e. bearer attributes) and higher layer capabilities will be necessary for the development of teleservices.

## 4.3	Supplementary services

A supplementary service modifies or supplements a basic telecommunication service. Consequently, it cannot be offered to a user as a stand alone service. It must be offered together or in association with a basic telecommunication service. The same supplementary service may be applicable to a number of basic telecommunication services.

Two methods are used for the characterisation of supplementary services;

-	The first method is used for the description of existing standardised supplementary services. These services are specified through the detailing of each of the operations involved in service provision and service usage (the provision/withdrawal, registration/erasure, activation/deactivation, invocation and interrogation operations). Clause 7 lists these services.

-	The second method enables the provision of HE/SN specific supplementary services. To make this possible, standardised building blocks referred to as service features are specified in clause 8. The combination and parametrisation of these service features allow the creation of supplementary services.

UMTS shall be able to handle multiple supplementary services within a call. Interactions shall be handled when several supplementary services are activated in the same call.. When multiple supplementary services can be activated concurrently, some prioritisation of the services will be necessary. Certain services may override or deactivate other services.

Interactions between operator specific supplementary services are not defined.

The following issues need consideration when interactions between services occur;

-	Different phases of a call.

-	A service spanning on more than one network.

-	Service interactions that may occur between services offered to a single user, as well as between services offered to different interacting users.

NOTE:	The methods defined for characterisation of services are description methods. They do not imply or restrict different implementations.

## 4.4	Service features

UMTS service features are based on functionality and mechanisms such as provided by SAT, MExE, IN and CAMEL. These toolkits are the basic building blocks for the VHE. These features can be used both by standardised and non-standardised services through the UMTS Application Programming Interface. The UMTS services and applications get access to UMTS service capabilities (bearers) for transport of user data through the UMTS adaptation layer. This lowest layer of the VHE is responsible for the selection of appropriate service capabilities according to the requirements of services and applications.

High level service features requirements :

. support of wide range of user applications,

. support of rapid application/service development,

. support of easy deployment of new services,

. scalability.

# 5	Bearer Services

## 5.1	Definition of bearer services

Bearer services provide the capability for information transfer between access points and involve only low layer functions. These functions are sometimes referred as low layer capabilities (in reference to OSI layers). The user may choose any set of high layer protocols for his communication and the UMTS network does not ascertain compatibility at these layers between users.

In the general case a communication link between access points provides a general service  for information transport. The communication link may span over different networks such as Internet, Intranets, LANs and ATM based transit networks, having network specific means for bearer control. Each network contributes to the end-to-end QoS perceived by the end-user.

## Description of bearer services

Bearer services are characterised by a set of end-to-end characteristics with requirements on QoS. The characteristics and requirements shall cover major network scenarios, i.e. the cases when the terminating network is PSTN, N-ISDN, GSM, IP networks/LANs, X.25 and a UMTS network.

Quality of Service is the quality of a requested service (Teleservice or Bearer Service or any other service, e.g. customer care) as perceived by the customer (ITU-T M.xxxx). QoS is always meant end-to-end. Network Performance of several network elements of the originating and terminating network(s) contribute to the QoS as perceived by the customer including terminals and terminal attachments. In order to offer the customer a certain QoS the serving network need to take into account network performance components of their network, reflect the performance of the terminal and ad sufficient margin for the terminating networks in case network performance requirements cannot be negotiated.

As far as the QoS to 3rd Generation subscriber is concerned 3G network elements have to provide sufficient performance (reflecting possible performance constraints in terminating networks) that 3G networks cannot be considered as a bottleneck.

This section outlines the requirements on bearer services in two main groups;

Requirements on information transfer, which characterise the networks transfer capabilities for transferring user data between two or more access points.

Information quality characteristics, which describe the quality of the user information transferred between two or more access points.

It shall be possible to negotiate / re negotiate the characteristics of a bearer service at session / connection establishment and during an on going session / connection.

### 5.2.1	Information transfer

- Connection oriented / conectionless services

- Both Connection oriented and connectionless services shall be supported.

- Traffic type  It is required that the bearer service provides guaranteed/constant bit rate and a non-guaranteed/dynamically variable bit rate.

- Real time and non real time applications shall be supported.

- Real time video, audio and speech must be supported. This implies the:

- ability to provide a real time stream of guaranteed bit rate, end to end delay and delay variation.

- ability to provide a real time conversational service of guaranteed bit rate, end to end delay and delay variation.

- Non real time interactive and file transfer service must be supported. This implies the:

- ability to support message transport with differentiation as regards QoS between different users.

- Multimedia applications shall be supported. This implies the:

- ability to support several user flows to/from one user having different traffic types (e.g. real time, non real time)

- Traffic characteristics

It shall be possible for an application to specify its traffic requirements to the network by requesting a bearer service with one of the following configurations

- 1)	Point-to-Point

-	Uni-Directional

-	Bi-Directional

-	Symmetric

-	Asymmetric

- 2)	Uni-Directional Point-to-Multipoint

-	Multicast

-	Broadcast

A multicast topology is one in which sink parties are specified before the connection is established, or by subsequent operations to add or remove parties from the connection. The source of the connection will always be aware of all parties to which the connection travels.

A broadcast topology is one in which the sink parties are not always known to the source. The connection to individual sink parties is not under the control of the source, but is by request of each sink party.

### In the case of a mobile termination with several active bearer services simultaneously, it shall be possible for each bearer service to have independent configurations and source/sink parties.5.2.2	Information quality

### 5.2.2 Information Quality

Information quality  characterizes the bit integrity and delay requirements of the applications.

Other parameters may be needed.

- Maximum transfer delay

- Transfer delay is the time between the request to transfer the information at one access point to its delivery at the other access point. In clause 5.5 requirements on maximum transfer delay is defined.

- Delay variation

The delay variation of the information received information over the bearer has tobe controlled to support real-time services. The possible values for delayvariation are not a limited set, but a continuous range of values.

Bit error ratio

The ratio between incorrect and total transferred information bits. The possible values for Bit error ratio are not a limited set, but a continuous range of values.

Data rate

The data rate is the amount of data thranserfered between the two access points in a given period of time.

## 5.3	Supported data rates

It shall be possible for one application to specify its traffic requirements to the network by requesting a bearer service with any of the specified traffic type, traffic characteristics, maximum transfer delay, delay variation, bit error ratios & data rates. It shall be possible for the network to satisfy these requirements without wasting resources on the radio and network interfaces due to granularity limitations in bit rates.

It shall be possible for one mobile termination to have several active bearer services simultaneously, each of which could be connection oriented or connectionless.

The only limiting factor for satisfying application requirements shall be the cumulative bit rate per mobile termination at a given instant (i.e. when summing the bit rates of one mobile termination’s simultaneous connection oriented and connectionless traffic, irrespective of the traffic being real time or non real time) in each radio environment:

- At least 144 kbits/s in satellite radio environment (Note 1).

- At least 144 kbits/s in rural outdoor radio environment.

- At least 384 kbits/s in urban/suburban outdoor radio environments.

- At least 2048 kbits/s in indoor/low range outdoor radio environment.

NOTE 1 : This Peak Bit Rate may only be achieved in a nomadic operating mode.

## 5.4	Range of QoS requirements

It shall be possible for one application to specify its QoS requirements to the network by requesting a bearer service with any  of the specified traffic type, traffic characteristics, maximum transfer delay, delay variation,bit error ratios & data rates.

The following table indicates the range of values that shall be supported by UMTS.. These requirements are valid for both connection and connectionless traffic. It shall be possible for the network to satisfy these requirements without wasting resources on the radio and network interfaces due to granularity limitations in QoS.

|  | Real Time (Constant Delay) | Non Real Time (Variable Delay) |
| --- | --- | --- |
| Operating environment | BER/Max Transfer Delay | BER/Max Transfer Delay |
| Satellite (Terminal relative speed to ground up to 1000 km/h for plane) | Max Transfer Delay less than 400 ms BER 10-3 - 10-7 (Note 1) | Max Transfer Delay 1200 ms or more (Note 2) BER = 10-5 to 10-8 |
| Rural outdoor (Terminal relative speed to ground up to 500 km/h) (Note 3) | Max Transfer Delay 20 - 300 ms BER 10-3 - 10-7 (Note 1) | Max Transfer Delay 150 ms or more (Note 2) BER = 10-5 to 10-8 |
| Urban/ Suburban outdoor (Terminal relative speed to ground up to 120 km/h) | Max Transfer Delay 20 - 300 ms BER 10-3 - 10-7 (Note 1) | Max Transfer Delay 150 ms or more (Note 2) BER = 10-5 to 10-8 |
| Indoor/ Low range outdoor (Terminal relative speed to ground up to 10 km/h) | Max Transfer Delay 20 - 300 ms BER 10-3 - 10-7 (Note 1) | Max Transfer Delay 150 ms or more (Note 2) BER = 10-5 to 10-8 |
| NOTE 1; There is likely to be a compromise between BER and delay. NOTE 2; The Max Transfer Delay should be here regarded as the target value for 95% of the data. NOTE 3; The value of 500 km/h as the maximum speed to be supported in the rural outdoor environment was selected in order to provide service on high speed vehicles (e.g. trains). This is not meant to be the typical value for this environment (250 km/h is more typical). |  |  |

## Supported End User QoS

This section outlines the QoS that shall be provided to the end user / applications. Figure 2 below summarises the major groups of application in terms of QoS requirements. Applications and new applications may be applicable to one more groups.

- The following tables further elaborate UMTS end user / application QoS requirements.

## Table 1: End-user Performance Expectations - Conversational / Real-time Services

| Medium | Application | Degree of symmetry | Data rate | Key performance parameters and target values |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | One-way Delay | Delay Variation | Information loss |
| Audio | Conversational voice | Two-way | 4-25 kb/s | <150 msec preferred <400 msec limit | < 1 msec | < 3% FER |
| Video | Videophone | Two-way | 32-384 kb/s | < 150 msec preferred <400 msec limit Lip-synch : < 100 msec |  | < 1% FER |
| Data | Telemetry - two-way control | Two-way | <28.8 kb/s | < 250 msec | N.A | Zero |
| Data | Interactive games | Two-way | < 1 KB | < 250 msec | N.A | Zero |
| Data | Telnet | Two-way (asymmetric) | < 1 KB | < 250 msec | N.A | Zero |

## Table 2: End-user Performance Expectations - Interactive Services

| Medium | Application | Degree of symmetry | Data rate | Key performance parameters and target values |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | One-way Delay | Delay Variation | Information loss |
| Audio | Voice messaging | Primarily one-way | 4-13 kb/s | < 1 sec for playback < 2 sec for record | < 1 msec | < 3% FER |
| Data | Web-browsing - HTML | Primarily one-way |  | < 4 sec /page | N.A | Zero |
| Data | Transaction services – high priority e.g. e-commerce, ATM | Two-way |  | < 4 sec | N.A | Zero |
| Data | E-mail (server access) | Primarily One-way |  | < 4 sec | N.A | Zero |

## Table 3: End-user Performance Expectations - Streaming Services

| Medium | Application | Degree of symmetry | Data rate | Key performance parameters and target values |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | One-way Delay | Delay Variation | Information loss |
| Audio | High quality streaming audio | Primarily one-way | 32-128 kb/s | < 10 sec | < 1 msec | < 1% FER |
| Video | One-way | One-way | 32-384 kb/s | < 10 sec |  | < 1% FER |
| Data | Bulk data transfer/retrieval | Primarily one-way |  | < 10 sec | N.A | Zero |
| Data | Still image | One-way |  | < 10 sec | N.A | Zero |
| Data | Telemetry - monitoring | One-way | <28.8 kb/s | < 10 sec | N.A | Zero |

## 5.6	Radio Interface optimisation

The following requirements shall lead the radio interface optimisation process;

-	support of high bit rate (around the Peak Bit Rate), bursty, asymmetric, non-real time bearer capabilities;

-	support of high bit rate (around the Peak Bit Rate), bursty, asymmetric, real time bearer capabilities;

-	the ability to extend or reduce bandwidth associated to a bearer capability in order to adapt to bit rate or radio condition variations, to add or drop service components.

However, the services provided by GSM (speech in particular) shall be supported in a spectrally efficient manner (at least as efficiently as in GSM) for the same quality of service.

In order to allow the support of flexible, bandwidth on demand services, bearer services should be provided with the finest possible granularity that can be efficiently supported.

## 5.7	Support of GSM general bearer services

UMTS shall support GSM General Bearer Services (GBS) and interworking scenarios as specified in 02.02.

# 6	Teleservices

## 6.1	Definition of teleservices

Teleservices provide the full capabilities for communications by means of terminal equipment, network functions and possibly functions provided by dedicated centres.

## 6.2	Description of teleservices

The basic reference in UMTS for the description of teleservices is the ITU-T F700 recommendation. F700 provides a generic, network independent, description of multimedia services. The methodology used covers both monomedia and multimedia services, the monomedia services being a particular type of multimedia services. Multimedia services are classified into categories with similar functional characteristics. The six categories are multimedia conference services, multimedia conversational services, multimedia distribution services ,multimedia retrieval services, multimedia messaging services and multimedia collection services.

The rest of clause 6 describes the teleservices and options that will be provided by UMTS networks.

A teleservice can be viewed as set of upper layer capabilities utilising the lower layer capabilities described by the set of attributes in clause 5.

Multimedia teleservices support the transfer (and in some case retrieval, messaging, distribution) of several types of information (service components). For this reason, there are service attributes (relating to all the components of a teleservice) and service component attributes (relating to only one service component).

## 6.3	Support of teleservices in UMTS networks

The realisation of teleservices requires the association of terminal and network capabilities. In the terminals and in the network, both upper layer capabilities and lower layer capabilities are necessary. The term upper layer capabilities is used because it relates to the OSI upper layers. Decoupling between upper layers and lower layers (transfer) is required. Even if this de-coupling may impact radio interface optimisation, it is nevertheless the only way of designing a system that is not outdated;

-	Each time the information rate associated with an already supported teleservice is decreased by more efficient source coding techniques.

-	Each time a new service is introduced that requires transfer capabilities not used by currently available teleservices.

Taking the example of two application that exchange information through a teleservice, the upper layer capabilities can be located in various places;

-	In the two terminals if the two applications are connected to a UMTS network.

- Figure 2; UMTS teleservice

In the terminal of the application connected to a UMTS network and in the upper layer interworking unit that is at the border of the UMTS network and the target network if one application is connected to a UMTS network and the other one is connected to another type of system. The upper layer interworking unit makes the adaptation between the UMTS network and the target network at a service level.

- Figure 3; Teleservice with upper layer interworking

In the terminal of the application connected to a UMTS network and in the terminal of the application connected to a target network if one application is connected to a UMTS network and the other one is connected to another type of system, but only lower layer interconnecting unit is used at the border of the two networks. In this case, the interconnecting unit makes the adaptation between the UMTS network and the target network at the transmission level.

- Figure 4; Teleservice with lower layer interworking

## 6.4	Existing Teleservices supported by UMTS networks

The subset of standardised teleservices shall be supported by UMTS for interworking with teleservices provided on other networks. The means to support the following set of teleservices will be standardised;

-	Speech;

-	Emergency call;

-	Short message service;

### 6.4.1	Speech

The speech service as defined in international standards should be supported by UMTS. The international reference for the speech is ITU E.105 recommendation. UMTS networks should contain interworking units which allow calls to be received from or destined to users of existing networks like PSTN, ISDN, GSM. This will include interworking units for generation of DTMF or other tones (the entire DTMF tone set would at minimum be available) and detection of DTMF tones.

Speech (7kHz) communications via bi-directional and symmetric channels between UMTS users or with fixed wireline or GSM users with equivalent or better quality than the audio quality of G.722 shall be supported in Phase 1, but not in Release 99.

A default speech codec shall be specified to provide speech service across the UTRAN. The selected speech codec shall be capable of operating with minimum discernible loss of speech on handover between the GSM access network and UTRAN.

### 6.4.2	Emergency Call

This service will use components of Speech. There are however compared to Telephony reduced authentication requirements and a requirement for specific routing. Additionally Emergency Calls may have higher priority than normal calls, etc.. The reference for the emergency call service is GSM 02.03.

### 6.4.3	Short Message Service  - Point to Point (SMS-PP)

The short message service point to point as specified in GSM 02.03 shall be supported in UMTS. A short message service shall be provided seamlessly (as far as the user or the users terminal equipment is concerned) across the UMTS and GSM access network. Additional features are planned for SMS in Release 99.

### 6.4.4	Short Message Service - Cell Broadcast (SMS-CB)

A short message service cell broadcast shall be provided seamlessly (as far as the user or the users terminal equipment is concerned) across the UMTS and GSM network.

## 6.5	Internet Access

UMTS shall provide means to interwork with external data networks. This interworking shall satisfy, within the constraints introduced by the mobile radio environment, the QoS requirements of the interworked-with network. For UMTS the Internet is seen as the most important interworked-with network, therefore the specification of an optimised access to Internet shall be part of the UMTS standard. The most important benefits achieved by the definition of Internet Access would be:

-	Optimised transmission of IP traffic over the UMTS radio interface to minimise the amount of information transmitted.

-	Optimised usage of encryption protocols/algorithms over the UMTS radio interface.

-	Inter-operation of QoS mechanisms used in both, UMTS and in Internet.

For the purposes of optimised access to Internet one or more of the UMTS generic bearers will be used. On top of the bearer a UMTS protocol profile will be defined. This profile would be based on the work done by IETF or other relevant fora, and will consist of a recommended set of parameters and standardised protocols providing similar services than the Internet ones but optimised for wireless access. In the case of Internet traffic it would be possible for the user to select the encryption to be used (e.g. no encryption, end-to-end encryption, encryption over UMTS radio, etc.). The QoS mechanisms defined for UMTS packet access mode shall be harmonised with those defined for Internet (e.g. Differentiated Services).

# 7	Supplementary Services

Supplementary services are used to complement and personalise the usage of basic telecommunication services (bearer services and teleservices). The capabilities standardised in UMTS shall enable all the supplementary services specified in GSM 02.04 and the 02.8x set to be provided.

# 8	Service features

Service features are building blocks which can be used to create services. The functionality offered by a service feature may depend upon the underlying service capability used to realise the service feature e.g. CAMEL, MExE etc.. Service features may be used to offer the user some control over a service such as the ability to modify a service, subscribe or unsubscribe to a service.

Service features are associated with call/session control, bearer control, mobility management. The term calls is used to encompass not only circuit-switched (e.g. voice) calls, but also virtual-circuit sessions set-up to handle packet data traffic.

The following service features are required;

-	security/privacy;

-	access control;

-	address translation;

-	call/session/bearer control;

-	location;

-	messaging;

-	service control;

-	user interaction.

## 8.1	Security/Privacy features

-	presentation of or restriction of information associated with a party involved in a call or a session (e.g. calling line ID, calling name, location...);

-	encryption of user data and signalling;

## 8.2	Access Control  features

The access control features are defined to provide access to the UMTS network to the UMTS users over the serving network’s air interface. These features include;

-	user registration;

-	user de-registration;

-	mutual authentication.

## 8.3	Address Translation Features

This address translation feature shall allow UMTS to offer the wide range of addressing options including;

-	E.164 Numbering (e.g. GSM MS-ISDN);

-	ASEA Numbering (ATM);

-	IP v6 Numbering;

-	X.25 Numbering;

-	Internet symbolic naming.

## 8.4	Call/Session/Bearer Control Features

These features will be used to establish, handle and terminate calls. The following service features shall be supported;

-	call/session set-up (point to point, point to multi-point, multi-point to multi-point);

-	add/delete a party from a call/session;

-	call/session termination;

-	call/session establishment e.g. answering of calls;

-	monitoring of call/session states and events;

-	modification of the bearer service attributes.

-	capability at initial call set-up to modify or reject the called party address;

-	capability for an incoming call to modify or reject the called party address both at early and late stage of the call;

-	capability to suspend and resume a call;

-	capability to re-route a call;

-	capability to be notified when a specified terminal is free or is ready to accept the call.

## 8.5	Location Features

Location features shall also be supported, to allow new and innovative location based services to be developed;

-	to identify and report in a standard format (e.g. geographical co-ordinates) the current location of the user’s terminal.

The precision of the location shall be network design dependent, i.e. an operator choice. This precision may vary from one part of a network to another. It may be chosen to be as low as hundreds of meters in some place and as accurate as 5 meters in other place. It is required that a minimum precision of around 50 meters can be achieved in all types of terrestrial radio environment. Technical issues may constrain the precision to be mobile state dependent as well (mobile idle / mobile in communication). Several design optional features (e.g. size of the cell, adaptive antenna technique, path loss estimation technique...) shall allow the network operator to reach cost effectively the target precision.

Because there may be very different uses of the location information;

-	It shall be possible to make the information available to the user, HE/SN and value added service providers. The user shall be able to restrict access to the location information (permanently or on a per call basis). The restriction can be overridden by the network operator when appropriate (e.g. emergency calls).

-	It shall be possible to set the delay to get the location information (the situation is quite different whether the information is needed for call routing or if it is needed by a user application).

-	It shall be possible to select the frequency of the location information update.

If the terminal is switched off, then the last known position and time/date shall be available. The time of last known location shall be recorded and be made available in universal time.

-	to identify and report when the user’s terminal enters or leaves a specified geographic area.

-	It shall be possible to specify the area as a circular zone (centre and radius) to a resolution that will be limited by the accuracy capability of the part of the serving network where the user is registered.

## 8.6	Messaging features

Messages are a block of data that may range from a few bytes to megabytes. Message delivery may involve store and forward of messages in transit. To be able to exchange and to control the exchange of messages between user the following service features shall be supported;

-	capability to send messages;

-	capability to receive messages;

-	capability to request confirmation of receipt;

-	capability to modify the content as well as the recipient of message;

-	capability to reject a outgoing and/or incoming message;

-	capability to re-route a message.

## 8.7	Service control features

To allow the support of HE/SN specific services the following service features shall be supported;

-	capability to download service software to network nodes;

-	capability to download service software to terminals;

-	capability to download service software to the USIM;

- -	capability to negotiate of supported capabilities between USIM, terminals, HE and SN;

-	capability to negotiate bearer services and service capabilities

## 8.8	User Interaction Features

To allow the support of HE/SN specific user interfaces, databases containing user profiles shall be provided. This user profile functionality shall provide the following interaction features :

-	capability to indicate information to the user;

- -	capability to collect user information;

-	capability to activate and deactivate a special user profile;

-	capability to change the user profile.

# 9	Standardised Protocols and Capabilities

This clause introduces a list of standardised protocols and capabilities that shall be supported by UMTS for the control and creation of services. The access protocols and the execution environment described below are essential for UMTS.

## 9.1	Access protocols

The access protocols shall allow the support of multimedia services. These services are characterised by the ability to dynamically change the number of participants and the number of connections during a call. The characteristics of the connections (confer the list of attributes used to describe a connection) may differ from one connection to another. They are negotiated during call set-up. They may be independently and dynamically re-negotiated on application (the telecommunication requirements of the application changes) or network initiative (change of network load conditions, during a handover procedure) during the call.

The application may require synchronisation between some of the connections. Later, this synchronisation shall not be lost during handover procedures.

Whenever a call is terminated in other types of networks, the negotiation shall take into account the limitations of these networks. Interworking shall be possible with PSTN, GSM, ISDN and Internet networks. Later it shall also be possible to interwork with B-ISDN networks.

The access protocols shall allow a mobile station to have several calls active simultaneously .

## 9.2	Execution Environment

The execution environment is a set of standardised capabilities that shall allow the support of HE/SN specific services (i.e. both applications, teleservices and supplementary services). The execution environment shall be distributed between the IC card, terminal and network nodes. The terminal and the serving network capabilities shall be the only limiting factor for the support of the services designed to run on the execution environment. The execution environment is composed of the following building blocks;

-	A standardised content description language for support of NO/SP specific user interfaces (both for information output and user input). This is intended only for platforms which are terminals.

-	A standardised procedural language for support of NO/SP specific scripts. This language shall be common to all types of platforms. The scripts could be used for e.g. improving the user interface, adding new features to the terminal like the latest version of a codec, controlling the execution of a service.

-	Standardised application programming interfaces for opening platform resources and capabilities to the scripts written with the standardised procedural language. These interfaces would be platform type dependent. The interfaces shall include primitives for accessing to the basic control functions, as illustrated on the figures 5 and 6 below.

- Figure 5: Execution Environment in the Mobile Station

- Figure 6: Execution Environment in the Network

-	Call states, messages, information elements, values of information elements shall serve as triggers for subsequent interaction with service logic. The list of triggers is for further study and is likely to incorporate CAMEL, SIM Toolkit, MExE.

-	Means to turn triggers on and off, and associate them with service logic shall be standardised.

-	A standardised certification scheme and security model with several levels of trusts in order to control the scripts access rights to the platform resources and capabilities. This would be used to allow e.g. the SP and the SP only to access to USIM data.

-	Standardised protocols for allowing the download of content description pages and scripts in the platform.

# 10	Existing GSM System features

Following GSM system features shall be supported by G-UMTS standards (for networks based on GSM evolution).

## 10.1	Network Identity and Time Zone (NITZ)

NITZ is specified in GSM 02.42.

## 10.2	Support of Localised Service Area (SoLSA)

SoLSA is specified in GSM 02.43.

Note : SoLSA modifications due to UTRAN related aspects are FFS.

## 10.3	Mobile station Execution Environment (MExE)

MExE is specified in GSM 02.57.

## 10.4	Location Services (LCS)

LCS is specified in GSM 02.71.

Note : LCS modifications due to UTRAN related aspects are FFS.

## 10.5	Customised Application for Mobile network Enhanced Logic (CAMEL)

CAMEL is specified in GSM 02.78.

## 10.6	Unstructured Supplementary Service Data (USSD)

USSD is specified in GSM 02.90

Note : USSD modifications due to UTRAN related aspects are FFS.

Annex A (informative):
Examples of services built from service features

- Call Barring

In standard GSM, the Call Barring services allow to prevent outgoing calls to certain sets of destinations, based on the number dialled and whether the user is roaming. In UMTS, it is proposed that this service allows to block outgoing calls based on a wider range of parameters which could include factors such as the time of day, day of week, location, type of call requested, cost of the service and/or destination. This would allow to develop Call Barring services tailored to business and personal markets to avoid abuse.

This service is invoked during the initial outgoing call set-up procedure and allow the call to be blocked prior to incurring any charges. This Service can be applied to any teleservice for both connection-oriented and connectionless-oriented services.

- Call Filtering/Forwarding

In standard GSM, there is no call filtering service. All calls are presented to the user unless a call forwarding service is used to re-direct calls; there is no different call handling depending on the incoming call parameters (although differentiation on call type (voice/data) is possible).

In UMTS, the call filtering service allows the control of whether incoming calls are accepted, forwarded or terminated. The parameters which can be used to determine the final destination of a call may include the caller ID (CLI), original number dialled, time of day, current user location/network, user profile settings and current state of the terminal.

This service shall be two-stage; immediate call filtering (handled regardless of whether the terminal is online or not) and late call filtering (handled only if the terminal is online). It shall be possible to create and operate new call filtering services which can access any of the key parameters to handle calls in this way.

- Hold

This service allows an established call to be maintained, whilst suspending use of the bearer from the incoming access point of the network. This saves on both air interface and network traffic resources when a call is temporarily suspended. The incoming access point in the network means either the originating UMTS terminal, or interworking point with another network.

- Transfer

This service allows either an established or held call to be redirected to another destination. This may either be used by setting up a new call to the destination first, or simply redirecting the existing call to the new destination. It shall be possible to revert such a call back to the diverting terminal at any time before it is accepted (answered) by the new destination. The UMTS system shall ensure that an optimal traffic route is used after the call has been answered by its new (final) destination.

- Call-back When Free

This service can be invoked where a call (or a connectionless message) cannot be delivered to its destination because it is in use. The UMTS system will inform the requesting entity when the destination is next able to accept the call, allowing a new call to be originated. This allows existing GSM services, such as Call-back When Free to be implemented. Where multiple requests are outstanding for a terminal which becomes available, the system will determine in which order the requests are handled, probably in a serial manner. Ideally, it shall be possible to create the service logic which determines the order used from a range of accessible parameters.

Annex B (informative) :
Description and analysis of communication schemes

This annex gives a high level classification and description of communications requirements from end users and applications.

# B.1	Communication schemes

The requirements on bearer services are based on an analysis of user and application needs. Four end-user groups are identified according to four distinctly different communication schemes; background traffic, interactive traffic, real time streams, real time conversation. For each scheme one (or two) fundamental characteristic(s) for QoS is identified and the resulting overall requirement(s) is derived. Of course, when the requested service/application bearer requirements are converted into bearer service attributes (as defined in section 5.2), these fundamental characteristics are reflected in the values allocated to the bearer service attributes.

## B.1.1	Background traffic

When the end-user, that typically is a computer, sends and receives data-files in the background, this scheme applies. Examples are background delivery of E-mails, SMS, download of databases and reception of measurement records.

Background traffic is one of the classical data communication schemes that on an overall level is characterised by that the destination is not expecting the data within a certain time. The scheme is thus more or less delivery time insensitive. Another characteristic is that the content of the packets must be transparently transferred (with low bit error rate).

Background traffic  - fundamental characteristics for QoS:

- the destination is not expecting the data within a certain time

- preserve payload content

The resulting overall requirement for this communication scheme is to support non-real time services without any special requirement on delay.

## B.1.2	Interactive traffic

When the end-user, that is either a machine or a human, is on line requesting data from remote equipment (e.g. a server), this scheme applies. Examples of human interaction with the remote equipment are: web browsing, data base retrieval, server access. Examples of machines interaction with remote equipment are: polling for measurement records and automatic data base enquiries (tele-machines).

Interactive traffic is the other classical data communication scheme that on an overall level is characterised by the request response pattern of the end-user. At the message destination there is an entity expecting the message (response) within a certain time. Round trip delay time is therefore one of the key attributes. Another characteristic is that  the content of the packets must be transparently transferred (with low bit error rate).

Interactive traffic  - fundamental characteristics for QoS:

request response pattern

preserve payload content

NOTE:	Interactive applications might differ between interactive games and interactive data transfer such as web browsing.

The resulting overall requirement for this communication scheme is to support interactive non-real time services with low round-trip delay.

## B.1.3	Real time streams

When the user is looking at (listening to) real time video (audio) the scheme of real time streams applies. The real time data flow is always aiming at a live (human) destination. It is a one way transport.

This scheme is one of the newcomers in data communication, raising a number of new requirements in both telecommunication and datacommunication systems. First of all it is a unidirectional stream with high continuous utilisation (i.e. having few idle/silent periods.) It is also characterised by that the time relations (variation) between information entities (i.e. samples, packets) within a flow must be preserved, although it does not have any requirements on low transfer delay.

The delay variation of the end-to-end flow must be limited, to preserve the time relation (variation) between information entities of the stream. But as the stream normally is time aligned at the receiving end (in the user equipment), the highest acceptable delay variation over the transmission media is given by the capability of the time alignment function of the application. Acceptable delay variation is thus much greater than the delay variation given by the limits of human perception.

Real time streams - fundamental characteristics for QoS:

- unidirectional continuous stream

- preserve time relation (variation) between information entities of the stream

The resulting overall requirement for this communication scheme is to support streaming real time services having unidirectional data flows with continuous utilisation. (There are less hard requirements on delay and packet loss ratio, i.e. the ratio of lost or corrupted packets out of all packets sent.)

## B.1.4	Real time conversation

The most wellknown use of this scheme is telephony speech (e.g. GSM), but with Internet and multimedia a number of new applications will require this scheme, for example voice over IP and video conferencing tools. Real time conversation is always performed between peers (or groups) of live (human) end-users. This is the only scheme where the required characteristics are strictly given by human perception (the senses). Therefore this scheme raises the strongest and most stringent QoS requirements.

The real time conversation scheme is characterised by that the transfer time must be low because of the conversational nature of the scheme and at the same time that the time relation (variation) between information entities of the stream must be preserved in the same way as for real time streams. The maximum transfer delay is given by the human perception of video and audio conversation. Therefore the limit for acceptable transfer delay is very strict, as failure to provide low enough transfer delay will result in unacceptable lack of quality. The transfer delay requirement is therefore both significantly lower and more stringent than the round trip delay of the interactive traffic case.

Real time conversation - fundamental characteristics for QoS:

- preserve time relation (variation) between information entities of the stream

- conversational pattern (stringent and low delay)

The resulting overall requirement for this communication scheme is to support conversational real time services with low transfer delay as given by the human perception. (There are less hard requirements on packet loss ratio.)

# B.2	Adaptability and bearer service negotiation

Applications using the interactive or real time conversational communication schemes can also be described according to their possibilities for adapting to different environmental conditions as follows:

- Rigid applications; these applications can not adapt at all (e.g. GSM full rate speech.)

- Adaptive applications; these applications can adapt to the environment; they therefore require the network to support service negotiation. (e.g. multi-rate speech codecs)

- Elastic applications; these applications adapt totally to the environment and do therefore not require service negotiation (e.g. web browsing.)

The resulting overall requirement is to support service negotiation.

Annex C (informative) :
Change history

| Change history |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SMG No. | TDoc. No. | CR. No. | Section affected | New version | Subject/Comments |
| SMG#25 |  |  |  | 3.0.0 | Approved at SMG#25 Sophia Antipolis 17-20 March 1998 |
| SMG#26 | 98-0325 | 001 | Section 3.1 (a new definition is added) and section 5 (the current text is clarified). | 3.1.0 | Clarification of the sections where the bearer services are characterised. |
| Pre-SMG#28 | SMG1 Tdoc 98-0864 | A002 | 3.2, 4.1, 4.3, 4.4, 6.1, 6.4.1, 6.4.3, 6.4.4., 6.4.5, 8.2, 8.3, 8.5, 8.7, 8.8, 9.1, 9.2 | Draft 3.2.0 | SMG1 Agreed at this stage Aligning this specification with 22.00 |
| Pre-SMG#28 | SMG1 Tdoc 98-0870 | 003 | Sections 2, 9 | Draft 3.2.0 | SMG1 Agreed at this stage References to relevant GSM specs has been added. |
| Pre-SMG#28 | SMG1 Tdoc 0895 (865#5) | 004 | Annex B | Draft 3.2.0 | SMG1 Agreed at this stage In line with views expressed by SMG2 and SMG12 about descriptions and analysis of communication schemes. |
| Pre-SMG#28 |  | 005 | Section 8 | Draft 3.2.0 | Service features are only used to create services (as building blocks) and not to modify and delete services. (Added Jan 27, 1999) |
| SMG#28 |  |  |  | Version 3.2.0 | Approved Versions |
| SA#2 Florida |  | 006 | 6.4 | Version 3.3.0 | Cell Broadcast Service in UMTS. |
|  |  |  |  | Version 3.3.0 | reformatted for TSG SA |
| SA#3, Japan |  | 007 | 6.4.1 | Version 3.4.0 | Add requirement for high quality speech. |
| SA#3, Japan |  | 008 | 2.1, 5.7 (added) | Version 3.4.0 | Support of GSM BSs |
| Post-SA#3 |  |  |  | Version 3.4.1 | Updated logo, foreword, etc … |

# History

| Document history |  |
| --- | --- |
| June 1998 | Version 3.1.0 Unpublished |
| January 1999 | v.3.2.0 - with 4 CRs accepted by SMG1 - Presented to TSG SA WG1 - To be presented to SMG#28 for Approval |
| February 1999 | 3.3.0 SMG#28 |
| March 1999 | 3.3.1 |
| May 1999 | Version 3.4.0 with approved CRs from SA#3, Yokohama, Japan |