TSG-SA Working Group 1 meeting #7	TSG S1 (00)050

Sophia Antipolis, France	Agenda Item: 5.14

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.121 |  | Version: |  |  | 3.1.0 |  |  |  |
| Submitted to TSG_SA | 7 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE |  | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Virtual Home Environment |
| --- | --- |

| Source: | Siemens/Ericsson | Date: | 15/1/00 |
| --- | --- | --- | --- |

| Subject: | Modification of section 5.1 |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| And one release | C Functional modification of feature |  |  | Release 98 |  |
| Only shall be | D Editorial modification |  |  | UMTS 99 | X |
| Marked with an X) |  |  |  |  |  |

| Reason for change: | There has been a long discussion on the section 5.1 concerning the introduction of MExE and SAT servers. The proposed change clarifies this issue by - introducing a modified figure 3 where the SCFs are presented in a neutral format (excluding a specific mapping to a concrete SCS) and the concrete denomination of the SCS with CSE, HLR, MExE and SAT are replaced in favour of the more general names SC1, .. SCS n. This has the advantage that the figure does no more suggest that the implementation of any of the SCS is mandatory. On the other hand this change does not limit the way how certain SCFs are implemented by SCS, i.e. which network resource they use to provide a certain SCF to the application. - considering all concrete network entities and their related interfaces (HLR, SAT, MExE,etc.) only as examples in the text by adding "e.g." or by "may be realised". |
| --- | --- |

| Clauses affected: | 5.1 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications | X |  List of CRs: | 23.127 |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 5.1 	Ways to realise services

The information contained in this clause is only to aid understanding and is not an extensive list.

Figure 3 illustrates how the concept of VHE makes use of the standardised application interface and how that fits to the service capability features and service capabilities for release 99. Note that the Service Capabilities (SCx) shown below are representatives of the different possible capabilities. It is not to be implied as the agreed architecture as this is a stage 2 issue.

[Note: SCS in figure renamed SC]

Figure 3	 Possible realisation of Framework for Services

- STANDARDISED SERVICES (Supplementary Services, Tele-Services, etc.) are implemented on existing GSM/UMTS entities (e.g. HLR , MSC/VLR and terminal) on a vendor specific basis, using standardised interfaces (MAP, etc.) for service communication (e.g. downloading of service data).   Availability and maintenance of these Services is also vendor dependent.

- OPERATOR SPECIFIC SERVICES (OSS) are not standardised and could be implemented at the GSM/UMTS entities (e.g. HLR) on a vendor specific basis or using GSM ph 2+ mechanisms (CAMEL, SAT, MExE). These tool-kits use standardised interfaces to the underlying network (e.g. CAP, MAP) or use GSM Bearers to transport applications and data e.g. from the MExE service environment or SAT to the MS/SIM. The implementation of these operator specific services on the different platforms (e.g. CSE, MExE/SAT Server, MSs) is done in a completely vendor specific way and uses only proprietary interfaces.

- Other APPLICATIONS are like OSS not standardised. These applications will be implemented using standardised interfaces to the Service Capabilities (Bearers, Mechanisms). The functionality offered by the different Service Capabilities are defined by Service Capability Features. These Service Capability Features will be standardised and can be used by the application designers to build their applications.

- Within the terminals Service Capabilities are accessible via existing APIs, the MExE and SAT APIs, i.e. there will be no service capability features within the terminal. 
The terminal can communicate, using GSM/UMTS bearers, with applications in the network via the service capability features which may be optionally realised for MExE-service environment and SAT.