| 3GPP SA1 # 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | S1-000058 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sophia Antipolis; 9-11 February 2000. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 22.060 |  |  |  |  |  | CR |  |  | 009 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.2.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | SA # 7 |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  | X | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  | X | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  | X |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Rapporteur |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 9 February 2000 |  |  |  |  |  |
| Subject: |  |  | Revision CR to 22.060 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | Restructuring to improve clarity of document and revision to align with Stage 2 description for Release 99. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | All |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

3G TS 22.060 V3.3.0 (1999-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

General Packet Radio Service (GPRS);

Service description, Stage 1

(3G TS 22.060 version 3.3.0)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTS/TSGSA-0122060U

Keywords

3GPP, SA

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

All rights reserved.

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 1999, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA,TTC).

Contents

Foreword	8

1	Scope	9

2	References	9

3	Definitions and abbreviations	11

3.1	Definitions	11

3.2	Abbreviations	16

4	Main Concepts	17

5	Service Requirements	18

5.1	GPRS reference model	18

5.1.1	Bearer services	19

5.1.2	Teleservices	20

5.1.3	GPRS Access Points	20

5.2	Network Interworking	20

5.2.1	Interworking with other data networks and other PLMNs	21

5.2.1.1	QoS when Interworking	21

5.2.2	GPRS Numbering Plan	21

5.2.3	Addressing and routeing requirements	21

5.2.3.1	PTP network layer services	21

5.2.4	Interworking for subscriber roaming	22

5.3	GPRS service description	22

5.3.1	Point-To-Point Connectionless Network Service (PTP-CLNS)	23

5.3.2	Point-To-Point Connection Orientated Network Service (PTP-CONS)	23

5.3.3	Point-To-Multipoint (PTM) service overview	23

5.3.3.1	Identification of subscribers and subscriber groups in PTM	24

5.3.3.2	PTM Group Call (PTM-G) Description	25

5.3.4	PTM-Group-Call Operations and User Interface	28

5.3.4.1	PTM Group Administration	29

5.3.4.2	PTM Group Call service description	29

5.3.4.3	PTM-G call initiate operation	30

5.3.4.4	PTM-G call terminate operation	32

5.3.4.5	PTM-G call status operation	32

5.3.4.6	PTM-G join call operation	34

5.3.4.7	PTM-G leave call operation	34

5.3.4.8	PTM-G data transfer operation	36

5.3.5	Point-To-Point Octet Stream Service	36

5.4	Service interworking	37

5.5	Transfer Characteristics	37

5.6	Service characteristics	37

5.6.1	Subscriber profile	37

5.6.2	Quality of Service (QoS)	38

5.6.2.1	QoS parameter definitions	38

5.6.2.1.1	Service precedence (priority)	38

5.6.2.1.2	Reliability	38

5.6.2.1.3	Delay	39

5.6.2.1.4	Throughput	39

5.6.2.2	QoS profile - PTP and PTM-G	39

5.6.2.3	Monitor	40

5.6.3	Packet size	41

6	Operational Requirements	41

6.1	GPRS MS Modes of Operation	41

6.2	Subscriber roaming	42

6.3	Construction of GPRS subscriber numbers	43

6.4	Battery life extension	43

6.5	Format Of Message User Data	43

6.6	Charging aspects	43

6.6.1	PTP and IP-M charging information	43

6.6.2	PTM-G charging information	44

6.6.3	Reverse Charging	44

6.7	Security services	44

6.8	Message Screening	45

6.9	GPRS Operator Determined Barring	45

7	Normal procedures with successful outcome	45

7.1	Provision	45

7.2	Withdrawal	46

7.3	GPRS-Attach, GPRS-Detach	46

7.4	Registration	46

7.5	Erasure	47

7.6	Interrogation	47

7.7	Activation	47

7.8	De-activation	47

7.9	Invocation and operation	48

7.10	PIN and Password Management	48

8	Exceptional procedures	48

8.1	Provision	48

8.2	Withdrawal	48

8.3	Registration	48

8.4	Erasure	49

8.5	Interrogation	49

8.6	Activation	49

8.7	De-activation	49

8.8	Invocation and operation	49

8.9	PIN and Password management	49

9	Addressing	49

9.1	Point to Point services	49

9.2	Point to Multipoint services	49

9.3	Overview	50

History	53

-

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TS, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version 3.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	Indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the specification;

# 1	Scope

This TS defines the stage one description of the General Packet Radio Service (GPRS). Stage one is an overall service description, primarily from the service subscriber's and user's points of view, but does not deal with the details of the human interface itself. This TS includes information applicable to network operators, service providers and terminal, switch and data base manufacturers.

This TS contains the core requirements for the packet switched networks specified by 3GPP which are sufficient to provide a complete service.

It defines  a set of bearer services that provide packet mode transmission within the PLMN and interwork with external networks.

The term GPRS shall be used in this document to refer to GPRS, EGPRS and the PS Domain of the 3G PLMN.

The GPRS shall not prevent the user's operation of other GSM services.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

- For this Release 1999 document, references to GSM documents are for Release 1999 versions (version 8.x.y).

[1]	TS 22.001: " Principles of telecommunication services supported by a GSM Public Land Mobile Network (PLMN)".

[2]	GSM 04.02: "Digital cellular telecommunications system (Phase 2+); GSM Public Land Mobile Network (PLMN) access reference configuration".

[3]	ITU-T Recommendation X.25: "Interface between data terminal equipment (DTE) and data circuit-terminating equipment (DCE) for terminals operating in the packet mode and connected to public data networks by dedicated circuit".

[4]	ISO 8208: "Information processing systems - data communications - X.25 packet level protocol for data terminal equipment".

[5]	ISO 8348: "Information processing systems - data communications - network service definition".

[6]	ISO 8878: "Information processing systems - data communications - use of X.25 to provide the OSI connection-mode network service".

[7]	Internet STD 5:RFC 791: Internet protocol, RFC 950: "Internet standard subnetting procedure", RFC 919: "Broadcasting internet datagrams", RFC 922: "Broadcasting internet datagrams in the presence of subnets", RFC 792: "Internet control message protocol", RFC 1112: "Host extensions for IP multicasting" RFC 1122:" Requirements for Internet hosts - communication layers". RFC 1920:" Internet official protocol standards", RFC 1458: "Requirements for multicast protocols", RFC 1301: "Multicast transport protocol".

[8]	ITU-T Recommendation X.3: "Packet Assembly/Disassembly facility (PAD) in a public data network".

[9]	ITU-T Recommendation X.28: "DTE/DCE interface for a start-stop mode Data Terminal Equipment accessing the Packet Assembly/Disassembly facility (PAD) in a public data network situated in the same country".
X.28 Add. 1  (7/94)  Addendum 1 to Recommendation X.28 to enable MAP support in accordance with Recommendation X.8.

[10]	ITU-T Recommendation X.29: "Procedures for the exchange of control information and user data between a Packet Assembly/Disassembly (PAD) facility and a packet mode DTE or another PAD".

[11]	ITU-T Recommendation X.75: "Packet-switched signalling system between public networks providing data transmission services".

[12]	ITU-T Recommendation X.121: "International numbering plan for public data networks".

-

[13]	TS22.004 (ETS 300 918): " General on supplementary services".

[14]	TS22.030 (ETS 300 907): " Man-Machine Interface (MMI) of the Mobile Station (MS)".

[15]	GSM 02.17 (ETS 300 922): "Digital cellular telecommunications system; Subscriber Identity Modules (SIM) Functional characteristics".

-

[16]	TS 22.041: "Operator Determined Barring (ODB)"

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of this TS the following definitions apply:

A

access delay: The value of elapsed time between an access request and a successful access (source: ITU-T X.140).

access protocol: a defined set of procedures that is adopted at an interface at a specified reference point between a user and a network to enable the user to employ the services and/or facilities of that network (source: ITU-T I.112).

accuracy: A performance criterion that describes the degree of correctness with which a function is performed. (The function may or may not be performed with the desired speed.) (source: ITU-T I.350).

B

bearer service: A type of telecommunication service that provides the capability for the transmission of signals between user-network interfaces (source: GSM 01.04, ITU-T I.112).

broadcast: A value of the service attribute "communication configuration", which denotes unidirectional distribution to all users (source: ITU-T I.113).

C

closed group: A group with a pre-defined set of members. Only defined members may participate in a closed group.

conversational service: An interactive service which provides for bi-directional communication by means of real-time (no store-and-forward) end-to-end information transfer from user to user (source: ITU-T I.113).

D

dependability: A performance criterion that describes the degree of certainty (or surety) with which a function is performed regardless of speed or accuracy, but within a given observational interval (source: ITU-T I.350).

destination user: Entity to which calls to the General Packet Radio Service (GPRS) are directed.

F

functional group: A set of functions that may be performed by a single equipment (source: ITU-T I.112).

G

group: A set of members allowed to participate in the group call service. The group is defined by a set of rules that identifies a collection of members implicitly or explicitly. These rules may associate members for the purpose of participating in a group call, or may associate members who do not participate in data transfer but do participate in management, security, control, or accounting for the group.

group call: The relationship that exists between the members of a group for the purpose of transferring data. More than one group call may exist in a group. A group call establishes an active group.

group call initiator: A member (or third party) authorised to initiate a group call. More than one member may initiate group calls.

group call participant: A member of a group participating in a particular group call at a given time.

group call service: A PTM service in which a relationship exists between participants of the group, and in which a single data unit transmitted by a source participant is received by multiple destination participants; it is a one-in, many-out service.

group controller: The member (or third party) responsible for the group creation and membership control.

I

interactive service: A service which provides the means for bi-directional exchange of information between users. Interactive services are divided into three classes of services: conversational services, messaging services and retrieval services (source: ITU-T I.113).

interface: The common boundary between two associated systems (source: GSM 01.04, ITU-T I.112).

M

mean bit rate: A measure of throughput. The average (mean) bit rate available to the user for the given period of time (source: ITU-T I.210).

messaging service: An interactive service which offers user-to-user communication between individual users via storage units with store-and-forward, mailbox and/or message handling, (e.g., information editing, processing and conversion) functions (source: ITU-T I.113).

mobile station: Equipment intended to access a set of PLMN telecommunication services. Services may be accessed while the equipment capable of surface movement within the system area is in motion or during halts at unspecified points.

mobile termination: The part of the mobile station which terminates the radio transmission to and from the network and adapts terminal equipment capabilities to those of the radio transmission (source GSM 01.04).

multicast service: A unidirectional PTM service in which a message is transmitted from a single source entity to all subscribers currently located within a geographical area. The message contains a group identifier indicating whether the message is of interest to all subscribers or to only the subset of subscribers belonging to a specific multicast group.

multipoint: A value of the service attribute "communication configuration", which denotes that the communication involves more than two network terminations (source: ITU-T I.113).

N

network operator: Entity which provides the network operating elements and resources for the execution of the General Packet Radio Service (GPRS).

network service data unit (NSDU): A unit of data passed between the user and the GPRS network across a Network Service Access Point (NSAP).

network termination: A functional group on the network side of a user-network interface (source: ITU-T I.112).

O

open group: A group that does not have a pre-defined set of members. Any user may participate in an open group.

P

packet: An information unit identified by a label at layer 3 of the OSI reference model (source: ITU-T I.113). A network protocol data unit (NPDU).

packet data protocol (PDP): Any protocol which transmits data as discrete units known as packets, e.g., IP, or X.25.

packet transfer mode: Also known as packet mode. A transfer mode in which the transmission and switching functions are achieved by packet oriented techniques, so as to dynamically share network transmission and switching resources between a multiplicity of connections (source: ITU-T I.113).

PLMN Operator: Public Land Mobile Network operator. The entity which offers a GPRS.

point-to-multipoint (PTM) service: A service type in which data is sent to ”all service subscribers or a pre-defined subset of all subscribers” within an area defined by the Service Requester.

point-to-point (PTP): A value of the service attribute "communication configuration", which denotes that the communication involves only two network terminations.

point-to-point (PTP) service: A service type in which data is sent from a single network termination to another network termination.

protocol: A formal set of procedures that are adopted to ensure communication between two or more functions within the within the same layer of a hierarchy of functions (source: ITU-T I.112).

protocol data unit (PDU): In the reference model for OSI, a unit of data specified in an (N)-protocol layer and consisting of (N)-protocol control information and possibly (N)-user data (source: ITU-T X.200 / ISO-IEC 7498-1).

Q

quality of service: The collective effect of service performances which determine the degree of satisfaction of a user of the service (ITU-T E.800). The set of performance parameters that can be directly observed and measured at the point at which the service is accessed by the user. There are three criteria by which performance is measured: speed, accuracy and dependability (source: ITU-T I.350).

R

reference configuration: A combination of functional groups and reference points that shows possible network arrangements (source: GSM 01.04, ITU-T I.112).

reference point: A conceptual point at the conjunction of two non-overlapping functional groups (source: GSM 01.04, ITU-T I.112).

retrieval service: An interactive service which provides the capability of accessing information stored in data base centres. The information will be sent to the user on demand only. The information is retrieved on an individual basis, i.e., the time at which an information sequence is to start is under the control of the user (source ITU-T I.113).

S

SDU loss probability: The ratio of total lost service data units (SDUs) to total transmitted service data units in a specified sample (source: ITU-T X.140).

NOTE:	the source document term "user information unit" has been replaced by the term "service data unit".

NOTE:	the source document term "user information unit" has been replaced by the term "service data unit".

SDU transfer delay: The value of elapsed time between the start of transfer and successful transfer of a specified service data unit (SDU) (source: ITU-T X.140).

NOTE:	the source document term "user information unit" has been replaced by the term "service data unit".

SDU transfer rate: The total number of successfully transferred service data units (SDUs) in a transfer sample divided by the input/output time for that sample. The input/output time is the larger of the input time or the output time for the sample (source: ITU-T X.140).

NOTE:	the source document term "user information unit" has been replaced by the term "service data unit".

service access point (SAP): In the reference model for OSI, the points through which services are offered to an adjacent higher layer (source: GSM 01.04, ITU-T X.200 / ISO-IEC 7498-1).

service attribute: A specified characteristic of a telecommunication service (source: ITU-T I.112).

NOTE:	the value(s) assigned to one or more service attributes may be used to distinguish that telecommunications service from others.

service bit rate: The bit rate that is available to a user for the transfer of user information (source: ITU-T I.113).

service category or service class: A service offered to the users described by a set of performance parameters and their specified values, limits or ranges. The set of parameters provides a comprehensive description of the service capability.

service data unit (SDU): In the reference model for OSI, an amount of information whose identity is preserved when transferred between peer (N+1)-layer entities and which is not interpreted by the supporting (N)-layer entities (source: ITU-T X.200 / ISO-IEC 7498-1).

service delay: The time elapsed from the invocation of the service request, to the corresponding service request indication at the Service Receiver, indicating the arrival of application data.

service model: A general characterisation of services based upon a QoS paradigm, without specifying the actual performance targets.

service provider: Entity which offers the General Packet Radio Service (GPRS) for subscription. The network operator may be the service provider.

service receiver: The entity which receives the service request indication primitive, containing the SDU.

service request: This is defined as being one invocation of the service through a service request primitive.

service requester: The entity which requests the initiation of a GPRS operation, through a service request.

service subscriber: Entity which subscribes to the General Packet Radio Service (GPRS) service.

signalling: The exchange of information specifically concerned with the establishment and control of connections, and with management, in a telecommunications network (source: ITU-T I.112).

speed: A performance criterion that describes the time interval required to perform a function or the rate at which the function is performed. (The function may or may not be performed with the desired accuracy.) (source: ITU-T I.350).

supplementary service: A service that modifies or supplements a basic telecommunication service (as defined in GSM 01.04). It cannot be offered to a customer as a stand-alone service. It must be offered together or in association with a basic telecommunication service (bearer or teleservice). The same supplementary service may be common to a number of telecommunication services (source: ITU-T I.210).

T

telecommunication service: That which is offered by a PLMN operator or service provider to its customers in order to satisfy a specific telecommunication requirement. (source: GSM 01.04, ITU-T I.112). Telecommunication services are divided into two broad families: bearer services and teleservices (source: ITU-T I.210).

teleservice: A type of telecommunication service that provides the complete capability, including terminal equipment functions, for communication between users according to protocols established by agreement between Administrations (source: GSM 01.04, ITU-T I.112).

terminal equipment: Equipment that provides the functions necessary for the operation of the access protocols by the user (source: GSM 01.04). A functional group on the user side of a user-network interface (source: ITU-T I.112).

throughput: A parameter describing service speed. The number of data bits successfully transferred in one direction between specified reference points per unit time (source: ITU-T I.113).

transit delay: A parameter describing service speed. The time difference between the instant at which the first bit of a protocol data unit (PDU) crosses one designated boundary (reference point), and the instant at which the last bit of the PDU crosses a second designated boundary (source: ITU-T I.113).

U

user access or user network access: The means by which a user is connected to a telecommunication network in order to use the services and/or facilities of that network (source: GSM 01.04, ITU-T I.112).

user-network interface: The interface between the terminal equipment and a network termination at which interface the access protocols apply (source: ITU-T I.112).

user-user protocol: A protocol that is adopted between two or more users in order to ensure communication between them (source: ITU-T I.112).

V

variable bit rate service: A type of telecommunication service characterised by a service bit rate specified by statistically expressed parameters which allow the bit rate to vary within defined limits (source: ITU-T I.113).

## 3.2	Abbreviations

For the purposes of this TS the following definitions apply:

BOC	Bell Operating Company

CLNP	Connectionless network protocol

CLNS	Connectionless network service

CONS	Connection-oriented network service

EGPRS	Enhanced GPRS

FFS	For further study

GPRS	General packet radio service

LATA	Local Access and Transport Area

IP	Internet protocol

IP-M	Internet protocol multicast

NSDU	Network service data unit

PDP	Packet data protocol

PDU	Protocol data unit

PLMN	Public land mobile network

PTM	Point to multipoint

PTM-G	Point to multipoint – group call

PTP	Point to point

QoS	Quality of service

SAP	Service access point

SVC	Switched virtual circuit

SDU	Service data unit

TBD	To be defined

TLLI	Temporary link level identity

# Main Concepts

The GPRS allows the service subscriber to send and receive data in an end-to-end packet transfer mode, without utilizing network resources in circuit switched mode.

GPRS enables the cost effective and efficient use of network resources for packet mode data applications e.g. for applications that exhibit one or more of the following characteristics:

-	intermittent, non-periodic (i.e., bursty) data transmissions, where the time between successive transmissions greatly exceeds the average transfer delay;

-	frequent transmissions of small volumes of data, for example transactions consisting of less than 500 octets of data occurring at a rate of up to several transactions per minute;

-	infrequent transmission of larger volumes of data, for example transactions consisting of several kilobytes of data occurring at a rate of up to several transactions per hour.

Within the GPRS, two different bearer service types are defined. These are:

-	Point-To-Point (PTP);

-	Point-To-Multipoint (PTM).

Based on standardized network protocols supported by the GPRS bearer services, a GPRS network administration may offer (or support) a set of additional services. This is outside the scope of this specification, however, a number of possible PTP interactive teleservices include:

-	retrieval services which provide the capability of accessing information stored in data base centres. The information is sent to the user on demand only. An example of one such service in the Internet's World Wide Web (WWW);

-	messaging services which offer user-to-user communication between individual users via storage units with store-and-forward mailbox, and/or message handling (e.g., information editing, processing and conversion) functions;

-	conversational services which provide bi-directional communication by means of real-time (no store-and-forward) end-to-end information transfer from user to user. An example of such a service is the Internet's Telnet application;

-	tele-action services which are characterized by low data-volume (short) transactions, for example credit card validations, lottery transactions, utility meter readings and electronic monitoring and surveillance systems.

Some examples of teleservices which may be supported by a PTM bearer service include:

-	distribution services which are characterized by the unidirectional flow of information from a given point in the network to other (multiple) locations. Examples may include news, weather and traffic reports, as well as product or service advertisements;

-	dispatching services which are characterized by the bi-directional flow of information from a given point in the network (dispatcher) and other (multiple) users. Examples include taxi and public utility fleet services;

-	conferencing services which provide multi-directional communication by means of real-time (no store-and-forward) information transfer between multiple users.

Capabilities that may be offered together with the PTM bearer services include:

-	geographical routeing capability, which provides the ability to restrict message distribution to a specified geographical area;

-	scheduled delivery capability, allowing store-and-forward type services to specify a future delivery time and a repetition rate.

It is possible to include these capabilities as part of the service request (i.e., as part of the packet). Some operators may offer PTM services only together with these capabilities.

EGPRS is an enhancement of GPRS allowing higher data rates on the radio interface. The same set of services provided by GPRS is available in EGPRS.

# 5	Service Requirements

## 5.1	GPRS reference model

The GPRS can be described using the simplified reference model in figure 1.

GPRS provides data transfer capabilities between a sending entity and one or more receiving entities. These entities may be an MS or a Terminal Equipment, the latter being attached either to a GPRS network or to an external data network. The base station provides radio channel access for MSs to the GPRS network. User data may be transferred between four types of MSs as described in TS22.001 [1] and GSM 04.02 [2]:

Figure 1: GPRS simplified reference model

### 5.1.1	Bearer services

The GPRS bearer services support the capability to transmit data between user-network access points (2, 4 and 7 in figure 2).

### 5.1.2	Teleservices

GPRS supports teleservices to provide complete communication service capabilities including the terminal equipment functions necessary for communication between network customers using the user-user protocols supported by the GPRS service provider(s) (see also TS 22.001 [1]).

Figure 2: Telecommunication services access points

### 5.1.3	GPRS Access Points

Considering the reference configuration in figure 1, users can access various telecommunication services at different access points as shown in figure 2:

-	at access points 1, 3, 5, and 6 (user-to-terminal interfaces), teleservices are accessed - the teleservice concept includes the terminal capabilities;

-	at access point 2 (the R reference point), non-ISDN bearer services may be accessed, for example ITU-T X- and V-Series Recommendations;

-	at access point 4 (the S reference point), ISDN compatible bearer services may be accessed;

at access point 7 external data network bearer services may be accessed;

-	access point 8 (the Gi reference point, which may not be identical for direct access and external data network access) provides bearer service access for direct access for a data terminal and/or for external data networks, for example X.25 networks.

## Network Interworking

Network interworking is required whenever a PLMN and a non-PLMN are involved in the execution of a GPRS Service request.

In general the mobile user of a GPRS network will receive and experience all the services provided by an external data network. In this case the external data network refers to the network that the GPRS PLMN interworks with as determined by the network operator. With this in mind it can be said that a user :-

1)	will require a universal identity(ies) of the form compatible with the interworked with network(s);

2)	and experience access to and provision of all services as offered by the interworked with networks (some reduction of capability may result from unavoidable restrictions due to the complexity of implementation).

### 5.2.1	Interworking with other data networks and other PLMNs

GPRS shall provide means to interwork with external data networks. The GPRS operator may provide an appropriate address to the external data network for the subscriber as part of the GPRS subscription. That address can be either dynamic (e.g. the user's IP address is allocated from a pool of unused IP addresses every time the subscriber activates the access to an IP network) or static (e.g. a certain IP address is permanently allocated to a particular subscriber). In addition, the GPRS network shall be able to notify  the server IP address (e.g. the gateway IP address)  to the subscriber, when the subscriber activates. When connected with some external data networks, the routeing protocols of these networks may limit the data network addresses that can be allocated. For example, when interworking with IP networks, the IP address for the GPRS subscriber shall belong to that GPRS operator's IP subnetwork that allocates the address. In the case of a simple point to point connection, a GPRS subscriber need not have an associated network address.

The type of interworking between a PLMN and data networks is determined by the network operator.

Interworking with the following types of data networks shall be defined:

-	X.25 [3] networks; (via X.75 [12]or X.75’ interfaces). Note: In the US, X.75’ interface is used for interworking with BOC’s data networks for intra-LATA packet data calls.

-	IP networks;

-	other PLMNs, directly or via a transit network;

-	other fixed networks (e.g. PSTN, ISDN);

The MS should interwork with the X.25 network using standardized X.3 [8], X.28 [9] and X.29 [12] mechanisms for asynchronous access and X.25 mechanisms for synchronous access.

#### 5.2.1.1	QoS when Interworking

GPRS shall satisfy, within the constraints introduced by the mobile radio environment, the QoS requirements of the interworked-with network.

### 5.2.2	GPRS Numbering Plan

It is required that GPRS MS addresses conform to the numbering plan already defined for GSM.

### 5.2.3	Addressing and routeing requirements

#### 5.2.3.1	PTP network layer services

A GPRS subscriber identified by an IMSI, may have a network layer address(es) temporarily and/or permanently associated with it that conforms to the standard addressing scheme of the respective network layer service used, e.g.:

-	E.164 number;

-	X.121 address [12] for X.25 CONS;

-	IP Version 4 and 6 addresses for Internet CLNS.

In the case of a simple point to point connection, a GPRS subscriber need not have an associated network layer address.

Routeing Information for Multipoint services is a combination of geographical area information and IMSI.

### 5.2.4	Interworking for subscriber roaming

Interworking between different GPRS PLMNs is required in order to support subscriber roaming.

It shall be possible for the VPLMN to provide access to the external data network when the external non-GSM data network address is dynamically assigned by the VPLMN.

## 5.3	GPRS service description

There are two categories of GPRS services:

-	Point to Point (PTP) services,

-	Point to Multipoint (PTM) services.

The PTP service provides a transmission of one or more packets between two users, initiated by a service requester and received by a receiver.

There are two PTP services:

-	PTP Connectionless Network Service (PTP-CLNS);

-	PTP Connection Orientated Network Service (PTP-CONS).

The PTM service provides a transmission of packets between a service requester and a receiver group.

There are two PTM services:

-	PTM Group Call (PTM-G);

-	IP Multicast (IP-M).

For PTM-G the data transmission is restricted to the members of a receiver group currently located within a geographical area. Both the receiver group and the geographical area are specified by the service requester.

The geographical area addressing mechanism is not applicable to IP-M.

An invocation of the service request by a service requester is possible from the fixed and mobile access points. Invocation and operation procedures (i.e., parallel sessions) are described in subclause 7.9.

Table 1 presents the relationship between service requests and the Service Requester/Receiver.

Table 1: Relationship of service request and service requester/receiver

| Service requester/receiver | Types of service request |  |  |  |
| --- | --- | --- | --- | --- |
| AP = Access Point (see note 1) | PTP-CONS and PTP-CLNS |  | PTM-G | IP-M |
| From fixed AP to mobile AP | Supported |  | Supported | Supported |
| From mobile AP to mobile AP (see note 3) | Supported |  | Supported | Supported |
| From mobile AP to fixed AP | Supported |  | Supported (limited, see note 2) | Supported |
| NOTE 1: Mobile bearer services access points are 2 and 4 from figure 2. Fixed bearer service access points are 7 and 8. NOTE 2: All PTM-G features may not be supported for fixed AP, e.g., paging. NOTE 3: It shall be possible to transfer data between two mobiles of the same operator without the use of external data networks. |  |  |  |  |

### 5.3.1	Point-To-Point Connectionless Network Service (PTP-CLNS)

Point-To-Point Connectionless Network Service (PTP-CLNS) is a service in which one (or more) single packet(s) is (are) sent from a single service subscriber "A" to a single destination user "B". Each packet is independent of the preceding and succeeding packet. This service is of the datagram type and is intended to support bursty applications. On the radio interface PTP-CLNS supports the acknowledged transfer mode for reliable delivery.

GPRS provides data communications services consistent with the connectionless network layer service definition ISO 8348 [5]. Specifically, GPRS shall be able to support applications based on the following network layer protocol:

-	Internet Protocol (IP);

-	IP is the network layer protocol of the Internet TCP/IP protocol suite defined by Internet STD 5 [7].

### 5.3.2	Point-To-Point Connection Orientated Network Service (PTP-CONS)

Point-To-Point Connection Orientated Network Service (PTP-CONS) is a service in which multiple packets are sent between a single service subscriber "A" and a single destination user "B". This service provides a logical relation between the users which may last between seconds to hours. The service is intended to support bursty transactive or interactive applications. On the radio interface PTP-CONS utilises the acknowledged transfer mode for reliable delivery.

GPRS provides data communications services consistent with the connection orientated network layer service definition ISO 8348 [5] and ISO 8878 [6]. This means that GPRS shall be able to support applications based on the following network layer protocol:

-	Connection Orientated Network Protocol (CONP).

-	CONP is the standard OSI connection orientated network protocol defined in ISO 8208 [4], i.e., the X.25 Packet Level Protocol for Data Terminal Equipment.

GPRS should provide the ability to maintain a connection orientated virtual circuit upon change of cell within a PLMN.

Any established SVC shall be released on irrecoverable failure of the radio data link.

### 5.3.3	Point-To-Multipoint (PTM) service overview

### A PTM service is the transmission of a single message to multiple subscribers. The GPRS PTM services provide the capability for a subscriber to send data to multiple destinations with a single service request. Transmission shall be performed within the specified service delay, over one or more, geographical areas as defined by the service requester. Repeated transmission shall be performed where applicable according to the schedule as defined/negotiated by the service requester.

There are two PTM services:

- PTM - Group Call (PTM-G) in which messages are transmitted to a specified receiver group.

- -	IP Multicast (IP-M), which is a service defined as a part of the IP protocol suite. In IP-M messages are sent between participants of an IP-M group. An IP-M group can be internal to the PLMN or distributed across the Internet. Participation and data transfer in the IP-M service shall be compliant with the Internet protocols. See RFC 1920 [7], RFC 1458 [7], RFC 1301 [7] and RFC 1112 [7].

- The Service Receiver of a point-to-multipoint service request shall be able to filter out packets at a network level, through use of the Packet Identities, which are of no interest either because they are for a service for which no subscription is held, or the packet belongs to a sub-group within the offered application service which is of no interest. It is required that the MS-Application resources shall not be utilized for this function.

Table 2: PTM Service Characteristics

| Characteristics |  | PTM-G | IP-M |
| --- | --- | --- | --- |
| Primary addressing mechanism |  | specified group of subscribers | specified group of subscribers |
| Secondary addressing mechanism |  | no | no |
| Present subscribers known |  | yes, group members shall join the PTM-G call to become participants | yes, group members shall join the IP-M call to become participants. |
| Delivery time |  | real time | real time |
| Direction of transmission |  | uni-directional bi-directional multi-directional | multi-directional |
| Reliable delivery |  | optional | acknowledged as normal IP PTP packets |
| Ciphering |  | yes | yes |

#### 5.3.3.1	Identification of subscribers and subscriber groups in PTM

#### For reasons of privacy and service management, the service requester shall be able to route PTM messages to a subset of all GPRS subscribers. This enables, for example, service providers to distinguish their application messages from the messages of other service providers using the same shared resource. Adequate protection of the storage on the SIM and for the transmission over the radio interface is required. The IMSI shall be used for authentication of the individual subscriber.

Similarly, the service receiver shall only receive, and use its resources to process messages from the underlying GPRS application to which it has subscribed.

In the following subclauses describing the PTM services references are made to "calling ID" and "called ID". Each of these terms indicate different forms of the same identity depending on "who" the receiver of the information is. For example a "calling ID" given to the network is the IMSI but the "calling ID" given to the called party is a user defined, human readable ID.

5.3.3.2	PTM Group Call (PTM-G) Description

PTM Group Call is a service in which the messages are transmitted to a specific subset of subscribers (PTM group participants). The following characteristics define the PTM Group Call service:

- Messages sent from the PTM-G service are addressed to individual participants in a group call.

- Messages sent from individual participants are addressed to the PTM-G service group call ID.

-	The network may not have knowledge of which participants have received the message.

-	Message delivery to each participant is in real-time according to the negotiated QoS,

-	Message transfer is by default one-way (unidirectional), but may optionally be two-way (bi-directional), or n-way (multi-directional). Two-way data transfer enables the group controller or initiator to send messages to the entire group (one to many), while other participants may send messages only to the group controller or initiator (many to one). N-way data transfer allows all participants to send and receive messages (many to many), as in a (voice) multiparty call.

-	Message reception may be unreliable, in that a particular participant is not guaranteed to correctly receive all PTM-G transmissions. Reliable delivery may be optionally provided per group call, whereby individual participants are able to request the retransmission of lost or incorrectly received messages.

-	Messages are transmitted or received only by group members which have explicitly joined the call by registering with the network.

-	Messages are transmitted with confidentiality, that is data ciphering is performed prior to transmission, and only group members which have joined the call are able to cipher and decipher the transmitted data.

The receiver receives the user data, calling ID and called group ID. No acknowledge is required.

Figure 3: PTM-M data transfer operation

#### The receiver (of the data transfer operation) is not involved in the cancel operation.

Figure 4: PTM-M data cancel operation

#### 5.

#### The receiver (of the data transfer operation) is not involved in the status operation.

Figure 5: PTM-M data status operation

The network may also initiate this operation (without a status request from the user) by using the PTM-M-Data-Status-Indication. This shall be used to inform the initiating service user of a significant change in the status of an on-going data-transfer-operation (e.g., the network has "aborted" the data-transfer-operation due to adverse network conditions).

Figure 6: PTM-M network invoked data status operation

### 5.3.4	PTM-Group-Call Operations and User Interface

The PTM-G service allows members of a PTM-G group to establish a logical relationship within the group for the purposes of transferring data; this relationship is known as a group call. Not all members of the PTM-G group need to participate in a group call. The group membership defines those users which are permitted to participate in the group call. Individual group members may join or leave the call at their discretion. A group member may participate simultaneously in multiple group calls of any group to which the member has a subscription.

#### 5.3.4.1	PTM Group Administration

A PTM-G group is identified by a group call ID. The PTM-G group database defines the group capabilities and the set of GPRS subscribers belonging to the group. A PTM-G group is created by administrative procedures specific to the individual PLMNs. PTM-G group membership is modified by administrative procedures.

Individual group members (subscribers) may be assigned different subscription options within the group. A group member shall have one or more of the following capabilities:

-	as a controller, authorized to create and delete the group and to control the membership;

-	as an initiator, authorized to initiate and terminate group calls, and to send and receive data to other group members;

-	as a participant, authorized to send or receive data to other group members.

PTM-G groups may be classified as either open or closed:

-	a closed group is a PTM-G group which has a set of members pre-defined by subscription. Only the pre-defined group members are authorized to join and participate in PTM-G communications with other participants;

-	an open group is a PTM-G group which does not have a pre-defined set of members. Any subscriber may attempt to participate in an open group call.

An initiator initiates a group call. Group members which have joined a group call including the initiator are termed participants.

In general, group membership consists primarily of mobile subscribers. However each GPRS group may contain one or more fixed network users, communicating with the network across the Gi reference point.

For each PTM group, the following group attributes may be configured within the network:

| join/leave call indication: | active or not-active |
| --- | --- |
| Group data transfer mode: | one-way, two-way or n-way |
| Group membership class: | open or closed |
| Group restrictions: | time-of-day, data transfer limits, quality of service class, or other network-specific restrictions. |

For each PTM group, the following individual member attributes may be configured within the network:

| Member privileges: | controller, initiator, participant |
| --- | --- |
| Member type: | mobile subscriber or fixed subscriber |
| Member restrictions: | time-of-day, data transfer limits, send or receive only, quality of service class, or other network-specific restrictions. |

#### 5.3.4.2	PTM Group Call service description

The following six operations are supported by the PTM-G service:

-	call initiate operation;

-	call terminate operation;

-	call status operation;

-	join call operation;

-	leave call operation;

-	data transfer operation.

A group call initiator initiates a group call by means of a call initiate operation. The network assigns a group call ID and confirms to the initiator that the call has been created. A group member may choose to join a group call and thereby become a group call participant, by informing the network using the join call operation. A participant may leave a group call at any time by informing the network using the leave call operation. Data transfer operations may occur at any time following group call initiation. Data transfers are distributed only to the participants including the initiator.

The initiator may terminate a group call by means of the call terminate operation. The network notifies participants of the group call termination.

Group members may query the status of a group call at any time using the call status operation.

The call status operation may also be used to request a list of group calls currently active at a PTM-G service provisioning point.

#### 5.3.4.3	PTM-G call initiate operation

This operation is used by a group call initiator to initiate a PTM-G call and establish the call characteristics.

Call attributes shall include:

-	the initiator´s member ID;

-	the data transfer mode (one-way, two-way, n-way);

-	the QoS associated with data transfers to and from the initiator.

Call attributes may include:

-	the (optional) join/leave call indication of the participants ID to the initiator;

The network shall respond by confirming the operation. If successful, the network shall provide a unique call ID which shall be used in subsequent PTM-G operations. It shall be possible for a group call initiator to initiate or participate in more than one group call simultaneously.

|  | Service Requester : initiator |  |  |  |  |  | Service Receiver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PTM-G-Call-Initiate-Request - member ID - data transfer mode - QoS - - (optional) join/leave call ind. (act/de-act) | ----- | ---> | PLMN Domain supporting GPRS |  |  |  |  |
|  | PTM-G-Call-Initiate-Confirm call ID - result code | <--- | ----- |  | ----- ----- . . ----- | ---> ---> . . ---> | ) Not involved |  |

Figure 3: PTM-G call initiate operation

#### 5.3.4.4	PTM-G call terminate operation

This operation is used by a group call initiator to terminate a group call. The network shall respond by confirming the operation. The network shall inform participants of the termination.

|  | Service Requester : initiator |  |  |  |  |  | Service Receiver : participant |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PTM-G-Call-Terminate-Request - member ID - call ID | ----- | ---> | PLMN Domain |  |  |  |  |
|  | PTM-G-Call-Terminate-Confirm - call ID - result code | <--- | ----- | supporting GPRS | ----- | ---> | PTM-G-Call-Terminate-Indication - call ID |  |

Figure 4: PTM-G call terminate operation

The network may invoke the call terminate operation due to administration reasons (e.g. if the initiator is no longer reachable).

|  | Service Requester |  |  |  |  |  | Service Receiver : initiator : participant |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Not involved |  |  | PLMN Domain supporting GPRS | ----- | ---> | PTM-G-Call-Terminate-Indication - call ID |  |

Figure 5: PTM-G network invoked call terminate operation

#### 5.3.4.5	PTM-G call status operation

This operation is used by the group members to obtain status information on all current group calls of which they are members. The interrogating group member may restrict the status request to one or more specific group calls by including the call IDs in the status request operation. The interrogating group member may be currently active in a group call. The network shall respond to the individual interrogating group member indicating the call status information.

The call status information shall include:

-	call active or not-active;

-	join/leave call indication active or not-active.

The service requestor may request and the network may optionally return the following additional information in the call status information:

-	a list of the current participants;

-	the number of current participants;

|  | Service Requester : member |  |  |  |  |  | Service Receiver |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PTM-G-Call-Status-Request - member ID - call ID - (optional) additional status information required | ----- | ---> | PLMN Domain |  |  |  |  |
|  | PTM-G-Call-Status-Confirm - call ID - result code - status information | <--- | ----- | supporting GPRS |  |  | Not involved |  |

Figure 6: PTM-G call status operation

The network may invoke the PTM-G-Call-Status-Indication operation to inform the controller, initiator, or participants of a call status (e.g. call termination due to network administration reasons).

|  | Service Requester |  |  |  |  |  | Service Receiver : member |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Not involved |  |  | PLMN Domain supporting GPRS | ----- | ---> | PTM-G-Call-Status-Indication - call ID - result code - status information |  |

Figure 7: PTM-G network invoked call status operation

#### 5.3.4.6	PTM-G join call operation

This operation is used by a group member to indicate the intention to join a PTM-G call. The user provides the call ID of the group call which the user wishes to participate in. The member ID and the call ID are transferred to the network. If there is a group call established with the specified call ID and the subscriber is authorized to join the group call, then the network responds with the call ID. The network shall indicate the member ID of each group member who joins the call to the call initiator if the "join/leave call indication" option is active. A group member which has successfully completed this operation is said to be a participant in the PTM-G call. It shall be possible for a group member to be a participant in more than one group call simultaneously. The join call operation is not applicable to the initiator of the group call.

|  | Service Requester : member |  |  |  |  |  | Service Receiver : initiator |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PTM-G-Join-Call-Request - member ID - call ID | ----- | ---> | PLMN Domain |  |  |  |  |
|  | PTM-G-Join-Call-Confirm - call ID - result code | <--- | ----- | supporting GPRS | ----- | ---> | PTM-G-Join/leave-Call-Indication (optional) - member ID - call ID |  |

Figure 8: PTM-G join call operation

#### 5.3.4.7	PTM-G leave call operation

This operation is used by a group call participant to indicate their intention to leave a PTM-G call. The participant provides the call ID of the group call which the user wishes to leave.

A group member which has successfully completed this operation is no longer a participant in the PTM-G call. This operation allows the network to remove the member from the list of participants. The network shall indicate the member ID of each participant who leaves the call to the call initiator if the "join/leave call indication" option is active.

The leave call operation is not applicable to the initiator of the group call.

|  | Service Requester : participant |  |  |  |  |  | Service Receiver : initiator : participant |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PTM-G-Leave-Call-Request - member ID - call ID | ----- | ---> | PLMN Domain |  |  |  |  |
|  | PTM-G-Leave-Call-Confirm - call ID - result code | <--- | ----- | supporting GPRS | ----- | ---> | PTM-G-Join/leave-Call-Indication (optional) - member ID - call ID |  |

Figure 9: PTM-G leave call operation

Figure 14: PTM-G location invoked leave call operation

#### 5.3.4.8	PTM-G data transfer operation

The service user performs this operation to transfer the user data to the participant(s) by informing the network of:

-	the service requestor´s member identity;

-	the call ID to which the data is addressed;

- the user data to be transmitted;

- the (optional) member ID to whom the message is to be transmitted.

The network shall either confirm the acceptance of the request or shall reject the request. If the request is accepted the network transfers the user data according to the request parameters.

In response to a user data transfer the service receiver may optionally send an acknowledgement.

|  | Service Requester : initiator : participant |  |  |  |  |  | Service Receiver : initiator : participant(s) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PTM-G-Data-Transfer-Request - member ID - call IDuser data | ----- | ---> | PLMN Domain |  |  |  |  |
|  | PTM-G-Data-Transfer-Confirm - call ID - result code | <--- | ----- | supporting GPRS | ----- <--- | ---> ----- | PTM-G-Data-Transfer-Indication - member ID - call ID - user data - (optional) acknowledge |  |

Figure 10: PTM-G data transfer operation

## 5.4	Service interworking

It shall be possible for the GPRS PTP services to be utilized as a bearer service for the SMS-MO and SMS-MT services.

## 5.5	Transfer Characteristics

The packet multiplexing mechanisms developed for GPRS shall be independent of a given channel type. It shall be possible to operate GPRS over low and high capacity channels. These may be existing and/or future channels whose capacities are as yet unspecified (e.g., a signalling channel, or two time-slots or an entire 200 kHz carrier). Table 3 is only applicable to the GPRS part of the connection.

Table 3: Transfer Characteristics

| Attributes |  | Supported capabilities |
| --- | --- | --- |
| Information Transfer Capability |  | Unrestricted digital information |
| Information | Connection mode | Connection orientated, connectionless |
| Transfer | Traffic type (a) | Variable bit rate and variable delay |
| Mode | Timing end-to-end (b) | Asynchronous |
| Information Transfer Rate (c) |  | Maximum bit rate. |
| Information Structure |  | - Service data unit integrity - Data sequence integrity |
| Communication Configuration |  | - Point to point - Point to multipoint group call |
| Establishment of Communication (d) |  | Demand mobile originated or mobile terminated |
| Negotiation |  | - Out of band - In band |
| Symmetry |  | - Bi-directional - Unidirectional |
| Radio Channel assignment |  | Shared, multi-user |

a)	Traffic type: describing data streams with constant bit rate or variable bit rate characteristics.

b)	Timing end-to-end: describing the timing relation between the source and destination of signals.

c)	Flexible channels with a maximum bit rate that depends on the type of channel and the number of time slots that are used. This capability defines the maximum capability associated with the channel(s). The actual bit rate for a particular user may be any value up to this maximum value.

d)	Establishment of communication: may be on demand, reserved or permanent.

## 5.6	Service characteristics

### 5.6.1	Subscriber profile

The subscriber profile holds subscription information about services and other parameters that have been assigned for an agreed contractual period. It includes the following information:

-	subscribed services (PTP-CLNS, PTP-CONS, PTM-G);

-	subscribed QoS profile (service precedence(priority), reliability, delay, throughput).

An invocation of a service by any user with the appropriate subscription profile shall be possible.

It shall be possible to validate a service request against a service subscriber's subscription profile.

### 5.6.2	Quality of Service (QoS)

The mapping of user application QoS parameters to GPRS QoS parameters is an implementation issue and is not part of GPRS specifications.

It shall be possible to respond to local data traffic conditions adaptively. GPRS shall include the functionality to increase or decrease the amount of radio resources allocated to GPRS on a dynamic basis. The criteria used to decide on dynamic changes of the GPRS part of the radio resource should not be specified. Thus, only the necessary procedure, including radio protocol and timers, needed to perform the change of radio resources shall be specified within the ETSI specifications.

Within GPRS the dynamic allocation of the radio resource for bursty or lengthy file transfer applications shall be such that it can be controlled by the network operator.

#### 5.6.2.1	QoS parameter definitions

The defined QoS parameter values, assume the user is at a location with acceptable -coverage and refer to and are valid for normal network operating conditions or, as in the case of the service precedence parameter, regulate how the network shall handle abnormal conditions.

#### 5.6.2.1.1	Service precedence (priority)

The service precedence indicates the relative priority of maintaining the service. For example under abnormal conditions (e.g. network congestion) packets which may be discarded can be identified. The following precedence levels are defined:

-	High precedence: Service commitments will be maintained ahead of all other precedence levels.

-	Normal precedence: Service commitments will be maintained ahead of low priority users.

-	Low precedence: Service commitments will be maintained after the high and normal priority commitments have been fulfilled.

#### 5.6.2.1.2	Reliability

The reliability parameter indicates the transmission characteristics that are required by an application. The reliability class defines the probability of loss of, duplication of, mis-sequencing of  or corruption of SDUs.

NOTE. 	If a user’s application is based on X.25, which requires reliable delivery of X.25 SDUs, it would be incorrect for an application to use an reliability class with a high loss probability.

Table 4 lists the three classes of the data reliability.

Table 4: Reliability classes

| Reliability class | Lost SDU probability (a) | Duplicate SDU probability | Out of Sequence SDU probability | Corrupt SDU probability (b) | Example of application characteristics. |
| --- | --- | --- | --- | --- | --- |
| 1 | 10-9 | 10-9 | 10-9 | 10-9 | Error sensitive, no error correction capability, limited error tolerance capability. |
| 2 | 10-4 | 10-5 | 10-5 | 10-6 | Error sensitive, limited error correction capability, good error tolerance capability. |
| 3 | 10-2 | 10-5 | 10-5 | 10-2 | Not error sensitive, error correction capability and/or very good error tolerance capability. |

a)	To protect against buffer overflow or a protocol malfunction, there is a maximum holding time for each SDU in the GPRS network after which the SDU is discarded. The maximum holding time depends on the protocols used (e.g., TCP/IP).

b)	Corrupt SDU probability: the probability that a SDU will be delivered to the user with an undetected error.

#### 5.6.2.1.3	Delay

GPRS is not a „store and forward“ service - although data is temporarily stored at network nodes during transmission - thus, any delay incurred is due to technical transmission characteristics (or limitations) of the system and is to be minimised for a particular delay class. The delay parameter thus defines the maximum values for the mean delay and 95-percentile delay to be incurred by the transfer of data through the GPRS network(s). The delay parameter defines the end-to-end transfer delay incurred in the transmission of SDUs through the GPRS network(s).

This includes the radio channel access delay (on uplink) or radio channel scheduling delay (on downlink), the radio channel transit delay (uplink and/or downlink paths) and the GPRS-network transit delay (multiple hops). It does not include transfer delays in external networks.

Delay is measured between the R or S (for MS) and Gi (for FS) reference points when applied to "MS to fixed station (FS)" or  "FS to MS" transmissions.

Table 5: Delay classes

|  | Delay (maximum values) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SDU size: 128 octets |  | SDU size: 1024 octets |  |  |
| Delay Class | Mean Transfer Delay (sec) | 95 percentile Delay (sec) |  | Mean Transfer Delay (sec) | 95 percentile Delay (sec) |
| 1. (Predictive) | < 0.5 | < 1.5 |  | < 2 | < 7 |
| 2. (Predictive) | < 5 | < 25 |  | < 15 | < 75 |
| 3. (Predictive) | < 50 | < 250 |  | < 75 | < 375 |
| 4. (Best Effort) | Unspecified |  |  |  |  |

#### 5.6.2.1.4	Throughput

The throughput parameter indicates the user data throughput requested by the user.

Throughput is defined by two negotiable parameters:

-	Maximum bit rate

-	Mean bit rate (includes, for example for "bursty" transmissions, the periods in which no data is transmitted.)

The maximum and mean bit rates can be negotiated to a value up to the Information Transfer Rate value (see table 3).

It shall be possible for the network to re-negotiate the throughput parameters at any time during a session.

#### 5.6.2.2	QoS profile - PTP and PTM-G

The subscriber´s QoS profile for the PTP and PTM-G services consists of the following parameters which are negotiated or set to default values:

-	service precedence (priority),

-	reliability,

-	delay,

-	user data throughput.

#### 5.6.2.3	Monitor

As an option, the MS may monitor the current QoS level. If this option is implemented, the following information shall be monitored:

-	user data throughput;

-	radio channel access delay;

-	round-trip-MS/PLMN delay; and

-	reliability,

The MS shall provide means of communicating this information to the application(s) via the R and S reference points as shown in Figure 2.

### Table 6: Protection of user identity and user data

| PTM-Multicast (receiver) | Yes a) |  |
| --- | --- | --- |

a)	The individual identities of the group members that actually receive the PTM-M traffic, are not transferred on the radio path and furthermore are also not known to the network. This is an important aspect for those applications where it is imperative that the location of the user cannot under any circumstances be traced. However, the group identity and the identity of the service requester are sent unciphered on the radio path.

b)	This does not preclude end-to-end ciphering of user data by the PTM-M application, this however, is outside the scope of this specification.

Security mechanisms available for existing teleservices and bearer services should be used if possible.

### 5.6.3	Packet size

Both PTP and PTM services shall allow the transfer of variable length Network Service Data Units (NSDU).

# 6	Operational Requirements

## 6.1	GPRS MS Modes of Operation

The purpose of the definition of the GPRS MS Classes is to enable the different needs of the various market segments to be satisfied by a number of MS types with distinct capabilities (e.g., simultaneous use and number of time-slots) .A means shall be provided to indicate the multi-slot capability and current configuration to the network when necessary.

Three GPRS MS modes of operationare identified:

NOTE 1:	The term simultaneous (attach, traffic, etc.) is the requirement to simultaneously support GSM GPRS services and GSM circuit switched services including SMS.

Class A:	The MS is attached to both GPRS and other GSM services. The MS supports simultaneous attach, simultaneous activation, simultaneous monitor, simultaneous invocation and simultaneous traffic. The mobile user can make and/or receive calls on the two services simultaneously subject to the QoS requirements.

A minimum of one time slot shall be available for each type of service (circuit switched and GPRS) when required.

Class B:	 The MS is attached to both GPRS and other GSM services, but the MS can only operate one set of services at a time. When the MS is in both idle mode and packet idle mode it should be able to monitor paging channels for both circuit-switched and packet-switched services depending on the mode of network operation.

At least one mode of network operation shall be defined so that when an MS is in both idle mode and packet idle mode it shall be able to respond to paging for both circuit-switched and packet-switched services. A mode of network operation where the network performs the paging for circuit-switched and packet-switched services on different paging channels is also defined. In such case an MS in both idle mode and packet idle mode should either attempt to listen to both paging channels with priority for the circuit-switched service or revert to class-C mode of operation.

If in a mode of network operation the network performs both the paging for circuit-switched and packet-switched services on the same paging channel, then the mobile station shall respond to paging messages for both services.

There is no requirement for the MS to monitor the packet paging channel when in dedicated mode.

One mode of network operation shall be defined so that when an MS is engaged in packet data transfer, it shall receive paging messages via the packet data channel without degradation of the packet data transfer. Modes of network operation where paging for other GSM services is not done via the packet data channel are also defined. In such cases an MS engaged in packet data transfer may attempt to receive paging messages.

When responding to a paging message for other GSM services, the MS shall establish the connection for that incoming service (i.e., enter dedicated mode) and suspend GPRS activity. GPRS activity is resumed upon return to idle mode.

If paged for an incoming circuit-switched call, the MS shall indicate the presence of the call to the user or user's application, and where possible provide to the user the CLI. It shall be possible for the user (or the user's application) to decide how to proceed with an incoming call (e.g., accept the call, indicate UDUB, or invoke Call Deflection).

It shall be possible for the MS to receive SMS-CB messages if it attached to GPRS but is not engaged in packet data transfer.

NOTE 2:	Users should be aware that monitoring paging (in some modes of network operation), responding to paging, alerting of circuit-switched service, or acceptance or establishment of a circuit-switched call during an active GPRS connection may degrade the performance of the established GPRS connection and, in some cases, may cause failures in an application using the GPRS connection (e.g., a file transfer might be aborted due to a timeout of the application protocol).

Class C:	The MS is attached to either GPRS or other GSM services. Alternate use only. If both services (GPRS and Circuit Switched) are supported then a Class C MS can make and/or receive calls only from the manually or default selected service, i.e., either GPRS or Circuit Switched service. The status of the service which has not been selected is detached i.e., not reachable. The capability for GPRS-attached class-C MSs to receive and transmit SMS messages is optional.

The network shall support SMS message reception and transmission for GPRS-attached class-C MSs.

It shall be possible for the MS to receive SMS-CB messages if it attached to GPRS but is not engaged in packet data transfer.

An MS may be reconfigured. E.g., a class A MS configured as 1 slot for circuit switched plus 1 slot for GPRS may be reconfigured as a class C configured as 0 slots for circuit switched plus 2 slots for GPRS.

Non-voice only MSs do not have to (but may) support emergency calls.

## 6.2	Subscriber roaming

GPRS shall provide the ability for subscribers to access the service while roaming between networks. This implies the pre-existence of an international signalling mechanism and a service agreement between the different network operators involved.

## 6.3	Construction of GPRS subscriber numbers

The number of a GPRS subscriber shall be composed such that the PLMN operator can maintain control of the numbering scheme, thereby ensuring subscriber numbering integrity. The ability to allocate a subset of the numbers privately shall be possible.

## 6.4	Battery life extension

The GPRS service shall provide for a means to extend the battery life of an MS, by allowing the MS to reduce current drain; e.g., by using a discontinuous reception (DRX) strategy. The power conservation strategy should allow the MS to remain in a very low current state for a considerable proportion of the time. Moreover, the power conservation strategy should ensure that no messages are lost and that bandwidth is not wasted.

When conserving power through the use of DRX, or other techniques, the MS shall still meet all QoS requirements of selected classes. Some classes of service may necessitate continuous reception (i.e., no DRX capability).

## 6.5	Format Of Message User Data

The user data is to be presented as an octet string between GPRS's access points, and is not interpreted by the PLMN.

## 6.6	Charging aspects

The charging information that the GPRS network collects for each MS shall be standardised. The information that the operator uses to generate a bill to a subscriber is operator specific. Billing aspects, e.g. a regular fee for a fixed period, are outside the scope of this specification.

The HPLMN Operator/Service Provider may be able to charge the GPRS subscriber for all costs incurred including those costs incurred in an external network be it a VPLMN or a data network.

### 6.6.1	PTP and IP-M charging information

The charging information of PTP and IP-M services is collected for the GPRS subscriber. As a minimum the following charging information shall be collected:

-	destination and source: The charging information shall describe the destination and source addresses;

-	usage of radio interface: The charging information shall describe the amount of data transmitted in MO and MT directions categorized with QoS and user protocols;

-	usage of the external data networks: The charging information shall describe the amount of data sent/received to/from the external data network;

-	usage of the packet data protocol addresses: The charging information shall describe how long the MS has used the packet data protocol addresses;

-	usage of the general GPRS resources: The charging information shall describe the usage of other GPRS related resources and MS's activity in the GPRS network (e.g. Attach/Detach, mobility management);

-	location of MS: HPLMN, VPLMN, plus optional higher accuracy location information.

### 6.6.2	PTM-G charging information

The charging information of PTM-G services is collected for the group controller, initiator, service requester and optionally members. The same charging information is collected per each participant as for the PTP service.

### 6.6.3	Reverse Charging

It shall be possible to provide reverse charging as a subscription option. However, reverse charging may not be applicable to certain external data network protocols. It shall be possible, as an option, to allow users to roam from one environment to another, between fixed and mobile, between public and private as well as between different public systems.

## 6.7	Security services

The use of radio communications for transmission to/from subscribers in mobile networks makes them particularly sensitive to:

1)	misuse of their resources by unauthorized persons using manipulated MSs;

2)	eavesdropping on the information being exchanged on the radio path.

Therefore, to protect the system in the two cases mentioned above, the following security features are provided for GPRS:

-	MS authentication; i.e., the confirmation by the land-based part of the system that the subscriber identity, transferred by the MS within the identification procedure on the radio path, is the one claimed. The purpose of this authentication is to protect the network against unauthorized use. It also enables the protection of GPRS subscribers by denying intruders the ability to impersonate authorized users;

-	access control; i.e., the network can support restrictions on access by or to different GPRS subscribers, such as restrictions by location, screening lists, and so on;

-	user identity confidentiality; i.e., the property that the user identity on the radio link is not made available or disclosed to unauthorized individuals, entities or processes. The purpose is to provide privacy of identities of the subscribers who are using GPRS radio resources. It allows for the improvement of other security features, e.g., user information confidentiality, and also provides for the protection against tracing the location of a mobile subscriber by listening to the signalling exchanges on the radio path;

-	user information confidentiality; i.e., the property that the user information is not made available or disclosed to unauthorized individuals, entities or processes. The purpose is to provide for confidentiality of user data, i.e., protection of the message part pertaining to layers 3 and above, that passes over the radio path.

Both user identity and user data shall be protected as shown in table 6:

Table 6: Protection of user identity and user data

| Service | User Identity Protection | User Data Protection |
| --- | --- | --- |
| PTP | Yes | Yes |
| PTM-Group Call | Yes | Yes |

Security mechanisms available for existing teleservices and bearer services should be used if possible.

## 6.8	Message Screening

The message screening function is concerned with filtering out unauthorized or unwanted messages. Message screening may be used to restrict the types of message or the volume of data which may be transferred across the GPRS network to/from an individual subscriber.

## 6.9	GPRS Operator Determined Barring

GPRS subscribers can request packet oriented services from access points within the HPLMN, or from access points within another GPRS enabled PLMN. Similarly, when roaming, a subscriber may request to be connected to an access point within the HPLMN which requires all of the packets to be sent over an international link, which may attract a higher charge from Service Providers. Therefore, similarly to circuit switched GSM, barring capability is required.

The specific requirements for ODB of GPRS can be found in 3G TS 22.041 (Operator Determined Barring) [16].

# 7	Normal procedures with successful outcome

## 7.1	Provision

The GPRS services shall be provided to the subscriber after prior arrangement with the service provider. The provision of each of the following GPRS services independently or any combination thereof shall be possible:

1)	Point To Point - Connectionless Network Service (PTP-CLNS);

2)	Point To Point - Connection Orientated Network Service (PTP-CONS);

3)	Point To Multipoint Group Call (PTM-G).

It shall be possible to subscribe to GPRS services with or without subscription to other GSM services. When a subscriber has subscribed to the Multiple Subscriber Profile (MSP) supplementary service, it shall be possible to assign GPRS services to each of the individual MSPs.

It shall be possible for the network operator to offer GPRS to the Service Provider(s) who may then offer GPRS-based services to the end-user.

GPRS services can be offered to a subscriber with the subscription option of using a password to control the services. A single GPRS-password per subscriber is supported by the network for all GPRS services.

As a subscription option modification of the subscriber’s service profile (e.g., registration, erasure) shall be possible only in conjunction with the GPRS-password.

The following Quality of Service (QoS) parameters shall be assigned to the subscriber profile upon provision of the GPRS service(s). These parameters may be negotiated or set to default values:

-	service precedence (priority),

-	reliability,

-	delay,

-	user data throughput.

-	scheduled repeated transmission.

## 7.2	Withdrawal

The service(s) shall be withdrawn at the subscriber's request or for administrative reasons. Withdrawal shall de-activate the service.

## 7.3	GPRS-Attach, GPRS-Detach

GPRS-Attach is performed when the MS indicates its presence to the PLMN for the purpose of using the GPRS PTP or PTM-G services. This can be immediately after the MS has been switched on or later as the user decides to use the GPRS services. The MS identifies itself with its identity (IMSI, TLLI). A GPRS-Attach shall be successfully performed only if the subscriber has a valid GPRS subscription.  It is assumed that the user will not, under normal circumstances, initiate a GPRS Attach without first ensuring that the data handling capability necessary in the terminating equipment has been activated.

GPRS-Detach is performed when the MS indicates to the PLMN that the MS will no longer be using the GPRS services. The MS identifies itself with its identity (IMSI, TLLI).

## 7.4	Registration

Registration of the service parameters shall enable the subscriber to optimize the actual (dynamic) service profile to the subscriber's present, actual requirements within the limitations of the (static) subscription profile (see subclause 6.1). The result of the registration shall be indicated (i.e. accepted/registered or rejected/not registered). The registered service profile is valid for all GPRS transactions until modified by re-registration or erasure or changes to the subscription profile.

As a subscription option registration of the service parameters shall be possible only in conjunction with a GPRS-password.

Table 7 defines which service parameters shall be able to be registered for the different GPRS services. The range of values for the service parameters is specified in subclause 5.6.

Table 7: Registration of service parameters

| Service Parameter | PTP-CLNS PTP-CONS PTM-G | IP-M |  |
| --- | --- | --- | --- |
| Service precedence (priority) | 2,3,4 | 2,3,4 |  |
| Reliability | 2,4 | 2,4 |  |
| Throughput | 2, 3, 4 | 2, 3, 4 |  |
| Simultaneous use class | 2 | 2 |  |
| Delay | 2, 4 | 2, 4 |  |
| Security management / encryption | 1 | 1 |  |
| Interworking profile (a) | 2 | 2 |  |
| Password Active / De-active | 2 | 2 |  |

Note: Service parameter for the PTM-G service are the same as for PTP and need not be registered.

Legend:	1	Network determined.

2	User determined per subscription and registration.

3	User determined per request.

4	Negotiable.

N/A	Not applicable.

a)	This is the set of subscription attributes (including a user-ID and/or address valid in the external data network if necessary) adopted by the subscriber when utilizing a particular destination network. A set of subscription attributes is required for each destination network. Multiple user-IDs and/or addresses for each destination network are allowed. See subclause 11.1 for address management.

Registration of an identity other than the IMSI (i.e. a user-ID/address in an external data network, e.g. Internet, X.25 etc.) is performed to couple this identity to the IMSI for the purpose of accessing (MO and/or MT) the external data network. Registration of an identity other than the IMSI is subject to valid subscription agreements with the network operator. This also provides an effective operator determined barring of access to external data networks.

Each of the registered interworking profiles can be individually activated and de-activated.

Screening parameters can be assigned to a particular interworking profile as a user protection against unwanted costs and/or data.

## 7.5	Erasure

The subscriber shall be able to erase previously registered service parameter values from the service profile.

Erasure of a non-GSM identity (e.g., Internet, X.25) which is currently registered for an interworking profile shall be possible without erasure of the other parameters of the interworking profile.

As a subscription option erasure shall only be possible in conjunction with the GPRS-password.

## 7.6	Interrogation

The subscriber shall be able to interrogate the status/value of registered service parameters as defined in the current service profile.

## 7.7	Activation

The subscriber shall be able to activate each of the registered interworking profiles independently within the limitations of the subscription profile.  As an option the GPRS network may request the activation of a specific interworking profile for a GPRS attached mobile, when an mobile terminated packet is received.  This option may be available for one or more PDP.  The activation of the inter-working profile(s) may be performed automatically by the MS immediately after GPRS Attach.  When an interworking profile  is activated a dynamic binding between the GSM/GPRS identity of the MS and the external data network user-ID and/or address is made by the system, and the appropriate service parameters are applied thus establishing a virtual connection  between the MS and the GPRS network. Activation of an interworking profile results in a dynamic binding until de-activation is performed.

## 7.8	De-activation

The subscriber shall be able to de-activate the previously activated interworking profiles independently within the limitations of the subscription profile thus releasing the previously established virtual connection. An interworking profile(s) shall be automatically de-activated if the provision for the supporting GPRS service is withdrawn. Interworking profiles are deactivated automatically upon GPRS detach..

De-activation of a particular interworking profile shall abort all presently on-going communication between the MS and the external data network.

## 7.9	Invocation and operation

The GPRS service is invoked upon transmission or reception of GPRS data.

It shall be possible for a MS to be a GPRS service requester and a service receiver. This allows generation and origination of GPRS messages from the mobile termination itself as a standalone GPRS data source. It also allows the mobile termination to provide DCE capabilities for user applications based in terminal equipment outside the mobile termination.

It shall be possible for a subscriber to set-up multiple GPRS PTP-CONS communication sessions and maintain these over prolonged periods (e.g. several hours). It shall be possible to establish these connections via more than one external data network. This enables more than one logical data channel to exist to a single mobile termination.

PTP-CONS, PTP-CLNS, IP-M, and/or PTM-G communications in any combination shall be possible in parallel. In the case of X.25 the concept of switched virtual circuits shall be maintained between the GPRS environment and the X.25 fixed network.

## 7.10	PIN and Password Management

GPRS-Attach shall only be possible after correct subscriber identification has been confirmed by entry of the current GSM PIN if this option is active. Thus only one PIN is required to gain access to either the GSM-Circuit Switched Services/SMS or the GSM-GPRS Services or both.

GPRS services can be offered to a subscriber with the subscription option of using a password to control the services. One password per subscriber is supported by the network for all GPRS services. The GPRS password shall be the specified password for supplementary services according to TS22.004 [13] and TS22.030 [14].

When this option is selected the following actions shall only be performed with the concurrent entry of the password:

-	registration, de-registration/erasure;

# 8	Exceptional procedures

## 8.1	Provision

None.

## 8.2	Withdrawal

None.

## 8.3	Registration

An attempt to register a service or service parameter not available to that subscriber shall result in an indication to the user. The reason shall be indicated:

-	the service or service parameter is not supported by the PLMN;

-	the subscriber’s present (static) subscription profile prohibits the use of this service or service parameter;

-	parameter is network determined;

-	the entered parameter value is outside the range of that supported by the PLMN.

If, in the case of interworking with an external data network, an attempt is made to set a parameter to a value outside the range of that supported by the interworking to the external data network, it shall be indicated to the user and confirmation required before the parameter value is changed to a value selected by the network.

## 8.4	Erasure

An attempt to erase a service parameter which is not registered shall result in an indication to the user, the service parameter shall remain in the not registered state.

Erasure of a non-GSM identity (e.g., Internet, X.25) which is presently registered as "active" shall result in an indication to the user with a request for confirmation before the de-activation and erasure is performed.

## 8.5	Interrogation

None.

## 8.6	Activation

An attempt to activate an interworking profile which is already in the active state shall result in an indication to the user, the interworking profile shall remain in the active state.

## 8.7	De-activation

An attempt to de-activate an interworking profile which is already in the de-active state shall result in an indication to the user, the interworking profile shall remain in the de-active state.

## 8.8	Invocation and operation

An attempt to send an MO message with an MS identity which is not valid in the network shall be rejected with an appropriate error status.

An MT message that cannot be delivered due to an invalid MS identity shall be discarded by the network. A message that is received by an incorrect MS shall be discarded by the MS.

## 8.9	PIN and Password management

PIN and password exceptional procedures are as defined in 02.17 [15] and 22.004 [13].

# 9	Addressing

## 9.1	Point to Point services

A GPRS-PTP subscription is identified by an IMSI.

## 9.2	Point to Multipoint services

## For mobile terminated PTM Group Call Services the network may identify the receiver by the IMSI thus allowing PTM-G messages to be delivered to the receiver as point to point messages.

For mobile originated PTM communications the IMSI shall be the basis for the access to the network.

## 9.3	Overview

How the identities are used within the different services is described in table 8.

Table 8: Services and Identities

| Services | mobile originated |  | mobile terminated |
| --- | --- | --- | --- |
|  | calling party | called party |  |
| GPRS PTP | IMSI | Address (note 1) | IMSI |
| GPRS PTM-G | IMSI | Address (note 1) | IMSI |
| IP-M | IMSI | Address (note 1) | IMSI |
| NOTE 1: The IMSI or in the case of network interworking refer to subclause 5.2.3. |  |  |  |

Annex A:
Change history

| Change history |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TSG SA# | Spec | Version | CR | <Phase> | New Version | Subject/Comment |
| Jun 1999 | GSM 02.60 | 8.1.0 |  |  |  | Transferred to 3GPP SA1 |
| SA#04 | 22.060 |  |  |  | 3.0.0 |  |
| SA#05 | 22.060 | 3.0.0 | 001 002 003 004 005 |  | 3.1.0 | Class B mode of operation The function that gives the notification of the server IP address from the GPRS network to the subscriber. Introduction of Barring for GPRS. Editorial update of references for GSM/3GPP use. R99 - GPRS Stage 1 rel 99 Editorial corrections (revised 1-99-352) |
| SA#06 | 22.060 | 3.1.0 | 006 007 008 | R99 | 3.2.0 | Support of Mobile IP in release 99 Deletion of Anonymous Service GPRS & SMS-CB Interworking |

# History

| Document history |  |  |
| --- | --- | --- |
| V3.0.0 | July 1999 | Transferred to TSG SA at ETSI SMG#29. Under TSG TSG SA Change Control. |
| V3.1.0 | October 1999 | Inclusions of CRs at SA#05. |
| V3.2.0 | December 1999 | Inclusions of CRs at SA#06. |