TSG-SA Working Group 1 (Services) meeting #7	TSG S1 (00) 109

Sophia Antipolis, France 7 - 11 February 2000	Agenda Item: 5.1

| ETSI SMG9 post-meeting #20 | Tdoc 9-00- 0162 |
| --- | --- |

# Liaison Statement

# Subject:	Fixed Dialling Numbers

# From:		SMG9 ad hoc meeting

Contact:	Samuel Blarre (Tel +33 1 4529 4272)

# To:		3GPP S1

SMG9 thanks S1 for their LS containing the CR of 22.101 (S1-99 1039) about FDN issues. SMG9 would like to make the following remarks:

- bearer capability indication related to ADN and FDN entries.

- A brief survey showed that most MEs do not use the CCP information. Due to this, no MMI exists to enter a reference to the CCP and, furthermore, the CCP information will not be checked by existing MEs. The check will only be based on the number stored in the FDN file. In order to implement the feature described in the CR, the ME must provide an MMI to enter one or several bearer services/teleservices.

- MEs supporting this new feature must  check that the bearer indicated in the FDN entry is available in the current PLMN. The behaviour of release 98 or earlier ME is not known.

- List of bearer services/teleservices excluded from the FDN check.

- This can be implemented for R’99 subject to the coding of bearer services/teleservices being defined. Two different coding are defined in 09.02/29.002 and in 04.08/24.008.

- R’98 or earlier MEs will not read the excluded bearer list. The check will be done on the FDN numbers if FDN is enabled, i.e. allowing calls to any number on a specific bearer which is on the excluded list will not work (e.g. GPRS calls to any number).

- Emergency calls

- Introducing the use of EFECC when FDN is enabled for R’99 will not necessary imply the use of the numbers stored in this file by R’98 or earlier MEs. According GSM 02.07 version 7.0.1, only emergency calls to 112 are allowed. R’98 or earlier MEs may not support emergency calls using numbers stored in Emergency Call Codes file (EFECC) when FDN is enabled.

- SMS

- R’98 or earlier MEs may not check SMS end destination addresses from the FDN file even if FDN is activated. It is also unlikely that the SMSC address is checked. Furthermore, according to our understanding, CCP coding does not apply to SMS.

- If this feature is introduced in R’99, R’98 or earlier MEs supporting FDN may not restrict the sending of SMS according to the R’99 definition of FDN.