TSG-SA Working Group 1 (Services) meeting #8	TSG S1 (00) 209

Beijing, China: 10-14th April 2000	Agenda Item:

TSG-RAN Working Group 3 meeting #11                       	TSGR3#11(00)0840

Sophia Antipolis, France, February 28 – March 03, 2000

To:	T2

CC:	SA1, SA2 and R2

Source: 	Iu-SWG (RAN3)

Title: 	LS on RAN3 decisions related to CBS

RAN3 would like to thank T2 for their liasion (TSG T2#8(00)137) on Use of Service Areas for CBS and liasion (TSG T2#8(00)138) on Conversion of GSM related CBS DRX Information.

During the recent RAN3#11 meeting in Sophia Antipolis the CBS subject has been further discussed. RAN3 would like to inform about their decisions which are stressed below:

Usage of Service Area

RAN3 will stick to the functional split which purpose is to keep CN and UTRAN related functions decoupled. Therefore it has been agreed to implement the term “Service Area” instead of “Cell” over the Iu-BC domain for Release 99. For CBS a Service Area is equal to one Cell and this one to one mapping is controlled by the RNC and managed by an O&M function.

We have taken this decision of one to one mapping because of the suspicion that the mapping of one or more cells to one SA could compromise the inclusion of CBS in Release 99 based on the T2 liasion. This more flexible mapping would require T2 to make some changes to the CBC-UE protocol. Some R3 members expressed concerns about the future proofness of CBS Release 99 with such limitation. The removal of this limitation in future releases is expected for service evolution but backward compatibility with R’99 UEs will have to be considered.

RAN3 would like to invite comments from T2 on the feasibility of changes to the CBC-UE protocol in future releases with the requirement that mapping of one or more cells to a service area is applicable.

Terminology of Service Area

Due to the fact that changing the terminology has an impact on other 3GPP Groups it has been agreed to use the terminology as shown in Table A.

|  | Release 99 | Release 2000 |
| --- | --- | --- |
| Service Name | Cell Broadcast Service – CBS | FFS |
| Network Node Name | Cell Broadcast Centre – CBC | FFS |
| Protocol Name | Service Area Broadcast Protocol – SABP | Service Area Broadcast Protocol – SABP |
| Interface Name | Iu-BC | Iu-BC |
| Table A |  |  |

Usage of conversion of GSM related CBS DRX information

The functionality to activate/deactivate DRX on CBS related radio resources is no more a requirement within UTRAN. Therefore is has been agreed to remove this function from the Service Area Broadcast Protocol SABP (3G TS 25.419).

Usage of procedure capacity indication/reservation

The benefit of implementing the two requested procedures capacity indication and capacity reservation within UTRAN have been discussed. Due to the fact that the capacity reservation can be basically administrated by O&M it has been agreed to remove this requirement.