TSG-SA Working Group 1 (Services) meeting #8	TSG S1 (00) 235

Beijing, China: 10-14th April 2000	Agenda Item: 6.5

# Source:		Fujitsu Europe Telcom R&D Centre

# Title:		Realisation of VHE in R00

# Action:		Discussion

The study on release 2000 services and capabilities TR 22.976 does not really cover the concept of VHE but focuses more on the toolkits for realisation of VHE in R99. It is necessary to have a main section that covers the concept of VHE, within which there can be a sub-section on the existing toolkits. This contribution proposes text for section 6 of TR 22.976.

VHE 22.121 R99 specification identified more than R99 requirements. The specification identified the overall requirement for VHE such as the requirement for the Home Environment, User requirement, and Serving network requirement. These requirements relate to the VHE in general in whatever releases.  A lot of time was spent in developing these requirements which have been agreed. A similar approach for R00 is not necessary.

R99 requirement that was realised was the service capability servers necessary to realise VHE in R99.  The approach in developing VHE R00 service specification should focus more on the requirement for the realisation VHE R00.

# 6	Virtual Home Environment.

Virtual Home Environment (VHE) is defined as a concept for personal service environment (PSE) portability across network boundaries and between terminals. The concept of the VHE is such that users are consistently presented with the same personalised features, User Interface customisation and services in whatever network and whatever terminal (within the capabilities of the terminal and the network), wherever the user may be located [8].

## Realisation of VHE in R00

This clause reviews issues and topics that are necessary for VHE in R00. Some of these topics were briefly discussed in TS 22.121 but advancement on these issues is necessary.

### 6.1.1			All IP architecture.

Although there is a strong discussion on All IP architecture as an option for R00, this as far as VHE is concerned should not make any difference to the service concept provided by VHE. VHE should be transparent to the transport mechanisms. There may be new service capabilities that are realised due to IP architecture and need to be realised in R00 specification.

Under this section some VHE service scenarios need to be considered e.g what happens when a CS only user roams into a PS domain what level of VHE should they expect.

### Personal Service Management.

Identify the additons to R99 required for R00 personal service management.

### 6.1.3	 		Applicability of existing toolkits.

Text as in TR  22.976

### 6.1.4			Interoperatability between toolkits

Are there cases were interoperatability between toolkits becomes an issue?

### 6.1.5			Fixed Mobile Convergence.

VHE should be access network independent. Requirement on how this is realised needs to be specified in R00 specification. Questions such as what level of VHE should be supported when a mobile terminal requires access from a fixed network and how will this be supported should be considered in the R00 VHE specification. Currently to realise this level of support there needs to be a close collaboration with other ETSI group in this area such as SPAN group.