3GPP TSG-SA Working Group 1 meeting #8	TSG S1 (00) 238

Beijing, CHINA	Agenda Item: 5.8

10-14th April 2000

Source : France Telecom

Title : Preferred access technology for PLMN selection

Document for : Discussion and Decision for release 99

The current Automatic Network Selection mode described in 22.011 already allows the user and the operator to specify in which order some PLMNs shall be selected. It is also possible to specify which access technology to search for a given PLMN.

However the Automatic Network Selection mode still suffers from a major limitation: some users will clearly want to favor one access technology above all (e.g. UMTS). The current selection algorithm does not allow for this situation (but if all the PLMN of every country are stored in the USIM).

Considering that:

- during the last plenary meeting of SA, PLMN selection was identified as an item for which improvements can be made until June,

- CN1 which is in charge of 23.122 has not yet worked on that specification and that their next meeting is in May,

France Telecom proposes:

- to add a Preferred Access Technology data field which will allow the user to set his preference in terms of access technology,

- to modify the current Automatic Network Selection mode to account with this Preferred Access Technology data field.

These proposals are made in one CR to be presented during this meeting. One other CR is also provided to align 22.011 v4.0.0 (R00) with the proposed modifications in 22.011 v3.2.0 (R99).

Finally one last CR proposes to account for the Preferred Access Technology data field in the periodic reselection attempts.