# Annex A		PS Domain feature list evaluation for release 2000 (Normative)

Key to Table

E = Essential for release 2000, launch of commercial IP multimedia services is not viable with these missing or required in Release 2000 terminal specifications to enable forward compatibility to future releases

D= Desirable for Release 2000, important features to enable a competitive and successful service launch, but could be slipped to Release 2001

- R99 = Features already supported in Release 99, shall also be supported as part of Release 2000

R01+ = could wait for these features, but hooks are required in Release 2000 to enable them to be added later

- No = Not needed in the PS domain

Note:	Circuit switched domain services are not considered at this annex. S1 has agreed to maintain the existing Release 99 requirements in Release 2000, allowing full service continuity.

Note: All Release 99 features shifted by any reason to Release 2000 shall be included as E R’00 (to be verified feature by feature)

| Feature Name | Short description | R99 | E R00 | D R00 | R01+ | No | Comments/Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DTAP CC |  |  |  |  |  |  | i.e. 04.08 based CC. Seamless support for existing GSM services This set of requirements means 04.08/04.80 CC and SS in PS domain |
| Rel 99 CS terminal and circuit service support by circuit switched domain (cf. Relationship A in figure 5). . | Support of R99 TS11, TS12, BS20, call offering SS, call completion SS, call restriction SS, CCBS SS, number identification SS etc. | X |  |  |  |  | How this requirement is supported (e.g. IP Transport, MSC servers, etc) in the circuit switched domain is out of S1 scope. |
| Rel 99 CS terminal and circuit service support by PS Domain (cf. Relationship B in figure 5).. | Support of R99 TS11, TS12, BS20, call offering SS, call completion SS, call restriction SS, CCBS SS, number identification SS etc. |  | X |  |  |  | How this requirement is supported (e.g. IP Transport, MSC servers, etc) in the PS domain is out of S1 scope. |
| IP CC |  |  |  |  |  |  | e.g. H.323 / SIP related IP multimedia services. |
| #### IP multimedia services |  |  |  |  |  |  |  |
| #### IP telephony | Single medium IP voice call (using H.323 or SIP) with end user perceived quality equal or better than 2G GSM voice call |  | X |  |  |  | It must be possible for the MMI to be identical to the standard telephony MMI (dialled digit, off hook, connection, on hook). Including end to end QoS support. |
| Multimedia IP Call | Includes IP telephony, all real time calls single and multi-media, processed by IP CC |  | X |  |  |  | Including end to end QoS support. It is desirable to limit the standard to one protocol only. S2 should decide on the standard protocol. The usage of any additional protocol may be based on the network transport function (bearer service). |
| Emergency Voice Call | Basic emergency voice call over IP |  | X |  |  |  | This must use the existing emergency numbering schemes (22.101). Must be compliant with FCC mandates, European and other regulatory requirements. |
| Group calling | This requirement covers various group call services (e.g. PMR/ASCI type of services) |  |  | X | X |  | Service requirements FFS |
| Short message service (CBS) | As specified by 23.041. | X |  |  |  |  | No additional standardisation work required for S1 |
| Short message service (SMS PTP) | As specified by 23.040. | X |  |  |  |  | No additional standardisation work required for S1 |
| #### Multimedia messages (MMS) | Support of multimedia messaging in PS domain | ? |  |  |  |  | Currently supported in R99 for MS/MS. Need to consider messaging to and from other access. |
| #### Facsimile service |  |  |  |  |  |  |  |
| #### Store and forward | Transfer of text or images from a MS to a store and forward unit for subsequent delivery to a fax machine. Faxes from PSTN/ISDN to mobile terminals are stored in a store-and-forward unit. |  |  |  |  | X | Support of bearer service is, however, necessary to allow customised solutions to be implemented (based on T.37 and/or T.38). |
| #### End-to-end | End-to-end fax between a PSTN/ISDN fax machine and a mobile terminal. |  | X |  |  |  |  |
| #### Services independence from transport technology | Possibility to operate in different transport environments (e.g. all IP or other different from IP) |  | X |  |  |  |  |
| #### IP bearer services |  |  |  |  |  |  |  |
| Point-to-Point | As in 22.060 | X |  |  |  |  | No additional standardisation work required for S1 |
| Point-to-Multipoint | PTM services such as PTM-Multicast, PTM Group Call, IP Multicast, IP Distribution Services (MDS) |  | X |  |  |  | Implementation of PTM services for example as defined within GPRS specifications |
| #### Asymmetric bearers | Separate parameters at the User Interface for the uplink and downlink data rate and QoS. This feature is already within R99 (e.g. TS 23.107) | X |  |  |  |  | No additional standardisation work required for S1 |
| #### Support of QoS mechanisms for real time services |  | ? | X |  |  |  | QoS is also part of R99, but the features for real time conversational services might be delayed |
| #### Multicall capability | Support of multiple active PS sessions (TS 22.060) | X |  |  |  |  | No additional standardisation work required for S1 |
| #### Interworking |  |  |  |  |  |  | Including end to end QoS support |
| IPv4 interworking |  | X |  |  |  |  | Same reqs as Rel 99 GPRS |
| IPv6 interworking |  | X |  |  |  |  | Same reqs as Rel 99 GPRS(?) |
| Speech to/from PSTN / ISDN / 2G CS mobile / 3G CS mobile | Full interoperability between corresponding services in PSTN/GSM environment and UMTS rel00 environment |  | X |  |  |  |  |
| Multimedia to/from Internet-H.323 | H.323 protocol interworking |  | X |  |  |  | Incl. Intranet. |
| Multimedia to/from Internet –SIP | SIP protocol interworking |  | X |  |  |  | Incl. Intranet. |
| Modem and ISDN interworking | Access to PSTN / ISDN dial up –data services |  |  |  |  | X | Removed from R99 at SA1#6 |
| Interworking with ISDN multimedia applications | Service compatibility between real time single/multimedia N-ISDN applications and single/multimedia UMTS rel 00 applications including H.324 |  | X |  |  |  |  |
| Interworking with other access networks (e.g. cable) |  |  | X |  |  |  |  |
| Interworking with intranets (including VPNs) | VPN functionality (firewall bypass) shall be supported |  |  | X |  |  | May possibly be supported at the application layer. |
| Roaming | Editor's note: Roaming section not reviewed, and roaming clause is under construction |  |  |  |  |  |  |
| CS/PS GSM/UMTS R99 to R00 PS CS only GSM/UMTS R99 to R00 PS PS or dual mode GSM/UMTS R99 to R00 PS | SIM ROAMING Terminal roaming |  | X X |  |  |  | (U)SIM and Multimode terminal Roaming. Roaming should be possible for both R99 and pre R99 GSM/GPRS networks. (Note: roaming to R00 CS is included but not within the scope) |
| R00 PS to GSM/GPRS |  |  | X |  |  |  | (U)SIM and Multimode terminal roaming. Roaming should be possible for both R99 and pre R99 GSM/GPRS networks. |
| ANSI-41 to R00 PS |  |  |  | X |  |  |  |
| R00 PS to ANSI-41 |  |  |  | X |  |  |  |
| Handover | Editor's note: Handover section not reviewed, and handover clause is under construction |  |  |  |  |  |  |
| Speech from PS Domain to CSGSM /UMTS Need to differentiate between handover requirements to 2G (GSM) and 3G (UMTS) | 3G PS to 2G CS – driven by islands of coverage of R99/R00 and need to handover real time services from 3G PS to 2G 3G packet to 3G circuit |  | X |  |  | X | Requirement for 3G(PS) to 2G(CS) handover could be alleviated by the support of real-time services in the 2G (PS) domain. |
| Speech from CS-GSM /UMTS to PS Domain Need to differentiate between handover requirements from 2G (GSM) and 3G (UMTS) | 2G CS (GSM) to PS Domain - 3G CS (UMTS) to PS Domain |  |  | X X |  |  | So users who start a CS call can then use PS domain for PS services and maintain CS call |
| Handover of parallel sessions/calls (with different QoS) | Where supported by target domains |  | X |  |  |  | Unclear exactly what is meant here – e.g. CS domain does not support multiple calls. Does this refer to handover between 2G (GPRS) and PS domain? |
| Multimedia handover between R00 UTRAN and GERAN Editor's note What about HO towards GPRS? Release 99 GPRS might offer similar capabilities (without the QoS for real time) as Release 2000 |  |  | X |  |  |  | To include real-time services in addition to R99 “best effort” |
| Multimedia and single-media handover between R00 UTRAN and 2G-GPRS |  |  | X |  |  |  | Need to include handover between 2G (GPRS) and 3G-PS domain |
| Multimedia services to/from alternative access technologies (e.g. HIPERLAN/2) |  |  |  | X | X |  | Alternative access technologies to include HIPERLAN/2 |
| #### Supplementary Services (PS Domain) |  |  |  |  |  |  | IP Multimedia / IP Telephony aware supplementary services to be considered only from end-user need view point. (Standardization, service capabilities and implementation FFS). S2 choice of MM CC protocol may support some MM services implicitly. Some of the following services can be provided at the application level (i.e. no standardisation required). The H.450 standards defines supplementary services for H.323, and SIP provides tools to build supplementary services. |
| Multimedia Call Barring | Enables mobile subscriber to have barring of certain categories of outgoing multimedia calls. |  | X |  |  |  | Also includes incoming multimedia calls and barring when roaming. A generic barring feature to allow barring of calls/sessions based on user preferences, operator settings and criteria of the call/session/service request (e.g. those listed in the 3 lines below) is required. |
| Network Barring | Editor's note: Description and justification to be supplied by Horst Rauch (T-Mobil) |  |  |  |  |  | It shall be possible for the subscriber to have the ability to bar access to certain networks (e.g. cost reasons) |
| Session Barring | Editor's note: Description and justification to be supplied by Horst Rauch (T-Mobil) |  |  |  |  |  | It might be valuable to limit the number of simultaneous sessionsFFS |
| Bearer Barring | Editor's note: Description and justification to be supplied by Horst Rauch (T-Mobil) |  |  |  |  |  | .Might be based on the QoS parameters (e.g. barring of bearers with high QoS for cost reasons). |
| Multimedia Call Forwarding | Forwarding of multimedia call (e.g. triggered by conditions of Unconditional, Busy, No Reply, Not Reachable etc). Triggers/activation will be different and more detailed in a MM environment for the different media components. |  | X |  |  |  | Conditions require to be evaluated. |
| Multimedia Call Transfer | Enables served mobile subscriber who has a multimedia call, to connect the other parties in the multimedia call and release the served mobile subscriber’s own connection. |  | X |  |  |  |  |
| Multimedia Call Deflection Service | Enables the served mobile subscriber to respond to an incoming multimedia call offered by the network by requesting redirection of this multimedia call to another address or location. |  | X |  |  |  |  |
| Multimedia Call Holding | Allows served mobile subscriber to interrupt communication on an existing active multimedia call and then subsequently re-establish communication |  | X |  |  |  |  |
| Multimedia Call Waiting | Permits mobile subscriber to be notified of an incoming multimedia call while the mobile subscriber is engaged in other multimedia call(s). Subscriber can either accept, reject, ignore, or deflect the incoming multimedia call. |  | X |  |  |  |  |
| Advice of Charge | Supply user sufficient information to allow real-time estimate of the call charge. |  |  |  | X |  |  |
| Caller Identification and restriction | Similar to CLIP, CLIR, CNAP and CNAR. |  | X |  |  |  | Could include additional IP related information such as IP address. Need to support European, FCC and other regulatory requirements. |
| Connected Line Identification and restriction | Similar to COLP and COLR |  |  |  |  | X | Could include additional IP related information such as IP address. Need to support European, FCC and other regulatory requirements. |
| Multimedia conferencing | Similar to Multiparty but is applicable to multimedia calls. |  | X |  |  |  |  |
| #### Multimedia call-back when free / CCBS | Editor's note: definition required in the MM case |  | X |  |  |  |  |
| #### Closed user group (CUG) / community of interest | Editor's note: definition required in the MM case |  | X |  |  |  |  |
| #### Precedence and Pre-emption service | Editor's note: definition required in the MM case |  | X |  |  |  |  |
| Network Services |  |  |  |  |  |  |  |
| Operator Determined Barring (ODB) | Allows service providers to regulate subscriber access to services by the barring of certain categories of outgoing or incoming multimedia calls and packet services. ODB could terminate ongoing multimedia calls and could bar future multimedia calls and packet services. | ? | X |  |  |  |  |
| CAMEL Support for Multimedia Services | Provides mechanisms to support multimedia services consistently & independently of the serving network. |  | X |  |  |  | CAMEL enhancements, but additionally needs to support multimedia calls. Implementation is FFS. Editor's note: DeWayne Sennet (AWS) to WIN support proposal |
| (U)SIM Toolkit | Feature provides a set of facilities which allow the (U)SIM to interact with external entities (e.g. the network, the Mobile Equipment, or the user) to enable value-added multimedia applications to exist in the (U)SIM. |  | X |  |  |  |  |
| OSA for new elements | Provision of an API for controlled, secure and accountable access to multimedia service capability features by applications, based on the user profile |  | X |  |  |  | E.g. H.323/SIP CSCF. |
| LCS for GPRS/PS domain | Support of LCS on the PS domain required to meet regulatory and commercial requirements (e.g., 3GPP 22.071) |  | X |  |  |  | Exact work required is for FFS. Need to support European, FCC and other regulatory requirements. |
| SoLSA | SoLSA shall facilitate user-dependent radio resource selection based on LSA (e.g. when user is located at his office, radio coverage provided with indoor radio solutions should be preferred). |  | X |  |  |  |  |
| Lawful Surveillance / Intercept |  |  | X |  |  |  | Need to support European, FCC and other regulatory requirements. |
| Number Portability | Ability for subscriber to change service providers while retaining the original directory number. Includes mobile to mobile, mobile to landline, & landline to mobile number portability scenarios. |  | X |  |  |  | Need to support European, FCC and other regulatory requirements. |
| Mobile Station Application Execution Environment (MExE) | Provides standardized execution environment in an MS, and an ability to negotiate its supported capabilities with a MExE service provider, allowing applications to be developed independently of any MS platform. |  |  |  | X |  |  |
| Personalization of Mobile Equipment (ME) | Storage of information in the ME which limits the SIMs which will operate with the ME. | X |  |  |  |  | No standardisation work required for R99 |
| Advanced Addressing | Support of symbolic and advanced addressing |  |  |  | X |  | Addressing depends on applications, e.g. e-mail addresses are used for e-mail, E.164 is used for telephony, ICQ uses IP addresses. This does not require standardisation by 3GPP. |
| System Selection | Ability for the mobile equipment to choose a preferred service provider, based upon geographic location, frequency band preferences, available operators, etc. Also the ability to force a mobile station to "disallow" service from a "forbidden" service provider, and to force a mobile station to use "home" services. Service provider lists must be downloadable over-the-air. The possibility for operators with multimode networks, e.g. with GSM and UMTS radio access networks, to control which RAN a user accesses. |  |  |  | X |  | Similar to ANSI-136 Intelligent Roaming. |
| Over-the-Air Service Provisioning | Ability to download parameters to either the SIM or ME for provisioning of services. This includes both subscription parameters as well as operator-specific parameters. |  | X |  |  |  | Similar to ANSI-136 OTASP and OTAPA. |
| Charging |  |  |  |  |  |  |  |
| Implementation of on-line charging mechanisms for the support of Pre-paid services | Definition of charging mechanisms for the support of IP multimedia pre-paid services |  | X |  |  |  |  |
| Event/transaction based charging mechanisms (e.g. content based) | Definition of charging mechanisms for the provisioning of IP based Value Added Services |  | X |  |  |  |  |
| Charging aspects – need to charge for each PDP context (PS sessions) independently |  |  | X |  |  |  |  |

.

# Annex B		Release 2000 document structure (Informative)

Release 2000 introduces a set of new requirements and service descriptions mainly related to IP multimedia telephony.  The IP based architecture offers an extended set of services, and may support services differentiating from services offered by circuit switched network.

Additional new specifications for IP multimedia services should be created when needed.

The following table presents the TSG-S WG1 document structure, with initial comments on their applicability for Release 2000 .

| New | Name | Release 2000 notes |
| --- | --- | --- |
| 22.001 | Principles of Circuit Telecommunication Services Supported by a Public Land Mobile Network(PLMN) | Not relevant for PS Domain. |
| 22.002 | Circuit Bearer Services Supported by a PLMN | Not relevant for PS Domain. |
| 22.003 | Circuit Teleservices supported by a PLMN | Not relevant for PS Domain. |
| 22.004 | General on Supplementary Services | Not relevant for PS Domain. |
| 22.011 | Service Accessibility | No major changes seen. |
| 22.016 | International Mobile Equipment Identities (IMEI) | Relevant for both CS and PS domains. No major changes seen. |
| 22.024 | Description of Charge Advice Information (CAI) | See 22.004 |
| 22.030 | Man-Machine Interface (MMI) of the Mobile Station (MS) | FFS |
| 22.034 | High Speed Circuit Switched Data (HSCSD) – Stage 1 | Not relevant for PS Domain. |
| 22.038 | SIM application toolkit (SAT); Stage 1 | Should be applicable to both sides as part of VHE. Further study for IP multimedia telephony needed. |
| 22.041 | Operator Determined Call Barring | Only applicable to CS domain. Further study for rel 2000 needed. |
| 22.042 | Network Identity and Time Zone (NITZ), stage 1 | Only applicable to CS domain. Further study for rel 2000 needed. |
| 22.043 | Support of Localised Service Area (SoLSA) - Stage 1 | Relevant for both CS and PS domains. Further study for rel 2000 needed. |
| 22.057 | Mobile Station Application Execution Environment (MExE); Stage 1 | Should be applicable to both sides as part of VHE. Further study for IP multimedia telephony needed. |
| 22.060 | General Packet Radio Service (GPRS); Stage 1 | Not relevant for CS domain. Detailed review needed. |
| 22.066 | Support of Mobile Number Portability (MNP); Stage 1 | IP multimedia telephony related changes FFS. |
| 22.067 | Priority Set-up Service(PSUS); Stage 1(ASCI spec) | See 22.004 |
| 22.071 | Location Services (LCS); Stage 1 | Applicable to both domains, GPRS status not clear . Further study for rel 2000 needed. |
| 22.072 | Call Deflection (CD); Stage 1 | See 22.004 |
| 22.078 | CAMEL; Stage 1 | IP multimedia telephony support need to be studied. |
| 22.079 | Support of Optimal Routing; Stage 1 | Only applicable to CS domain. Similar requirement needed for rel 2000. |
| 22.081 | Line Identification Supplementary Services; Stage 1 | See 22.004 |
| 22.082 | Call Forwarding (CF) Supplementary Services; Stage 1 | See 22.004 |
| 22.083 | Call Waiting (CW) and Call Hold (HOLD) Supplementary Services; Stage 1 | See 22.004 |
| 22.084 | MultiParty (MPTY) Supplementary Service; Stage 1 | See 22.004 |
| 22.085 | Closed User Group (CUG) Supplementary Services; Stage 1 | See 22.004 |
| 22.086 | Advice of Charge (AoC) Supplementary Services; Stage 1 | See 22.004 |
| 22.087 | User-to-user signalling (UUS); Stage 1 | See 22.004 |
| 22.088 | Call Barring (CB) Supplementary Services; Stage 1 | See 22.004 |
| 22.090 | Unstructured Supplementary Service Data (USSD); Stage 1 | See 22.004 |
| 22.091 | Explicit Call Transfer (ECT) Supplementary Service; Stage 1 | See 22.004 |
| 22.093 | Call Completion to Busy Subscriber (CCBS); Stage 1 | See 22.004 |
| 22.094 | Follow Me; Stage 1 | See 22.004 |
| 22.096 | Calling Name Presentation (CNAP); Stage 1 (T1P1) | See 22.004 |
| 22.097 | Multiple Subscriber Profile (MSP); Stage 1 | PS domain FFS. |
| 22.100 | UMTS phase 1 capabilities | Not applicable to rel 2000. |
| 22.101 | UMTS service principles | Applicable to both domains. No major changes seen. Introduction to IP multimedia services and IP multimedia telephony might be good. |
| 22.105 | Services and service capabilities | Applicable to both domains. IP multimedia services are FFS |
| 22.115 | Service aspects: charging and billing | Applicable to both domains. IP multimedia services are FFS |
| 22.121 | Virtual home environment | Applicable to both domains. IP multimedia services are FFS |
| 22.129 | Handover requirements between UMTS and GSM or other radio systems | Applicable to both domains. IP multimedia services are FFS |
| 22.135 | Multicall | Applicable to CS domain only. GERAN applicability studied with EDGE. |
| 22.140 | Multimedia Messaging Service Stage 1 | Applicable to both domains. IP multimedia services are FFS |
| 22.960 | Mobile multimedia services including mobile intranet and internet services | No impact identified. |
| 22.971 | Automatic establishment of roaming relations | No impact identified. |
| 22.975 | Advanced addressing | No impact identified. |
| 22.976 | Study on PS domain services and capabilities | Basis for S1 rel 2000 IP multimedia services work. |

# Annex C		Regional Regulatory Requirements (Informative)

Release 2000 shall enable compliance for IP multimedia services with regional regulatory mandates. The following list is neither exhaustive nor complete, and requires further elaboration in this annex.

US regional regulatory mandates services may include (and are not limited to):-

- emergency services (e.g. in the U.S., the FCC Phase 2 Emergency Services mandate which requires provision of geographic location information).

- lawfully authorized surveillance

- number portability

- text telephone services (e.g. TTY in the U.S.)

- priority services for Critical Communications (e.g. ITU-T Study Group 2 recommendation E.ieps (International Emergency Preference Scheme to support the need for telecommunications among essential users of public telecommunications networks in crises situations)).

# Change History

| Document history |  |  |
| --- | --- | --- |
| 0.0.0 | 17/11/1999 | Skeleton |
| 0.1.0 | 14/1/2000 | Editor's draft input to Rome, Italy R2000 ad-hoc 17-19/1/2000 |
| 0.2.0 | 18/1/2000 | Produced during the Rome, Italy R2000 ad-hoc 17-19/1/2000 |
| 0.3.0 | 19/1/2000 | Produced during the Rome, Italy R2000 ad-hoc 17-19/1/2000 |
| 0.4.0 | 19/1/2000 | Produced during the Rome, Italy R2000 ad-hoc 17-19/1/2000 |
| 0.5.0 | 20/1/2000 | Produced during the Rome, Italy R2000 ad-hoc 17-19/1/2000 |
| 0.6.0 | 18/2/2000 | Editor's draft input to the Helsinki, Finland R2000 ad-hoc 29/2-2/3/2000 |
| 0.7.0 | 02/3/2000 | Produced following the Helsinki, Finland R2000 ad-hoc 29/2-2/3/2000 |
| 0.8.0 | 10/3/2000 | Produced during the e-mail discussion period following the Helsinki, Finland R2000 ad-hoc 29/2-2/3/2000 |
| 1.0.0 | 14/3/2000 | Version 1.0.0 produced for presentation to SA #7 |
| Editor: Mark Cataldo, Motorola E-mail: mcatald1@email.mot.com Telephone: +44 1793 566297 |  |  |

# History

| Document history |  |  |
| --- | --- | --- |
| 14 March 00 | V1.0.0 | Presented to SA#7 for information. |