Subject:	Comments to: Global "Cause of no ID"

###### Source:	Mannesmann Mobilfunk

Contact:	stephan.kleier@d2mannesmann.de

Mannesmann Mobilfunk has some concerns about the CR presented in Tdoc 309. The Calling Party Number information element, in special the screening indicator, is coded according ITU Q.931 (see below).

To be compatible with the fixed network the alignment need to be granted.

Only the first 3 states can be mapped (and are already mapped) without changing the fixed network standard.

Following four cases  NO calling party number is informed.

- Unavailable 		    		In case of the service is unavailable.

This is currently the case if the complete CLI information element is missing.

- User rejection		     		In case of user do not wish to tell his number.

Better wording: “user restricted” – This is mapped to the second entry in the list below

- Interaction with other services 		In case of service interaction.

This is mapped to the third entry in the list below.

- Coin line			     		In case of a coin line call.

There is no interworking defined yet. Could be probably mapped to the forth entry but need to be confirmed by ITU.

Mannesmann Mobilfunk proposes to change the CR accordingly and remove the last bullet.

TABLE  4-11/Q.931 (concluded)

Calling party number information element

| Presentation indicator (octet 3a) |  |
| --- | --- |
| Bits |  |
| 7 6 | Meaning |
| 0 0 | Presentation allowed |
| 0 1 | Presentation restricted |
| 1 0 | Number not available due to interworking |
| 1 1 | Reserved |
| NOTE 1 – The meaning and the use of this field is defined in 3/Q.951 and 4/Q.951. |  |
| Screening indicator (octet 3a) |  |
| Bits |  |
| 2 1 | Meaning |
| 0 0 | User-provided, not screened |
| 0 1 | User-provided, verified and passed |
| 1 0 | User-provided, verified and failed |
| 1 1 | Network provided |
| NOTE 2 – The meaning and the use of this field is defined in 3/Q.951 and 4/Q.951. |  |
| Number digits (octets 4, etc.) |  |
| This field is coded with IA5 characters, according to the formats specified in the appropriate numbering/dialling plan. |  |