3GPP TSG-SA WG1 	TSG S1 (00) 328

Beijing, China,	 Agenda Item: 5.12

10th – 14th April 2000

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.082 |  | Version: |  |  | 3.0.1 |  |  |  |
| Submitted to TSG_SA | 8 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  | X |  |
| list TSG plenary meeting no. Here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE |  | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Call Forwarding Enhancements |
| --- | --- |

| Source: | Vodafone Airtouch | Date: | 13/04/00 |
| --- | --- | --- | --- |

| Subject: | Support for long forwarded-to numbers |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| And one release | C Functional modification of feature |  |  | Release 98 |  |
| Only shall be | D Editorial modification |  |  | UMTS 99 | X |
| Marked with an X) |  |  |  |  |  |

| Reason for change: | To provide a consistent set of documents for release 99 it is necessary to align stage 1 with stage 2 . |
| --- | --- |

| Clauses affected: | 1.2.3,1.4.2,1.4.7,1.5,2.2.3,2.4.2,2.4.7,2.5,3.2.3,3.4.2,3.4.7,3.5,4.2.3,4.4.2,4.4.7,4.5 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 1.2.3	Terminology

A served mobile subscriber is a mobile subscriber of a particular PLMN access who is requesting that calls to her number be forwarded. This subscriber may also be referred to as the forwarding subscriber or the called subscriber.

A forwarded-to subscriber is a subscriber to whom the call shall be forwarded. A long forwarded-to number is a forwarded-to number of length greater than 15 digits and less or equal to 28 digits.

## 1.3	Normal procedures with successful outcome

### 1.3.1	Provision

The supplementary service will be provisioned for all Basic Services subscribed to and to which it is applicable.

The service can be offered with several subscription options. Options apply to all Basic services subscribed to. For each subscription option, only one value can be selected.

Subscription options	Value

Calling subscriber receives 	- No

notification the call has been forwarded	- Yes

MSISDN of the served subscriber can be	- No

presented to the forwarded-to subscriber	- Yes

### 1.3.2	Withdrawal

The service will be withdrawn at the subscriber's request or for administrative reasons.

### 1.3.3	Registration

The following information has to be registered in the network:

1)	the forwarded-to number, which may be accompanied by a forwarded-to sub-address;

2)	information as to whether all calls or all calls of one (or several) specific Basic service groups should be forwarded.

If no Basic Service code is inserted by the user this is interpreted as All Basic Services.

Registration can take place either by the service provider or with an appropriate control procedure by the subscriber.

Verification, where possible, of the forwarded-to number should be accomplished before accepting the call forwarding request. This verification is done by a simple check of the forwarded-to number to see if the number is within the allowed number range.

When the mobile subscriber so registers call forwarding unconditional, the network will return notification of acceptance or rejection of the request. This notification will include the forwarded-to number to which call forwarding unconditional is registered.

### 1.3.4	Erasure

A previous registration can be erased in either of three ways:

Firstly, the subscriber can specifically erase a previous registration with an appropriate control procedure.

When call forwarding unconditional is active and operative, the ability of the served mobile subscriber to originate calls is not affected. However, an indication that the call forwarding service is currently active and operative on a number will be given to the forwarding party each time an outgoing call is made.

As a subscription option, the served mobile subscriber can request that the calling subscriber receives a notification that the call has been forwarded.

NOTE:	In case the notification is implemented using intermediate tones or announcements, the tones and/or announcements to the calling party should not be provided if the information transfer capability is set to UDI.

### 1.3.11	Interrogation

Data request

The data request procedure enables the mobile subscriber to obtain information about the data stored in the PLMN. After having requested this procedure the network shall return the following information:

-	in response to a general data request the served mobile subscriber should be given a list of all Basic service groups to which call forwarding unconditional is registered, including information whether or not it is active and operative, and the associated forwarded-to numbers;

-	in response to a specific request concerning one particular Basic service group, the served mobile subscriber should be informed whether or not call forwarding unconditional is registered, including information whether or not it is active and operative for that Basic service group. If CFU is registered, the associated forwarded-to number shall be given.

### 1.3.12	Charging requirements

The forwarding subscriber may be charged for the forwarded part of the call.

## 1.4	Exceptional procedures or unsuccessful outcome

### 1.4.2	Registration

If the system cannot accept a registration request, the served mobile subscriber should receive a notification that call forwarding unconditional registration was not successful. Possible causes are:

-	service not subscribed to;

-	forwarded-to number is invalid directory number;

-	use of an operator access prefix;

-	insufficient information;

-	forwarded-to number is a special service code (e.g. police);

-	forwarded-to number is the directory number of the mobile subscriber herself;

-	conflicting situation with other supplementary services (e.g. incoming call barring has been activated);

- no basic service provisioned within the basic service group(s);

- no support for long forwarded-to numbers.

The network is not required to validate information related to the forwarded-to number.

### 1.4.6	Invocation

Within a PLMN or different PLMNs the number of tandem forwarding connections should be limited. The maximum number of tandem forwarding connections should be limited to a value between 1 and 5. This is to prevent infinite looping.

If the limit of successive forwardings of a call has already been reached, an unsuccessful call set up indication is sent backwards.

If the forwarded call cannot be completed to the forwarded-to destination, then the network will clear the forwarded part of the call and an unsuccessful call set up indication is sent backwards.

NOTE:	Call forwarding unconditional applies only to the Basic services subscribed to. Calls to a directory number requesting a Basic service which is not subscribed to will never be forwarded.

### 1.4.7 Interrogation

It is possible that a user has registered a long forwarded-to number but has an MS that does not support long forwarded-to numbers. For this case if the user interrogates call forwarding then the service state shall be indicated with the last 16 digits of the number being displayed.

## 1.5	Alternate procedures

### 1.5.1 Non-support of Long Forwarded-to Numbers

If the HPLMN receives a request for routeing information from a VPLMN which does not support long forwarded-to numbers and the destination user has registered a long forwarded-to number, the HPLMN shall:

- Supply an alternative forwarded-to number of less than 16 digits, or

- Instruct the VPLMN to release the call with a suitable error code, e.g. forwarding violation

## 1.6	Interactions with other supplementary services

#### 1.6.81.1	Calling line identification presentation

See GSM 02.81.

#### 1.6.81.2	Calling line identification restriction

See GSM 02.81.

#### 1.6.81.3	Connected line identification presentation

See GSM 02.81.

#### 1.6.81.4	Connected line identification restriction

See GSM 02.81.

### 2.2.2	Applicability to telecommunication services

The applicability of this supplementary service is defined in TS 22.004.

### 2.2.3	Terminology

A served mobile subscriber is a mobile subscriber of a particular PLMN access who is requesting that calls to her number be forwarded. This subscriber may also be referred to as the forwarding subscriber or the called subscriber.

A forwarded-to subscriber is a subscriber to whom the call shall be forwarded. . A long forwarded-to number is a forwarded-to number of length greater than 15 digits and less or equal to 28 digits.

## 2.3	Normal procedures with successful outcome

### 2.3.1	Provision

The supplementary service will be provisioned for all Basic Services subscribed to and to which it is applicable.

The service can be offered with several subscription options. Options apply to all Basic services subscribed to. For each subscription option, only one value can be selected.

Subscription options are summarized below:

Subscription options	Value

Forwarding subscriber receives 	- No

notification that the call has been forwarded	- Yes

Calling subscriber receives 	- No

notification that the call has been forwarded	- Yes

MSISDN of the served subscriber can be	- No

presented to the forwarded-to subscriber	- Yes

In case of User Determined User Busy (UDUB) a notification towards the forwarding subscriber will not be given.

### 2.3.2	Withdrawal

The service will be withdrawn at the subscriber's request or for administrative reasons.

### 2.3.3	Registration

The following information has to be registered in the network:

1)	the forwarded-to number, which may be accompanied by a forwarded-to sub-address;

2)	information as to whether all calls or all calls of one (or several) specific Basic service group(s) should be forwarded.

If no Basic Service Code is inserted by the user this is interpreted as All Basic Services.

Registration can take place either by the service provider or with an appropriate control procedure by the subscriber. Verification, where possible, of the forwarded-to number should be accomplished before accepting the call forwarding

### - Activation

The user shall be allowed to activate Call Forwarding on Busy (CFB) by e.g. using the MMI command described in TS 22.030. This activation request may include information of which Basic Service Group(s) it shall apply to. If no such information is included, the Activation request applies to all Basic Service Groups.

When receiving an activation request the network shall activate CFB for the Basic Service Groups comprised in the BS group information given by the user against which a CFB forwarded-to-number is registered. The Activation(s) shall be in accordance with the rules set out in TS 22.004.

## 2.4	Exceptional procedures or unsuccessful outcome

### 2.4.2	Registration

If the system cannot accept a registration request, the served mobile subscriber should receive a notification that call forwarding on mobile subscriber busy registration was not successful. Possible causes are:

-	service not subscribed to;

-	forwarded-to number is invalid directory number;

-	use of an operator access prefix;

-	insufficient information;

-	forwarded-to number is a special service code (e.g. police);

-	forwarded-to number is the directory number of the mobile subscriber herself;

-	conflicting situation with other supplementary services (e.g. incoming call barring has been activated);

- no basic service provisioned within the basic service groups.

- No support for long forwarded-to numbers

The network is not required to validate information related to the forwarded-to number.

### 2.4.3	Erasure

If the network cannot accept a mobile subscriber's request for erasure, cause will be returned to the subscriber, such as:

-	insufficient information;

-	inconsistent with registration.

If the subscriber does not completely specify which call forwarding on mobile subscriber busy request is to be erased, the network will reject the erasure request with appropriate cause.

If no Basic Service Code is inserted by the user, CF will be erased from all Basic Services.

### 2.4.4	Activation

If there is no CFB forwarded-to-number registered against a Basic Service group, the activation request for that Basic Service group shall be denied.

Examples of causes for a not accepted activation request:

-	service not provisioned;

-	conflicting situation with other supplementary services;

### 2.4.5	Deactivation

If the network cannot accept a deactivation request, an appropriate cause will be returned to the user, e.g. Service not Active.

### 2.4.6	Invocation

Within a PLMN or different PLMNs the number of tandem forwarding connections should be limited. The maximum number of tandem forwarding connections should be limited to a value between 1 and 5. This is to prevent infinite looping.

If the limit of successive forwardings of a call has already been reached, an unsuccessful call set up indication is sent backwards.

If the forwarded call cannot be completed to the forwarded-to destination, then the network will clear the forwarded part of the call and an unsuccessful call set up indication is sent backwards.

### 2.4.7 Interrogation

It is possible that a user has registered a long forwarded-to number but has an MS that does not support long forwarded-to numbers. For this case if the user interrogates call forwarding then the service state shall be indicated with the last 16 digits of the  number being displayed.

## 2.5	Alternate Procedures

### Non-support of Long Forwarded-to Numbers

If a user with a long forwarded-to number registered roams to a VPLMN not supporting long forwarded-to numbers then the HPLMN shall:

- Supply an alternative forwarded-to number of less than 16 digits, or

- Make CFB quiescent

## 3.1	Definition

This service permits a called mobile subscriber to have the network send all incoming calls, or just those associated with a specific Basic service group, addressed to the called mobile subscriber's directory number and which meet no reply to another directory number. The ability of the served mobile subscriber to originate calls is unaffected. If this service is activated, a call is forwarded only if the call meets no reply.

## 3.2	Description

### 3.2.1	Description

The served mobile subscriber can request a different forwarded-to number for each Basic service group containing a basic service to which she has subscribed.

### 3.2.2	Applicability to telecommunication services

The applicability of this supplementary service is defined in TS 22.004.

### 3.2.3	Terminology

A served mobile subscriber is a mobile subscriber of a particular PLMN access who is requesting that calls to her number be forwarded. This subscriber may also be referred to as the forwarding subscriber or the called subscriber.

A forwarded-to subscriber is a subscriber to whom the call shall be forwarded. A long forwarded-to number is a forwarded-to number of length greater than 15 digits and less or equal to 28 digits.

## 3.3	Normal procedures with successful outcome

### 3.3.1	Provision

The supplementary service will be provisioned for all Basic Services subscribed to and to which it is applicable.

The service can be offered with several subscription options. Options apply to all Basic services subscribed to. For each subscription option, only one value can be selected.

Subscription options are summarized below:

Subscription options	Value

Forwarding subscriber receives 	- No

notification that the call has been forwarded	- Yes

Calling subscriber receives 	- No

notification that the call has been forwarded	- Yes

MSISDN of the served subscriber can be	- No

presented to the forwarded-to subscriber	- Yes

### 3.3.2	Withdrawal

The service will be withdrawn at the subscriber's request or for administrative reasons.

## 3.4	Exceptional procedures or unsuccessful outcome

### 3.4.2	Registration

If the system cannot accept a registration request, the served mobile subscriber should receive a notification that call forwarding on no reply registration was not successful. Possible causes are:

-	service not subscribed to;

-	forwarded-to number is invalid directory number;

-	use of an operator access prefix;

-	insufficient information;

-	forwarded-to number is a special service code (e.g. police);

-	forwarded-to number is the directory number of the mobile subscriber herself;

-	conflicting situation with other supplementary services (e.g. incoming call barring has been activated);

- no basic service provisioned within the basic service group(s).

- No support for long forwarded-to numbers

hThe network is not required to validate information related to the forwarded-to number.

### 3.4.3	Erasure

If the network cannot accept a mobile subscriber's request for erasure, cause will be returned to the subscriber, such as:

-	insufficient information;

-	inconsistent with registration.

If the subscriber does not completely specify which call forwarding on no reply request is to be erased, the network will reject the erasure request with appropriate cause.

If no Basic Service Code is inserted by the user, CF will be erased from all Basic Services.

### 3.4.4	Activation

If there is no CFNRy forwarded-to-number registered against a Basic Service group, the activation request for that Basic Service group shall be denied. [However, if there is CFNRy forwarded-to-numbers stored against other Basic Service groups contained in the same activation request, these will be activated.]

Examples of causes for a not accepted activation request:

-	service not provisioned;

-	conflicting situation with other supplementary services;

-	service not registered.

### 3.4.5	Deactivation

If the network cannot accept a deactivation request, an appropriate cause will be returned to the user, e.g. Service not

### 3.4.6	Invocation

Within a PLMN or different PLMNs the number of tandem forwarding connections should be limited. The maximum number of tandem forwarding connections should be limited to a value between 1 and 5. This is to prevent infinite looping.

If the limit of successive forwardings of a call is reached and an attempt is made to forward the call an additional time, then the forwarded call shall be treated as in the following paragraph.

If the forwarded call cannot be completed to the forwarded-to destination, then the network will clear the call towards the calling user with an indication that the call cannot be completed. This indication shall not include information that the call has been forwarded.

NOTE 1:	The alerting of the forwarding user shall have ceased on invocation of the CFNRy Service.

NOTE 2:	The alerting of the calling subscriber should, if possible, not cease on invocation of CFNRy.

NOTE 3:	Call forwarding on no reply applies only to the Basic services subscribed to. Calls to a directory number requesting a Basic service which is not subscribed to will never be forwarded.

### 3.4.7 Interrogation

It is possible that a user has registered a long forwarded-to number but has an MS that does not support long forwarded-to numbers. For this case if the user interrogates call forwarding then the service state shall be indicated with the last 16 digits of the number being displayed.

## 3.5	Alternate Procedures

### 3.5.1 Non-support of Long Forwarded-to Numbers

If a user with a long forwarded-to number registered roams to a VPLMN not supporting long forwarded-to numbers then the HPLMN shall:

- Supply an alternative forwarded-to number of less than 16 digits, or

- Make CFNRy quiescent

outside the home PLMN country.

## 3.7	Interworking considerations

If the forwarded-to number is not a PLMN- or ISDN-number, then an interworking situation is said to exist.

If a forwarded call meets an interworking situation, then an interworking indication should be sent to the calling party. When interworking with non-PLMN or non-ISDN networks, tones and announcements will be required.

NOTE1:	The number of times a call has been forwarded once it has exited the PLMN or ISDN may not be limited.

NOTE2:	Tones and/or announcements should not be provided to the calling party if the information transfer capability is set to UDI.

# 4	Call Forwarding on Mobile Subscriber Not Reachable

## 4.1	Definition

This service permits a called mobile subscriber to have the network send all incoming calls, or just those associated with a specific Basic service group, addressed to the called mobile subscriber's directory number, but which is not reachable, to another directory number. The ability of the served mobile subscriber to originate calls is principally unaffected, but practically it is affected if the mobile subscriber is de-registered, if there is radio congestion or if the mobile subscriber for example is being out of radio coverage. If this service is activated, a call is forwarded only if the mobile subscriber is not reachable.

## 4.2	Description

### 4.2.1	Description

The served mobile subscriber can request a different forwarded-to number for each Basic service group containing a basic service to which he has subscribed.

### 4.2.2	Applicability to telecommunication services

The applicability of this supplementary service is defined in TS 22.004.

### 4.2.3	Terminology

A served mobile subscriber is a mobile subscriber of a particular PLMN access who is requesting that calls to his number be forwarded. This subscriber may also be referred to as the forwarding subscriber or the called subscriber. A forwarded-to subscriber is a subscriber to whom the call shall be forwarded. A long forwarded-to number is a forwarded-to number of length greater than 15 digits and less or equal to 28 digits.

## 4.4	Exceptional procedures or unsuccessful outcome

### 4.4.2	Registration

If the system cannot accept a registration request, the served mobile subscriber should receive a notification that call forwarding on mobile subscriber not reachable registration was not successful. Possible causes are:

-	service not subscribed to;

-	forwarded-to number is invalid directory number;

-	use of an operator access prefix;

-	insufficient information;

-	forwarded-to number is a special service code (e.g. police);

-	forwarded-to number is the directory number of the mobile subscriber herself;

-	conflicting situation with other supplementary services (e.g. incoming call barring has been activated);

- no basic service provisioned within the basic service group(s).

- No support for long forwarded-to numbers

The network is not required to validate information related to the forwarded-to number.

### 4.4.3	Erasure

If the network cannot accept a mobile subscriber's request for erasure, cause will be returned to the subscriber, such as:

-	insufficient information;

-	inconsistent with registration.

If the subscriber does not completely specify which call forwarding on mobile subscriber not reachable request is to be erased, the network will reject the erasure request with appropriate cause.

If no Basic Service Code is inserted by the user, CF will be erased from all Basic Services.

### 4.4.4	Activation

If there is no CFNRc forwarded-to-number registered against a Basic Service group, the activation request for that Basic Service group shall be denied.

Examples of causes for a not accepted activation request:

-	service not provisioned;

-	conflicting situation with other supplementary services;

-	service not registered.

### 4.4.5	Deactivation

If the network cannot accept a deactivation request, an appropriate cause will be returned to the user, e.g. Service not Active.

4.4.4

### 4.4.6	Invocation

Within a PLMN or different PLMNs the number of tandem forwarding connections should be limited. The maximum number of tandem forwarding connections should be limited to a value between 1 and 5. This is to prevent infinite looping.

If the limit of successive forwardings of a call has already been reached, an unsuccessful call set up indication is sent backwards.

If the forwarded call cannot be completed to the forwarded-to destination, then the network will clear the forwarded part of the call and an unsuccessful call set up indication is sent backwards.

NOTE:	Call forwarding on mobile subscriber not reachable applies only to the Basic services subscribed to. Calls to a directory number requesting a Basic service which is not subscribed to will never be forwarded.

### 4.4.7 Interrogation

It is possible that a user has registered a long forwarded-to number but has an MS that does not support long forwarded-to numbers. For this case if the user interrogates call forwarding then the service state shall be indicated with the last 16 digits of the number being displayed

## 4.5 Alternate Procedures

### 4.5.1 Non-support of Long Forwarded-to Numbers

If a user with a long forwarded-to number registered roams to a VPLMN not supporting long forwarded-to numbers then the HPLMN shall:

- Supply an alternative forwarded-to number of less than 16 digits, or

- Make CFNRc quiescent

If the HPLMN receives a request for routeing information from a VPLMN which does not support long forwarded-to numbers and the destination user has registered a long forwarded-to number and the HPLMN determines that the called mobile subscriber is not reachable, the HPLMN shall:

- Supply an alternative forwarded-to number of less than 16 digits, or

- Instruct the VPLMN to release the call with a suitable error code, e.g. forwarding violation