3GPP TSG SA WG 1 Meeting #102 	S1-231018

Berlin, Germany, 22-26 May 2023

Source:	ZTE

pCR Title:	Annex A1 – List of Potential Requirements

Draft Spec:	3GPP TR 22.851 V1.1.0

Agenda item:	7.4

Document for:	Approval

Contact:	Yvette Koza

Yvette.koza@zte.com.cn

Abstract: Requirement Consolidation-Other aspects on Network Sharing.

1. Introduction

New Annex A2 added

2. Reason for Change

List of defined potential requirements

3. Conclusions

None.

4. Proposal

It is proposed to approve the following changes to 3GPP TR 22.851 V1.1.0.

* * * First Change * * * *

Annex A1 (informative):
Summary of Potential Pequirements

UC1:	Network sharing without direct connections between the shared access and the core networks of the participating operators (Sect 5.1)

[PR 5.1.6-001] The 5G system shall be able to support network sharing with indirect connection between the shared access network and one or more participating operators’ core networks.

[PR 5.1.6-002] The 5G system shall be able to support means for participating operators to provide their operator name to a registered UE, for display to the user.

UC2:	Service Continuity and QoS (Sect 5.2)

[PR 5.2.6-001] The 5G core network shall be able to support collection of charging information associated with a UE accessing a shared network using indirect network sharing.

[PR 5.2.6-002] In case of Indirect Network Sharing and subject to hosting and participating operators’ policies, the 3GPP system shall support service continuity for UEs that are moving between different Shared RANs or between a Shared RAN and a non-shared RAN (managed by hosting and participating operators).

[PR 5.2.6-003] In case of Indirect Network Sharing and subject to hosting and participating operators’ policies, the 3GPP system shall be able to minimize the impact to the user experience (e.g. QoS, QoE) of UEs in an active communication moving between different Shared RANs or between a Shared RAN and a non-shared RAN (managed by hosting and participating operators).

[PR 5.2.6-004] In case of Indirect Network Sharing, the 5G system shall support a mechanism for a UE to access the subscribed PLMN services when entering a Shared RAN.

UC3:	Network access control and mobility between sharing parties (Sect 5.3)

[PR 5.3.6-001] In case of indirect network sharing, the 3GPP system shall support mechanisms to minimize service interruptions for UEs that are moving between different Shared RANs or between a Shared RAN and a non-shared RAN (managed by hosting and participating operators).

[PR 5.3.6-002] In case of Indirect Network Sharing and subject to hosting and participating operators’ policies, the 3GPP system shall support access control for a UE accessing a shared network.

[PR 5.3.6-003] In case of Indirect Network Sharing and subject to hosting and participating operators’ policies, the 3GPP system shall be able to select an appropriate radio access network (i.e. 4G, 5G) for a UE.

[PR 5.3.6-004] In case of Indirect Network Sharing and subject to hosting and participating operators’ policies, the 3GPP system shall support a mechanism to enable a UE with a subscription to a participating operator to access an authorized shared network

UC4:	International Roaming Users in a Shared Network (Sect 5.4)

[PR 5.4.6-001] The 5G system shall enable the shared access network of a hosting operator with indirect connection between the shared access network and a participating operator’s core network to provide services for inbound roaming users.

UC5:	Hosted Services (Sect 5.5)

[PR 5.5.6-001] In case of Indirect Network Sharing, the 5G system shall support a mechanism to enable a UE subscribed to the participating operator to access the participating operator’s Hosted Services.

UC6:	Emergency call (Sect 5.6)

[PR 5.6.6-001] Subject to national or regional regulatory and operator policies, the 5G system shall support emergency call from a UE accessing a shared network via Indirect Network Sharing.

[PR 5.6.6-002] Subject to national or regional regulatory and operator policies, the 5G system shall provide to the participating operator the location information of an emergency caller accessing a shared network via Indirect Network Sharing.

UC7:	Long-distance mobility in and across shared networks (Sect 5.7)

[PR 5.7.6-001] Subject to the agreement between hosting operators and a participating operator, the 5G system shall support a mechanism to authorize UE with a subscription to the participating operator to obtain services in a Shared RAN of hosting operators using Indirect Network Sharing.

Editor’s Note: Requirement #001 is FFS.

[PR 5.7.6-002] In case of Indirect Network Sharing, based on the hosting and participating operators’ policies, the 5G system shall enable the participating operator to provide steering information to the hosting operator.

NOTE 1: Such steering information for a given UE of the participating operator could be applied by the hosting operator to select amongst the hosting operator’s available shared access network(s).

[PR 5.7.6-003] In case of Indirect Network Sharing, the 5G system shall be able to apply different access control for different access networks of Shared RANs based on the hosting and participating operator’s agreement and policies.

UC8:   Support of PWS in Shared NG-RAN in Indirect Interconnection with the Participating Operators (Sect 5.8)

[PR 5.8.6-001] Subject to regulatory requirements and mutual agreement between the participating operators and the hosting operator, the 5G system shall be able to support PWS when connectivity is provided via Indirect Network Sharing.

[PR 5.8.6-002] Subject to regulatory requirements and mutual agreement between the participating operators and the hosting operator, the shared NG-RAN in Indirect Network Sharing shall be able to broadcast PWS messages originated from the core network of the hosting operator.

* * * Next Changes * * * *

Annex A2 (informative):
Change history

* * * End of Changes * * * *