3GPP TSG SA WG 1 Meeting #102 	S1-231048

Berlin, Germany, 22-26 May 2023	(revision of S1-23XXXX)

Source:	China Unicom, Charter Communications?, ZTE Corporation? and CATT?

pCR Title:	Updated Consolidation on general and charging reqs

Draft Spec:	3GPP TR 22.851 V1.1.0

Agenda item:	7.4

Document for:	Approval

Contact:	Qun wei

Weiqun5@chinaunicom.cn

Abstract: Consolidation of requirements on general and charging aspects from the use cases on Network Sharing.

1. Introduction

New definitions were introduced and requirements categories was discussed since last meeting. The consolidation of general aspects of this pCR includes the following parts:

- General aspects // basic capability

[PR 5.1.6-001], [PR 5.1.6-002]

-General aspects // Service accessibility

[PR 5.2.6-004], [PR 5.5.6-001]

-Charging // or target for 22.261 Chapter 9

[PR 5.2.6-001]

2. Reason for Change

To consolidate requirements. The large consolidation document was discussed at the last meeting, which has been divided into parts to make the entire process more effective.

3. Conclusions

None.

4. Proposal

It is proposed to approve the following changes to 3GPP TR 22.851 V1.1.0.

* * * Copy of 22.851 v110 for reference * * * *

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Indirect Network Sharing: describes the communication between the shared access NG-RAN and the Participating NG-RAN Operator’s core network being routed through the Hosting NG-RAN Operator’s core network.

Hosted Service: a service containing the operator's own application(s) and/or trusted third-party application(s) in the Service Hosting Environment, which can be accessed by the user.

Service Hosting Environment: the environment, located inside of 5G network and fully controlled by the operator, where Hosted Services are offered from.

Hosting NG-RAN Operator: the operator that has operational control of a shared NG-RAN.

NOTE 1: Hosting NG-RAN Operator can also be a Hosting RAN Operator. See 3GPP TS 22.101 [2].

Participating NG-RAN Operator: authorized operator that is sharing NG-RAN resources provided by a Hosting NG-RAN Operator.

NOTE 2: Participating NG-RAN Operator can also be participating operator. See 3GPP TS 22.101 [2].

Shared NG-RAN: NG-RAN that is shared among a number of operators.

* * * First Change * * * *

7.2	Consolidated Potential Requirements

7.2.1	Introduction

The following consolidated requirements are specified. Different aspects related network sharing with indirect connection between the hosting and the participating operators are specially taken into account. See Tables below.

7.2.2	General aspect

Table 7.2.2-1 General requirement

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.2.2-001 | The 5G system shall be able to support Indirect Network Sharing between the Shared NG-RAN and one or more Participating NG-RAN Operators’ core networks, by means of the connection being routed through the Hosting NG-RAN Operator’s core network. | [PR 5.1.6-001] The 5G system shall be able to support network sharing with indirect connection between the shared access network and one or more Participating Operators’ core networks | Align with the definitions. |
| CPR 7.2.2-002 | The 5G system shall be able to support means for Participating NG-RAN Operator to provide their operator name to a registered UE, for display to the user. | [PR 5.1.6-002] The 5G system shall be able to support means for participating operators to provide their operator name to a registered UE, for display to the user. | Align with the definitions. participating operators->Participating NG-RAN Operator |
| CPR 7.2.2-003 | In case of Indirect Network Sharing the registered UEs shall be able to access the subscribed PLMN services when entering a Shared NG-RAN. | [PR 5.2.6-004] In case of Indirect Network Sharing, the 5G system shall support a mechanism for a UE to access the subscribed PLMN services when entering a Shared RAN. | Align with the definitions. Updated wording |
| CPR 7.2.2-004 | In case of Indirect Network Sharing, the 5G system shall support a mechanism to enable a UE subscribed to the Participating NG-RAN Operator to access the participating operator’s Hosted Services. | [PR 5.5.6-001] In case of Indirect Network Sharing, the 5G system shall support a mechanism to enable a UE subscribed to the participating operator to access the participating operator’s Hosted Services. | Align with the definitions. participating operators->Participating NG-RAN Operator |

Editor’s Note: Consolidated requirements for further aspects are FFS.

* * * Second Change * * * *

7.2.X	Charging aspects

Table 7.2.X-1 Charging requirement

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.2.3-001 | The 5G core network shall be able to support collection of charging information associated with a UE accessing a shared network using Indirect Network Sharing. | [PR 5.2.6-001] The 5G core network shall be able to support collection of charging information associated with a UE accessing a shared network using indirect network sharing. | Align with the definitions. |

* * * End of Changes * * * *