3GPP TSG SA WG 1 Meeting #102 	S1-231249

Berlin, Germany,  22 - 26 May 2023	(revision of S1-23xxxx)

Source:	Xiaomi

pCR Title:	Pseudo-CR on the additional security requirement in the use case 5

Draft Spec:	3GPP TR 22.840 v1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Chunhui Zhu, zhuchunhui@xiaomi.com

Abstract: It is proposed to add the security requirement in the use case 5.

1. Introduction

There is a FFS in the use case 5 (intralogistics in automobile manufacturing) in TR 22.840 v1.1.0:

Editor’s Note: 	Additional requirements for this use case are FFS.

2. Reason for Change

Since the Ambient IoT device is solely dependent on harvested ambient energy, with battery-less or limited energy storage capability, extremely small, thin, and of low complexity, very light-weight, and with a long lifespan, how to reduce the unnecessary 3GPP operation including 3GPP security operation should be considered in the Ambient IoT device.

The security requirement should consider the real deployment use case. In use case 5 (intralogistics in automobile manufacturing), the Ambient IoT device is only used in the intralogistics for production facilities in automobile factories, it means the location of using the device is determined and the environment can be under strictly controlled by the automobile factory’s operator. In such a closed and controllable location and environment, the security risk and issue can be managed and minimized by the automobile factory’s operator. Then based on the agreement and policy from the Ambient IoT network and automobile factory’s operator, the unnecessary 3GPP security operation in the Ambient IoT device can be skipped in the intralogistics in automobile manufacturing.

3. Conclusions

It is proposed to add the security requirement in the use case 5 in TR 22.840 v1.1.0.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.1.0.

* * * First Change * * * *

5.5	Use case on intralogistics in automobile manufacturing

5.5.1	Description

The automobile manufacturing industries are constantly looking for ways to increase productivity by improving inventory accuracy and material flows. Therefore, intralogistics for production facilities in automobile factories have been targeting at these goals by achieving timely visibility of inventories (a.k.a. precise materials scheduling) to achieve optimum efficiency in production. Specifically, it involves stocking, dispatching, and sorting. It is no surprise that there are competing technologies for supporting small-scale inventory, which at times would require human involvement. With the advent of Industry 4.0, manufacturing requires a much higher automatic intralogistics performance [10]. This means more key areas inside large manufacturing facilities would require highly-efficient and automated inventory of materials and parts. Ambient power-enabled IoT (Ambient IoT) service provided by 5G can be expected to meet the demanding needs by providing communication to Ambient IoT devices with good performance, with the Ambient IoT solely dependent on harvested ambient energy, being maintenance-and-battery-less or with limited energy storage capability, extremely small, thin, and of low complexity, very light-weight, and with a long lifespan. For this use case, communication service availability with sufficient 5G network coverage in the service area is important. Additionally, the 5G system can provide Ambient IoT devices with positioning services, as full automation would require AGVs or forklifts to fetch materials at accurate locations within the large storage areas. Moreover, the 5G system provides intrinsic core network functions to manage and authenticate Ambient IoT connections, enabling operators to build more interesting cases together with their business partners.

Automobile company A uses standardized load containers to realize flow of materials. The load containers are purchased, owned and managed by Company A. With its business growing, Company A is integrating the latest and advanced production and logistics management systems to ensure the production facilities are modernized for Industry 4.0. In quest of this, intralogistics at Company A is expected to automatically identify and track individual goods and materials throughout production facilities: not only at the dock where materials enter the production facilities, but also in the large floor-level storage areas, further in the sorting areas and down at production lines. Similar to the norm of the automobile manufacturing industry, a typical production facility of Company A covers a total area of around 600,000 square meters. Company A uses unique identifiers (e.g. 96 bit or 240 bit EPC codes) to distinctly identify load containers.

Fig. 5.5.1-1: Intralogistics in automobile manufacturing

Prior to deciding on a future-proof solution, a comprehensive analysis (e.g., ROI) is carried out by Company A to verify Ambient IoT’s suitability for the long-term business objectives. By virtue of many of Ambient IoT’s attractive wireless communication characteristics, Company A came to the conclusion of a positive case. To effectively meet their intralogistics demand, it entails installing around 1300 stationary “readers” strategically per typical production facility (600,000 square meters), where in total around 800,000 Ambient IoT devices would be physically present at the same time. To keep track of the production process, normally 20 Ambient IoT devices need to be read per second per “reader” (e.g., fast-moving AGVs passing a gate). After verifying with operator O, Company A adopts Ambient IoT by having a service contract with the operator, which is commissioned to design and deploy 5G network coverage within Company A’s automobile production facilities and then to accordingly enable and manage Ambient IoT service. Wherever needed in the production facilities, sufficient network coverage is assumed.

It is in the huge floor storage area where the loaded containers are kept, accurate positioning service is needed by AGVs for pick-up, once certain materials or parts are needed by a production line. At Company A, the floor storage area is divided into blocks with the dimension of 18m by 18m, in each of which a stationary “reader” is deployed. Each block is further divided into grids of the size slightly larger than a load container, whose length is around 1.5m. To achieve uninterrupted material flow, Company A decide to use the space of two grids for parking load containers holding the same materials. In this way, as a load container holding certain material at a given grid is carried away by an AGV, the same material loaded in another container would always be available at the grid just next to it.

The personnel Sylvain at Company A attaches each of the load containers with an Ambient IoT device, where a unique code is written in its microchip. Ambient IoT devices are very thin non-battery powered or with limited energy storage capability IoT devices, which instead function using an energy harvesting mechanism that produces a limited amount of power. A typical energy harvesting device can produce up to a few hundred microwatts [11] (e.g., less than 250 micro-Watts). In the production facilities of Company A, an Ambient IoT device can be connected to the 5G network and the communication is enabled.

5.5.2	Pre-conditions

An Ambient IoT device can obtain energy by collecting energy sources such as solar and radio waves. Each load container is attached with an Ambient IoT device for supporting intralogistics. Ambient IoT devices have the capability of storing information needed by the inventory process. Base stations installed inside car manufacturing facilities provide network coverage. The communication service availability is achieved by providing seamless 5G network coverage inside the service areas. The 5G system delivers application data to the intralogistics management system of the car manufacturer. For saving the power and resource in the Ambient IoT device, based on the agreement and policy from the Ambient IoT network and automobile factory’s operator, the unnecessary 3GPP security operation in the Ambient IoT device can be skipped.

5.5.3	Service Flows

- Company A uses Ambient IoT devices attached to their load containers to support automated intralogistics process for improved productivity. They send load containers to various suppliers in order to bring back ordered goods, materials and parts.

- By returning to Company A’s automobile manufacturing facilities, large amount of load containers first enter the dock area. There the 5G network transmits signals intending to start inventory process. Since the Ambient IoT devices harvest power from the environment, once detecting the signals from the 5G network the Ambient IoT devices can respond by starting random access.

- 5G network (i.e., core network) authenticates and authorizes Ambient IoT devices.

- Based on the requests issued by the application function, the 5G network performs the automatic inventory operation (read out pre-written unique IDs in Ambient IoT devices) of the large number of incoming load containers entering the dock area fast and efficiently. The read-out information is sent by 5GC to Company A’s inventory system (connected to ERP/APS), where the inventoried information is updated or saved.

- Once registered in the inventory system, load containers are stored in the floor storage areas. The load containers (still loaded with various materials and parts) can be inventoried by Company A either periodically or when requested.

- As automobile manufacturing process continues (almost non-stop), a picking list is automatically generated according to the production plan managed by the APS (Advanced Planning and Scheduling) system.

- According to the information in the inventory system, AGVs (or forklifts) with their respective picking lists are sent to the floor storage areas to pick up the corresponding load containers (holding different materials or parts). As the floor storage area is extremely huge, Company A utilize the 5G network positioning service for AGVs to quickly fetch the target load containers holding the exact materials requested in the pick-up list. Since the floor storage area is divided into 18m by 18m blocks with each block being further divided into grids of 1.5m by 1.5m, the 5G system always provides the AGVs with the accurate positioning information of the target load containers.

- When AGVs find the needed load containers in the floor storage area, convey them further to the sorting areas. It is in the sorting areas that different materials and parts brought in by AGVs are grouped in accordance with precise manufacturing schedules to be followed at the production lines.

- At the production lines, materials and parts as logically grouped (by sorting machines at the sorting areas) are eventually fed into production. The empty load containers will be brought to the recycling area. These empty load containers await to be sent to suppliers at the next schedule determined by ERP/APS.

- In all the key areas (floor storage areas, sorting areas, production lines and empty container recycling area) inside the manufacturing facility, the load containers need to be inventoried efficiently, timely, fast, accurately. When AGVs enter these areas, step 4 is repeated and the precise material scheduling is updated in the inventory system.

5.5.4	Post-conditions

Thanks to the Ambient IoT service provided by the 5G system, automobile manufacturing can enjoy automatic intralogistics, largely improve the efficiency and productivity.

5.5.5	Existing features partly or fully covering the use case functionality

SA1 has performed various studies on IoT in previous releases, where related normative stage 1 requirements are introduced in TS 22.011 [9], TS 22.278 [7], TS 22.368 [6], and TS 22.261 [8].

TS 22.011 introduces access control for MTC, examples of periodic network selection attempts are:

For UEs only supporting any of the following, or a combination of, NB-IoT, GERAN EC-GSM-IoT [18], and  Category M1[13] of E-UTRAN enhanced-MTC, the UE shall interpret the interval value to be between 2 and 240 hours, with a step size of 2 hours between 2 and 80 hours and a step size of 4 hours between 80 and 240 hours.

In the absence of a permitted value in the SIM/USIM, or the SIM/USIM is phase 1 and therefore does not contain the datafield, then a default value of 60 minutes, shall be used by the UE except for those UEs only supporting any of the following, or a combination of: NB-IoT, GERAN EC-GSM-IoT [18], and Category M1 [17] of E-UTRAN enhanced-MTC. For those UEs a default value of 72 hours shall be used.

NOTE:	Use of values less than 60 minutes may result in excessive UE battery drain.

TS 22.368 addresses features of MTC communication and service requirements related to MTC device triggering, addressing, identifiers, low mobility, small data transmission, infrequent MT communication, security, remote MTC device management, group-based MTC features including policing and addressing, etc. Example requirements are:

The system shall provide mechanisms to lower power consumption of MTC Devices.

The system shall provide mechanisms for the network operator to efficiently manage numbers and identifiers related to MTC Subscribers.

TS 22.261 captures some important service requirements for IoT, e.g.

The 5G system shall support a secure mechanism for a home operator to remotely provision the 3GPP credentials of a uniquely identifiable and verifiably secure IoT device.

The 5G system shall support a secure mechanism for the network operator of an NPN to remotely provision the non-3GPP identities and credentials of a uniquely identifiable and verifiably secure IoT device.

An IoT device which is able to access a 5G PLMN in direct network connection mode using a 3GPP RAT shall have a 3GPP subscription.

The 5G system shall allow the operator to identify a UE as an IoT device based on UE characteristics (e.g. identified by an equipment identifier or a range of equipment identifiers) or subscription or the combination of both.

An IoT device which is able to connect to a UE in direct device connection mode shall have a 3GPP subscription, if the IoT device needs to be identifiable by the core network (e.g. for IoT device management purposes or to use indirect network connection mode).

The 5G system shall support operator-controlled alternative authentication methods (i.e. alternative to AKA) with different types of credentials for network access for IoT devices in isolated deployment scenarios (e.g. for industrial automation).

The 5G system shall support a suitable framework (e.g. EAP) allowing alternative (e.g. to AKA) authentication methods with non-3GPP identities and credentials to be used for UE network access authentication in non-public networks.
           NOTE:	Non-public networks can use 3GPP authentication methods, identities, and credentials for a UE to access network. Non-public networks are also allowed to utilize non-AKA based authentication methods such as provided by the EAP framework, for which the credentials can be stored in the ME.

The 5G system shall enable an NPN to be able to request a third-party service provider to perform NPN access network authentication of a UE based on non-3GPP identities and credentials supplied by the third-party service provider.

In these specifications, albeit the service requirements addressing traits for IoT in terms of low device power consumption, small and infrequent data transmissions, long service lifetime, and resource efficiently, the IoT devices considered in 3GPP have been assumed to be powered by at least batteries up till now. To enable extremely small, thin, light-weight, battery-less or with limited energy storage capability, or even disposable Ambient IoT devices that provide basic IoT data transaction at appropriate performance level suitable for the target scenarios, new challenges to the 5G system are foreseen and need to be addressed.

5.5.6	Potential New Requirements needed to support the use case

[PR 5.5.6-001] The 5G system shall support communication for an Ambient IoT device which is battery-less or with limited energy storage capability.

[PR 5.5.6-002] The 5G system shall support collection of charging information based on different charging policies for Ambient-IoT type of communication, i.e., total number of communication (e.g. data payload) per charging period, or total number of Ambient IoT devices per charging period.

[PR 5.5.6-003] Based on the agreement and policy from the operator and user, the 5G system shall support skipping the unnecessary 3GPP security operation in the Ambient IoT device in the certain location.

[PR 5.5.6-004] The 5G system shall provide the network connection to address the KPIs for the use of Ambient IoT devices for intralogistics in automobile manufacturing, see table 5.3.6.1-1.

Table 5.5.6-1: Potential key performance requirements for the use of Ambient IoT devices for intralogistics in automobile manufacturing

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Automatic Intralogistics in automobile manufacturing | >100 ms (note 1) | 99% | NA | <1 kbit/s (note 2) | 96 bits (note 3) | <1,5 Million/km2 (note 4) | <30 meters Indoors | 600 000 m2 (note 5) | NA | NA | NA | NA | [3] m |
| NOTE 1: This value corresponds to peak reading rate of 100 tags per second. The average tag reading rate is lower. NOTE 2: This value is calculated as the instant data rate for transmitting 96 bits within 100 ms time period. The need for data transmission is infrequent. NOTE 3: EPC Tag Data standard [5], the length of the EPC number ranges from 96 bits to 496 bits. For intralogistics, EPC length of 96 bits is the most common EPC lengths to satisfy the use case. NOTE 4: Daily around 1 million units of materials are used in the manufacturing area, but they are not used at the same time. NOTE 5: A typical car manufacturing plant takes up to 600 000 m2 in surface. |  |  |  |  |  |  |  |  |  |  |  |  |  |