3GPP TSG SA WG 1 Meeting #102 	S1-231383

Berlin, Germany, 22 - 26 May 2023	(revision of S1-231087)

Source:	Novamint, Huawei, ETRI, Nokia, Gatehouse, China Telecom

pCR Title:	Pseudo-CR on consolidated requirements for TR 22.865

Draft Spec:	3GPP TR 22.865 1.0.0

Agenda item:	7.7

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document includes a text proposal for section Consolidated Requirements of TR 22.865, capturing a set of final consolidated potential requirements, based on associated discussion paper and proposal in S1-231086.

1. Introduction

The “Consolidated Requirements” of TR 22.865 are currently not included in version 1.0.0

2. Reason for Change

adding texts and tables for section Consolidated Requirements of TR 22.865, capturing a set of final consolidated potential requirements, based on associated discussion paper and proposal in  S1-231086r06.

The present document has been updated to merge the documents S1-231072 & S1-231224.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.865 v1.0.0.

===============================  All new Text ================================

# 6	Consolidated Requirements

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to the 5G system with satellite access.

## 6.2	Store & Forward Satellite operation

The potential requirements corresponding to the support of Store & Forward Satellite information are listed in the table below

Table 6.2-1 – Consolidated Requirements for Store & Forward Satellite operation

| CPR # |  | Consolidated Potential Requirement |  | Original PR # |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CPR 6.2-1 |  | Subject to operator’s policies, a 5G system with satellite access shall be able to support Store & Forward Satellite operation for authorized UEs e.g. store data on the satellite when the feeder link is unavailable; and forward the data when feeder link between the satellite and the ground segment is available. |  | [PR 5.1.6-001] [PR.5.3.6-001] [PR 5.4.6-001] |  |  |  |
| CPR 6.2-2 |  | A 5G system with satellite access shall be able to inform a UE whether Store & Forward Satellite operation is applied. |  | [PR 5.1.6-002] |  |  |  |
| CPR 6.2-3 | A 5G system with satellite access shall be able to inform a trusted application server or 3rd party application whether Store & Forward Satellite operation is applied for communication with a UE. |  | [PR 5.2.6-001] |  |
| CPR 6.2-4 |  | Subject to operator’s policies, a 5G system with satellite access supporting Store and Forward Satellite operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE and/or satellite basis, an S&F data retention period. |  | [PR 5.1.6-003] [PR 5.2.6-002] |  |  |  |
| CPR 6.2-5 |  | Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE and/or satellite basis, an S&F data storage quota. |  | [PR 5.1.6-004] [PR 5.2.6-003] [PR.5.3.6-003] [PR 5.4.6-004] |  |  |  |
| CPR 6.2-6 |  | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to configure and provision specific required QoS and policies for UE’s data subject to store and forward operation (e.g. forwarding priority, acknowledgment policy). |  | [PR 5.1.6-005] [PR 5.2.6-004] |  |  |  |
| CPR 6.2-7 |  | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to inform an authorized UE about how long the data will be stored before being delivered. |  | [PR.5.16.6-001] |  |  |  |

## 6.3	UE-Satellite-UE communication

The potential requirements corresponding to the support of UE-Satellite-UE communication (without going through the ground network) are listed in the table below

Table 6.3-1 – Consolidated Requirements for UE-Satellite-UE communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support communication between UEs via one or multiple serving satellites without going through the ground network. NOTE: UEs can be connected via their HPLMN or a VPLMN. | [PR 5.6.6-003] [PR 5.7.6-001] |  |
| CPR 6.3-2 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support service continuity (with minimum service interruption) of the communication between UEs not going through the ground network when the UE communication path moves between serving satellites (due to the movement of the UE and/or the satellites). | [PR 5.6.6-004] [PR 5.14.6-001] |  |
| CPR 6.3-3 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity (with minimum service interruption) of the communication between UEs not going through the ground network when the communication path between UEs extends from one to several satellites (through inter satellite links). | [PR 5.14.6-002] |  |
| CPR 6.3-4 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide QoS control of the communication between UEs using satellite access without going through the ground network. | [PR 5.8.6-001] |  |
| CPR 6.3-5 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support different types of communication (e.g. services including unicast, multicast, broadcast) using satellite access without going through the ground network. | [PR 5.8.6-002] |  |

| CPR 6.3-6 | Subject to the operator’s policy, a 5G system shall be able to support two UEs using different satellite access paths to communicate with each other without going through the ground network. (e.g. one UE using a direct satellite connection path, and one UE using a connection via a terrestrial relay with satellite access ) | [PR 5.10.6-002] |  |
| --- | --- | --- | --- |

## 6.4	GNSS independent operation & positioning enhancements for satellite access

The potential requirements corresponding to the support of GNSS independent operation & positioning enhancements for satellite access are listed in the table below

Table 6.4-1 – Consolidated Requirements for GNSS independent operation & positioning enhancements for satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4-1 | Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide services to an authorized UE registered to a HPLMN or a VPLMN independently of the UE’s GNSS capability. | [PR 5.11.6-001] [PR 5.11.6-002] |  |
| CPR 6.4-2 | Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to determine the location of a UE using only satellite access (e.g. based on 3GPP positioning technologies, based on the information from reliable and trusted sources) in order to provide services. | [PR 5.11.6-003] | Simplify and align the wording |
| CPR 6.4-3 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support 3GPP positioning methods for UEs using only satellite access. | [PR 5.13.6-001] [PR 5.12.6-001] | Merge of these two PRs Simplify and align the wording |
| CPR 6.4-4 | A 5G system with satellite access shall be able to provide to a UE using only satellite access with information on positioning services (e.g. supported positioning performance). | [PR 5.12.6-002] |  |
| CPR 6.4-5 |  | A 5G system with satellite access shall be able to support negotiation of positioning methods, between UE or network, according e.g. to 3GPP RAT and UE positioning capability, the availability of non-3GPP positioning technologies (e.g. GNSS). |  | [PR 5.13.6-002] |  |  |  |

## 6.5	General and other aspects for satellite access

The potential requirements corresponding to the support of enhancements of general and other aspects of satellite access are listed in the table below

Table 6.5-1 – Consolidated Requirements for general and other aspects of satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6;5-1 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support an efficient communication path and resource utilization for a UE using only satellites access, e.g. to minimize the latencies introduced by satellite links involved. | [PR 5.5.6-001] [PR 5.5.6-002] [PR 5.15.6-001] |  |

| CPR 6.5-2 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite, for network (e.g. terrestrial) planning. | [PR 5.9.6-001] |  |
| --- | --- | --- | --- |

## 6.6	Security aspects

The potential requirements corresponding to the security aspect are listed in the table below.

Table 6.6-1 – Consolidated Requirements for security aspects

| CPR # |  | Consolidated Potential Requirement |  | Original PR # |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CPR 6.6-1 | Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support forwarding of the stored data received from authorized UEs using S&F operation from one satellite to another satellite through Inter-Satellite Links while preserving integrity protection, confidentiality and security of the data. | [PR.5.3.6-002] |  |
| CPR 6.6-2 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support mechanisms to authorize a UE to use the S&F operation when satellite connectivity is intermittently/temporarily unavailable. NOTE: The authorizing entity could be an operator, a user/subscriber (UE owner) or a third party (e.g., a business customer having service level agreement with the operator and interacting with the 5G network via an application server). | [PR.5.3.6-004] [PR 5.4.6-003] [PR 5.4.6-002] |  |
| CPR 6.6-3 | A 5G system with satellite access shall be able to support mechanisms to authorize the communication between UEs using satellite access without going through the ground network, based on e.g., location information and subscription. NOTE: UEs can use satellite access directly or via a terrestrial relay. | [PR 5.6.6-001] [PR 5.10.6-001] [PR 5.1.6-006] [PR 5.2.6-005] |  |

## 6.7	Charging aspects

The potential requirements corresponding to the charging aspects are listed in the table below.

Table 6.7-1 – Consolidated Requirements for charging aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.7-1 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to collect charging information per UE or per application (e.g., number of UEs, data volume, duration, involved satellites). | [PR 5.4.6-005] [PR 5.4.6-006] |  |
| CPR 6.7-2 | A 5G system with satellite access shall be able to collect charging information for a UE registered to a HPLMN or a VPLMN, without going through the ground network. | [PR 5.6.6-002] [PR 5.7.6-002] |  |

FFS:

The following CPR will be considered once their corresponding PRs have been agreed in their respective contribution.

| CPR 6.2-x | Potential revised requirement from S1-231235 Subject to regulatory requirement, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to know the data sent from a UE is for emergency purpose, shall forward the emergency report from an UE as soon as possible, and shall immediately notify the UE about the successful forwarding. | Potential revised PR [PR.5.16.6-001] | Potential revised PR from S1-231235 |
| --- | --- | --- | --- |
| CPR 6.2-x | Potential New requirement from S1-231120 Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support forwarding of the stored data from one satellite to another satellite, which has an available feeder link to the ground network, through Inter-Satellite Links. | Potential new PR [PR.5.1.6-00x] | Potential new PR from S1-231120 Is a complement to CPR 6.2-1 Or addressed by CPR 6.6-1? |
| CPR 6.2-x | Potential New requirement from S1-231117 Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to optimize the accumulated and stored MO messages delivery once the feeder link is available (e.g., priority, timer, timestamp, message size). | Potential new PR [PR.5.1.6-00y] | Potential new PR from S1-231117 |
| CPR 6.2-x | Potential New requirement from S1-231143 [PR 5.2.6-00x] A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to provide the information about the store and forward operation applied to a UE (e.g. how long the data will be stored before being delivered) to a trusted application server or third party application. | Potential new PR [PR 5.2.6-00x] | Potential new PR from S1-231143 |
| CPR 6.2-x | Potential New requirement from S1-231118 [PR 5.2.6-00y] A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to optimize the stored MT messages delivery once the feeder link is available (e.g. priority, timer, timestamp, message size). | Potential new PR [PR 5.2.6-00y] | Potential new PR from S1-231118 |
| CPR 6.2-x | Potential New requirement from S1-231118 [PR 5.2.6-00z] A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support an optimized paging mechanism to deliver stored MT messages to UEs once the service link is available. | Potential new PR [PR 5.2.6-00z] | Potential new PR from S1-231118 |
| CPR 6.2-x | Potential New requirement from S1-231119 A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to inform an active UE with ongoing satellite services of the feeder link status when the feeder link is becoming unavailable. | Potential new PR [PR 5.6.6-005] | Potential new PR from S1-231119 Partially already covered covered by CPR 6.2-1 ? |
| CPR 6.2-x | Potential New requirement from S1-231119 A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to handle ongoing on board satellite traffic without going through the ground when the feeder link is becoming unavailable (e.g., notifying UE of the feeder link status, then release traffic, or keep traffic). | Potential new PR [PR 5.6.6-006] | Potential new PR from S1-231119 Partially already covered covered by CPR 6.2-7? |
| CPR 6.3-x | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support the establishment of a communication path between UEs via one or multiple serving satellites without going through the ground network. | Potential new PR [PR 5.14.6-00x] | Potential new PR from S1-231121 May be merged in CPR 6.3-1 |
| CPR 6.4-x | Potential new PR from S1-231142 A 5G system with satellite access shall be able to provide positioning services in compliance with regulatory requirements (e.g. for public safety services) independently of UE’s GNSS capability when the UE is using only satellite access. The minimum performance shall be compliant with the following KPIs. | Potential new PR [PR 5.13.6-003] | Potential new PR from S1-231142 To decide where to put this PR as there is KPIs To incorporate in CPR 6.4-1? |